3GPP TSG RAN WG1 #109-e			R1-2204176

e-Meeting, May 9th – 20th, 2022

Source:	Sharp

Title:	Discussion on potential solutions to further reduce UE complexity

Agenda Item:	9.6.1

Document for:	Discussion and Decision

Introduction

In RAN94e, a new SID[1] on further NR RedCap UE complexity reduction was approved. In this SID, further expansion for RedCap use cases is pursued at relatively lower cost and lower energy consumption. The maximum bandwidth for eRedCap devices will be reduced to 5MHz and lower peak data rates are targeted. Some potential solutions for eRedCap UE with smaller bandwidth and lower peak rate are discussed in this contribution.

Discussions

To further reduce the complexity and cost of user equipment, smaller maximum bandwidth and lower peak data rate compared with R17 redcap UE are required. The accordingly system impacts are discussed for limitation of bandwidth and peak data rate respectively.

Further UE complexity reduction techniques on bandwidth

Based on the evaluation methodology in section 7 of TR 38.875 [2], It is found that bandwidth reduction contributes a significant gain in reducing UE complexity. Hopefully, further reduction of the maximum UE bandwidth of the eRedCap device can lead to even more reduction in UE power consumption. One objective of this SID is to study the system impacts when the maximum UE bandwidth is further reduced from 20MHz to 5MHz in FR1 bands. We can first evaluate the impacts on the access procedure. Since the eRedCap UE should coexist in a cell with R17 RedCap and non-RedCap UEs, the SSB will be reused directly, where the synchronization signal structure occupies 20 PRBs. For FR1, the SCS of SSB can be 15kHz or 30KHz and the bandwidth of SSB with 20 RBs is 3.6MHz and 7.2MHz, respectively. Within the maximum 5MHz bandwidth limit, the eRedCap UE can only detect an SSB with 15KHz SCS.

Proposal 1: R18 eRedCap UE should only support SSB with 15KHz SCS

After monitoring a valid SSB in a cell, UE will detect PDCCH according to the CORESET0 and search space configuration in MIB. In NR cells, when the SCS of SSB is limited to 15kHz, the SCS combination of {SSB,PDCCH} can be {15,15}kHz or {15,30}kHz. When the SCS of PDCCH is 15kHz, the PRB number of CORESET can be one of {24,48,96}, and the total bandwidths are {4.32,8.64,17.28}MHz respectively. For type0-PDCCH, the CCE AL of PDCCH candidates can be one of the values {4,8,16}. In Table1, we list supported ALs set for each CORESET PRBs and symbol combinations. If an eRedCap UE will not receive channels with bandwidth over 5MHz, the UE can only support CORESET with 24 RBs and cannot monitor a type0-PDCCH candidate with AL of 16, which will narrow the application scenario of eRedCap UE.

Table1: ALs supported by SCS 15kHz CORESET0 configuration combinations

| PRBs | 24 |  | 48 |  |  | 96 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| bandwidth | 4.32MHz |  | 8.64MHz |  |  | 17.28MHz |  |  |
| symbols | 2 | 3 | 1 | 2 | 3 | 1 | 2 | 3 |
| Support ALs | 4,8 | 4,8 | 4,8 | 4,8,16 | 4,8,16 | 4,8,16 | 4,8,16 | 4,8,16 |

When the SCS of type0-PDCCH is 30kHz, all available CORESET bandwidths will exceed 5MHz. To avoid narrowing the eRedCap usage, we believe that eRedCap UE should support CORESET0 with a bandwidth larger than 5MHz.

Proposal 2: R18 eRedCap UE should support CORESET0 with a bandwidth larger than 5MHz

If the eRedCap UE needs to support data bandwidth beyond its maximum bandwidth, the UE with a smaller bandwidth can retune its center frequency to cover all data PRBs. Following the methodology of 38.875, the functional block of the NR device can be divided into two parts: RF and BB. RF includes most of the analog processing of the transceiver, including the power amplifier/filter/duplexer, and the other functional blocks are grouped into the BB section. Considering that digital samples may be more easily buffered, an eRedCap UE may support partial BB modules with a maximum bandwidth of 5MHz and the other modules with a maximum bandwidth of 20MHz to support the cases of CORESET with a bandwidth of larger than 5MHz. For example, the RF and ADC/FFT modules of UE can reuse the modules with maximum 20MHz bandwidth for R17 RedCap UE and reduce other BB modules to reduce the complexity with relaxed processing time.

Observation 1: eRedCap UE can be equipped with RF modules with a bandwidth of 20MHz and partial BB modules with a bandwidth of 5MHz to support channels with bandwidth larger than 5MHz

Further UE complexity reduction techniques on peak data rate

According to the SID, the supported peak data rate for R18 eRedCap targets to 10Mbps. By applying the evaluation formula for the approximate data rate for a band in 38.306[3], the peak data rates with some typical configurations are listed in table 2.

Table 2: peak data rates with different configuration combinations

| SCS | NRB | Layer | Mod Q | OH（DL/UL） | Peak Data Rate（DL/UL） |
| --- | --- | --- | --- | --- | --- |
| 15KHz | 25 | 1 | 6 | 0.14/0.08 | 20.0 Mbps/21.5 Mbps |
| 15KHz | 25 | 1 | 4 | 0.14/0.08 | 13.4 Mbps/14.3 Mbps |
| 15KHz | 13 | 1 | 6 | 0.14/0.08 | 10.4 Mbps/11.1 Mbps |
| 30KHz | 11 | 1 | 6 | 0.14/0.08 | 17.6 Mbps/18.8 Mbps |
| 30KHz | 11 | 1 | 4 | 0.14/0.08 | 11.7 Mbps/12.6 Mbps |
| 30KHz | 7 | 1 | 6 | 0.14/0.08 | 11.2 Mbps/12.0 Mbps |

Referring to 38.104-1[4], the maximum available PRB number is 25 for SCS 15KHz and 11 for SCS 30KHz within 5MHz bandwidth. The peak data rate of DL and UL is about 20 Mbps when scheduling all available PRBs in 5 MHz bandwidth with a modulation mode of 64 QAM. It is approximate twice the target rate. There are several schemes to reduce the complexity of the device while lowing the peak data rate. For example, limiting the maximum modulation order will be beneficial to simplify the receiver of eRedCap UE. With a modulation order of 4, the peak data rate would drop to 13.4Mbps or 11.7Mbps for the downlink with SCS of 15 and 30kHz.

In another way, TBS limitation also helps to reduce cost/complexity, for example by limiting the maximum number of PRBs for PDSCH/PUSCH scheduling. With TBS limit, the complexity of channel decoding-related modules (e.g. decoding and demodulation blocks) can be reduced. In Table 2, the peak data rates of DL are 10.4 and 11.2 Mbps, which can meet the requirements of this SID when scheduling 13 NRB with SCS 15 kHz and 7 NRB with SCS 30 kHz.

Observation 2: Limiting the modulation order and/or limiting the maximum number of scheduled PRBs can be used to reduce the complexity of the eRedCap UE

Conclusion

In this contribution, we have the following observations and proposals:

Proposal 1: R18 eRedCap UE should only support SSB with 15KHz SCS

Proposal 2: R18 eRedCap UE should support CORESET0 with a bandwidth larger than 5MHz

Observation 1: eRedCap UE can be equipped with RF modules with a bandwidth of 20MHz and  partial BB modules with a bandwidth of 5MHz to support channels with bandwidth larger than 5MHz

Observation 2: Limiting the modulation order and/or limiting the maximum number of scheduled PRBs can be used to reduce the complexity of the eRedCap UE

References

RP-213661, New SID on Study on further NR RedCap UE complexity reduction, Ericsson, 2022. 10

3GPP TR 38.875 V17.0.0, TR Study on support of reduced capability NR devices

3GPP TS 38.101-1 V17.4.0, User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone

3GPP TS 38.306, NR; User Equipment (UE) radio access capabilities