3GPP TSG RAN WG1 #109-e                                                                                                  R1-2204177

Meeting, 9th May – 20th May, 2022

Source:               TCL Communication

Title:                    Discussion on XR specific power saving techniques

Agenda item:      9.11.1

Document for:    Discussion and Decision

- Introduction

In the RAN #94e meeting, a new SID on NR XR enhancements was approved [1]. One objective is to discuss XR specific power saving techniques.

Objectives on XR-specific Power Saving (RAN1, RAN2): - Study XR specific power saving techniques to accommodate XR service characteristics (periodicity, multiple flows, jitter, latency, reliability, etc...). Focus is on the following techniques: - C-DRX enhancement. - PDCCH monitoring enhancement.

eXtended Reality (XR) and Cloud Gaming are some of the most important 5G media applications under consideration in the industry. XR is an umbrella term for different types of realities and refers to all real-and-virtual combined environments and human-machine interactions generated by computer technology and wearables. It includes representative forms such as Augmented Reality (AR), Mixed Reality (MR) and Virtual Reality (VR) and the areas interpolated among them. Power saving for XR has been studied in R-17 SI and the purpose of power study is to understand the NR UE power consumption performance for XR applications, and identify any issues and performance gaps, which could be useful for understanding i) the limitation of current NR systems in supporting XR applications and ii) the potential directions for future necessary enhancements to improve power efficiency. In this contribution, we provide our views on XR specific power saving.

- Discussion

Traffic characteristics of XR services were intensively studied in the Rel-17 RAN1 meetings, and the DL/UL traffic models for the video stream and Pose/control for XR and cloud gaming services were agreed. Based on the outcomes of the studies so far, the following characteristics of the XR traffic are identified and should be considered for the potential enhancements of NR to support XR.

- Non-integer periodicity

In RAN1 meetings of Rel-17, it was agreed that 60 frame per second (fps) is baseline for both DL and UL video stream and 30 fps, 90 fps as well as 120 fps can be also optionally evaluated. Based on the arrival time of packet with 30 fps, 60fps, 90fps, 120fps per second, which the corresponding periodicities are {33.33ms, 16.67ms, 11.11ms, 8.33ms} respectively, it’s not an integer periodic of symbol or slot.

- Jitter of packet arrival time

As agreed in RAN1 meeting in Rel-17, a truncated Gaussian distribution is used to model the jitter of DL and UL video stream for XR services. The range of jitter is agreed to be [-4, 4]ms (baseline) and [-5, 5]ms (optional). This means the XR packets may arrive at gNB or UE within a time window of total 8ms or 10ms length, and the exact arrival time is not known in advance.

- Low latency and large packet size

To provide good immersive experience, the latency of XR traffic should be as low as possible. As agreed in RAN1 meetings in Rel-17, air interface PDB for DL video stream is as follows.

VR/AR

  - 10ms

CG

  - 15ms

According to the agreed traffic model in Rel-17, mean packet size is very large. Taking AR/VR 60Mbps as example, mean packet size is 135000 bytes.

This is very different from R-15/6 URLLC or eMBB services, for URLLC, the most challenge is latency and for eMBB, the most challenge is high transmission data. However, for XR services, both high transmission rate and low latency should be satisfied.

- Varying packet size

In Rel-17 RAN1 meeting, parameters of Truncated Gaussian distribution for packet size of DL video stream in case of single stream evaluation has been agreed and is shown below, we can observed that the packet size of XR is not fixed.

- [STD, Max, Min]: [10.5, 150, 50]% of Mean packet size

Observation 1: XR services have the following characteristics.

- The non-integer periodicity

- Jitter of packet arrival time

- Low latency and large packet size

- Varying packet size

e-CDRX

- CDRX periodicity matched with XR traffic

DRX is one of the efficient method for UE power saving, when UE steps into the DRX-OFF state, it will be stop to monitor PDCCH and can go to sleeping for UE power saving. In current specification, the periodicity of DRX cycle can only be configured as an integer number of slots. The DRX cycle can be configured as {10ms, 20ms, 32ms, 40ms, 60ms, 64ms…} for long DRX cycle and the cycle values for short DRX can be configured as {2ms, 3ms, 4ms, 5ms, 6ms…}, as show in table 1. However, the periodicity for XR service is not an integer of slot, as discussion in section 2. Therefore, it is not suitable to configure a DRX cycle to match with the XR DL traffic perfectly. If the DRX cycle is configured to an integer number of slots, there will be a gap between XR packet arrival time and DRX configuration, so large delay will be caused.

Table 1: DRX-Config information element

-- ASN1START

-- TAG-DRX-CONFIG-START

DRX-Config ::=                      SEQUENCE {

drx-onDurationTimer                 CHOICE {

subMilliSeconds INTEGER (1..31),

milliSeconds    ENUMERATED {

ms1, ms2, ms3, ms4, ms5, ms6, ms8, ms10, ms20, ms30, ms40, ms50, ms60,

ms80, ms100, ms200, ms300, ms400, ms500, ms600, ms800, ms1000, ms1200,

ms1600, spare8, spare7, spare6, spare5, spare4, spare3, spare2, spare1 }

},

drx-InactivityTimer                 ENUMERATED {

ms0, ms1, ms2, ms3, ms4, ms5, ms6, ms8, ms10, ms20, ms30, ms40, ms50, ms60, ms80,

ms100, ms200, ms300, ms500, ms750, ms1280, ms1920, ms2560, spare9, spare8,

spare7, spare6, spare5, spare4, spare3, spare2, spare1},

drx-HARQ-RTT-TimerDL                INTEGER (0..56),

drx-HARQ-RTT-TimerUL                INTEGER (0..56),

drx-RetransmissionTimerDL           ENUMERATED {

sl0, sl1, sl2, sl4, sl6, sl8, sl16, sl24, sl33, sl40, sl64, sl80, sl96, sl112, sl128,

sl160, sl320, spare15, spare14, spare13, spare12, spare11, spare10, spare9,

spare8, spare7, spare6, spare5, spare4, spare3, spare2, spare1},

drx-RetransmissionTimerUL           ENUMERATED {

sl0, sl1, sl2, sl4, sl6, sl8, sl16, sl24, sl33, sl40, sl64, sl80, sl96, sl112, sl128,

sl160, sl320, spare15, spare14, spare13, spare12, spare11, spare10, spare9,

spare8, spare7, spare6, spare5, spare4, spare3, spare2, spare1 },

drx-LongCycleStartOffset            CHOICE {

ms10                                INTEGER(0..9),

ms20                                INTEGER(0..19),

ms32                                INTEGER(0..31),

ms40                                INTEGER(0..39),

ms60                                INTEGER(0..59),

ms64                                INTEGER(0..63),

ms70                                INTEGER(0..69),

ms80                                INTEGER(0..79),

ms128                               INTEGER(0..127),

ms160                               INTEGER(0..159),

ms256                               INTEGER(0..255),

ms320                               INTEGER(0..319),

ms512                               INTEGER(0..511),

ms640                               INTEGER(0..639),

ms1024                              INTEGER(0..1023),

ms1280                              INTEGER(0..1279),

ms2048                              INTEGER(0..2047),

ms2560                              INTEGER(0..2559),

ms5120                              INTEGER(0..5119),

ms10240                             INTEGER(0..10239)

},

shortDRX                            SEQUENCE {

drx-ShortCycle                      ENUMERATED  {

ms2, ms3, ms4, ms5, ms6, ms7, ms8, ms10, ms14, ms16, ms20, ms30, ms32,

ms35, ms40, ms64, ms80, ms128, ms160, ms256, ms320, ms512, ms640, spare9,

spare8, spare7, spare6, spare5, spare4, spare3, spare2, spare1 },

drx-ShortCycleTimer                 INTEGER (1..16)

}                                                                                                           OPTIONAL,   -- Need R

drx-SlotOffset                      INTEGER (0..31)

}

-- TAG-DRX-CONFIG-STOP

-- ASN1STOP

Observation 2：There is a gap between XR periodic DL traffic and current C-DRX configuration, large delay will be caused.

To align the start time of DRX cycle with XR traffic arrive, when jitter is not considered for XR services, UE and gNB know the arrival time of XR traffic according to the periodicity. A straightforward way is to introduce a set of non-integer periodicity for XR services (such as 8.33ms, 11.11ms, 16.66ms, 33.33ms), however, the set of non-integer periodicity is not matched with integer symbol or slot and the symbol or slot is the minimum time unit for data transmission in current specification. To solve the problem, as show in figure 1(take 60 fps as an example), it can be obviously observed that the mismatch issue appears every three arrival intervals, in others words, within 50ms periodicity, there are three packets will be arrived, so pre-defined  a fixed pattern for DRX within an integer periodicity can be considered.

Figure 1. Mismatch between DL traffic with 60 fps and C-DRX with 16ms

Proposal 1: A fixed pattern for DRX within an integer periodicity for XR power saving can be considered.

- Jitter

As agreed in Rel-17 RAN1 meeting, a truncated Gaussian distribution is used to model the jitter of DL and UL video stream for XR services. The range of jitter is agreed to be [-4, 4]ms (baseline) and [-5, 5]ms (optional). This means the XR packets may arrive at gNB or UE within a time window of 8ms or 10ms length, and the exact arrival time is not known in advance. When the XR traffic is arrived before DRX-ON, then a scheduling grant will be monitored in DRX on duration timer. However, when the XR traffic is arrived after DRX-ON, UE need to wait for a scheduling grant until the next DRX cycle, then a large latency will be caused. To solve this problem, an additional PDCCH monitor occasion can be configured on DRX-OFF state.

Proposal 2: An additional PDCCH monitor occasion can be configured on DRX-OFF state.

Enhanced PDCCH monitoring adaptation

In Rel-17 power saving WI, layer-1 based dynamic PDCCH monitoring reduction schemes, i.e. the PDCCH skipping and search space group switching have been discussed. In general, PDCCH skipping can reducing energy consumption of UE obviously, the more PDCCH monitor occasion skipping is, the more energy UE can be saved. As the XR packet is pseudo periodic, so the gNB and UE know the arrive location of the packet of XR. When a packet is successful transmitted, there is an idleness time before a new packet arrives, to save energy, the PDCCH monitoring during the idleness time can be skipped. However, as discussion in section 2, when the XR packet arrive with jitter, as a results, gNB and UE does not know the location of packet arrived anymore, if the skipped number of PDCCH monitoring is too small, then UE need to monitor some unnecessary PDCCH, power of UE is wasted. If the skipped number of PDCCH monitoring is too large, then large delay will be caused, which may exceeds the delay budget of a packet. Therefore, a reasonable setting of PDCCH monitoring skipping for UE power saving with gentle capacity loss for XR should be considered.

Similarly, for search space group switching, large search space set periodicity will be caused higher transmission latency and lower power consumption, so a trade-off between power saving and capacity for XR should be considered.

Proposal 3: PDCCH skipping and search space group switching can be considered for XR power saving.

- Conclusion

In this contribution, the following observations and proposals have been made:

Observation 1: XR services have the following characteristics.

- The non-integer periodicity

- Jitter of packet arrival time

- Low latency and large packet size

- Varying packet size

Observation 2：There is a gap between XR periodic DL traffic and current C-DRX configuration, large delay will be caused.

Proposal 1: A fixed pattern for DRX within an integer periodicity for XR power saving can be considered.

Proposal 2: An additional PDCCH monitor occasion can be configured on DRX-OFF state.

Proposal 3: PDCCH skipping and search space group switching can be considered for XR power saving.

References

[1] 3GPP RAN#94-e, RP-213587, New SID “Study on XR Enhancements for NR”, Nokia.