3GPP TSG RAN WG1 #109-e		R1-2204198

e-Meeting, May 9th – 20th, 2022

Source: 	Apple Inc.

Title:                     On remaining issues for NR-NR DC Power Control

Agenda item:	7.2.10

Document for:	Discussion and Decision

# 1 Introduction

RAN4 has sent an LS to both RAN1 and RAN2 on power control for NR-DC with the following overall description [1]:

1. Overall Description: RAN4 would like to thank RAN1 for the Further Reply LS on power control for NR-DC. Regarding the feasibility of independent power control for the mentioned two cases: 1) uplink CCs of MCG and uplink CCs of SCG are in different frequency bands in FR2. 2) uplink CCs of MCG and uplink CCs of SCG are in the same frequency band in FR2. RAN4 has discussed the definition of independent power control, and the understanding is that it means power control is per CG, and there is no total power limitation. Up to now RAN4 hasn’t introduce NR-DC band combination for FR2, therefore UL CA is referred here and it is RAN4 understanding that same conclusion can be applied to NR-DC. For intra-band UL CA, total max TRP and max EIRP limitation are defined in section 6.2A.4 of TS 38.101-2, thus power sharing is needed when the maximum power limitation was reached. Therefore, it is not independent power control. For inter-band UL CA: RAN4 has discussed two kinds of UEs to support inter-band CA, one is called CBM (Common Beam Management), and the other is called IBM (Independent Beam Management), and in Rel-17 RAN4 only defined requirements for IBM in inter-band UL CA. It is agreed that independent power control might be possible in some case for IBM but is not guaranteed UE behavior. RAN4 hasn’t discussed CBM for inter-band UL CA up to Rel-17. In addition, RAN4 do not plan to discuss p-NR-FR2 or p-UE-FR2 in Rel-17.

This contribution contains a discussion on the potential RAN1 impact related to the LS from RAN2 on NR-NR DC power control procedure.

# 2. Discussions

In clause 7.6.2. of TS 38.213, the parameter p-NR-FR2 is used to configure the maximum transmission power of CG on FR2 for NR-DC power control, which was as shown in below text excerpt.

“If a UE is configured with an MCG and a SCG using NR radio access in FR1 and/or in FR2, the UE is configured a maximum power PMCG for transmissions on the MCG by p-NR-FR1 and/or by p-NR-FR2 and a maximum power PSCG for transmissions on the SCG by p-NR-FR1 and/or by p-NR-FR2 and with an inter-CG power sharing mode by nrdc-PCmode-FR1 for FR1 and/or by nrdc-PCmode-FR2 for FR2. The UE determines a transmission power on the MCG and a transmission power on the SCG per frequency range.”

However, according to RAN4 LS [1][2], the required parameter p-NR-FR2 will not be specified in Rel-17. Since the parameter p-NR-FR2 forms the basis of FR2-FR2 NR-DC power control framework, RAN4 decision impacts all of power control schemes designed in RAN1, i.e., not only semi-static power sharing schemes but also the dynamic power sharing operation.

In RAN1 104bis meeting, how to solve this problem was extensively discussed and one LS [3] was sent to RAN4 to collect the inputs on the feasibility of independent power control for FR2 MCG and FR2 SCG. Referring to the latest RAN4 LS reply [1], it appears that the independent power control is not consistent with the current RAN4 requirement defined for intra-band NR-DC in section 6.2A.4 of TS 38.101-2. For inter-band NR-DC case, RAN4 indicates that independent PC may be possible for IBM case but still lack of specified UE behaviour at least from RAN4 perspective and the feasibility for CBM has not been studied yet in RAN4.

On a high level, a common power control methodology was used to handle power limitation case for different DC cases, including EN-DC, NE-DC and NR-NR DC, which can be summarized as follows:

- Various cross-CG power prioritization rule for different use cases such as EN-DC, NE-DC and NR-DC.

- Commonly, power prioritization rule defined for CA is applied within a CG.

Figure 1: Potential impacts on the current NR-DC PC due to lack of p-NR-FR2

In our view, the lack of parameter p-NR-FR2 only impacts on the cross-CG prioritization operation defined in current RAN1 spec. It should be noted that the FR2 transmission power in RAN4 specification is characterized by TRP and EIRP, a combination of emitted power and beamforming gain. It makes challenges or even practically impossible to use p-NR-FR2 for power sharing across CGs in case of FR2 NR-NR DC. On the other hand, it is not preferrable to specify a new power sharing rule for FR2 NR-NR DC during the maintenance stage.

In such situation, if p-NR-FR2 is not provided to UE, one alternative means to handle FR2 NR-NR DC power control is to apply CA prioritization rule within a CG and then use the RAN4 requirement to handle the cross-CG power transmission. The resulting FR2 NR-NR DC power control is listed in Table 1:

Table 1: FR2 NR-NR DC power control without p-NR-FR2 configured by RRC

| Index | Use case | Description |
| --- | --- | --- |
| 1 | Intra-band and inter-band FR2 NR-NR DC | - A UE applies the CA rule within CG and additionally controls the total radiated power PTMAX not to exceed the UE maximum output power TRPmax specified in sub-clause 6.2A.1 of TS 38.101-2. - The exact prioritization ordering across CG is left to UE implementation as long as the total power across CG is kept within the maximum power limit TRPmax set by RAN4 spec. |

We therefore propose to handle this issue using the below TP, which implements the consideration above:

--------------- start TP1 for 38.213 sub-clause 7.6.2 ------------------------ 7.6.2 NR-DC If a UE is configured with an MCG using NR radio access in FR1 or in FR2 and with a SCG using NR radio access in FR2 or in FR1, respectively, the UE performs transmission power control independently per cell group as described in Clauses 7.1 through 7.5. If a UE is configured with an MCG and a SCG using NR radio access in FR2 and is not configured with p-NR-FR2, the UE performs transmission power control independently per cell group as described in Clauses 7.1 through 7.5 and across cell groups as defined in [xx, TS 38.101-2]. If a UE is configured with an MCG and a SCG using NR radio access in FR1 and/or in FR2, the UE is configured a maximum power PMCG for transmissions on the MCG by p-NR-FR1 and/or by p-NR-FR2 and a maximum power PSCG for transmissions on the SCG by p-NR-FR1 and/or by p-NR-FR2 and with an inter-CG power sharing mode by nrdc-PCmode-FR1 for FR1 and/or by nrdc-PCmode-FR2 for FR2. The UE determines a transmission power on the MCG and a transmission power on the SCG per frequency range. <unchanged parts omitted> --------------- end TP1 ------------------------

# 3. Conclusion

In this contribution, we have presented our views on NR-NR DC power control and proposed the following:

Proposal 1: Adopt the following TP for section 7.6.2 of TS 38.213 for FR2-FR2 NR-NR DC:

Reason for change: RAN4 agreed not to define p-NR-FR2 in Rel-17, which results in the current power control defined for FR2-FR2 NR-DC in section 7.6.2 of TS 38.213 does not work. The CR is to implement a solution of power control such that the FR2-FR2 NR-DC can be supported in Rel-16. Summary of change: specify a UE behaviour for FR2-FR2 NR-NR DC for the case that RRC parameter p-NR-FR2 is not configured. Consequence if not approved: The UE behaviour for power sharing for FR2 NR-DC in case when RRC parameter p-NR-FR2 is not configured is not specified.

--------------- start TP1 for 38.213 sub-clause 7.6.2 ------------------------ 7.6.2 NR-DC If a UE is configured with an MCG using NR radio access in FR1 or in FR2 and with a SCG using NR radio access in FR2 or in FR1, respectively, the UE performs transmission power control independently per cell group as described in Clauses 7.1 through 7.5. If a UE is configured with an MCG and a SCG using NR radio access in FR2 and is not configured with p-NR-FR2, the UE performs transmission power control independently per cell group as described in Clauses 7.1 through 7.5 and across cell groups as defined in [xx, TS 38.101-2]. If a UE is configured with an MCG and a SCG using NR radio access in FR1 and/or in FR2, the UE is configured a maximum power PMCG for transmissions on the MCG by p-NR-FR1 and/or by p-NR-FR2 and a maximum power PSCG for transmissions on the SCG by p-NR-FR1 and/or by p-NR-FR2 and with an inter-CG power sharing mode by nrdc-PCmode-FR1 for FR1 and/or by nrdc-PCmode-FR2 for FR2. The UE determines a transmission power on the MCG and a transmission power on the SCG per frequency range. <unchanged parts omitted> --------------- end TP1 ------------------------

# References

- R1-2203030 	Reply LS on power control for NR-DC		RAN4

- R1-2102303 	Further Reply LS on power control for NR-DC		RAN4

- R1-2104018	Further Reply LS on power control for NR-DC		RAN1