3GPP TSG RAN WG1 #109-e		R1-2204201

e-Meeting, May 9th – 20th, 2022

Source: 	Apple Inc.

Title:                     Remaining issues for initial access aspects

Agenda item:	8.2.1

Document for:	Discussion and Decision

# 1 Introduction

In RAN1 108e meeting, the following was agreed for initial access aspects of FR2-2:

Working assumption - Use 1 bit for Q in MIB - SubcarrierSpacingCommon field will be used to convey value of {32, 64} for operation with shared spectrum channel access - Note that this is revising the working assumption made in RAN1#107-e on “use 2 bits for Q, {SubcarrierSpacingCommon, spare bit in MIB}” Conclusion - For operation with shared spectrum channel access, support default DBTW length of 5ms if discoveryBurstWindowLength is not provided in FR2-2. - No change to specification is needed Working Assumption - The following table is used for set of resource blocks and slot symbols of CORESET for Type0-PDCCH search space set when {SS/PBCH block, PDCCH} SCS is {120, 120}, {480, 480}, and {960, 960} kHz for FR2-2. - FFS: whether/how to define X, if defined, candidate values {56, 76} - Text Proposal #4-2D (for 38.213, Section 13) in section 3 of R1-2202503 is endorsed - Note: this working assumption can be revisited once RAN4 finalizes the further details of the channelization. Index; SS/PBCH block and CORESET multiplexing pattern; Number of RBs; Number of Symbols; Offset (RBs). 0; 1; 24; 2; 0. 1; 1; 24; 2; 4. 2; 1; 48; 1; 0. 3; 1; 48; 1; 14. 4; 1; 48; 1; 28. 5; 1; 48; 2; 0. 6; 1; 48; 2; 14. 7; 1; 48; 2; 28. 8; 1; 96; 1; 0. 9; 1; 96; 1; X. 10; 1; 96; 2; 0. 11; 1; 96; 2; X. 12; 3; 24; 2; -20 if -21 if > 0. 13; 3; 24; 2; 24. 14; 3; 48; 2; -20 if -21 if > 0. 15; 3; 48; 2; 48.

In addition, a couple of TPs were agreed for different open issues [1], such as Q value indication of licensed band, NR Carrier RSSI measurement and editorial correction.

This contribution discussed the remaining issues of initial access aspects to complete the designs.

# 2. Discussions

## 2.1 Value range of Q

In RAN2 #116 bis meeting, it was agreed [2] that the sparse bit in MIB cannot be used for Q value indication. How to indicate the Q value was extensively discussed in RAN1 108 meeting. A working assumption was agreed to use 1-bit SubcarrierSpacingCommon field to convey value of NSSBQCL={32, 64}. Some concerns were raised during discussions that supporting two values negates the benefit of Q parameter for shared spectrum. To mitigate this problem, it was proposed to borrow another bit in MIB payload e.g., LSB of KSSB field assuming only even KSSB values would be used by putting some restriction on channelization and sync raster. Typically, a large amount of antenna elements and beams is expected for FR2-2 deployment. The potential benefit of supporting one more value of ‘NSSBQCL={16}’ does not seem to justify the restriction on deployment flexibility.

We therefore propose the following:

Proposal 1: Confirm the following working assumption:

- Use 1 bit for Q in MIB

  - SubcarrierSpacingCommon field will be used to convey value of {32, 64} for operation with shared spectrum channel access

## 2.2 CORESET#0 Configuration

In RAN1 108 meeting, good progress was made on the CORESET#0 configuration for FR2-2 and a working assumption was made for set of resource blocks and slot symbols of CORESET for Type0-PDCCH search space set when {SS/PBCH block, PDCCH} SCS is {120, 120}, {480, 480}, and {960, 960} kHz for FR2-2.

One open issue is whether/how to define a single X value from two identified candidate values {56, 76}. In general, our preference is to take one from two candidate values to maximum the flexibility for licensed band with floating channelization design. On the exact value, we conducted some numerical checking. our observation is that both two candidate values are able to meet the basic design requirement and ensure at least one GSCN point and its associated CORESET#0 fits within the transmission bandwidth. As already pointed out in RAN1 108 meeting and depicted in FIG.1 below, one potential advantage of ‘X=76’ is that it provides some additional benefits in terms of addressable resources for RMSI transmission. Referring to FIG.1, there are 76 contiguous RBs available for RMSI transmission if X=76. The RB number is reduced to 56 if X = 56, which may substantially increase the coding rate of RMSI and further reduce the cell coverage.

Figure 1: Available RBs for RMSI transmission with different X offset value

We therefore propose the following for SSB-CORESET0 offset values:

Proposal 2: Confirm the working assumption of SSB-CORESET0 offset Table with X = 76.

| Index | SS/PBCH block and CORESET multiplexing pattern | Number of RBs NRBCORESET | Number of Symbols NsymbCORESET | Offset (RBs) |
| --- | --- | --- | --- | --- |
| 0 | 1 | 24 | 2 | 0 |
| 1 | 1 | 24 | 2 | 4 |
| 2 | 1 | 48 | 1 | 0 |
| 3 | 1 | 48 | 1 | 14 |
| 4 | 1 | 48 | 1 | 28 |
| 5 | 1 | 48 | 2 | 0 |
| 6 | 1 | 48 | 2 | 14 |
| 7 | 1 | 48 | 2 | 28 |
| 8 | 1 | 96 | 1 | 0 |
| 9 | 1 | 96 | 1 | 76 |
| 10 | 1 | 96 | 2 | 0 |
| 11 | 1 | 96 | 2 | 76 |
| 12 | 3 | 24 | 2 | -20 if kSSB=0 -21 if kSSB>0 |
| 13 | 3 | 24 | 2 | -24 |
| 14 | 3 | 48 | 2 | -20 if kSSB=0 -21 if kSSB>0 |
| 15 | 3 | 48 | 2 | -48 |

# 3. Conclusion

In this contribution, we have presented our views on remaining issues related to initial access aspects of FR2-2. Based on the discussions, we proposed the following:

Proposal 1: Confirm the following working assumption:

- Use 1 bit for Q in MIB

  - SubcarrierSpacingCommon field will be used to convey value of {32, 64} for operation with shared spectrum channel access

Proposal 2: Confirm the working assumption of SSB-CORESET0 offset Table with X = 76.

| Index | SS/PBCH block and CORESET multiplexing pattern | Number of RBs NRBCORESET | Number of Symbols NsymbCORESET | Offset (RBs) |
| --- | --- | --- | --- | --- |
| 0 | 1 | 24 | 2 | 0 |
| 1 | 1 | 24 | 2 | 4 |
| 2 | 1 | 48 | 1 | 0 |
| 3 | 1 | 48 | 1 | 14 |
| 4 | 1 | 48 | 1 | 28 |
| 5 | 1 | 48 | 2 | 0 |
| 6 | 1 | 48 | 2 | 14 |
| 7 | 1 | 48 | 2 | 28 |
| 8 | 1 | 96 | 1 | 0 |
| 9 | 1 | 96 | 1 | 76 |
| 10 | 1 | 96 | 2 | 0 |
| 11 | 1 | 96 | 2 | 76 |
| 12 | 3 | 24 | 2 | -20 if kSSB=0 -21 if kSSB>0 |
| 13 | 3 | 24 | 2 | -24 |
| 14 | 3 | 48 | 2 | -20 if kSSB=0 -21 if kSSB>0 |
| 15 | 3 | 48 | 2 | -48 |

# References

- RAN1 108 e-meeting Chairman notes

- R1-2200874 (R2-2201720)		 Reply LS on initial access for 60 GHz	 RAN1#108-e.