3GPP TSG- RAN WG1 Meeting #109-e                                             R1- 2204202

e-Meeting, May 9th – 20th, 2022

Agenda Item:	8.2.2

Source:	Apple Inc.

Title:	On remaining issues for PDCCH Monitoring

Document for:	Discussion/Decision

# Introduction

Based on the SI for RAN1 above 52.6 GHz [1] [2], an updated WID was agreed to in RAN #90-e with the following objectives [3] :

- In addition to 120kHz SCS, specify new SCS, 480kHz and 960kHz, and define maximum bandwidth(s), for operation in this frequency range for data and control channels and reference signals, only NCP supported.

  - Note: Except for timing line related aspects, a common design framework shall be adopted for 480kHz to 960kHz

- Support enhancement to PDCCH monitoring, including blind detection/CCE budget, and multi-slot span monitoring, potential limitation to UE PDCCH configuration and capability related to PDCCH monitoring.

In this contribution, we address the remaining issues for PDDCH monitoring for the new SCSs (480 kHz and 960 kHz) for NR operation between 52.6GHz and 71 GHz.

# Discussion

## Multi-slot Monitoring (MSM) and MSM capability indication for Rel-17

To enable SS configuration for multi-slot PDCCH monitoring and for the purpose of BD/CCE budget determination, the following agreement and working assumption have been made in RAN1#108-e as follows [6]:

Agreement Revise the RAN1#107bis-e agreement/working assumption: For search space set configuration of multi-slot PDCCH monitoring: - monitoringSlotPeriodicityAndOffset and duration are appended with "-r17", and - For monitoringPeriodicityAndOffset-r17 - The values represent slots - Add periodicity values {32,64,128,5120,10240,20480} to the existing values in monitoringSlotPeriodicityAndOffset - Note: Total list of supported periodicity values that can be configured for SCS 480kHz and 960kHz: {1,2,4,5,8,10,16,20,32,40,64,80,128,160,320,640,1280,2560,5120,10240,20480} - For each periodicity value Xp - The value range for the offset O is {0 .. Xp-1} slots Offset O values that can be configured: {0, 4, 8, …, } slots - Note: There may be no need to introduce the term "Xp" in the specifications - The configured periodicity at least for Group (1) SSs is restricted to be an integer multiple of XsL slots - FFS: details of offset The configured offset is restricted to be an integer multiple of L slots - For duration-r17 - The values represent slots - The value range is {8, 12, …, 20476} - If duration-r17 is absent, the UE assumes the duration in slots is equal to L - The UE ignores duration-r17 for DCI format 2_0 - The configured duration is restricted to be an integer multiple of XsL slots at least for Group (1) SSs - The maximum duration is one less slot group of L slots than the configured periodicity - FFS: need to revise the definition of durationduration-r17 is the total number of slots in consecutive groups of L slots in which a Search Space can exist in every period as given by monitoringPeriodicityAndOffset-r17 - monitoringSymbolsWithinSlot applies to each slot in a slot group configured for multi-slot PDCCH monitoring - Note: This parameter can be directly re-used from earlier releases. - Introduce new parameter monitoringSlotsWithinSlotGroup-r17 - Working assumption: - The size is 8 bitsTwo sizes L are supported for this parameter: L=4 bits and L=8 bits - Each bit in monitoringSlotsWithinSlotGroup-r17 represents a slot in a slot group of L slots - The parameter monitoringSlotsWithinSlotGroup-r17 is applied in each of the L slots as determined by monitoringSlotPeriodicityAndOffset-r17 and duration-r17. - A slot in the slot each group of L slots is configured for multi-slot PDCCH monitoring if the corresponding bit in the slot group monitoringSlotsWithinSlotGroup-r17 is set to '1' - Note: Further configuration of the monitoring symbols in such a slot is done by monitoringSymbolsWithinSlot - For Group (1) SSs - The slots indicated in the bitmap should be consecutive per group of L slots - The number of slots configured for multi-slot PDCCH monitoring per slot group of slots should be no larger than according to at least one of the supported by a UE - Working assumption: For Group (2) SSs - For Type0/0A/2 CSS - The slots indicated in monitoringSlotsWithinSlotGroup-r17 are not restricted to be consecutive - The number of slots configured for multi-slot PDCCH monitoring in monitoringSlotsWithinSlotGroup-r17 can be up to L - For Type1 CSS without dedicated RRC - The number of slots configured for multi-slot PDCCH monitoring in monitoringSlotsWithinSlotGroup-r17 per slot group of slots should be no larger than M, where M is FFS Working assumption For the purpose of BD/CCE budget determination for PDCCH monitoring for , if more than one of the PDCCH monitoring capability combinations supported by a UE comply with all configured search space sets, then the UE selects Xs and thereby and values corresponding to the complying combination with the largest and values. Note1: This determination of Xs may be independent of the size of monitoringSlotsWithinSlotGroup discussed in the context of search space configuration

The following elements should be considered:

- In RAN1#108-e, a working assumption on Group (2) SS has been made. To finalize the design, the working assumption with the value of M=1 should be agreed:

    - For Type0/0A/2 CSS

      - The slots indicated in monitoringSlotsWithinSlotGroup-r17 are not restricted to be consecutive

      - The number of slots configured for multi-slot PDCCH monitoring in monitoringSlotsWithinSlotGroup-r17 can be up to L

    - For Type1 CSS without dedicated RRC

      - The number of slots configured for multi-slot PDCCH monitoring in monitoringSlotsWithinSlotGroup-r17 per slot group of  slots should be no larger than M, where M is 1 FFS

- In RAN1#108-e, another working assumption was made to determine how the UE selects Xs to decidon the  BD/CCE budget for PDCCH monitoring for . To finalize the BD/CCE budget determination aspect, the working assumption should be confirmed as an agreement.

Proposal 1: Confirm the following working assumption from RAN1#107bis-e with M=1 value:

Working assumption: For Group (2) SSs

  - For Type0/0A/2 CSS

    - The slots indicated in monitoringSlotsWithinSlotGroup-r17 are not restricted to be consecutive

    - The number of slots configured for multi-slot PDCCH monitoring in monitoringSlotsWithinSlotGroup-r17 can be up to L

  - For Type1 CSS without dedicated RRC

    - The number of slots configured for multi-slot PDCCH monitoring in monitoringSlotsWithinSlotGroup-r17 per slot group of  slots should be no larger than M, where M is 1FFS

Proposal 2: Confirm the following working assumption from RAN1#107bis-e with M=1 value:

Working assumption:

For the purpose of BD/CCE budget determination for PDCCH monitoring for , if more than one of the PDCCH monitoring  capability combinations supported by a UE comply with all configured search space sets, then the UE selects Xs and thereby  and  values corresponding to the complying combination with the largest  and  values.

Note1: This determination of Xs may be independent of the size of monitoringSlotsWithinSlotGroup discussed in the context of search space configuration

## MSM capability for multiple cells

In RAN1#108-e, following agreement on MSM capability for multiple cells was made [6]:

Agreement - For serving cells configured with 480 or 960 kHz SCS, the serving cells with the same SCS and value are grouped together to determine a total BD/CCE budget for that group and the per-cell BD/CCE budget within the group. - Support UE capability signaling for 4 additional cases : - Case 4: Capability on the number of CCs with Rel-17 monitoring capability only - Range of pdcch-BlindDetectionCA-R17: {[4, 5, …, , 16]} - Case 5: Capability on the number of CCs with Rel-15 monitoring capability and Rel-17 monitoring capability on different serving cells - pdcch-BlindDetectionCA-R15 for Rel-15 PDCCH monitoring capability - pdcch-BlindDetectionCA-R17 for Rel-17 PDCCH monitoring capability - Range of pdcch-BlindDetectionCA-R17 and pdcch-BlindDetectionCA-R15: {[1, 2, …, 15]} - Range of pdcch-BlindDetectionCA-R15 + pdcch-BlindDetectionCA-R17: {[4, 5, …, 16]} - Case 6: Capability on the number of CCs with Rel-16 monitoring capability and Rel-17 monitoring capability on different serving cells - pdcch-BlindDetectionCA-R16 for Rel-16 PDCCH monitoring capability - pdcch-BlindDetectionCA-R17 for Rel-17 PDCCH monitoring capability - Range of pdcch-BlindDetectionCA-R17 and pdcch-BlindDetectionCA-R16: {[1, 2, …, 15]} - Range of pdcch-BlindDetectionCA-R16 + pdcch-BlindDetectionCA-R17: {[3, 4, …, 16]} - Case 7: Capability on the number of CCs with Rel-15 monitoring capability , Rel-16 monitoring capability and Rel-17 monitoring capability on different serving cells - pdcch-BlindDetectionCA-R15 for Rel-15 PDCCH monitoring capability - pdcch-BlindDetectionCA-R16 for Rel-16 PDCCH monitoring capability - pdcch-BlindDetectionCA-R17 for Rel-17 PDCCH monitoring capability - Range of pdcch-BlindDetectionCA-R17, pdcch-BlindDetectionCA-R16, and pdcch-BlindDetectionCA-R15: {[1, 2, …, 15]} - Range of pdcch-BlindDetectionCA-R15 + pdcch-BlindDetectionCA-R16 + pdcch-BlindDetectionCA-R17 : {[4, 5, …, 16]} - For the case with Rel-15 monitoring capability, Rel-16 monitoring capability and Rel-17 monitoring capability on different serving cells (case 7) or any combination of 2 of the capabilities (i.e. case 5, and case 6), the UE will report one or more combination of (pdcch-BlindDetectionCA-R15, pdcch-BlindDetectionCA-R16, pdcch-BlindDetectionCA-R17) as UE capability. If UE reports more than one combination of (pdcch-BlindDetectionCA-R15, pdcch-BlindDetectionCA-R16, pdcch-BlindDetectionCA-R17), as in Rel-16, the gNB configures which combination for the UE to use for scaling PDCCH monitoring capability if the number of CCs configured is larger than the reported capability. - FFS: Extension to NR-DC scenario

To finalize the design, the values for the range of pdcch-BlindDetectionCA for cases 4-6 should be agreed based on the numbers in the brackets of the agreement from RAN1#108-e.

Proposal 3: Accept current parameters for PDCCH Multi-cell Capability for the agreed Case 4, Case 5, Case 6, and Case 7 in RAN1#108-e:

- Example: Case 4: Capability on the number of CCs with Rel-17 monitoring capability only

Range of pdcch-BlindDetectionCA-R17: {[4, 5, …, , 16]}

## Search Space Set Group Switching

Search Space Set Group Switching was introduced in Rel-16 NR-U for power saving. It operates on CSS Type 3 and USS and allows the UE to switch between two groups of search space sets to either increase or decrease the UE search space set monitoring to save power as needed e.g. within and outside a COT in unlicensed access.

To finalize the design, the following WA made in RAN1 #107-e should be confirmed:

Working assumption

The following values are adopted as minimum value of  for 120/480/960 kHz

- Support only search space set group switching processing capability 1 with the following values

|  | Minimum value for UE processing capability 1 [symbols] |
| --- | --- |
| 3 | 40 |
| 5 | 160 |
| 6 | 320 |

Proposal 4: The following WA made in RAN1 #107-e should be confirmed:

Working assumption

The following values are adopted as minimum value of  for 120/480/960 kHz

- Support only search space set group switching processing capability 1 with the following values

|  | Minimum value for UE processing capability 1 [symbols] |
| --- | --- |
| 3 | 40 |
| 5 | 160 |
| 6 | 320 |

# Conclusion

In this contribution, we have discussed PDCCH monitoring enhancements for NR between 52.6GHz and 71 GHz and have the following proposals:

Proposal 1: Confirm the following working assumption from RAN1#107bis-e with M=1 value:

Working assumption: For Group (2) SSs

  - For Type0/0A/2 CSS

    - The slots indicated in monitoringSlotsWithinSlotGroup-r17 are not restricted to be consecutive

    - The number of slots configured for multi-slot PDCCH monitoring in monitoringSlotsWithinSlotGroup-r17 can be up to L

  - For Type1 CSS without dedicated RRC

    - The number of slots configured for multi-slot PDCCH monitoring in monitoringSlotsWithinSlotGroup-r17 per slot group of  slots should be no larger than M, where M is 1FFS

Proposal 2: Confirm the following working assumption from RAN1#107bis-e with M=1 value:

Working assumption:

For the purpose of BD/CCE budget determination for PDCCH monitoring for , if more than one of the PDCCH monitoring  capability combinations supported by a UE comply with all configured search space sets, then the UE selects Xs and thereby  and  values corresponding to the complying combination with the largest  and  values.

Note1: This determination of Xs may be independent of the size of monitoringSlotsWithinSlotGroup discussed in the context of search space configuration

Proposal 3: Accept current parameters for PDCCH Multi-cell Capability for the agreed Case 4, Case 5, Case 6, and Case 7 in RAN1#108-e:

- Example: Case 4: Capability on the number of CCs with Rel-17 monitoring capability only

Range of pdcch-BlindDetectionCA-R17: {[4, 5, …, , 16]}

Proposal 4: The following WA made in RAN1 #107-e should be confirmed:

Working assumption

The following values are adopted as minimum value of  for 120/480/960 kHz

- Support only search space set group switching processing capability 1 with the following values

|  | Minimum value for UE processing capability 1 [symbols] |
| --- | --- |
| 3 | 40 |
| 5 | 160 |
| 6 | 320 |

# Reference

- RP-193259, New SID: Study on supporting NR from 52.6GHz to 71 GHz

- 3GPP TR 38.808 V1.0.0 (2020-12), Study on supporting NR from 52.6 GHz to 71 GHz (Release 17)

- RP-202925, 3GPP Work Item Description on Extending current NR operation to 71GHz

- Chairman’s Notes, RAN1 #107-e, November 2021

- Chairman’s Notes, RAN1 #107-bis-e, January 2022

- Chairman’s Notes, RAN1 #108-e, February-March 2022