3GPP TSG- RAN WG1 Meeting #109-e                                             R1- 2204203

e-Meeting, May 9th – 20th, 2022

Agenda Item:	8.2.3

Source:	Apple Inc.

Title:	On remaining issues for PDSCH/PUSCH Enhancements

Document for:	Discussion/Decision

# Introduction

Based on the SI [1] [2], an updated WID was agreed to in RAN #90-e with the following objectives [3]:

- define maximum bandwidth(s), for operation in this frequency range for data and control channels and reference signals

- Timeline related aspects adapted to 480kHz and 960kHz, e.g., BWP and beam switching timing, HARQ timing, UE processing, preparation, and computation timelines for PDSCH, PUSCH/SRS and CSI, respectively.

- Evaluate, and if needed, specify the PTRS enhancement for 120kHz SCS, 480kHz SCS and/or 960kHz SCS, as well as DMRS enhancement for 480kHz SCS and/or 960kHz SCS.

- Support enhancements for multi-PDSCH/PUSCH scheduling and HARQ support with a single DCI

In this contribution, we address the following subjects covered by the objectives including timeline related aspects adapted to each of the new numerologies 480kHz and 960kHz and scheduling particularly w.r.t. multi-PDSCH/PUSCH with a single DCI and it corresponding HARQ support.

# Multi-PDSCH/PUSCH Scheduling and HARQ with a single DCI

In the WI for NR operation between 52.6 GHz and 71 GHz, a multi-slot framework for PDCCH monitoring to limit the overall PDCCH decoding complexity is under discussion. This feature requires that RAN1 consider enabling scheduling to take place over these multiple slots in an efficient manner. To reduce the control signaling overhead within the larger PDCCH monitoring unit, the WID proposes enhancements for multi-PDSCH/PUSCH scheduling support with a single DCI and corresponding HARQ enhancements to support this.  In the last few meetings, agreements have been made on the design of multi-PDSCH and multi-PUSCH transmission with a single DCI and the associated HARQ enhancements.

## Multi-PDSCH and multi-PUSCH Transmission

### Configured vs. valid PxSCH

In RAN1 #107-bis-e and in RAN1#108-e, there was a discussion on whether “scheduled PXSCH” in previous agreements implies scheduled (configured) or valid PXSCH. The Table below summarizes the remaining cases that are pending and Apple’s Proposals. Basically, our preference is to have valid PXSCH for both cases 5 and 6.

Proposal 1: The following table indicates our preferences on “configured” vs “valid” for the existing agreements:

| Case | Issue | Agreement: Configured vs Valid | Apple’s Proposals |
| --- | --- | --- | --- |
| Case 5 | Out-of-order behavior | Pending | Valid |
| Case 6 | NN-K1 | Pending | Valid |

### Determining HARQ process number

In RAN1#106b-e meeting, it has been agreed that if one of multiple PDSCHs (or PUSCHs) scheduled by the DCI collides with a flexible symbol (indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated), and if that PUSCH is collided with SSB symbols indicated by ssb-PositionsInBurst, the HARQ process number increment is skipped for the PUSCH. Otherwise, the HARQ process number increment is not skipped for that PDSCH (or PUSCH).

For the case if PUSCH collides with CORESET#0, it is still under discussion whether the HARQ process number is incremented or not. Based on the current specification, the UE is not expected to be scheduled with PUSCH on the symbols indicated for CORESET#0. Therefore, since such a case is not expected, it should not impact the HARQ process increment.

Proposal 2: HARQ Process Number is NOT skipped if PUSCH collides with CORESET #0.

# HARQ Feedback for Multi-PDSCH Scheduling with a Single DCI

To support multi-slot PDCCH monitoring, multi-slot scheduling and multi-PDSCH transmission in Rel-17, the HARQ codebook design has been updated for Rel-17.

## Type-2 Codebook for multi-PDSCH Transmission

In RAN1#108-e, a working assumption was made for the text proposal for construction of type-2 codebook construction based on configured SLIVs.

Working assumption:

The TP#1 below for TS 38.213 section 9.1.3.1 is endorsed.

- TP#1 (Construction of bundling group based on “configured” SLIVs)

If a UE is provided numberOfHARQ-BundlingGroups and is not provided harq-ACK-SpatialBundlingPUCCH for a serving cell c, the UE generates HARQ-ACK information over transport block groups (TBGs) for PDSCH receptions where, for a maximum number of NPDSCHmax PDSCH receptions scheduled by a DCI format on the serving cell, a maximum number of TBGs NHARQ-ACK,cTBG,max is provided by numberOfHARQ-BundlingGroups. If the UE detects a DCI format scheduling NPDSCH,c PDSCH receptions on the serving cell c, the UE generates NHARQ-ACK,cTBG,max HARQ-ACK information bits for first TBs and, if applicable, generates NHARQ-ACK,cTBG,max HARQ-ACK information bits for second TBs in the NPDSCH,c PDSCH receptions as described in clause 9.1.1 by setting NHARQ-ACKCBG/TB,max=NHARQ-ACK,cTBG,max and C=NPDSCH,c. For a TBG with at least one PDSCH not overlapping with an UL symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated, if provided, the UE assumes that a PDSCH in the TBG is correctly received if the PDSCH overlaps with an UL symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated. For a TBG that includes only PDSCH(s) overlapping with UL symbol(s) indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated, if provided, NACK is generated for the TBG.

In RAN1#109, the above working assumption should be confirmed.

Proposal 3: For multi-PDSCH scheduling with a single DCI and Type 2 HARQ-ACK codebook, confirm the working assumption for the TP for TS 38.213 based on “configured” SLIVs.

## Multi-PDSCH and COT

RAN1 needs to decide whether a multi-PxSCH transmission can occur across multiple COTs and the specify the UE behavior in the case that one or more of the transmissions occurs outside a valid COT. If cross-COT multi-PDSCH scheduling occurs, RAN1 needs to decide whether the HARQ feedback for all the transmissions occurs in one PUCCH at the end of the transmissions or it occurs separately per COT.

Three different scenarios are shown in the Figures below:

- Single HARQ-ACK feedback for Multi-PDSCH transmissions within a single COT.

- Single HARQ-ACK feedback for multi-PDSCH transmissions across multiple COTs

- Per COT HARQ-ACK feedback for multi-PDSCH transmissions across multiple COTs

Figure 1: Single HARQ-ACK feedback for multi-PDSCH transmissions within a  single COT.

Figure 2: Single HARQ-ACK feedback for multi-PDSCH transmissions across multiple COTs

Figure 3: Per COT HARQ-ACK feedback for multi-PDSCH transmissions across multiple COTs

Ideally, to limit signaling overhead and reduce UE complexity, the UE should not expect a multi-PDSCH transmission to occur across COTs. Note that Option 3 is not valid based on the agreement inRAN1 #107-e that a UE should not support HARQ-ACK information corresponding to different PDSCHs scheduled by the DCI being carried by different PUCCH(s).

Proposal 4: RAN1 should support single HARQ-ACK feedback for multi-PDSCH transmissions within a single COT only.

# Miscellaneous Issues for Multi-PDSCH Scheduling with a Single DCI

## HARQ ACK Codebook and BWP switching

In Rel-15/Rel-16 if slot n_u starts at a same time as or after a slot for an active DL BWP change on serving cell  c or an active UL BWP change on the PCell and slot   is before the slot for the active DL BWP change on serving cell  c or the active UL BWP change on the PCell then we skip this downlink slot in the codebook.

To address this issue for multi-PDSCH transmission, the UE should have the following behavior:

- The UE does not expect an UL or DL BWP change on the serving cell after the DCI scheduling the multi-PDSCH transmission and until the PUCCH is transmitted

Note that in the case of UCI multiplexing on multi-PUSCH transmission, RAN1 should discuss if each PUSCH can be multiplexed on or if UCI multiplexing is allowed on only one of the PUSCH transmissions.

Proposal 5: In the case of BWP switching during multi-PxSCH transmission the UE does not expect an UL or DL BWP change on the serving cell after the DCI scheduling the multi-PDSCH transmission and until the PUCCH is transmitted.

# Conclusion

In this contribution, we have the following observations, proposals, and conclusions:

Proposal 1: The following table indicates our preferences on “configured” vs “valid” for the existing agreements:

| Case | Issue | Agreement: Configured vs Valid | Apple’s Proposals |
| --- | --- | --- | --- |
| Case 5 | Out-of-order behavior | Pending | Valid |
| Case 6 | NN-K1 | Pending | Valid |

Proposal 2: HARQ Process Number is NOT skipped if PUSCH collides with CORESET #0.

Proposal 3: For multi-PDSCH scheduling with a single DCI and Type 2 HARQ-ACK codebook, confirm the working assumption for the TP for TS 38.213 based on “configured” SLIVs.

Proposal 4: RAN1 should support single HARQ-ACK feedback for multi-PDSCH transmissions within a single COT only.

Proposal 5: In the case of BWP switching during multi-PxSCH transmission the UE does not expect an UL or DL BWP change on the serving cell after the DCI scheduling the multi-PDSCH transmission and until the PUCCH is transmitted.

# References

- RP-193259, New SID: Study on supporting NR from 52.6GHz to 71 GHz

- 3GPP TR 38.808 V1.0.0 (2020-12), Study on supporting NR from 52.6 GHz to 71 GHz

- RP-202925, 3GPP Work Item Description on Extending current NR operation to 71GHz

- Chairman’s Notes, RAN1 #107-e

- Chairman’s Notes, RAN1 #107-bis-e

- Chairman’s Notes, RAN1 #108-e