3GPP TSG RAN WG1 #109-e	                                    	   R1-2204206

e-Meeting, May 9th – 20th, 2022

Agenda Item:	8.3.2

Source:	Apple Inc.

Title:	Remaining issues in intra-UE multiplexing/prioritization

Document for:	Discussion/Decision

1	Introduction	1

2	Discussion on Rel-17 UCI multiplexing framework	1

3	Interlace number adjustment	2

3.1	Rel-16 design	2

3.2	Rel-17 PRB number adjustment/interlace number adjustment design	2

3.2.1	Interlace number adjustment	3

3.3	Summary	3

4	UCI multiplexing over PUSCH	4

5	Power control aspects	4

5.1	Design history in NR	4

5.2	Design considerations on PUCCH power control for Rel-17	6

5.3	Proposed change for PUCCH power control	7

6	References	10

# Introduction

In this contribution, we provide our views on remaining issues in intra-UE multiplexing.

# Discussion on Rel-17 UCI multiplexing framework

In TS 38.213 release 15, it is prescribed there can be up to 2 PUCCHs in a slot, e.g. two short PUCCHs, one short PUCCH + one long PUCCH, etc. With the introduction of sub-slot based feedback, then such a prescription also applies to sub-slot based transmissions.

It is noted that on the outcome of PUCCH multiplexing, i.e. resources Z, there is no restriction on the number of PUCCHs generated. Actually, it is not clear where in the PUCCH multiplexing flow, PUCCHs should be trimmed to satisfy the requirement of allowing up to 2 PUCCHs in a slot/sub-slot, e.g. prior to UCI multiplexing over PUSCH or after UCI multiplexing over PUSCH. In Rel-15, such a question was raised. The situation may become more severe as in Rel-17 PUCCH multiplexing is executed three times:  two times in Step 2.1 (one for LP PUCCHs, another for HP PUCCHs), and one time in Step 2.2 (for LP PUCCH(s) and HP PUCCH(s)). And UCI multiplexing over PUSCH is also executed three times: two times in Step 1.2 and one time in Step 2.2. Any different processing/assumption between gNB and UE in those procedures concerning the two PUCCH restriction could lead to misunderstanding between gNB and UE. From that, we feel it is important to clarify the trimming procedure to satisfy the two PUCCHs per slot/sub-slot requirement.

We have

Proposal 2: Clarify the number of PUCCH resources Z after PUCCH UCI multiplexing.

# Interlace number adjustment

## Rel-16 design

In Rel-16, interlaced transmission of PUCCH is supported from the NR-U work item. For interlaced transmission of PF3, the interlace number adjustment and CSI omission are captured below:

From the review, we have

Observation 3-1: in Rel-16, for PRB number adjustment/interlace number adjustment, the configured PRB number of the  selected PUCCH resource (if it is at PF2 or PF3), OUCI, OCRC, maxCodeRate according to the PUCCH format of the selected PUCCH resource,  OUCI, OCRC, maxCodeRate according to the PUCCH format of the selected PUCCH resource are used as inputs, and the same coding rate is used for two CSI parts.

## Rel-17 PRB number adjustment/interlace number adjustment design

At RAN1 #106bis-e, the following was agreed:

Agreement

For determining the PUCCH resource to carry the multiplexed high-priority and low-priority HARQ-ACKs,

- The number of RBs for multiplexing HP HARQ-ACK and LP HARQ-ACK on a PUCCH format 3 is determined as following:

  - If OHP_UCI+OCRC, HP_UCIrHP_UCI∙Qm+OLP_UCI+OCRC, LP_UCIrLP_UCI∙Qm ≤MRBPUCCH∙Nsc, ctrlRB∙Nsymb-UCIPUCCH, the minimum number of RBs is determined as the number of MRB, minPUCCH, satisfying OHP_UCI+OCRC, HP_UCIrHP_UCI∙Qm+OLP_UCI+OCRC, LP_UCIrLP_UCI∙Qm≤MRB,minPUCCH∙Nsc, ctrlRB∙Nsymb-UCIPUCCH and OHP_UCI+OCRC, HP_UCIrHP_UCI∙Qm+OLP_UCI+OCRC, LP_UCIrLP_UCI∙Qm>MRB,minPUCCH-1∙Nsc, ctrlRB∙Nsymb-UCIPUCCH.

    - Note: rHP_UCI∙rLP_UCI∙Qm is multiplied at both sides to avoid mismatch between gNB and UE due to floating point operation. Editor to capture as suggested.

  - Otherwise,

    - Alt1: the number of RBs is MRBPUCCH. FFS: Whether/How LP HARQ-ACK is dropped.

    - Alt2: the number of RBs is determined by HP ACK payload size. LP HARQ-ACK is fully dropped.

    - Other alternatives are not precluded.

  - r_HP_UCI is maxCodeRate configured for HP bits and r_LP_UCI is maxCodeRate configured for LP bits in the second PUCCH-Config (the PUCCH-config containing the PUCCH resource of the HP HARQ-ACK).

    - FFS whether more than one maxCodeRate can be configured for one priority.

  - If  MRB, minPUCCH is not equal to  according to [4, TS 38.211], MRB, minPUCCH is increased to the nearest allowed value of nrofPRBs for PUCCH-format3 provided by the second PUCCH-Config [12, TS 38.331].

  - HP coded bits and LP coded bits are not transmitted using the same RE(s)

- FFS for PUCCH format 2.

The support of intra-L1 priority MUXing for PUCCH format 2 was agreed in RAN1 #107bis-e.

### Interlace number adjustment

For interlace number adjustment, instead of performing PRB number adjustment for potentially many PRB number candidates, only two PRB number candidates are examined: one corresponds to the number of PRBs in the first interlace, another corresponds to the number PRBs in the first and second interlace. The same consideration provided in the previous section applies.

Further details can be found below:

If a UE is provided a first interlace of MInterlace,0PUCCH PRBs by interlace0 in InterlaceAllocation, the UE has HARQ-ACK, SR and wideband or sub-band CSI reports to transmit, and the UE determines a PUCCH resource with PUCCH format 2, or the UE has HARQ-ACK, SR and wideband CSI reports to transmit and the UE determines a PUCCH resource with PUCCH format 3, or the UE has HARQ-ACK, SR and sub-band CSI reports to transmit, and the UE determines a PUCCH resource with PUCCH format 3, where

- the UE determines the PUCCH resource using the PUCCH resource indicator field in a last of a number of DCI formats with a value of a PDSCH-to-HARQ_feedback timing indicator field indicating a same slot for the PUCCH transmission, from a PUCCH resource set provided to the UE for HARQ-ACK transmission, and after the UE determines the PUCCH resource set

- if

n=1NUCI-part2totalOUCI-part2,n+OCRC,UCI-part2/Qm⋅r2+⌈n=1NUCI-part1totalOUCI-part1,n+OCRC,UCI-part1/Qm⋅r1⌉≤MInterlace,0PUCCH⋅Nsc,ctrlRB⋅Nsymb-UCIPUCCH,

the UE transmits the HARQ-ACK, SR, and CSI reports bits in a PUCCH over the first interlace

- else, if the UE is provided a second interlace of MInterlace,1PUCCH PRBs by interlace1 and if

n=1NUCI-part2totalOUCI-part2,n+OCRC,UCI-part2/Qm⋅r2+⌈n=1NUCI-part1totalOUCI-part1,n+OCRC,UCI-part1/Qm⋅r1⌉≤MInterlace,0PUCCH+MInterlace,1PUCCH⋅Nsc,ctrlRB⋅Nsymb-UCIPUCCH,

the UE transmits the HARQ-ACK, SR, and CSI reports bits in a PUCCH over both the first and second interlaces

- else, the UCI omission procedure is same as the corresponding one when the UE is provided PUCCH-ResourceSet by replacing MRBPUCCH with MInterlace,0PUCCH, or, if the UE is provided interlace1, by MInterlace,0PUCCH+MInterlace,1PUCCH.

## Summary

In this section, we treat PRB number adjustment in Rel-17. We have

Proposal 3-1: If a UE is provided a first interlace of MInterlace,0PUCCH PRBs by interlace0 in InterlaceAllocation, the UE has HARQ-ACK, SR and wideband or sub-band CSI reports to transmit, and the UE determines a PUCCH resource with PUCCH format 2, or the UE has HARQ-ACK, SR and wideband CSI reports to transmit and the UE determines a PUCCH resource with PUCCH format 3, or the UE has HARQ-ACK, SR and sub-band CSI reports to transmit, and the UE determines a PUCCH resource with PUCCH format 3, where

- the UE determines the PUCCH resource using the PUCCH resource indicator field in a last of a number of DCI formats with a value of a PDSCH-to-HARQ_feedback timing indicator field indicating a same slot for the PUCCH transmission, from a PUCCH resource set provided to the UE for HARQ-ACK transmission, and after the UE determines the PUCCH resource set

- if

n=1NUCI-part2totalOUCI-part2,n+OCRC,UCI-part2/Qm⋅r2+⌈n=1NUCI-part1totalOUCI-part1,n+OCRC,UCI-part1/Qm⋅r1⌉≤MInterlace,0PUCCH⋅Nsc,ctrlRB⋅Nsymb-UCIPUCCH,

the UE transmits the HARQ-ACK, SR, and CSI reports bits in a PUCCH over the first interlace

- else, if the UE is provided a second interlace of MInterlace,1PUCCH PRBs by interlace1 and if

n=1NUCI-part2totalOUCI-part2,n+OCRC,UCI-part2/Qm⋅r2+⌈n=1NUCI-part1totalOUCI-part1,n+OCRC,UCI-part1/Qm⋅r1⌉≤MInterlace,0PUCCH+MInterlace,1PUCCH⋅Nsc,ctrlRB⋅Nsymb-UCIPUCCH,

the UE transmits the HARQ-ACK, SR, and CSI reports bits in a PUCCH over both the first and second interlaces

- else, the UCI omission procedure is same as the corresponding one when the UE is provided PUCCH-ResourceSet by replacing MRBPUCCH with MInterlace,0PUCCH, or, if the UE is provided interlace1, by MInterlace,0PUCCH+MInterlace,1PUCCH.

# UCI multiplexing over PUSCH

One remaining issue is UCI multiplexing over PUSCH when CG-UCI is present. We note there was an agreement to support the multiplexing of CG-UCI and HARQ-ACK reached in the unlicensed spectrum access session under Rel-17 URLLC. We believe that agreement is only about the support of UCI types in CG PUSCH, not about how that is supported, which belongs to the intra-UE multiplexing discussion.

We have

Observation 4: agreement from Rel-17 IIoT/URLLC unlicensed spectrum access session is only about the support of UCI types in CG PUSCH, not about how that is supported, which belongs to the intra-UE multiplexing discussion.

A simple treatment is for UCI multiplexing over PUSCH, if multiplexing of HARQ-ACK and CG-UCI is supported, then CG-UCI is concatenated with HP HARQ-ACK if present, in any case CG-UCI (in the absence of HP HARQ-ACK) or CG-UCI/HP-HARQ-ACK (in the presence of HP HARQ-ACK) is carried in the legacy HARQ-ACK way.

We have

Proposal 4: For Rel-17 intra-UE multiplexing, reuse R15 HARQ-ACK rate matching/puncturing and RE mapping for CG-UCI (in the absence of HP HARQ-ACK) or CG-UCI/HP-HARQ-ACK (in the presence of HP HARQ-ACK). LP-HARQ-ACK is treated as if CG-UCI is absent.

# Power control aspects

In NR, important decisions on PUCCH power control were taken at RAN1 #92, which leveraged LTE’s design. By reviewing the discussion history, it can be seen much simulation work is required to arrive at an optimized design. Given the diverse issues to tackle just under UCI multiplexing, reopening all the design choices is not desirable. In Section 13.1, we first review the discussion history on power control in NR. In Section 5.2, we provide general design consideration for Rel-17. In Section 5.3, we provide our concrete proposal for power control.

## Design history in NR

In Rel-15, for PUCCH formats 2/3/4, evaluations on the candidates for ΔPUCCH_TF,c(i) were conducted. It was assumed that

  - For  2 < OUCI ≦11 bits, When RM code is used, the coding gain is considered linear.

  - For  12 ≦ OUCI ≦19 bits,  Polar code with 6-bit CRC is used.

  - For  19 bits <  OUCI ,  Polar code with 11-bit CRC is used.

It is clear that for small UCI payload size, Reed-Muller code was the target for curve-fitting, and for larger UCI payload size, polar code was the target for curve-fitting.

In Rel-15/16, only UCIs of the same L1 priority can be multiplexed in a PUCCH. The power control rules for PUCCH formats are captured in TS 38.213. For PF0/PF1, the payload size is not considered in the power control formula. For PF2/3/4, the payload size is considered. For PF2/3/4, joint encoding is supported, and for PF3/4, separate encoding is supported. It can be seen from the agreement in RAN1 #92, when the agreements on power control formula were reached, it seems joint encoding was assumed in the specification work, and separate encoding was not considered.

At RAN1 #92, the following were agreed for NR:

Agreement:

- For PUCCH format 2, 3, and 4, for the case of small UCI payload size (less than or equal to 11)

  - ΔPUCCH_TF,c(i) = 10log10(K2∙BPRES(i)) where

    - BPRES(i) = OUCI(i)/NRE(i)

    - OUCI(i) is the actual number of UCI bits transmitted in i excluding the known bits as in LTE

    - NRE(i) = MPUCCH,c(i) x number of subcarriers per PRB x number of DFT-s-OFDM/CP-OFDM symbols excluding DMRS symbols/tones

    - FFS: K2 (there is no new RRC parameter introduced)

Agreement:

For the case of large UCI payload size (greater than 11), ΔPUCCH_TF,c(i) is equal to ΔPUCCH_TF,c(i) = 10log10(2K1∙BPRE(i) – 1)

- BPRE(i) = OUCI(i)/NRE(i)

- OUCI(i) is the number of UCI bits including CRC bits in i

- NRE(i) = MPUCCH,c(i) x number of subcarriers per PRB x number of DFT-s-OFDM/CP-OFDM symbols excluding DMRS symbols/tones

- FFS: K1 (there is no new RRC parameter introduced)

While the use of OUCI(i) is more straightforward, the derivation of  can be found from the specification text below:

is a number of HARQ-ACK information bits that the UE determines as described in Clause 9.1.2.1 for Type-1 HARQ-ACK codebook and as described in Clause 9.1.3.1 or 9.1.3.3 for Type-2 HARQ-ACK codebook.is the same as  as described in Clause 9.1.4 for Type-3 HARQ-ACK codebook. If the UE is not provided any of pdsch-HARQ-ACK-Codebook, pdsch-HARQ-ACK-Codebook-r16, or pdsch-HARQ-ACK-OneShotFeedback,  if the UE includes a HARQ-ACK information bit in the PUCCH transmission; otherwise,

And tracing back the discussion history in LTE, nHARQ (which is the LTE counterpart of nHARQ-ack(i) in NR) was introduced in Rel-10. The discussion in LTE (e.g. [8]) first led to an agreement in RAN1 #63:

Agreement (RAN1 #63)

n_harq based on:

-  number of received TBs (also including SPS release)

And the calculation of  nHARQ was discussed and agreed in [9] from the email discussion in May 2011 after the RAN1 #65 meeting “[65-09] R1-111886 36213CR0299 CR on power control for HARQ-ACK transmission on PUCCH”, the calculation was incorporated in Clause 10.1 of TS 36.213v10.2.0.

One consideration of using nHARQ instead of the HARQ-ACK size is to avoid un-necessarily high transmission power of LTE PUCCH format 3 when the number of received PDSCHs is actually small.

We note  is intended to be used for the case with Reed-Muller code.

## Design considerations on PUCCH power control for Rel-17

In previous sections, we provide our view on PUCCH resource set selection, PUCCH resource determination, omission rules, etc. We can see it is necessary to reflect the payloads at different coding rates for them. The coding rates can be associated with UCI parts, e.g. UCI part 1, UCI part2, the coding rates can be also associated with UCI types including the nature of the UCI payload, e.g. whether it is for HARQ-ACK, SR or CSI, further the L1 priority (HP or LP) can be used for further associations. As discussed in Section 4.1, there are a number of choices (4.1-option 1 to 4.1-option 4) to derive coding rates (r1 and r2).  It can be seen in the Rel-16 power control formula, the payload size at PF 2/3/4 is reflected in the power adjustment. Hence, we propose necessary updates to support power control for UCIs at two L1 priorities.

Here we reproduce the figures in Section 9 to make it easier to follow the discussion below.

With separate encoding, the following mapping can be considered.

Figure 5-1 Separate encoding for HARQ-ACK/SR only

Figure 5-2 Delta factors for Reed-Muller code and Polar code

(Note: per symbol is equivalent to per RE in this case)

In one example, it may happen the number of UCI bits for UCI part 1 is no larger than 11, and the number of UCI bits for UCI part 2 is no larger than 11, hence each UCI part is protected through a Reed-Muller code. However, if the sum of the UCI bits from two parts exceeds 11, then the power adjustment term follows that for polar code following the current specification.

In another example, it may also happen that HP HARQ-ACK in UCI part 1 has a few bits, yet LP HARQ-ACK in UCI part 2 has tens of bits. If the Rel-16 design is used, then according to the total number of UCI bits, the power adjustment for Polar code is used, which leads to a much lower Tx power than that for Reed-Muller code if the code rate is below 2/3 with QPSK as shown in Figure 12. In that case, the protection of the HP HARQ-ACK can be lacking.

To avoid reopening the design for Rel-17, and we can derive the adjustment factors for two UCI parts separately. Purely from protecting HP UCIs’ point of view, the adjust factor for UCI part 1 should be the determining factor.

## Proposed change for PUCCH power control

Since UCI part 1 and UCI part 2 are separately encoded, the delta factors can be determined from UCI part 1 and UCI part 2. We can consider the delta factor from UCI part 1 only to prioritize the transmission of HP UCI(s) in UCI part 1. To avoid power spectral density change in the frequency domain and/or time domain, then the determined delta factor is applied for both UCI parts. To derive the delta factor for UCI part 1, its payload size and the number of used RE resource elements for UCI part 1 need to be identified. Instead of using NREi, which is the total number of resource elements to carry both UCI parts, a UCI part specific resource element parameter can be used for UCI part 1.

As the rate matching procedure can guarantee on one resource element, only coded bits from a single UCI part are present. Then we can use the number of coded bits for UCI part 1 to calculate the number of resource elements taken by UCI part 1.

1

EUCI-part1=min(Etot,⌈(OH-ACK+OH-SR+L)/r1/Qm⌉⋅Qm)

EUCI-part2=Etot-min(Etot,⌈(OH-ACK+OH-SR+L)/r1/Qm⌉⋅Qm)

EUCI-part1=min(Etot,⌈(OH-ACK+OH-SR+OH-CSI+L)/r1/Qm⌉⋅Qm)

EUCI-part2=Etot-min(Etot,⌈(OH-ACK+OH-SR+OH-CSI+L)/r1/Qm⌉⋅Qm)

EUCI-part1=min(Etot,⌈(OH-ACK+OH-SR+OCSI-part1+L)/r1/Qm⌉⋅Qm)

EUCI-part2=Etot-min(Etot,⌈(OH-ACK+OH-SR+OCSI-part1+L)/r1/Qm⌉⋅Qm)

1

EUCI-part1=min(Etot,⌈(OH-ACK+OH-SR+L)/r1/Qm⌉⋅Qm)

EUCI-part2=Etot-min(Etot,⌈(OH-ACK+OH-SR+L)/r1/Qm⌉⋅Qm)

EUCI-part1=min(Etot,⌈(OH-ACK+OH-SR+OH-CSI+L)/r1/Qm⌉⋅Qm)

EUCI-part2=Etot-min(Etot,⌈(OH-ACK+OH-SR+OH-CSI+L)/r1/Qm⌉⋅Qm)

EUCI-part1=min(Etot,⌈(OH-ACK+OH-SR+OCSI-part1+L)/r1/Qm⌉⋅Qm)

EUCI-part2=Etot-min(Etot,⌈(OH-ACK+OH-SR+OCSI-part1+L)/r1/Qm⌉⋅Qm)

1

EUCI-part1=min(Etot,⌈(OH-ACK+OH-SR+L)/r1/Qm⌉⋅Qm)

EUCI-part2=Etot-min(Etot,⌈(OH-ACK+OH-SR+L)/r1/Qm⌉⋅Qm)

EUCI-part1=min(Etot,⌈(OH-ACK+OH-SR+OH-CSI+L)/r1/Qm⌉⋅Qm)

EUCI-part2=Etot-min(Etot,⌈(OH-ACK+OH-SR+OH-CSI+L)/r1/Qm⌉⋅Qm)

EUCI-part1=min(Etot,⌈(OH-ACK+OH-SR+OCSI-part1+L)/r1/Qm⌉⋅Qm)

EUCI-part2=Etot-min(Etot,⌈(OH-ACK+OH-SR+OCSI-part1+L)/r1/Qm⌉⋅Qm)

1

EUCI-part1=min(Etot,⌈(OH-ACK+OH-SR+L)/r1/Qm⌉⋅Qm)

EUCI-part2=Etot-min(Etot,⌈(OH-ACK+OH-SR+L)/r1/Qm⌉⋅Qm)

EUCI-part1=min(Etot,⌈(OH-ACK+OH-SR+OH-CSI+L)/r1/Qm⌉⋅Qm)

EUCI-part2=Etot-min(Etot,⌈(OH-ACK+OH-SR+OH-CSI+L)/r1/Qm⌉⋅Qm)

EUCI-part1=min(Etot,⌈(OH-ACK+OH-SR+OCSI-part1+L)/r1/Qm⌉⋅Qm)

EUCI-part2=Etot-min(Etot,⌈(OH-ACK+OH-SR+OCSI-part1+L)/r1/Qm⌉⋅Qm)

Figure 5-3 UCI mapping for separate encoding of HP/LP HARQ-ACK (1).

Figure 5-4 UCI mapping for separate encoding of HP/LP HARQ-ACK (2).

In the tables above, L is the number of CRC bits which can be zero, Qmis the modulation order. Etot and  EUCI-part1 are the number of coded bits for both UCI parts and UCI part 1 respectively,  NREi and  NRE-part1  are the number of resource elements for both UCI parts and the number of resource elements for UCI part 1 respectively.

For PUCCH formats 2/3/4, the delta factor ΔTF,b,f,ci is determined from UCI part 1:

- The number of resource elements for UCI part 1 NRE-part1(i)=EUCI-part1EtotNRE(i),  where   EUCI-part1 is the number of coded bits for UCI part 1

- If  n=1NUCI-part1total‍OUCI-part1,n  is smaller or equal to 11,

  - ΔTF,b,f,c,part1i=10⋅log10(K1⋅n=1NUCI-part1total‍OUCI-part1,n/NRE-part1i)

  - If a HARQ-ACK codebook with OUCI-part1,n'  bits is included in UCI part 1,  is used instead of OUCI-part1,n'  for the HARQ-ACK codebook:

    - ΔTF,b,f,c,part1i=10⋅log10(K1⋅n=1NUCI-part1total‍OUCI-part1,n-OUCI-part1,n' +nHARQ-ACK(i)/NRE-part1i)

  - If more than one HARQ-ACK codebooks are included in UCI part 1 (e.g. one due to SPS HARQ deferral, another for HARQ feedback for dynamic grant PDSCH(s)), then replacement of the number of HARQ-ACK codebook size by the associated nHARQ-ACK can be applied to each HARQ-ACK codebook.

    - The subscript i in  is dropped as there are multiple nHARQ-ACK.

- otherwise

  - ΔTF,b,f,c,part1i=10log10(2K2⋅BPREi-1)  where

    - and

    - BPREi=n=1NUCI-part1total‍OUCI-part1,n+OCRC,UCI-part1/NRE-part1i

- And ΔTF,b,f,ci = ΔTF,b,f,c,part1i is applied to both UCI parts.

We have

Proposal 5-1:  For PUCCH formats 2/3/4, the delta factor ΔTF,b,f,ci is determined from UCI part 1:

- The number of resource elements for UCI part 1 NRE-part1(i)=EUCI-part1EtotNRE(i),  where   EUCI-part1 is the number of coded bits for UCI part 1

- If  n=1NUCI-part1total‍OUCI-part1,n  is smaller or equal to 11,

  - ΔTF,b,f,c,part1i=10⋅log10(K1⋅n=1NUCI-part1total‍OUCI-part1,n/NRE-part1i)

  - If a HARQ-ACK codebook with OUCI-part1,n'  bits is included in UCI part 1,  is used instead of OUCI-part1,n'  for the HARQ-ACK codebook:

    - ΔTF,b,f,c,part1i=10⋅log10(K1⋅n=1NUCI-part1total‍OUCI-part1,n-OUCI-part1,n' +nHARQ-ACK(i)/NRE-part1i)

  - If more than one HARQ-ACK codebooks are included in UCI part 1 (e.g. one due to SPS HARQ deferral, another for HARQ feedback for dynamic grant PDSCH(s)), then replacement of the number of HARQ-ACK codebook size by the associated nHARQ-ACK can be applied to each HARQ-ACK codebook.

- otherwise

  - ΔTF,b,f,c,part1i=10log10(2K2⋅BPREi-1)  where

    - and

    - BPREi=n=1NUCI-part1total‍OUCI-part1,n+OCRC,UCI-part1/NRE-part1i

- And ΔTF,b,f,ci = ΔTF,b,f,c,part1i is applied to both UCI parts.

Looking back on Rel-15 design, for PUCCH formats 3 / 4 with two UCI parts, it might be possible to determine adjustment factors for two UCI parts first, similar to what is proposed for UCI part 1 here, then define a rule (e.g., taking the higher adjustment factor between them) to arrive at ΔTF,b,f,c,part1i. Given existing deployments and implementation, it is not desirable to reopen the discussion for Rel-15/16. With that said, given the difference can clearly see from Figure 12, the Rel-15/16 practice should not be used to block Rel-17 design.

At RAN1 #107-e, the following was agreed:

Agreement

For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUCCH in R17,

- At least for PUCCH format 3/4, use the HP UCI bit number and HP RE number for ∆TF,b,f,c(i)∆TF, b, f,ci  formula selection and calculation

- For PUCCH format 1, use the total UCI bit number for ∆TF,b,f,c(i) calculation.

- FFS for PUCCH format 2.

In TS 38.213 v17.0.0, the specification text to capture the RAN1 #107-e agreement reads:

If a UE transmits a PUCCH that includes HARQ-ACK information bits of priority 0 and 1 using PUCCH format 3 or PUCCH format 4, the UE determines a power for the PUCCH transmission, as described in clause 7.2.1, assuming that the PUCCH includes only UCI bits of priority 1, where NRE(i)=minMRBPUCCH⋅Nsc,ctrlRB⋅Nsymb-UCIPUCCH,OUCI,1+OCRC,1Qm⋅r1.

For PUCCH format 3, the above specification text is correct w.r.t. the agreement from RAN1 #107-e. For PUCCH format 4, as spreading factor is used to determine REs after encoding processing, for example with the HP HARQ bits at 6 bits OUCI,1+OCRC,1=6, Qm=2, r1=13, then 62 ×13=9, which does not include the effect of spreading through OCC. And the issue can be also looked at by the formula in the CR itself, the first term in the bracket considering OCC already, but the second does not.

NRE(i)=minMRBPUCCH⋅Nsc,ctrlRB⋅Nsymb-UCIPUCCH,OUCI,1+OCRC,1Qm⋅r1.

We have

Proposal 5-2: the issue for PUCCH format 4 needs to be tackled. Proposal 5-1 above provides a solution for that.

As for PUCCH format 2, the design in Proposal 13-1 naturally covers it.

Note the difference between  and OUCI,1 was identified as important in LTE and NR essentially inherits the design. We don’t see the point to abandon that in Rel-17.

We have

Proposal 5-3: For Rel-17 intra-UE multiplexing, retain the difference between  and OUCI,1 .

# Conclusion

In this contribution, we discuss remaining issues in Rel-17 intra-UE multiplexing.

Proposal 2: Clarify the number of PUCCH resources Z after PUCCH UCI multiplexing.

Proposal 3-1: If a UE is provided a first interlace of MInterlace,0PUCCH PRBs by interlace0 in InterlaceAllocation, the UE has HARQ-ACK, SR and wideband or sub-band CSI reports to transmit, and the UE determines a PUCCH resource with PUCCH format 2, or the UE has HARQ-ACK, SR and wideband CSI reports to transmit and the UE determines a PUCCH resource with PUCCH format 3, or the UE has HARQ-ACK, SR and sub-band CSI reports to transmit, and the UE determines a PUCCH resource with PUCCH format 3, where

- the UE determines the PUCCH resource using the PUCCH resource indicator field in a last of a number of DCI formats with a value of a PDSCH-to-HARQ_feedback timing indicator field indicating a same slot for the PUCCH transmission, from a PUCCH resource set provided to the UE for HARQ-ACK transmission, and after the UE determines the PUCCH resource set

- if

n=1NUCI-part2totalOUCI-part2,n+OCRC,UCI-part2/Qm⋅r2+⌈n=1NUCI-part1totalOUCI-part1,n+OCRC,UCI-part1/Qm⋅r1⌉≤MInterlace,0PUCCH⋅Nsc,ctrlRB⋅Nsymb-UCIPUCCH,

the UE transmits the HARQ-ACK, SR, and CSI reports bits in a PUCCH over the first interlace

- else, if the UE is provided a second interlace of MInterlace,1PUCCH PRBs by interlace1 and if

n=1NUCI-part2totalOUCI-part2,n+OCRC,UCI-part2/Qm⋅r2+⌈n=1NUCI-part1totalOUCI-part1,n+OCRC,UCI-part1/Qm⋅r1⌉≤MInterlace,0PUCCH+MInterlace,1PUCCH⋅Nsc,ctrlRB⋅Nsymb-UCIPUCCH,

the UE transmits the HARQ-ACK, SR, and CSI reports bits in a PUCCH over both the first and second interlaces

- else, the UCI omission procedure is same as the corresponding one when the UE is provided PUCCH-ResourceSet by replacing MRBPUCCH with MInterlace,0PUCCH, or, if the UE is provided interlace1, by MInterlace,0PUCCH+MInterlace,1PUCCH.

Observation 4: agreement from Rel-17 IIoT/URLLC unlicensed spectrum access session is only about the support of UCI types in CG PUSCH, not about how that is supported, which belongs to the intra-UE multiplexing discussion.

Proposal 4: For Rel-17 intra-UE multiplexing, reuse R15 HARQ-ACK rate matching/puncturing and RE mapping for CG-UCI (in the absence of HP HARQ-ACK) or CG-UCI/HP-HARQ-ACK (in the presence of HP HARQ-ACK). LP-HARQ-ACK is treated as if CG-UCI is absent.

Proposal 5-1:  For PUCCH formats 2/3/4, the delta factor ΔTF,b,f,ci is determined from UCI part 1:

- The number of resource elements for UCI part 1 NRE-part1(i)=EUCI-part1EtotNRE(i),  where   EUCI-part1 is the number of coded bits for UCI part 1

- If  n=1NUCI-part1total‍OUCI-part1,n  is smaller or equal to 11,

  - ΔTF,b,f,c,part1i=10⋅log10(K1⋅n=1NUCI-part1total‍OUCI-part1,n/NRE-part1i)

  - If a HARQ-ACK codebook with OUCI-part1,n'  bits is included in UCI part 1,  is used instead of OUCI-part1,n'  for the HARQ-ACK codebook:

    - ΔTF,b,f,c,part1i=10⋅log10(K1⋅n=1NUCI-part1total‍OUCI-part1,n-OUCI-part1,n' +nHARQ-ACK(i)/NRE-part1i)

  - If more than one HARQ-ACK codebooks are included in UCI part 1 (e.g. one due to SPS HARQ deferral, another for HARQ feedback for dynamic grant PDSCH(s)), then replacement of the number of HARQ-ACK codebook size by the associated nHARQ-ACK can be applied to each HARQ-ACK codebook.

- otherwise

  - ΔTF,b,f,c,part1i=10log10(2K2⋅BPREi-1)  where

    - and

    - BPREi=n=1NUCI-part1total‍OUCI-part1,n+OCRC,UCI-part1/NRE-part1i

- And ΔTF,b,f,ci = ΔTF,b,f,c,part1i is applied to both UCI parts.

Proposal 5-2: the issue for PUCCH format 4 needs to be tackled. Proposal 5-1 above provides a solution for that.

Proposal 5-3: For Rel-17 intra-UE multiplexing, retain the difference between  and OUCI,1 .

# References

- RP-193233, “New WID on enhanced Industrial Internet of Things (IoT) and URLLC support”, Nokia, Nokia Shanghai Bell, RAN #86, December 2019

- RP-201310, “Revised WID: Enhanced Industrial Internet of Things (IoT) and ultra-reliable and low latency communication (URLLC) support for NR”, Nokia, Nokia Shanghai Bell, RAN #88e, June 2020

- R1-2008434, “Maintenance of configured grant design in Physical Layer Enhancements for NR URLLC”, Apple, RAN1 #103-e, October 2020

- R1-2103083, UCI multiplexing and PUSCH skipping design in URLLC, Apple, RAN1 #104bis-e, April 2021

- R1-1801462, Remaining details of UL power control design	Huawei, HiSilicon, RAN1 #92, Feb. 2018

- R1-1801746, Correction on NR PUCCH Power Control Formula	CATT, RAN1 #92, Feb. 2018

- R1-1801994, On Delta_TF for PUCCH Power Control	Samsung, RAN1 #92, Feb. 2018

- R1-106020, PUCCH Format 3 Transmission Power for DL CA, Samsung, RAN1 #63, November 2011.

- R1-112009, CR on power control for HARQ-ACK transmission on PUCCH, ZTE, CATT, Samsung, Alcatel-Lucent, Alcatel-Lucent Shanghai Bell, Huawei, HiSilicon, RAN1 #65, May 2011.

- R1-2106120, Design of Rel-17 intra-UE multiplexing/prioritization, Apple, RAN1 #105-e, May 2021

- R1-2107735, Design of Rel-17 intra-UE multiplexing/prioritization, Apple, RAN1 #106-e, August 2021