3GPP TSG RAN WG1 #109-e		R1- 2204210

e-Meeting, May 9th – 20th, 2022

Agenda Item:	8.7.1

Source:	Apple Inc.

Title:	Remaining issues on idle/inactive-mode UE power saving

Document for:	Discussion/Decision

# Introduction

The work item on UE Power Saving Enhancements for NR (RP-200938 [1]) was completed in RAN1#107-e. In this contribution, we discuss one remaining issue for TRS indication for idle/inactive UEs.

# TRS indication for idle/inactive UEs

One remaining issue is the reference point for the start of validity duration, for which we need to down-select from the two alternatives as shown in the agreement below.

Agreement

Down-select one of the TPs to clarify the current default DRX cycle used for determining the reference point for start of validity duration for an availability indication

- Alt1: Adopt TP#1

- Alt2: Adopt TP#2

- Note: TRS availability indication transmitted in all PDCCH monitoring occasions of the same PEI-O or PO is same.

- Note: Reference point for TRS availability indication in a given DCI is identical for all UEs regardless of UE specific DRX value.

============================= start of TP#1========================================== ## 10.4B Indication of TRS resources ## <Unchanged text omitted> A value of '1' for a bit of the bitmap indicates presence of associated TRS resource sets for the multiple of the number of frames, starting from a SFN determined from [17, TS 38.304] that corresponds to the frame within the DRX cycle that includes a the PDCCH providing the DCI format 2_7, or the DCI format 1_0 with CRC scrambled by P-RNTI, with the TRS availability indication field indicating the TRS resource sets, where is provided by defaultPagingCycle. A value of '0' for a bit of the bitmap is ignored by the UE. A UE can receive first and second PDCCHs that provide DCI format 2_7 or DCI format 1_0 with CRC scrambled by P-RNTI that indicate presence of TRS resource sets for the multiple of the number of frames, where the second PDCCH reception after the first PDCCH reception by a time that is smaller than the multiple of the number of frames. ## <Unchanged text omitted> ============================= end of TP#1========================================== ============================= start of TP#2========================================== ## 10.4B Indication of TRS resources ## <Unchanged text omitted> A value of '1' for a bit of the bitmap indicates presence of associated TRS resource sets for the multiple of the number of frames, starting from a SFN determined from [17, TS 38.304] that corresponds to the frame of the DRX cycle associated with the first PDCCH MO of the PEI-O or PO that includes a the PDCCH providing the DCI format 2_7, or the DCI format 1_0 with CRC scrambled by P-RNTI, with the TRS availability indication field indicating the TRS resource sets, where is provided by defaultPagingCycle. A value of '0' for a bit of the bitmap is ignored by the UE. A UE can receive first and second PDCCHs that provide DCI format 2_7 or DCI format 1_0 with CRC scrambled by P-RNTI that indicate presence of TRS resource sets for the multiple of the number of frames, where the second PDCCH reception after the first PDCCH reception by a time that is smaller than the multiple of the number of frames. ## <Unchanged text omitted> ============================= end of TP#2==========================================

Alt 1 essentially means that we stick with the existing agreement. If the MOs of PO/PEI-O happen to span across two DRX cycle, the TRS indication carries in different MOs would have different meaning due to different reference point, and gNB needs to make sure the TRS indication is valid regardless of in which MO the UE receives the indication.

On the other hand, Alt 2 uses the first MO as the reference point, so TRS indication is always interpreted in the same way and does not depend on in which MO the UE receives the indication. This makes it slightly easier for the gNB.

As we are in maintenance phase already and only essential corrections should be considered, we have slight preference for Alt 1.

Proposal: For determining the reference point for start of validity duration for an TRS availability indication, adopt Alt 1/TP#1.

# Conclusion

Proposal: For determining the reference point for start of validity duration for an TRS availability indication, adopt Alt 1/TP#1.

# Reference

- RP-200938, Revised WID UE Power Saving Enhancements for NR, MediaTek Inc., RAN#88e, June 2020.