3GPP TSG RAN WG1 #109-e			R1-2204214

e-Meeting, May 9th – 20th, 2022

Agenda Item:	8.11.1.1

Source:	Apple

Title:	On Sidelink Resource Allocation for Power Saving

Document for:	Discussion/Decision

# Introduction

The Release-17 NR sidelink enhancement work item is declared as complete in RAN #95e meeting.

In this contribution, we discuss the remaining open issues of sidelink resource allocation for power saving.

# Discussion

## Sensing during sidelink DRX inactive time

It was agreed that a UE performing sidelink reception of PSCCH and RSRP measurement for partial sensing during its sidelink DRX inactive time is enabled/disabled by resource pool (pre)configuration. If enabled, UE is required to only monitor the default periodic sensing occasion for PBPS and monitor a minimum of M slots for CPS.

Agreement Whether UE performs SL reception of PSCCH and RSRP measurement for partial sensing on slots in SL DRX inactive time is enabled/disabled by (pre-)configuration per resource pool when partial sensing is configured in the UE by a higher layer. - When it is enabled, - When UE performs periodic-based partial sensing for a given Preserve, UE monitors only the default periodic sensing occasion. - When UE performs contiguous partial sensing, UE monitors a minimum of M slots for CPS. - Note, when it is disabled, the UE is not required to perform SL reception of PSCCH and RSRP measurement in SL DRX inactive time. - Note: no further optimization on the resource (re)selection procedure with regard to SL DRX operation is specified in Rel.17. - FFS the case when full sensing is configured in the UE by a higher layer

It should be noted that for periodic transmission, a UE performs CPS for resource selection from M logical slots earlier than slot ty0SL till Tproc,0SL+Tproc,1SL slots earlier than ty0SL, where ty0SL is the first slot of the selected Y candidate slots of PBPS. In other words, the total number of logical slots that UE performs sensing can be less than M. The similar conclusion applies for UE performing CPS for resource re-evaluation and pre-emption checking.

For aperiodic transmission, the CPS monitoring window starts at least M logical slots before ty0SL till Tproc,0SL+Tproc,1SL slots earlier than ty0SL. In other words, the minimum number of logical slots that UE performs sensing can be less than M.

Hence, the agreement on “when UE performs contiguous partial sensing, UE monitors a minimum of M slots for CPS” seems not aligned with the intention of power saving. We propose that UE monitors a minimum of (M-Tproc,0SL'-Tproc,1SL') logical slots for CPS, in case sensing during SL DRX inactive time is enabled by resource pool (pre)configuration. Here, Tproc,0SL' and Tproc,1SL' are equal to Tproc,0SL and Tproc,1SL converted to logical slots.

Proposal 1: Whether UE performs SL reception of PSCCH and RSRP measurement for partial sensing on slots in SL DRX inactive time is enabled/disabled by (pre-)configuration per resource pool when partial sensing is configured in the UE by a higher layer.

- When it is enabled,

  - When UE performs periodic-based partial sensing for a given Preserve, UE monitors only the default periodic sensing occasion.

  - When UE performs contiguous partial sensing, UE monitors a minimum of (M-Tproc,0SL'-Tproc,1SL') slots for CPS, where Tproc,0SL' and Tproc,1SL' are equal to Tproc,0SL and Tproc,1SL converted to logical slots, and M is equal to resource pool (pre)configured parameters “contiguousSensingWindowAperiodic” or  “contiguousSensingWindowPeriodic”.

Proposal 2: Adopt the following text proposal for the update of the minimum monitoring window duration for contiguous partial sensing in sidelink DRX inactive time.

TS38.214 [1] ### 8.1.4 UE procedure for determining the subset of resources to be reported to higher layers in PSSCH resource selection in sidelink resource allocation mode 2 *** < Unchanged parts are omitted> *** Whether the UE is required to performs SL reception of PSCCH and RSRP measurement for partial sensing on slots in SL DRX inactive time is enabled/disabled by higher layer parameter partialSensingInactiveTime. When it is enabled, if UE performs periodic-based partial sensing on the slots in SL DRX inactive time for a given Prsvp_TX, UE monitors only the default periodic sensing occasions (most recent sensing occasion) from the slots; if UE performs contiguous partial sensing on the slots in SL DRX inactive time, UE monitors a minimum of M M-Tproc,0SL'-Tproc,1SL' slots from the slots, where Tproc,0SL' and Tproc,1SL' are equal to Tproc,0SL and Tproc,1SL converted to logical slots and M is (pre-)configured with contiguousSensingWindowAperiodic for Prsvp_TX=0 or contiguousSensingWindowPeriodic for Prsvp_TX≠0, otherwise M is 31. *** < Unchanged parts are omitted> ***

Another open issue is the behaviour of a full sensing UE configured with sidelink DRX. In our view, it is possible that a full sensing UE is configured with sidelink DRX for power saving. In this case, the full sensing UE performs full sensing in its sidelink DRX inactive time if the resource pool (pre)configuration enables it. The full sensing UE is not required to perform sidelink reception of PSCCH and RSRP measurement in SL DRX inactive time, if the resource pool (pre)configuration disables it. To avoid the RAN2 impact, the existing parameter “partialSensingInactiveTime” could be re-used to enable/disable the full sensing on slots in sidelink DRX inactive time.

Proposal 3: Whether UE performs SL reception of PSCCH and RSRP measurement for full sensing on slots in SL DRX inactive time is enabled/disabled by (pre-)configuration per resource pool when full sensing is configured in the UE by a higher layer.

- Parameter “partialSensingInactiveTime” indicates the enabling/disabling.

Proposal 4: Adopt the following text proposal for the update of the minimum monitoring window duration for contiguous partial sensing in sidelink DRX inactive time.

TS38.214 [1] ### 8.1.4 UE procedure for determining the subset of resources to be reported to higher layers in PSSCH resource selection in sidelink resource allocation mode 2 *** < Unchanged parts are omitted> *** Whether the UE is required to performs SL reception of PSCCH and RSRP measurement for partial sensing or full sensing on slots in SL DRX inactive time is enabled/disabled by higher layer parameter partialSensingInactiveTime. When it is enabled, if UE performs periodic-based partial sensing on the slots in SL DRX inactive time for a given Prsvp_TX, UE monitors only the default periodic sensing occasions (most recent sensing occasion) from the slots; if UE performs contiguous partial sensing on the slots in SL DRX inactive time, UE monitors a minimum of M slots from the slots. *** < Unchanged parts are omitted> ***

## Update of resource exclusion in step 6

In resource selection procedure step 6c) of TS 38.214 Section 8.1.4, we have the following agreement.

Agreement In Step 6 c) of TS38.214 Section 8.1.4, when UE is configured with partial sensing by its higher layer, adopt the following changes: - if slot belongs to the set , otherwise, slot is the first slot after slot belonging to the set . - Option D: converted to milliseconds, where slot is the last slot of the Y or Y’ candidate slots. - Slot is the first slot of the selected/remaining set of Y or Y’ candidate slots.

There is an open issue [2] about the update of resource exclusion in step 6.

It was agreed that for PBPS, UE monitors the most recent sensing occasion for a given reservation periodicity before the first slot of the set of Y or Y’ candidate slots by default. If (pre)configured, UE additionally monitors the second most recent sensing occasion for a given reservation periodicity before the first slot of the set of Y or Y’ candidate slots. For the latter case, the value Q in step 6c) of TS 38.214 Section 8.1.4 should be adjusted accordingly.

In the current specification, if Prsvp_RX< Tscal and  n'-m≤Prsvp_RX', the value Q is equal to TscalPrsvp_RX; otherwise, Q=1. The condition n'-m≤Prsvp_RX' is to ensure the next periodic resource reserved at slot t'mSL is before the reference slot t'n'SL, while the condition Prsvp_RX< Tscal indicates the possibility of more than one periodic resource reserved at slot t'mSL is within the resource selection window.

When the parameter “additionalPeriodicSensingOccasion” is (pre)configured, if Prsvp_RX< Tscal and  Prsvp_RX'<n'-m≤2⋅Prsvp_RX', then the value Q is equal to TscalPrsvp_RX+1. Here, the condition Prsvp_RX'<n'-m≤2⋅Prsvp_RX', is to ensure the second most recent sensing occasion for a given reservation periodicity is counted in the resource selection window, and the value Q is increased by 1 to compensate the additional reservation periodicity before the starting of the resource selection window.

This is illustrated in Figure 1. Consider the example of Prsvp_RX=8 ms, Prsvp_RX'=8 slots and Tscal=20 ms. If t'mSL is at slot 0 and t'n'SL is at slot 14, then the range of q=2, 3, 4 are within the resource selection window. This implies that value of Q is extended by 1.

Figure 1: Illustration of the extended Q value

For the case of Prsvp_RX≥ Tscal and Prsvp_RX'< n'-m≤2⋅Prsvp_RX', the value of Q should also be extended by 2, to capture the second most recent sensing occasion for a given reservation periodicity. This is illustrated in Figure 2.

Figure 2: Illustration of the extended Q value

For the case of n'-m≤Prsvp_RX', the legacy formula to calculate the Q value is applied.

Proposal 5: In step 6c) of TS 38.214 Section 8.1.4, for partial sensing, when the parameter “additionalPeriodicSensingOccasion” is (pre)configured,

- If Prsvp_RX< Tscal and Prsvp_RX'< n'-m≤2⋅Prsvp_RX', Q=TscalPrsvp_RX+1;

- If Prsvp_RX≥ Tscal and Prsvp_RX'< n'-m≤2⋅Prsvp_RX', Q=2;

- If Prsvp_RX< Tscal and  n'-m≤Prsvp_RX', Q=TscalPrsvp_RX;

- Otherwise, Q=1;

Proposal 6: Adopt the following text proposal for the update of Q value in step 6c) of TS 38.214.

TS38.214 [1] ### 8.1.4 UE procedure for determining the subset of resources to be reported to higher layers in PSSCH resource selection in sidelink resource allocation mode 2 *** < Unchanged parts are omitted> *** 6) The UE shall exclude any candidate single-slot resource Rx,y from the set SA if it meets all the following conditions: a) the UE receives an SCI format 1-A in slot t'mSL, and 'Resource reservation period' field, if present, and 'Priority' field in the received SCI format 1-A indicate the values Prsvp_RX and prioRX, respectively according to Clause 16.4 in [6, TS 38.213]; b) the RSRP measurement performed, according to clause 8.4.2.1 for the received SCI format 1-A, is higher than ThprioRX,prioTX; c) the SCI format received in slot t'mSL or the same SCI format which, if and only if the 'Resource reservation period' field is present in the received SCI format 1-A, is assumed to be received in slot(s) t'm+q×Prsvp_RX'SL determines according to clause 8.1.5 the set of resource blocks and slots which overlaps with Rx,y+j×Prsvp_TX' for q=1, 2, …, Q and j=0, 1, …, Cresel-1. Here, Prsvp_RX' is Prsvp_RX converted to units of logical slots according to clause 8.1.7, when the parameter “additionalPeriodicSensingOccasion” is not (pre)configured, Q=TscalPrsvp_RX if Prsvp_RX< Tscal and n'-m≤Prsvp_RX', where if the UE is configured with full sensing by its higher layer, t'n'SL = n if slot n belongs to the set t'0SL,t'1SL,...,t'T'max-1SL, otherwise slot t'n'SL is the first slot after slot n belonging to the set t'0SL,t'1SL,...,t'T'max-1SL; If UE is configured with partial sensing by its higher layer, tn''SL=tyiSL-Tproc,1SL if slot tyiSL-Tproc,1SL belongs to the set (t0'SL, t1'SL,⋯, tTmax'-1'SL), otherwise, slot tn''SL is the first slot after slot tyiSL-Tproc,1SL belonging to the set (t0'SL, t1'SL,⋯, tTmax'-1'SL). otherwise Q=1. When the parameter “additionalPeriodicSensingOccasion” is (pre)configured, Q=TscalPrsvp_RX+1 if Prsvp_RX< Tscal and Prsvp_RX'< n'-m≤2⋅Prsvp_RX', or Q=2 if Prsvp_RX≥ Tscal and Prsvp_RX'< n'-m≤2⋅Prsvp_RX', or Q=TscalPrsvp_RX if Prsvp_RX< Tscal and n'-m≤Prsvp_RX', or otherwise, Q=1. If the UE is configured with full sensing by its higher layer, Tscal is set to selection window size T2 converted to units of msec. If UE is configured with partial sensing by its higher layer, Tscal=tyLSL-(tyiSL-Tproc,1SL) shall be converted to milliseconds, where slot tyLSL is the last slot of the Y or Y' candidate slots. The slot tyiSL is the first slot of the selected/remaining set of Y or Y' candidate slots. *** < Unchanged parts are omitted> ***

# Conclusion

In this contribution, we provided our views on the remaining open issues of NR sidelink resource allocation for power saving. Our proposals are as follows:

Proposal 1: Whether UE performs SL reception of PSCCH and RSRP measurement for partial sensing on slots in SL DRX inactive time is enabled/disabled by (pre-)configuration per resource pool when partial sensing is configured in the UE by a higher layer.

- When it is enabled,

  - When UE performs periodic-based partial sensing for a given Preserve, UE monitors only the default periodic sensing occasion.

  - When UE performs contiguous partial sensing, UE monitors a minimum of (M-Tproc,0SL'-Tproc,1SL') slots for CPS, where Tproc,0SL' and Tproc,1SL' are equal to Tproc,0SL and Tproc,1SL converted to logical slots, and M is equal to resource pool (pre)configured parameters “contiguousSensingWindowAperiodic” or  “contiguousSensingWindowPeriodic”.

Proposal 2: Adopt the following text proposal for the update of the minimum monitoring window duration for contiguous partial sensing in sidelink DRX inactive time.

TS38.214 [1] ### 8.1.4 UE procedure for determining the subset of resources to be reported to higher layers in PSSCH resource selection in sidelink resource allocation mode 2 *** < Unchanged parts are omitted> *** Whether the UE is required to performs SL reception of PSCCH and RSRP measurement for partial sensing on slots in SL DRX inactive time is enabled/disabled by higher layer parameter partialSensingInactiveTime. When it is enabled, if UE performs periodic-based partial sensing on the slots in SL DRX inactive time for a given Prsvp_TX, UE monitors only the default periodic sensing occasions (most recent sensing occasion) from the slots; if UE performs contiguous partial sensing on the slots in SL DRX inactive time, UE monitors a minimum of M M-Tproc,0SL'-Tproc,1SL' slots from the slots, where Tproc,0SL' and Tproc,1SL' are equal to Tproc,0SL and Tproc,1SL converted to logical slots. *** < Unchanged parts are omitted> ***

Proposal 3: Whether UE performs SL reception of PSCCH and RSRP measurement for full sensing on slots in SL DRX inactive time is enabled/disabled by (pre-)configuration per resource pool when full sensing is configured in the UE by a higher layer.

- Parameter “partialSensingInactiveTime” indicates the enabling/disabling.

Proposal 4: Adopt the following text proposal for the update of the minimum monitoring window duration for contiguous partial sensing in sidelink DRX inactive time.

TS38.214 [1] ### 8.1.4 UE procedure for determining the subset of resources to be reported to higher layers in PSSCH resource selection in sidelink resource allocation mode 2 *** < Unchanged parts are omitted> *** Whether the UE is required to performs SL reception of PSCCH and RSRP measurement for partial sensing or full sensing on slots in SL DRX inactive time is enabled/disabled by higher layer parameter partialSensingInactiveTime. When it is enabled, if UE performs periodic-based partial sensing on the slots in SL DRX inactive time for a given Prsvp_TX, UE monitors only the default periodic sensing occasions (most recent sensing occasion) from the slots; if UE performs contiguous partial sensing on the slots in SL DRX inactive time, UE monitors a minimum of M slots from the slots. *** < Unchanged parts are omitted> ***

Proposal 5: In step 6c) of TS 38.214 Section 8.1.4, for partial sensing, when the parameter “additionalPeriodicSensingOccasion” is (pre)configured,

- If Prsvp_RX< Tscal and Prsvp_RX'< n'-m≤2⋅Prsvp_RX', Q=TscalPrsvp_RX+1;

- If Prsvp_RX≥ Tscal and Prsvp_RX'< n'-m≤2⋅Prsvp_RX', Q=2;

- If Prsvp_RX< Tscal and  n'-m≤Prsvp_RX', Q=TscalPrsvp_RX;

- Otherwise, Q=1;

Proposal 6: Adopt the following text proposal for the update of Q value in step 6c) of TS 38.214.

TS38.214 [1] ### 8.1.4 UE procedure for determining the subset of resources to be reported to higher layers in PSSCH resource selection in sidelink resource allocation mode 2 *** < Unchanged parts are omitted> *** 6) The UE shall exclude any candidate single-slot resource Rx,y from the set SA if it meets all the following conditions: a) the UE receives an SCI format 1-A in slot t'mSL, and 'Resource reservation period' field, if present, and 'Priority' field in the received SCI format 1-A indicate the values Prsvp_RX and prioRX, respectively according to Clause 16.4 in [6, TS 38.213]; b) the RSRP measurement performed, according to clause 8.4.2.1 for the received SCI format 1-A, is higher than ThprioRX,prioTX; c) the SCI format received in slot t'mSL or the same SCI format which, if and only if the 'Resource reservation period' field is present in the received SCI format 1-A, is assumed to be received in slot(s) t'm+q×Prsvp_RX'SL determines according to clause 8.1.5 the set of resource blocks and slots which overlaps with Rx,y+j×Prsvp_TX' for q=1, 2, …, Q and j=0, 1, …, Cresel-1. Here, Prsvp_RX' is Prsvp_RX converted to units of logical slots according to clause 8.1.7, when the parameter “additionalPeriodicSensingOccasion” is not (pre)configured, Q=TscalPrsvp_RX if Prsvp_RX< Tscal and n'-m≤Prsvp_RX', where if the UE is configured with full sensing by its higher layer, t'n'SL = n if slot n belongs to the set t'0SL,t'1SL,...,t'T'max-1SL, otherwise slot t'n'SL is the first slot after slot n belonging to the set t'0SL,t'1SL,...,t'T'max-1SL; If UE is configured with partial sensing by its higher layer, tn''SL=tyiSL-Tproc,1SL if slot tyiSL-Tproc,1SL belongs to the set (t0'SL, t1'SL,⋯, tTmax'-1'SL), otherwise, slot tn''SL is the first slot after slot tyiSL-Tproc,1SL belonging to the set (t0'SL, t1'SL,⋯, tTmax'-1'SL). otherwise Q=1. When the parameter “additionalPeriodicSensingOccasion” is (pre)configured, Q=TscalPrsvp_RX+1 if Prsvp_RX< Tscal and Prsvp_RX'< n'-m≤2⋅Prsvp_RX', or Q=2 if Prsvp_RX≥ Tscal and Prsvp_RX'< n'-m≤2⋅Prsvp_RX', or Q=TscalPrsvp_RX if Prsvp_RX< Tscal and n'-m≤Prsvp_RX', or otherwise, Q=1. If the UE is configured with full sensing by its higher layer, Tscal is set to selection window size T2 converted to units of msec. If UE is configured with partial sensing by its higher layer, Tscal=tyLSL-(tyiSL-Tproc,1SL) shall be converted to milliseconds, where slot tyLSL is the last slot of the Y or Y' candidate slots. The slot tyiSL is the first slot of the selected/remaining set of Y or Y' candidate slots. *** < Unchanged parts are omitted> ***

# References

- R1-2202963, “Corrections on NR sidelink enhancements,” Mar. 2022.

- R1-2202565, “FL summary for AI 8.11.1.1 – NR sidelink resource allocation for power saving (EOM)”, Feb. 2022.