3GPP TSG RAN WG1 #109-e			R1-2204216

e-Meeting, May 9th – 20th, 2022

Agenda item:	8.12.1

Source:	Apple Inc.

Title:	Remaining issues on MBS reliability improvement for RRC_CONNECTED UEs

Document for:	Discussion/Decision

# Introduction

In RAN#86 meeting, new work item on NR Multicast and Broadcast Services [1] was agreed. Some of objectives of this study item are showing below,

- Specify RAN basic functions for broadcast/multicast for UEs in RRC_CONNECTED state [RAN1, RAN2, RAN3]:

  - Specify a group scheduling mechanism to allow UEs to receive Broadcast/Multicast service [RAN1, RAN2]

    - This objective includes specifying necessary enhancements that are required to enable simultaneous operation with unicast reception.

  - Specify support for dynamic change of Broadcast/Multicast service delivery between multicast (PTM) and unicast (PTP) with service continuity for a given UE [RAN2, RAN3]

  - Specify support for basic mobility with service continuity [RAN2, RAN3]

  - Assuming that the necessary coordination function (like functions hosted by MCE, if any) resides in the gNB-CU, specify required changes on the RAN architecture and interfaces, considering the results of the SA2 SI on Broadcast/Multicast (SP-190625) [RAN3]

  - Specify required changes to improve reliability of Broadcast/Multicast service, e.g. by UL feedback. The level of reliability should be based on the requirements of the application/service provided.[RAN1, RAN2]

  - Study the support for dynamic control of the Broadcast/Multicast transmission area within one gNB-DU and specify what is needed to enable it, if anything [RAN2, RAN3]

Some MBS reliability related agreements were made in RAN1#108-e meeting [2]. In this contribution, we discuss the MBS reliability enhancement aspects, including the HARQ-ACK feedback mechanism, Type 1 HARQ codebook, etc.

# MBS reliability improvements for RRC_CONNECTED UEs

After last RAN1 meeting, the following issues are still open.

Multiplexing NACK-only feedback with SR

For NACK-only based feedback multiplexing with SR, after several round discussion, the followings were proposed. For up to two HARQ-ACK bits case, the two HARQ-ACK bits could come from NACK-only feedback or one bit is from NACK-only feedback, another bit is from unicast. The HARQ-ACK multiplexing with SR is UE specific, so the UE dedicated PUCCH resource should be used. The Alt-1 is straight forward and preferred.

For more than two HARQ-ACK bits case, it’s up to which NACK-only feedback mode is configured. If Alt 1 is configured, Alt 2-2 can be adopted. Alt4 only supports up to four bits, then Alt2-3 can be applied, as PUCCH resource for NACK-only feedback can be shared by group of users, the impact of dropping the NACK-only bits can be acceptable.

#### Proposal 3.2.1.4-1 (GTW) When PUCCH transmission for the NACK-only based feedback for multicast collides with PUCCH transmissions for SR for the same priority, support the following: - For up to two HARQ-ACK bits for NACK-only feedback is transformed into ACK/NACK HARQ bits and is multiplexed with SR as legacy procedure of Rel-15/16. - Alt1-1: PUCCH resources configured for unicast will be used. - Note: either the PUCCH resource for SR or for ACK/NACK is used per the legacy procedure of Rel-15/16. - the PUCCH resource is determined by the PRI in the last DCI for multicast; - Alt1-2: the PUCCH resource is determined by RRC configuration; - Alt1-3: the PUCCH resource for NACK-only feedback transmission is SR resource. - For the case of more than two TBs of NACK-only, FFS which alternative from the following is supported: - Alt2-1: SR is dropped - Alt2-2: NACK-only feedback is transformed into ACK/NACK HARQ bits and PF 2/3/4 is used for the transmission per the legacy SR and HARQ-ACK multiplexing rule of Rel-15/16. - Alt2-3: NACK-only is dropped #### Proposal 3.2.1.4-2 (if proposal 3.2.1.4-1 is not agreeable) Multiplexing NACK-only based feedback for multicast with SR is not supported in Rel-17, - It is up to UE implementation when the PUCCH transmission for the NACK-only based feedback for multicast collides with PUCCH transmissions for SR.

Proposal 1:  When PUCCH transmission for the NACK-only based feedback for multicast collides with PUCCH transmissions for SR for the same priority, support the following:

  - For up to two HARQ-ACK bits for NACK-only feedback is transformed into ACK/NACK HARQ bits and is multiplexed with SR as legacy procedure of Rel-15/16.

    - Alt1-1: PUCCH resources configured for unicast will be used.

      - Note: either the PUCCH resource for SR or for ACK/NACK is used per the legacy procedure of Rel-15/16.

      - the PUCCH resource is determined by the PRI in the last DCI for multicast;

  - For the case of more than two TBs of NACK-only, FFS which alternative from the following is supported:

    - If NACK-only feedback mode Alt 1 is configured. Alt2-2: NACK-only feedback is transformed into ACK/NACK HARQ bits and PF 2/3/4 is used for the transmission per the legacy SR and HARQ-ACK multiplexing rule of Rel-15/16.

    - If NACK-only feedback mode Alt 4 is configured. Alt2-3: SR is dropped

NACK-only overlapping with ACK/NACK

In last RAN1 meeting, the following agreement was reached on codebook construction for NACK-only feedback and ACK/NACK feedback multiplexing. Down-selection is needed between two alternatives. We agreed to configured type 1 or type 2 codebook for ACK/NACK based feedback. However, there is no codebook type is configured for NACK-only based feedback, as the intention of NACK-only based feedback just send one HARQ-ACK bit. For Alt 1, NACK-only feedback is considered as configured with type 2 codebook, whatever the ACK/NACK based feedback is configured with type 1 or type 2 codebook. Comparing with Alt2, this will introduce more additional HARQ-ACK bits for NACK-only feedback, if type1 codebook is configured for ACK/NACK based feedback. As the same TDRA table is applied to all G-RNTIs. Alt2 just follows the design principle for multiplexing HARQ-ACK bits between ACK/NACK based feedback and unicast, UE assumes NACK-only feedback codebook is the same as the ACK/NACK based codebook type if configured, otherwise follows the codebook type of unicast.

Agreement For multiplexing NACK-only feedback for the first G-RNTI with ACK/NACK based feedback for the second G-RNTI or for unicast, down-select from: - Alt1: the converted NACK-only bits are concatenated to the ACK/NACK feedback. - Alt2: the codebook construction/concatenation is the same as when the feedback mode is ‘ACK/NACK’ for both G-RNTIs.

Proposal 2: For multiplexing NACK-only feedback for the first G-RNTI with ACK/NACK based feedback for the second G-RNTI or for unicast, down-select from:

- Alt2: the codebook construction/concatenation is the same as when the feedback mode is ‘ACK/NACK’ for both G-RNTIs.

NACK-only based HARQ feedback

In last RAN1 meeting, the following agreement was reached to support more than one bit with NACK-only based HARQ feedback. But there are several open issues.

Agreement For supporting more than one NACK-only feedback in the same PUCCH transmission, define RRC configuration to configure between Alt1 and Alt4 (from previous agreements): - Alt1: Support UE multiplexing the HARQ-ACK bits by transforming NACK-only into ACK/NACK HARQ bits. - FFS: how to determine PUCCH resource - Alt4: Define combination of NACK-only which corresponds to a specific sequence or a PUCCH transmission. - define up to 15 orthogonal PUCCH resources to select from according to combinations of up to 4 TBs with NACK-only feedback, - FFS: The PUCCH slot for the transmission is based on the K1 in the “last DCI” scheduling multicast. - FFS: The PUCCH resource for the transmission is from PUCCH-config configured for NACK-only based feedback according to the mapping between number of TBs with PUCCH resource ID. - FFS mapping details. - How to determine the number of TBs is discussed separately, e.g., Type-1-like and/or Type-2-like codebook. - FFS: whether this applies to a single G-RNTI or multiple G-RNTIs - Alt4 is not supported for more than 4 TBs - FFS: whether RRC configuration between Alt1 and Alt4 is per G-RNTI or per CFR - FFS: UE capability

For NACK-only feedback mode Alt1 transforming NACK-only into ACK/NACK HARQ bits, how to determine the number of HARQ-ACK bits is not clear, as the DAI field in DCI format 4_2 is associated with dynamic codebook. There is no codebook configured for NACK-only feedback. In addition, NACK-only feedback multiplexing with CSI on PUCCH or PUSCH was agreed in RAN1#106bis-e meeting. More than one bit case is still open, if multiplexing is supported, the DAI field would be necessary for NACK-only feedback. Otherwise, multiplexing is not supported, dropping rule need to be defined.

Agreement: When PUCCH transmission for the NACK-only based feedback for multicast collides with PUCCH transmissions for HARQ-ACK feedback/CSI for unicast for the same priority or PUSCH transmission for the same priority, support UE multiplexing the NACK-only based feedback with the HARQ-ACK feedback/CSI on PUCCH or on to PUSCH by transforming NACK-only into the ACK/NACK HARQ bit. - This applies to at least the case of the feedback addressing one TB. NACK-only based feedback for more than one TBs is to be handled separately. - Note: When the TB is correctly decoded, the ACK will be transmitted and multiplexed with others. - FFS the case of PUCCH for SR.

Proposal 3: 2bit DAI field in DCI format 4_2 is applied for NACK-only feedback as counter DAI.

For NACK-only feedback mode Alt4, the mapping between the HARQ-ACK bit and PUCCH resource need to be specified. Alt4 only supports up to 4TBs, which requires four HARQ-ACK bits. The detailed HARQ-ACK bit to PUCCH resource mapping is showing in below table. In below table, for the same PUCCH resource, if one more TB is scheduled, one bit with NACK status is appended on top of existing HARQ-ACK feedback. This can avoid the error if the last scheduled TB is DTX. For instance, three TBs are scheduled by gNB, but the DCI for last TB, i.e., TB3, was missed by UE. UE would report (N,A) with the 2nd PUCCH resource. After gNB detect the 2nd PUCCH resource, gNB would know third TB is not corrected received by UE.

- Table 1: HARQ-ACK bit mapping to PUCCH resource

| HARQ-ACK feedback |  |  |  | PUCCH resource |
| --- | --- | --- | --- | --- |
| TB1 | TB1,TB2 | TB1,TB2,TB3 | TB1,TB2,TB3,TB4 |  |
| N | N,N | N,N,N | N,N,N,N | 1st PUCCH resource |
|  | N,A | N,A,N | N,A,N,N | 2nd PUCCH resource |
|  | A,N | A,N,N | A,N,N,N | 3rd PUCCH resource |
|  |  | N,N,A | N,N,A,N | 4th PUCCH resource |
|  |  | N,A,A | N,A,A,N | 5th PUCCH resource |
|  |  | A,N,A | A,N,A,N | 6th PUCCH resource |
|  |  | A,A,N | A,A,N,N | 7th PUCCH resource |
|  |  |  | N,N,N,A | 8th PUCCH resource |
|  |  |  | N,N,A,A | 9th PUCCH resource |
|  |  |  | N,A,N,A | 10th PUCCH resource |
|  |  |  | A,N,N,A | 11th PUCCH resource |
|  |  |  | N,A,A,A | 12th PUCCH resource |
|  |  |  | A,N,A,A | 13th PUCCH resource |
|  |  |  | A,A,N,A | 14th PUCCH resource |
|  |  |  | A,A,A,N | 15th PUCCH resource |

Proposal 4: Adopt the above HARQ-ACK bit mapping to PUCCH resource table for NACK-only feedback mode Alt4.

Multicast HARQ-ACK on PUSCH

If the type 2 codebook is configured for multicast and multicast HARQ-ACK bits are multiplexing on PUSCH, the UL DAI field can be used for checking the missing HARQ-ACK bit of last scheduled multicast PDSCH. However, even considering the case of HAQ-ACK bits of multiple G-RNTIs multiplexing on the same PUCCH, if the last counter DAI for a G-RNTI is missed, the HARQ-ACK bits ordering could be wrong, as the sub-codebook for a G-RNTI is appended to a sub-codebook of another G-RNTI. The issue can be solved by allowing 4bits DL DAI in DCI format 4_2 in case multiple G-RNTIs are configured, 2 MSB bits are the counter DAI and the 2 LSB bits are the total DAI for the scheduled G-RNTI. The total DAI is all the DAIs for the scheduled G-RNTI within the k1 set, i.e., the DL slots within the k1 set will feed back the HARQ-ACK bits in the same UL slot. The total DAI value is the same for all slots in the k1 set.

Proposal 5: If multiple G-RNTIs are configured, 2bit total DAI in DCI format 4_2 is to indicate the all the scheduled PDSCH for a G-RNTI and the total DAIs in all the DCIs have the same value.

In addition to allow 2bit total DAI to assist type 2 codebook construction, if HARQ-ACK bits are multiplexing on PUSCH, the UL DAI in UL grant could provide additional information to check PDSCH missing which the PDSCH is the only scheduled PDSCH for a G-RNTI.

Agreement If Type-2 codebook is configured for unicast and multicast, the following option2-2 (from the previous agreement) is adopted: - Option2-2: 2-bit UL DAI(s) are included in DCI for multicast, in addition to the 2-bit UL DAI for unicast. - The 2-bit UL DAI for multicast is applied for all configured G-RNTIs. - FFS: how to count the total number of HARQ-ACK bits for all configured G-RNTIs. - Alt1-1: UL-DAI indicates the sum of DL-DAIs - Alt1-2: DL-DAIs have the same value. - Alt1-3: largest DL DAI value among the configured G-RNTIs. - Other alternatives are not precluded

Proposal 6:  If Type-2 codebook is configured for unicast and multicast, the following option2-2 (from the previous agreement) is adopted:

- Option2-2: 2-bit UL DAI(s) are included in DCI for multicast, in addition to the 2-bit UL DAI for unicast.

    - The 2-bit UL DAI for multicast is applied for all configured G-RNTIs.

        - Alt1-1: UL-DAI indicates the sum of DL-DAIs

# Summary

In this contribution, we discuss MBS HARQ feedback mechanism and potential reliability improvement techniques, and have the following proposals:

Proposal 1:  When PUCCH transmission for the NACK-only based feedback for multicast collides with PUCCH transmissions for SR for the same priority, support the following:

  - For up to two HARQ-ACK bits for NACK-only feedback is transformed into ACK/NACK HARQ bits and is multiplexed with SR as legacy procedure of Rel-15/16.

    - Alt1-1: PUCCH resources configured for unicast will be used.

      - Note: either the PUCCH resource for SR or for ACK/NACK is used per the legacy procedure of Rel-15/16.

      - the PUCCH resource is determined by the PRI in the last DCI for multicast;

  - For the case of more than two TBs of NACK-only, FFS which alternative from the following is supported:

    - If NACK-only feedback mode Alt 1 is configured. Alt2-2: NACK-only feedback is transformed into ACK/NACK HARQ bits and PF 2/3/4 is used for the transmission per the legacy SR and HARQ-ACK multiplexing rule of Rel-15/16.

    - If NACK-only feedback mode Alt 4 is configured. Alt2-3: SR is dropped

Proposal 2: For multiplexing NACK-only feedback for the first G-RNTI with ACK/NACK based feedback for the second G-RNTI or for unicast, down-select from:

- Alt2: the codebook construction/concatenation is the same as when the feedback mode is ‘ACK/NACK’ for both G-RNTIs.

Proposal 3: 2bit DAI field in DCI format 4_2 is applied for NACK-only feedback as counter DAI.

Proposal 4: Adopt the above HARQ-ACK bit mapping to PUCCH resource table for NACK-only feedback mode Alt4.

Proposal 5: If multiple G-RNTIs are configured, 2bit total DAI in DCI format 4_2 is to indicate the all the scheduled PDSCH for a G-RNTI and the total DAIs in all the DCIs have the same value.

Proposal 6:  If Type-2 codebook is configured for unicast and multicast, the following option2-2 (from the previous agreement) is adopted:

- Option2-2: 2-bit UL DAI(s) are included in DCI for multicast, in addition to the 2-bit UL DAI for unicast.

    - The 2-bit UL DAI for multicast is applied for all configured G-RNTIs.

        - Alt1-1: UL-DAI indicates the sum of DL-DAIs

# References

- RP-193248, “New Work Item on NR Multicast and Broadcast Services”, Huawei, HiSilicon

- R1-2203011, “Report of RAN1#108-e meeting”, ETSI MCC

- R1-2202883, “FL summary#5 on improving reliability for MBS for RRC_CONNECTED UEs”, Moderator (Huawei)