3GPP TSG RAN WG1 #109-e			R1-2204221

e-Meeting, May 9th – 20th, 2022

Agenda Item:	8.16.4

Source:	Apple

Title:	Views on Rel-17 NR NTN UE Features

Document for:	Discussion/Decision

# Introduction

The updated RAN1 UE features for Release-17 NR from WI rapporteurs were summarized in [1]. These include the UE features on NR NTN.

In this contribution, we provide our views on the RAN1 UE features on NR NTN.

# Discussion

The feature 26-1 was updated or expanded to uplink time and frequency pre-compensation and timing relationship enhancements. This feature is mandatory in supporting NR communication via satellite. In the latest version of this feature, the field of “Need for the gNB to know if the feature is supported” is No. However, we think gNB needs to know if a UE supports this feature to serve it via satellite. Hence, we have the following proposal.

Proposal 1: It is necessary for the gNB to know if the feature 26-1 is supported.

The features 26-6, 26-6a, 26-6b are respectively defined for Type-1, Type-2, Type-3 HARQ codebook enhancement when there are feedback disabled HARQ processes. The pre-requisite features are open for these three features. In our view, UE needs to support the PDSCH reception associated with HARQ processes with feedback disabling, before it can construct the enhanced HARQ-ACK codebook construction. The feature of downlink HARQ disabling is defined in RAN2, which could serve as pre-requisite of features 26-6, 26-6a and 26-6b.

Proposal 2: The pre-requisite of features 26-6, 26-6a and 26-6b are the RAN2 feature of downlink HARQ disabling.

# Conclusion

We discussed Rel-17 NR NTN UE features. Our proposals are as follows:

Proposal 1: It is necessary for the gNB to know if the feature 26-1 is supported.

Proposal 2: The pre-requisite of features 26-6, 26-6a, 26-6b are the RAN2 feature of downlink HARQ disabling.

# References

- R1-2202928, “Updated RAN1 UE features list for Rel-17 NR after RAN1 #108-e,” Feb. 2022.