3GPP TSG RAN WG1 #109-e		                                R1-2204224

e-Meeting, May 9th – 20th, 2022

Agenda Item:	8.16.11

Source:	Apple

Title:	Views on Rel-17 NR Sidelink UE Features

Document for:	Discussion/Decision

# Introduction

The updated RAN1 UE features for Release-17 NR from WI rapporteurs were summarized in [1]. These include the UE features on NR sidelink enhancement. Furthermore, it was agreed in RAN #95-e meeting [2] that FG 32-2 is split into two sub-FGs, where one sub-FG is for support of only S-SSB reception and another sub-FG for support of PSFCH reception with pre-requisite of S-SSB reception.

In this contribution, we provide our views on the remaining RAN1 UE features on NR sidelink enhancement.

# Discussion

It was agreed that FG 32-2 is split into two sub-FGs, where one sub-FG is for support of only S-SSB reception and the other sub-FG for support of PSFCH reception with pre-requisite of S-SSB reception. The remaining details of these two feature groups are still open.

For the feature “receiving NR sidelink of S-SSB”, we think UE needs to report it to gNB but does not need to report it to other UEs. This is similar to FG 15-4. The granularity of this feature is per band.

For the feature “receiving NR sidelink of PSFCH”, we think UE needs to report it to gNB but does not need to report it to other UEs. This is similar to FG 15-11. The granularity of this feature is per band.

Proposal 1: For the feature “receiving NR sidelink of S-SSB”,

- need for the gNB to know if the feature is supported

- no need for other UEs to know if the feature is supported

- defined per band

Proposal 2: For the feature “receiving NR sidelink of PSFCH”,

- need for the gNB to know if the feature is supported

- no need for other UEs to know if the feature is supported

- defined per band

The pre-requisite for the feature 32-4 and 32-4a is open. As discussed in previous RAN1 meetings, the sidelink synchronization is necessary for NR sidelink transmission with reduced sensing. Here, the feature 32-4b or 32-4c could serve as pre-requisite. Additionally, the feature of “receiving NR sidelink S-SSB” serves as pre-requisite for features 32-4 and 32-4a.

Proposal 3: In both feature 32-4 and feature 32-4a, pre-requisite feature is at least one of 32-4b, 32-4c and the feature of “receiving NR sidelink of S-SSB”.

The feature 32-5b-2 is to receive inter-UE coordination information scheme 2 in NR sidelink mode 2. The reception of inter-UE coordination scheme 2 is based on transmitting NR sidelink mode 2. Hence, the pre-requisite feature of 32-5b-2 is one of features 15-3, 32-4 and 32-4a.

Proposal 4: In feature 32-5b-2, the pre-requisite feature group is at least one of features 15-3, 32-4 and 32-4a.

The feature 32-6-1 is to receive scheme 1 inter-UE coordination information over 2nd SCI. The pre-requisite of this feature is the reception of inter-UE coordination information, i.e., at least one of features 32-5a-2 and 32-5a-3. This is because a UE with capability of receiving inter-UE coordination information over 2nd SCI must have the capability of receiving inter-UE coordination information over MAC CE.

Proposal 5: In feature 32-6-1, the pre-requisite feature group is at least one of 32-5a-2 and 32-5a-3.

The feature 32-6-2 is to receive scheme 1 explicit request over 2nd SCI. The pre-requisite of this feature is the reception of explicit request, i.e., feature 32-5a-1, since a UE with capability of receiving explicit request over 2nd SCI must have the capability of receiving explicit request over MAC CE.

Proposal 6: In feature 32-6-2, the pre-requisite feature group is 32-5a-1.

The feature 32-5a-1 involves sensing operations to identify a set of preferred or non-preferred resources. These sensing operations require a lot of computations. A UE may be limited by its amount of computational capability. Hence, instead of defining this feature in per band, we prefer it is defined per FS. This could reflect UE’s total computational capability.

The features 32-5b-1 and 32-7 involve sensing operations to determine a potential resource collision. These sensing operations require a lot of computations. Similar to feature 32-5a-1, we prefer it is defined per FS. This could reflect UE’s total computational capability.

Proposal 7: In features 32-5a-1, 32-5b-1 and 32-7, the granularity is per FS.

The feature 32-6-2 involves the SCI format 2-C decoding. This is similar to feature 32-6-1. Hence, this feature could be defined per band.

Proposal 8: In feature 32-6-2, the granularity is per band.

# Conclusion

We discussed NR UE sidelink enhancement features. Our proposals are as follows:

Proposal 1: For the feature “receiving NR sidelink of S-SSB”,

- need for the gNB to know if the feature is supported

- no need for other UEs to know if the feature is supported

- defined per band

Proposal 2: For the feature “receiving NR sidelink of PSFCH”,

- need for the gNB to know if the feature is supported

- no need for other UEs to know if the feature is supported

- defined per band

Proposal 3: In both feature 32-4 and feature 32-4a, pre-requisite feature is at least one of 32-4b, 32-4c and the feature of “receiving NR sidelink of S-SSB”.

Proposal 4: In feature 32-5b-2, the pre-requisite feature group is at least one of features 15-3, 32-4 and 32-4a.

Proposal 5: In feature 32-6-1, the pre-requisite feature group is at least one of 32-5a-2 and 32-5a-3.

Proposal 6: In feature 32-6-2, the pre-requisite feature group is 32-5a-1.

Proposal 7: In features 32-5a-1, 32-5b-1 and 32-7, the granularity is per FS.

Proposal 8: In feature 32-6-2, the granularity is per band.

# References

- R1-2202928, “Updated RAN1 UE features list for Rel-17 NR after RAN1 #108-e,” Feb. 2022.

- Chairman’s Notes, 3GPP TSG RAN1 WG #95-e Meeting, Mar. 2022.