3GPP TSG RAN WG1 #109-e			R1-2204225

e-Meeting, May 9th – 20th, 2022

Agenda item:	8.16.12

Source:                Apple Inc.

Title:                     Views on Rel-17 MBS UE Features

Document for:	Discussion/Decision

# Introduction

In this contribution, we provide our analysis on UE features for NR MBS. The discussion is based on  the upated RAN1 UE features list for Rel-17 NR in [1].

# Discussion

FG33-1 Broadcast

In last RAN1 meeting, there were discussions on the broadcast PDSCH rate matching around LTE CRS. It was clarified the rate-matching around LTE CRS is Rel-15 FG 5-28. Both FG 5-26 and 5-28 are mandatory feature in Rel-15. And there were agreements that multicast PDSCH could be configured with rate matching pattern. So FG 5-26 and 5-28 can be prerequisite of FG33-1 or add a note to require supporting FG5-26 and FG5-28 for broadcast.

Proposal 1: Add FG5-26 and 5-28 as prerequisite of FG33-1 or add a note in FG 33-1: A UE supporting FG 33-1 must indicate support of FGs 5-26 and 5-28 for broadcast.

Regarding the type of FG 33-1, should it per UE, per band or per FSPC. As indicated by RAN2, RRC connected UE could receive the MBS broadcast on Scell or non-serving cell. In addition, RAN1 agreed the related UE capability is to be defined in RAN2. Thus, leaving it to RAN2 to make decision is enough.

- Agreement - From RAN1 perspective, it is feasible for UE in RRC_CONNECTED state to receive MBS broadcast on an activated SCell as long as UE has capability of supporting MBS broadcast on SCell. From RAN1 perspective, if a UE is to receive MBS broadcast on SCell, - The capability of supporting MBS broadcast on SCell is separate capability from the one of CA for unicast. - The UE is not required to monitor DCI formats associated with SI-RNTI, P-RNTI, RA-RNTI in SCell. - Overbooking for SCell is not supported. - MBS broadcast reception on SCell can be supported only for RRC_CONNECTED UEs only with self-scheduling. - Type0-PDCCH CSS set is only configured on the primary cell of the MCG. - Configuring the search space on SCell for PDCCH monitoring of MBS DCI formats is via unicast RRC signaling. - The UE capability is expected to be defined by RAN2. - E.g. the total number of component carriers for receiving broadcast on SCell may be subject to UE capability - The UE is not required to receive broadcast on PCell and SCell simultaneously

Proposal 2: It’s up to RAN2 to determine FG33-1 is per UE, per band or per SFPC.

ACK/NACK feedback and NACK-only based feedback

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported |
| --- | --- | --- | --- | --- | --- |
| 33. NR_MBS | 33-2a | Support of ACK/NACK based HARQ-ACK feedback andRRC-based enabling/disabling ACK/NACK-based feedback for dynamic scheduling for multicast | 1) Support of ACK/NACK based HARQ-ACK feedback, and support of enabling/disabling ACK/NACK based HARQ-ACK feedback configured by RRC signalling 2) Support of PTM retransmission for multicast 3) support of Type-1 and Type-2 HARQ-ACK CB for multicast feedback only 4) Support of shared PUCCH resource configurations with unicast FFS: Whether to define separate PUCCH resource configurations from unicast as separate FG from FG 33-2a | 33-2 | Yes |
| 33. NR_MBS | 33-4 | NACK-only based HARQ-ACK feedback for multicast | - [Support NACK-only based HARQ-ACK feedback.] - Support of shared PUCCH resource configurations with unicast FFS: Whether to define separate PUCCH resource configurations from unicast as separate FG from FG 33-4 | [33-2] | Yes |

Regarding ACK/NACK based feedback and NACK-only based feedback, one open issue is whether introduce a separate FG for separate PUCCH resource configuration. For ACK/NACK HARQ feedback, the separated PUCCH resource configuration can be another FG, the shared PUCCH resource configuration with unicast is the basic function of FG33-2a. If multicast HARQ-ACK bit multiplexing with HARQ bit of unicast, the unicast PUCCH resource will be applied according to the agreement that PRI indicates last DCI for unicast. If there is no separated PUCCH resource configuration for multicast, the HARQ feedback function works well. For NACK-only based HARQ feedback, the shared and separated PUCCH resource configurations can be merged into FG33-4, as one of benefits to introduce the NACK-only feedback is to save the PUCCH resource by sharing the resource among the users, and to reduce the gNB blind detection PUCCH complexity.

Proposal 3: For ACK/NACK based HARQ feedback, define separate PUCCH resource configurations from unicast as separate FG from FG 33-2a.

Proposal 4: For NACK-only based feedback, support both shared and separate PUCCH resource configurations with unicast.

NACK-only based HARQ-ACK feedback

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported |
| --- | --- | --- | --- | --- | --- |
| 33. NR_MBS | 33-4 | NACK-only based HARQ-ACK feedback for multicast | - [Support NACK-only based HARQ-ACK feedback.] - Support of shared PUCCH resource configurations with unicast FFS: Whether to define separate PUCCH resource configurations from unicast as separate FG from FG 33-4 | [33-2] | Yes |

In last RAN1 meeting, it was agreed to support more than one bit feedback for NACK-only based feedback. RRC signaling can configure either Alt1 or Alt 4.  Supporting two alternatives shall be independent UE capabilities to provide the UE implementation flexibility.

Agreement For supporting more than one NACK-only feedback in the same PUCCH transmission, define RRC configuration to configure between Alt1 and Alt4 (from previous agreements): - Alt1: Support UE multiplexing the HARQ-ACK bits by transforming NACK-only into ACK/NACK HARQ bits. - FFS: how to determine PUCCH resource - Alt4: Define combination of NACK-only which corresponds to a specific sequence or a PUCCH transmission. - define up to 15 orthogonal PUCCH resources to select from according to combinations of up to 4 TBs with NACK-only feedback, - FFS: The PUCCH slot for the transmission is based on the K1 in the “last DCI” scheduling multicast. - FFS: The PUCCH resource for the transmission is from PUCCH-config configured for NACK-only based feedback according to the mapping between number of TBs with PUCCH resource ID. - FFS mapping details. - How to determine the number of TBs is discussed separately, e.g., Type-1-like and/or Type-2-like codebook. - FFS: whether this applies to a single G-RNTI or multiple G-RNTIs - Alt4 is not supported for more than 4 TBs - FFS: whether RRC configuration between Alt1 and Alt4 is per G-RNTI or per CFR - FFS: UE capability

Proposal 5:  Define two separate NACK-only feedback modes.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported |
| --- | --- | --- | --- | --- | --- |
| 33. NR_MBS | 33-4-2 | NACK-only HARQ-ACK bits transforming into ACK/NACK HARQ bits | Supporting more than one NACK-only feedback in the same PUCCH transmission by transforming NACK-only HARQ-ACK bits to ACK/NACK HARQ bits. | 33-4 | Yes |
| 33. NR_MBS | 33-4-3 | PUCCH resource table for NACK-only feedback | Supporting more than one NACK-only feedback in the same PUCCH transmission by defining HARQ-ACK bit to PUCCH resource mapping table | 33-4 |  |

# Summary

In this contribution, we discuss the MBS related UE features and give the following proposals.

Proposal 1: Add FG5-26 and 5-28 as prerequisite of FG33-1 or add a note in FG 33-1: A UE supporting FG 33-1 must indicate support of FGs 5-26 and 5-28 for broadcast.

Proposal 2: It’s up to RAN2 to determine FG33-1 is per UE, per band or per SFPC.

Proposal 3: For ACK/NACK based HARQ feedback, define separate PUCCH resource configurations from unicast as separate FG from FG 33-2a.

Proposal 4: For NACK-only based feedback, Support both shared and separate PUCCH resource configurations with unicast.

Proposal 4:  Keeping FG33-4 as independent feature and include FG33-2c as prerequisite feature group.

Proposal 5:  Define two separate NACK-only feedback modes.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported |
| --- | --- | --- | --- | --- | --- |
| 33. NR_MBS | 33-4-2 | NACK-only HARQ-ACK bits transforming into ACK/NACK HARQ bits | Supporting more than one NACK-only feedback in the same PUCCH transmission by transforming NACK-only HARQ-ACK bits to ACK/NACK HARQ bits. | 33-4 | Yes |
| 33. NR_MBS | 33-4-3 | PUCCH resource table for NACK-only feedback | Supporting more than one NACK-only feedback in the same PUCCH transmission by defining HARQ-ACK bit to PUCCH resource mapping table | 33-4 |  |

# References

- R1-2202928, “Updated RAN1 UE features list for Rel-17 NR after RAN1 107bis-e”, Moderators (AT&T, NTT DOCOMO, INC.)

- R1-2201520, “Summary on UE features for NR MBS”, Moderator (NTT DOCOMO, INC.)