3GPP TSG RAN WG1 #109-e	R1- 2204228

e-Meeting, May 9th – 20th, 2022

Agenda Item:	18.7

Source:	Apple Inc.

Title:	Dropping Timeline Considerations for SRS Carrier Switching

Document for:	Discussion and Decision

# Introduction

Within the past couple of meetings, RAN1 has been discussing some open aspects regarding SRS carrier switching, when UE switches to a carrier of the serving cell without configured PUCCH/PUSCH transmission to transmit SRS. In RAN1#108-e, three issues were discussed

- Issue #1: SRS carrier switching and prioritization rules

- Issue #2: SRS carrier switching for half-duplex UE with TDD CA

- Issue #3: Back-to-back SRS carrier switching

In this contribution, we provide our views on the first issue.

# Prioritization Rules for SRS Carrier Switching

It was agreed in RAN1#105-e that the prioritization rules of SRS carrier switching apply to at least the source CC. RAN1 continued discussions to determine which UL CCs other than the source CC should be used for SRS carrier switching priority rules, where in RAN1#107, the following conclusion was agreed:

Conclusion

Regarding SRS carrier switching priority rules:

- For Rel-16, it is concluded that no modification in specifications should be made to clarify the current UE behaviour or to introduce a new UE behaviour regarding SRS carrier switching priority rules.

- For releases later than Rel-16, it is concluded to consider introducing a new UE capability for indicating simultaneous transmission while switching, and/or clarify the UE behaviour in the case of intra-band CA.

  - Note: If introduced, the new UE capability should always assume no simultaneous transmission while SRS carrier switching for the bands in the band combinations that are signalled to not support simultaneous transmission within BandCombinationList-UplinkTxSwitch.

The discussion on prioritization rules corresponding to SRS CS, for releases later than Rel-16, was continued in RAN1#108-e, where the following were agreed

Agreement

For SRS carrier switching in Rel-17,

- Introduce prioritization rules for carriers that are in the same band as the source CC for intra-band CA

- Introduce prioritization rules for carriers that are in a different band as the source CC for inter-band CA

  - Introduce a new capability

Working Assumption

A new UE capability is defined as below,

- For each “source-target” pair (as indicated by srs-SwitchingTimesListNR), the UE can indicate which other bands in the band combination are affected by the SRS switch. If this new indication is missing, the UE defaults to Rel-15 behavior.

- If the UE indicates the new list of bands, the dropping rules / timelines apply to the bands indicated by the list (requires update in RAN1 specs).

Note: the new UE capability has no impact on the legacy capability txSwitchImpactToRx and txSwitchWithAnotherBand

In [1], a possible implementation of this new capability is shown. Further optimization of such an implementation, like signaling overhead, can be left to RAN2.

Proposal 1: Confirm the working assumption.

Following the agreements that were made in and before RAN1#108-e, and by confirming the WA, we have the following set of CCs for which SRS CS prioritization rules are defined in Rel-17.

- Source CC

- Carriers intra-band with the source CC

- Carriers in bands within the band combination that are inter-band with source CC and are indicated by UE capability for each pair of source-target, that are impacted by SRS carrier switching to target

Now for the case that some UL transmissions from the set of UL CCs defined above are lower priority than SRS on target CC and some are higher priority than SRS on target CC, SRS CS shall be dropped and no dropping on low priority UL transmissions due to overlap with SRS is expected, since there is no longer a SRS CS case to begin with.

Proposal 2: If SRS transmission on target carrier, including any interruption due to UL/DL RF retuning time, overlaps in time with UL transmissions on UL carriers for which the SRS CS prioritization rules are applied, SRS shall be dropped if there is any UL transmission among the set of carriers that is higher priority than SRS transmission on the target carrier. In this case, none of the UL transmissions among the set of carriers can be defined as low priority (as there is no longer a SRS CS transmission to begin with).

# SRS switching and cancellation timeline across carriers

Dropping an UL transmission is associated with a cancellation timeline which shall be met, otherwise UE cannot cancel the low priority uplink transmission in time. While NR defines cancellation timeline for various scenarios, like intra/inter-UE cancellation for URLLC, cancellation of configured uplink transmission by dynamic SFI or another DCI, etc, current RAN1 specification lacks the proper definition of cancellation timeline when SRS carrier switching is involved. We should note that without specification of cancellation timeline for the case of SRS carrier switching, gNB does not know the latest time that the UE can drop a low priority channel. Therefore, it cannot schedule the high priority transmission accordingly; and in case UE is not given enough time (which is not specified yet) to cancel the low priority channel, UE behavior becomes undefined.

In RAN1#104b-e, a CR was endorsed in [2] that specifies the required timeline to cancel the low priority transmission on a single carrier c, where c is restricted to be either the source carrier, i.e., the “switching from” carrier, or the target carrier, i.e., “switch to” carrier. Given that with SRS carrier switching, in addition to source CC, simultaneous UL transmissions on carriers intra band with the source carrier, and subject to UE capability carriers inter-band with source carrier, may be impacted, the cancellation timeline needs to be extended to include transmissions on such carriers. In this contribution, we extend the timeline specification to the general case, where due to SRS carrier switching, carriers that based on prioritization rules shall be dropped and/or DCIs that schedule high priority transmissions on other carriers, can be more than one.

## Cancellation timeline for UL carriers intra/inter-band with source

In the following, we define a cancellation timeline for scenarios where SRS transmission on the target carrier requires simultaneous low priority UL transmissions on source, and/or carriers intra band with source, and/or (subject to UE capability) carriers inter-band with source, to be dropped. Here, the formulation reuses the basics of UCI multiplexing, given by 38.213, Sec. 9.2.5.

Figure 1: Timeline for cancellation due to higher priority SRS transmission on target carrier.

Proposal 3: For the case that aperiodic SRS transmission on the target cell has higher priority than overlapping UL transmissions on the impacted UL carriers:

- UE does not expect that the gap between the last symbol of DCI indicating A-SRS on target CC and the first symbol of the earliest low priority UL transmission overlapping with A-SRS transmission, to be less than Tprocmax, with Tprocmax=max{Tproc1, …,Tproci, …}

where Tproci is based on N2, which itself is determined based on the UE processing capability on the i-th low priority carrier, and the minimum of (µDL, µith-UL), where the µDL corresponds to the SCS of the PDCCH scheduling A-SRS, and µith-UL corresponds to the SCS of the uplink channel on the i-th low priority carrier.

Note: Based on 38.214, Sec. 6.2.1.3, the overlapping includes any interruption due to UL/DL RF retuning time as defined by higher layer parameters switchingTimeUL and switchingTimeDL of SRS-SwitchingTimeNR

## Cancellation timeline for SRS on target cell

Following the same line of discussion, we can have a scenario where SRS transmission on the target cell is dropped due to high priority scheduling on other carriers (those are source carrier, intra-band carrier with source, and subject to capability signalling inter-band carrier with source). An example is given by Figure 2, where UE is configured to switch from C3 (not shown in the figure) to C0 (C0 is DL carrier) for periodic SRS switching, but UE is also dynamically indicated by DCI_1 and DCI_2 to transmit PUSCH on C1 and C2, and simultaneous transmission on (C0, C1, C2) is beyond UE’s capability. Based on current specification, P-SRS on the target cell shall be dropped. Although this cancellation needs a timeline that shall be met, similar to the cancellation timeline for CG PUSCH in current specification.

Figure 2: Timeline for cancellation of SRS transmission on the target carrier.

Proposal 4: For the case that UE is scheduled by a DCI, or a set of DCIs, to transmit a high priority UL transmission on a serving cell overlapping with a low priority SRS transmission on a carrier without configured PUSCH/PUCCH, and simultaneous transmission is beyond UE’s capability:

- UE does not expect the gap between the first symbol of the earliest low priority SRS transmission on the target cell and a last symbol of the last DCI among all DCIs indicating high priority transmissions on another carriers within the set of UL carriers on which SRS CS prioritization rules are applied, overlapping with SRS transmission on target, to be less than Tprocmax, with Tprocmax=max{Tproc1, …,Tproci, …}

where Tproci is based on SRS-SwitchingTime + N2, and N2 is determined based on the UE processing capability on the target carrier, and the minimum of (µith-DL, µUL), where the µith-DL corresponds to the SCS of the PDCCH scheduling a high priority UL transmission on the i-th high priority carrier, and µUL corresponds to the SCS of the SRS on the target cell. SRS-SwitchingTime represents the UL or DL RF retuning time [11, TS 38.133] as defined by higher layer parameters switchingTimeUL and switchingTimeDL of SRS-SwitchingTimeNR,

## Text proposal for SRS Carrier Switching

Based on what we discussed, the following TP is proposed:

[Start of TP, 38.214]

#### 6.2.1.3	UE sounding procedure between component carriers

For a carrier of a serving cell c1 with slot formats comprised of DL and UL symbols, not configured for PUSCH/PUCCH transmission, denote as c2 the corresponding carrier of a serving cell whose UL transmissions are temporarily suspended as signalled by higher layer parameter srs-SwitchFromServCellIndex and srs-SwitchFromCarrier. Define the set Sc2={c2, s1(c2)…sN-1(c2)} as the set of carriers of serving cells that each carrier meets one of the following conditions:

-               si(c2) is in the same band as c2, or si(c2) is an inter-band CA with c2 and si(c2) is indicated through the capability signalling ImpactedBands-SRS-CS-v17 to be affected by the SRS switch from c2 to c1.

-     si(c2) is in the same TAG as c2.

Where 1≤i≤N-1.

----- unchanged part omitted-----

For an SRS transmission starting in symbol Nc1 of carrier c1 and a conflicting transmission in any carrier c2 within the set Sc2 starting in symbol Nsc2, the UE shall apply the prioritization / dropping rules in the remainder of this clause taking into account:

-       DCI(s) for which the time interval between the last symbol of PDCCH and Nc1 is at least N2 symbols and an additional time duration TSRSCS,  and the time interval between the last symbol of PDCCH and Nsc2 is at least N2 symbols; and

-       semi-persistent CSI reports or SRS considered active at leastN2 symbols and an additional time durationTSRSCS before Nc1, and considered active at least N2 symbols before Nsc2.

Where TSRSCS=max⁡{switchingTimeUL,switchingTimeDL}, and the time interval unit of OFDM symbol is counted based on the smaller subcarrier spacing across any carrier within the set Sc2, c1,  c2 and their corresponding scheduling cells.

The following prioritization rules shall be applied in case of collision between a transmission of SRS over carrier and transmission of a physical signal/channel over a carrier of a serving cell in set Sc2

-     For a carrier of a serving cell with slot formats comprised of DL and UL symbols, not configured for PUSCH/PUCCH transmission, the UE shall not transmit SRS whenever SRS transmission (including any interruption due to uplink or downlink RF retuning time [11, TS 38.133] as defined by higher layer parameters switchingTimeUL and switchingTimeDL of SRS-SwitchingTimeNR) on the carrier of the serving cell c2 and PUSCH/PUCCH transmission carrying HARQ-ACK/positive SR/RI/CRI/SSBRI and/or PRACH on a carrier of a serving cell in set Sc2 happen to overlap in the same symbol and that can result in uplink transmissions beyond the UE’s indicated uplink carrier aggregation capability included in [13, TS 38.306].

-     For a carrier of a serving cell with slot formats comprised of DL and UL symbols, not configured for PUSCH/PUCCH transmission, the UE shall not transmit a periodic/semi-persistent SRS whenever periodic/semi-persistent SRS transmission (including any interruption due to uplink or downlink RF retuning time [11, TS 38.133] as defined by higher layer parameters switchingTimeUL and switchingTimeDL of SRS-SwitchingTimeNR) on the carrier of the serving cell c2 and PUSCH transmission carrying aperiodic CSI on a carrier of a serving cell in set Sc2 happen to overlap in the same symbol and that can result in uplink transmissions beyond the UE’s indicated uplink carrier aggregation capability included in [13, TS 38.306].

-     For a carrier of a serving cell with slot formats comprised of DL and UL symbols, not configured for PUSCH/PUCCH transmission, the UE shall drop PUCCH/PUSCH transmission carrying periodic/semi-persistent CSI comprising only CQI/PMI/L1-RSRP/L1-SINR, and/or SRS transmission on a carrier of a serving cell in set Sc2 another serving cell configured for PUSCH/PUCCH transmission whenever the transmission and SRS transmission (including any interruption due to uplink or downlink RF retuning time [11, TS 38.133] as defined by higher layer parameters switchingTimeUL and switchingTimeDL of SRS-SwitchingTimeNR) on the carrier of the serving cell c2  happen to overlap in the same symbol and that can result in uplink transmissions beyond the UE’s indicated uplink carrier aggregation capability included in [13, TS 38.306].

-     For a carrier of a serving cell with slot formats comprised of DL and UL symbols, not configured for PUSCH/PUCCH transmission, the UE shall drop PUSCH transmission carrying aperiodic CSI comprising only CQI/PMI/L1-RSRP/L1-SINR on a carrier of a serving cell in the set Sc2 whenever the transmission and aperiodic SRS transmission (including any interruption due to uplink or downlink RF retuning time [11, TS 38.133]) as defined by higher layer parameters switchingTimeUL and switchingTimeDL of SRS-SwitchingTimeNR) on the carrier of the serving cell c2  happen to overlap in the same symbol and that can result in uplink transmissions beyond the UE’s indicated uplink carrier aggregation capability included in [13, TS 38.306].

[text unchanged…]

[End of TP, 38.214]

Proposal 5: Endorse the proposed TP for TS 38.214, 6.2.1.3.

# Conclusions

In this contribution, we shared our views on procedure to apply priority rules for SRS carrier switching. Next, we provided a formulation that is aligned with the rest of specification for timeline cancellation. Based on views shared in this paper, we make the following proposals:

Proposal 1: Confirm the working assumption.

Proposal 2: If SRS transmission on target carrier, including any interruption due to UL/DL RF retuning time, overlaps in time with UL transmissions on UL carriers for which the SRS CS prioritization rules are applied, SRS shall be dropped if there is any UL transmission among the set of carriers that is higher priority than SRS transmission on the target carrier. In this case, none of the UL transmissions among the set of carriers can be defined as low priority (as there is no longer a SRS CS transmission to begin with).

Proposal 3: For the case that aperiodic SRS transmission on the target cell has higher priority than overlapping UL transmissions on the impacted UL carriers:

- UE does not expect that the gap between the last symbol of DCI indicating A-SRS on target CC and the first symbol of the earliest low priority UL transmission overlapping with A-SRS transmission, to be less than Tprocmax, with Tprocmax=max{Tproc1, …,Tproci, …}

Proposal 4: For the case that UE is scheduled by a DCI, or a set of DCIs, to transmit a high priority UL transmission on a serving cell overlapping with a low priority SRS transmission on a carrier without configured PUSCH/PUCCH, and simultaneous transmission is beyond UE’s capability:

- UE does not expect the gap between the first symbol of the earliest low priority SRS transmission on the target cell and a last symbol of the last DCI among all DCIs indicating high priority transmissions on another carriers within the set of UL carriers on which SRS CS prioritization rules are applied, overlapping with SRS transmission on target, to be less than Tprocmax, with Tprocmax=max{Tproc1, …,Tproci, …}

Proposal 5: Endorse the proposed TP for TS 38.214, 6.2.1.3.

# References

- R1-2202112, “Discussion on SRS carrier switching”, Qualcomm

- R1-2104043, “Timelines for SRS carrier switching”, Qualcomm, ZTE