3GPP TSG RAN WG1 #109-e	                                         	           R1-2204231

e-Meeting, May 9th – 20th, 2022

Agenda Item:	9.1.2

Source:	Apple Inc.

Title:	Views on Rel-18 MIMO CSI enhancement

Document for:	Discussion/Decision

# Introduction

In the following, we summarize the NR CSI design evolution from Rel-15/16/17.

- In Rel-15, both Type I and Type II codebook were introduced for advanced PMI reporting. The codebook structure relies on W=W1*W2 in which W1 is the spatial bases selection matrix, and W2 is the combination coefficient matrix. Spatial bases are constructed based on the oversampled two-dimensional DFT matrix

  - For Type I codebook, both single panel and multiple panel codebook are supported

  - For Type II codebook, both regular Type II and port selection codebook are supported

- In Rel-16, to reduce Type II codebook CSI overhead, frequency domain compression was introduced for enhanced Type II codebook based on frequency bases. Frequency bases are constructed based on DFT matrix. The codebook structure of the enhanced Type II codebook is W=W1*W2*Wf in which W1 is the spatial bases selection matrix, W2 is the combination coefficient matrix, and Wf is frequency bases selection matrix.

- In Rel-17, Type II port selection is further enhanced by allowing more flexible port selection while restricting the frequency bases selection. In addition, CSI report enhancement was introduced to support Non-Coherent Joint Transmission (NCJT). However, the scope of CSI enhancement for NCJT is only limited to Type I codebook and only covers the Single-DCI Multi-TRP scheme 1a, i.e., SDM scheme

In Rel-18, as part of the approved WID in RP-213598, [1], the following two objectives are considered for CSI enhancement

- Study, and if justified, specify CSI reporting enhancement for high/medium UE velocities by exploiting time-domain correlation/Doppler-domain information to assist DL precoding, targeting FR1, as follows: - Rel-16/17 Type-II codebook refinement, without modification to the spatial and frequency domain basis - UE reporting of time-domain channel properties measured via CSI-RS for tracking - Study, and if justified, specify enhancements of CSI acquisition for Coherent-JT targeting FR1 and up to 4 TRPs, assuming ideal backhaul and synchronization as well as the same number of antenna ports across TRPs, as follows: - Rel-16/17 Type-II codebook refinement for CJT mTRP targeting FDD and its associated CSI reporting, taking into account throughput-overhead trade-off - SRS enhancement to manage inter-TRP cross-SRS interference targeting TDD CJT via SRS capacity enhancement and/or interference randomization, with the constraints that 1) without consuming additional resources for SRS; 2) reuse existing SRS comb structure; 3) without new SRS root sequences - Note: the maximum number of CSI-RS ports per resource remains the same as in Rel-17, i.e., 32

In contribution, we provide our views on the CSI enhancement in the following two areas

- CSI enhancement by exploiting channel time-domain properties

- CSI enhancement for supporting Coherent Joint Transmission (CJT)

# CSI enhancement by exploiting channel time-domain properties

When UE speed is medium/high, the wireless channel can change rapidly and CSI aging becomes a severe problem. Also as wireless channel changes rapidly, the optimal precoder (PMI) and/or CQI for multiple slots with PDSCH transmissions may change. Logically it is even possible the optimal precoder may change within a single slot. To make the enhancement tractable, it seems reasonable to assume that a single rank indication is applied to multiple slots.

According to the WID shown above, the spatial domain basis and frequency domain basis are not modified in Rel-18

for CSI enhancements exploiting time-domain correlation. In our understanding, basically bases other than DFT bases, such as sine/cosine bases, are not to be discussed for spatial and frequency domain bases. Yet the use of spatial beam bases and frequency domain bases are not necessarily exactly the same as in Rel-15/16/17. It would be good to achieve a common understanding on that among companies at the beginning of the WI to avoid future controversy.

We have

Observation: The WID prescribes that only DFT bases are used for spatial beam bases and FD bases, yet the WID does not prescribe whether the same spatial beam bases and FD bases selection as in Rel-17 is used.

It is customary to agree the evaluation methodology at the beginning of an MIMO WI. For this particular objective, it is important to first agree on the design target. As wireless channel profile, UE speed and carrier frequency affect time domaining correlation, and consequently they affect the performance on various proposed schemes, we need to agree on the deployment scenario and channel model first. In our view, UMa can be a suitable choice as delay spread is large enough so characterization of frequency selectivity in wireless channel is necessary in CSI feedback.  Note for UE to estimate/predict channels, then clustered CSI-RS resources are needed.

Since FR1 is targeted, a carrier frequency at 2.1 GHz, 3.5 GHz, etc. can be selected. As both Rel-16 and Rel-17 design are referred in WID, then one TDD band carrier frequency and one FDD band carrier frequency can be chosen. For TDD, DDDSU can be considered for DL/UL split.

As for UE speed, there are at least two considerations for its choice. In the first consideration, the UE speed should be high enough so the legacy design from Rel-16/Rel-17 does not perform well, and the Rel-18 design can fill a gap so MU-MIMO pairing at that speed performs well. In the second consideration, as wireless channel models in RAN1 design are just abstraction of actual wireless channels, the wireless channel may change substantially when a UE moves over a long distance in a short period of time and the validity of a channel realization may become an issue. In theory, spatial consistence can be used to deal with that, so the change in the wireless channel’s parameters is modelled. However, modeling & exploiting two or more wireless channel realizations may require too much effort than can be handled in Rel-18.

We have

Proposal 1.1, for evaluation methodology on time-domain correlation:

- UMa is selected as the channel model

- Both TDD band and FDD band are considered, one carrier frequency is selected for each. And for TDD, DDDSU is used for DL/UL split.

- Select a UE speed large enough so Rel-18 design will provide performance improvement over legacy design which would have degraded performance at such speed, and small enough so channel validity is assured.

Before starting on the detailed design, a first question needs answer is how predictable the channel really is. Since Type II CSI feedback already involves substantial feedback overhead, adding the time correlation aspect can make it worse, then whether subband feedback should be used or wideband feedback with much lower feedback overhead should be used should be studied.

Assume channels are predicable, the channel model provided in TR 38.901 can be used to guide the design exploiting time-domain correlation. It can be seen in previous releases, the time-domain variation is not explicitly considered in PMI and CQI feedback (though there were proposals to define the validity time of a CSI report). In the channel model, the Doppler frequency can be different for ray m in cluster n. Similar to the treatment of frequency selection, for which DFT beams are used in Rel-16 Type II codebook to reduce CSI feedback overhead, DFT beams can be used for the treatment of time selection so CSI feedback overhead for multiple applicable PDSCH occasions can be reduced.

Excerpt from TR 38.901

Step 13: Generate channel coefficients for each cluster n and each receiver and transmitter element pair u, s.

In case of NLOS, the channel coefficients of ray m in cluster n for a link between Rx antenna u and Tx antenna s at time t in k-th frequency bin can be calculated as

(8.4-27)

where Frx,u,θ and Frx,u,ϕ are the receive antenna element u field patterns in the direction of the spherical basis vectors,  and  respectively, Ftx,s,θ and Ftx,s,ϕ are the transmit antenna element s field patterns in the direction of the spherical basis vectors,  and  respectively. The delay (TOA) for ray m in cluster n for a link between Rx antenna u and Tx antenna s is given by:

(8.4-28)

For the m-th ray within n-th cluster,  is the spherical unit vector with azimuth arrival angle  and elevation arrival angle , given by

(8.4-29)

is the spherical unit vector with azimuth departure angle  and elevation departure angle , given by

(8.4-30)

Also, is the location vector of receive antenna element u and is the location vector of transmit antenna element s, n,m is the cross polarisation power ratio in linear scale. If polarisation is not considered, the 2x2 polarisation matrix can be replaced by the scalar  and only vertically polarised field patterns are applied.

The Doppler frequency component is calculated from the arrival angles (AOA, ZOA), and the UT velocity vector  with speed v, travel azimuth angle ϕv, elevation angle θv and is given by

,	(8.4-31)

Then with Rel-16, two DFT bases (spatial beam bases and FD bases) are used as shown on the top of the figure below.

In Rel-18, three DFT bases (spatial beam bases, FD bases, time-domain bases) as shown on the bottom of the figure below can be used. Then instead of feeding back linear combination coefficients on a single plane (2D matrix) as in Rel-16, linear combination (LC) coefficients on a 3D matrix are fed back to network,  or a single plane/”page” (without considering Doppler frequency) is split into multiple planes/”pages”, each plane/”page” corresponding to a Doppler frequency.

We have

Proposal 1.2: Consider DFT bases for representing a time-varying Type II codeword.

Subject to RAN1’s decision on deployment scenarios, UE speed and carrier frequency, the “pages” may be densely or sparsely populated with non-zero LC coefficients. UE’s measurement over TRS can reveal the Doppler frequencies seen by the UE, and the UE feedback can help network to select a suitable configuration for CSI reporting. It is noted the assistance information provided by the UE from the measurement over TRS is separate from the CSI reporting on time-varying Type II precoder and/or CQI, and they can take place at different time scales at network’s discretion. For example, the assistance information reporting can be sent at a slower pace than that for time-varying precoder and/or CQI.

We have

Proposal 1.3: UE can provide CSI report setting configuration assistance information from measurement over TRS.

Figure 1 From Rel-16 design (top) to Rel-18 design (bottom)

If the “pages” are sparsely populated with non-zero LC coefficients, and each “page” may have different top spatial beam bases and/or top FD bases, then selection of spatial beam bases, FD component bases and TD component bases need careful design to ensure the sparseness of non-zero LC coefficients is not un-exploited in CSI feedback design.

On the other hand, if the top spatial beam bases remain common among “pages” and all the “pages” are densely populated, the 3D representation of time-varying precoders may not offer any advantage over multiple 2D representations for time-invariant precoders. Then there can be a simpler design where the W1/W2 structure in Rel-16 is kept, and W1 is shared for multiple CSI reports (or W1 can be included in the first CSI report), and for each of the multiple CSI reports targeting different PDSCH slots, W2 is reported.

It is also possible to define a CSI feedback scheme that the same W1 is shared for multiple slots with PDSCH transmissions, only W2 for each such slot is fed back. While Rel-16/Rel-17 design is still used as the baseline to perform throughput/feedback overhead evaluation on candidate schemes for CSI feedback, candidate schemes need to show better throughput/feedback overhead than such a modification to Rel-16 design.

We have

Proposal 1.4: If the planes for Doppler frequency are densely populated with non-zero linear combination coefficients and top spatial beam bases are common for those planes, consider different feedback schedules for W1/W2. Consider such a scheme in the tradeoff of throughput/feedback overhead.

# CSI enhancement for supporting Coherent Joint Transmission (CJT)

Figure below illustrates one example of the Coherent Joint Transmission (CJT) from 4 TRPs. Each TRP is configured with 2x2 linear antenna array in which there are 2 V-Pol and 2 H-Pol antenna elements per vertical and per horizontal direction.

Figure 2 Illustration of Coherent Joint Transmission (CJT) from 4 TRP

According to the WID objective, we should assume “ideal backhaul and synchronization” among TRPs participating in the CJT. With this strong assumption, in our understanding, the goal is to design a Multi-Panel Type II codebook similar as Multi-Panel Type I codebook.

Firstly, to design CSI enhancement for CJT, we need to have a common codebook structure. In our view, a reasonable codebook structure can be

W=C0W0C1W1⋮CT-1WT-1

In which

- T is the total number of TRPs in CJT

- Wt, t=0,1,…,T-1 is the PMI for TRP t.

  - Wt can be reported based on the current Type II codebook, i.e., Wt=W1t*W2t*Wft,

- Ct, t=0,1,…,T-1 is the linear weighting coefficient for TRP t.

Proposal 2.1, for CSI enhancement for CJT, we can consider codebook structure

W=C0W0C1W1⋮CT-1WT-1

In which

- T is the total number of TRPs in CJT

- Wt, t=0,1,…,T-1 is the PMI for TRP t.

  - Wt can be reported based on the current Type II codebook, i.e., Wt=W1t*W2t*Wft,

- Ct, t=0,1,…,T-1 is the linear weighting coefficient for TRP t.

Once the codebook structure can be agreed, during the rest of Rel-18 discussion, we can consider the following design details

- For Wt, which component(s) in  Wt can be common for all TRPs, and which component(s) in  Wt can be independent for each TRP

- For Ct, the resolution of Ct in terms of

  - Whether Ct is per layer, or, per TRP, etc.

  - Whether Ct contains only phase or contains both phase and amplitude

- Whether and how to support dynamic TRP selection

- CMR configuration, i.e., how to logically associate different CSI-RS ports/resources/resource sets to different TRP

- IMR configuration including ZP IMR and NZP IMR

- CBSR restriction, and rank restriction configuration

- UCI/CSI assembly and omission design

# Conclusion

In contribution, we provide our views on the CSI enhancement in the following two areas for Rel-18 MIMO evolution

- CSI enhancement by exploiting channel time-domain properties

- CSI enhancement for supporting Coherent Joint Transmission (CJT)

Regarding CSI enhancement by exploiting channel time-domain properties, we have the following proposals

Observation: The WID prescribes that only DFT bases are used for spatial beam bases and FD bases, yet the WID does not prescribe whether the same spatial beam bases and FD bases selection as in Rel-17 is used.

Proposal 1.1, for evaluation methodology on time-domain correlation:

- UMa is selected as the channel model

- Both TDD band and FDD band are considered, one carrier frequency is selected for each. And for TDD, DDDSU is used for DL/UL split.

- Select a UE speed large enough so Rel-18 design will provide performance improvement over legacy design which would have degraded performance at such speed, and small enough so channel validity is assured.

Proposal 1.2: Consider DFT bases for representing a time-varying Type II codeword.

Proposal 1.3: UE can provide CSI report setting configuration assistance information from measurement over TRS.

Proposal 1.4: If the planes for Doppler frequency are densely populated with non-zero linear combination coefficients and top spatial beam bases are common for those planes, consider different feedback schedules for W1/W2. Consider such a scheme in the tradeoff of throughput/feedback overhead.

Regarding CSI enhancement for supporting Coherent Joint Transmission (CJT), we have the following proposals

Proposal 2.1, for CSI enhancement for CJT, we can consider codebook structure

W=C0W0C1W1⋮CT-1WT-1

In which

- T is the total number of TRPs in CJT

- Wt, t=0,1,…,T-1 is the PMI for TRP t.

  - Wt can be reported based on the current Type II codebook, i.e., Wt=W1t*W2t*Wft,

- Ct, t=0,1,…,T-1 is the linear weighting coefficient for TRP t.

# Reference

[1] RP-213598, New WID: MIMO Evolution for Downlink and Uplink, 3GPP TSG RAN Meeting #94e, Electronic Meeting, Dec. 6 - 17, 2021