3GPP TSG RAN WG1 #109-e	                                         	           R1-2204232

e-Meeting, May 9th – 20th, 2022

Agenda Item:	9.1.3.1

Source:	Apple Inc.

Title:	Views on supporting increased number of orthogonal DMRS ports

Document for:	Discussion/Decision

# Introduction

In current NR, for CP-OFDM waveform, we have symmetric DMRS design for both PDSCH and PUSCH. We support two types of DMRS, DMRS configuration type 1 and DMRS configuration type 2

- To support up to 12 ports DMRS, the basic design unit of DMRS is CDM group

  - In frequency domain, each CDM group employs 2 REs. With FD-OCC 2, it can create two orthogonal ports in the frequency domain

  - In time domain, each CDM group employ either 1 or 2 symbols. When CDM group employs two symbols, TD-OCC 2 is used to create two orthogonal ports in the time domain

  - As results

    - With 1 symbol, each CDM group can support 2 orthogonal ports

    - With 2 symbols, each CDM group can support 4 orthogonal ports

- For DMRS configuration type 1, it contains two CDM groups

  - Support up to 4 ports for single symbol DMRS

  - Support up to 8 ports for two symbol DMRS

- For DMRS configuration type 2, it contains three CDM groups

  - Support up to 6 ports for single symbol DMRS

  - Support up to 12 ports for two symbol DMRS

Figure below illustrates the CDM group arrangement for both DMRS type 1 and DMRS type 2.

Figure 1 the CDM group arrangement for both DMRS type 1 and DMRS type 2

In Rel-18, as part of the approved WID in RP-213598, [1], the following two objectives are considered for CSI enhancement

- Study, and if justified, specify larger number of orthogonal DMRS ports for downlink and uplink MU-MIMO (without increasing the DM-RS overhead), only for CP-OFDM, - Striving for a common design between DL and UL DMRS - Up to 24 orthogonal DM-RS ports, where for each applicable DMRS type, the maximum number of orthogonal ports is doubled for both single- and double-symbol DMRS

In contribution, we provide our views on how to increase (double) the maximum number of DMRS ports for both PDSCH and PUSCH for CP-OFDM waveform

# General view

In our view, when designing the new DMRS pattern to support increased number of DMRS ports for CP-OFDM, we need to consider as the following three issues

- We should keep the new DMRS design also symmetric between PDSCH and PUSCH for CP-OFDM

- We should design the new DMRS to be backward compatible as much as possible, i.e., to facilitate the MU-MIMO co-scheduling of both the legacy UE that does not support the new DMRS and the Rel-18 UE that supports the new DMRS in the same symbol

- We should reuse the antenna port indication table in 38.212 as much as possible for both PDSCH and PUSCH

Proposal 1, for DMRS enhancement to support increased (doubled) number of DMRS ports for CP-OFDM

- We should keep the new DMRS design symmetric between PDSCH and PUSCH for CP-OFDM

- We should design the new DMRS to be backward compatible as much as possible, i.e., to facilitate the MU-MIMO co-scheduling of both the legacy UE that does not support the new DMRS and the Rel-18 UE that supports the new DMRS in the same symbol

- We should reuse the antenna port indication table in 38.212 as much as possible or both PDSCH and PUSCH

It is also important to note that, the current WID objective limits the DMRS enhancement “without increasing the DM-RS overhead”.  With the above design consideration and the WID objective restriction in mind, in our view, there are three different directions to increase the number of DMRS ports

- Double the number of CDM groups, the same number of ports per CDM group

  - Frequency domain sub-sampling

  - Time domain sub-sampling

- The same number of CDM groups, double the number of ports per CDM group

In the following sections, we provide our view regarding the above direction for DMRS enhancement

# Double the number of CDM groups, the same number of ports per CDM group

One way to double the number of DMRS ports without increasing DMRS overhead is to increase the number of DRMS groups but keep the same number of DMRS ports supportable by each CDM group. This solution effectively reduces the density of each CDM group by 50%, i.e., each CDM group only occupies 50% amount of REs compared to the legacy design. This design can be achieved by either performing frequency domain subsampling or time domain subsampling

In the following two figures, we illustrate both the frequency domain subsampling and time domain subsampling. There are two things worth further note

- For frequency domain subsampling, what we illustrated is the PRB level subsampling. There are other solutions such as CDM group level subsampling which might be friendlier and have less impact on UE channel estimate implementation

- For time domain subsampling, it relies on additional DMRS location in the time domain which, to certain extend, can be argued to violate the “without increasing the DM-RS overhead” restriction. Furthermore, it may impact the PDSCH/PUSCH demodulation pipeline since the full channel estimate available later. Therefore, in our view, time domain subsampling is not a preferred solution.

Figure 2 Frequency domain subsampling to double the number of CDM groups with the same number of ports per CDM group

Figure 3 Time domain subsampling to double the number of CDM groups with the same number of ports per CDM group

# The same number of CDM groups, double the number of ports per CDM group

Another solution to increase the number of DMRS ports is to double the number of ports supported per CDM group while keep the same number of CDM groups. To achieve this, in the frequency domain, each CDM group can occupy 4 REs and FD-OCC 4 can be used to double the number of DMRS ports.

Figure below illustrates the two possible solutions to double the number of ports per CDM group while keep the same number of CDM groups

- Option 1 basically combines to two adjacent CDM groups with the same index into a CDM group with twice the size to support twice the number of ports

  - This solution is friendlier for backward compatibility, but performance may suffer for channel with very high frequency selectivity due to the larger frequency domain footprint

- Option 2 tries to reduce the frequency domain footprint of the CDM group by doubling the number of REs consecutively in frequency domain

  - This solution is not friendly for backward compatibility, but performance can be more robust for channel with very high frequency selectivity due to the smaller frequency domain footprint

Figure 4 two design options to double the number of ports per CDM group while keep the same number of CDM groups

Overall, comparing the three possible solutions listed above, we prefer to use frequency domain subsampling to double the number of DMRS ports for CP-OFDM. This solution keeps the same number of DMRS ports supported per CDM group but double the amount of CDM groups

Proposal 2, to increase the number of orthogonal DMRS ports, we can consider frequency domain subsampling to double the number of DMRS ports for CP-OFDM.

- This solution keeps the same number of DMRS ports supported per CDM group but double the amount of CDM groups

# Conclusion

In contribution, we provide our views on how to increase (double) the maximum number of DMRS ports for PDSCH/PUSCH for CP-OFDM waveform. We have the following proposals

Proposal 1, for DMRS enhancement to support increased (doubled) number of DMRS ports for CP-OFDM

- We should keep the new DMRS design symmetric between PDSCH and PUSCH for CP-OFDM

- We should design the new DMRS to be backward compatible as much as possible, i.e., to facilitate the MU-MIMO co-scheduling of both the legacy UE that does not support the new DMRS and the Rel-18 UE that supports the new DMRS in the same symbol

- We should reuse the antenna port indication table in 38.212 as much as possible or both PDSCH and PUSCH

Proposal 2, to increase the number of orthogonal DMRS ports, we can consider frequency domain subsampling to double the number of DMRS ports for CP-OFDM.

- This solution keeps the same number of DMRS ports supported per CDM group but double the amount of CDM groups

# Reference

[1] RP-213598, New WID: MIMO Evolution for Downlink and Uplink, 3GPP TSG RAN Meeting #94e, Electronic Meeting, Dec. 6 - 17, 2021