3GPP TSG- RAN WG1 Meeting #109-e                                                  R1-2204234

e-Meeting, May 9th – 20th, 2022

Agenda Item:	9.1.4.1

Source:	Apple Inc.

Title:	Views on UL precoding indication for multi-panel simultaneous PUSCH transmissions

Document for:	Discussion/Decision

# Introduction

In Rel-16, multi-TRP based simultaneous PDSCH transmissions using a single DCI was specified for both FDM and SDM PDSCH transmissions. While Rel-17 supports multi-TRP based PUSCH, where different repetitions in time may be associated with different beams, simultaneous PUSCH transmission across different panels is one of the objectives to be studied in R18. In this contribution, we provide our views on multi-panel simultaneous PUSCH transmissions, including resource indication, TB calculation, PUSCH mapping, antenna port indication and precoding indication.

# Simultaneous multi-beam PUSCH transmissions

## Indication of PUSCH Resources by a s-DCI

Multi-TRP based simultaneous PDSCH transmissions using a single DCI is achieved in Rel-16 by indicating the PDSCH resources associated to each TCI state (i.e., each beam). More precisely,

- For FDM simultaneous PDSCH transmissions through multi-TRP and by a single DCI indication, a UE is configured by higher layer parameter repetitionScheme set to one of 'fdmSchemeA', or 'fdmSchemeB', and the UE is indicated a TCI codepoint associated with two TCI states, and the indicated DM-RS port(s) are within one CDM group. FDRA bit-field indicates frequency domain resources that will be eventually split between the two TCI states, as part of PRB bundling and depending on the precoding granularity size, such that PDSCH occasions associated to each TCI state has non-overlapping frequency domain resource allocation with respect to the other PDSCH transmission occasion, as described in TS 38.214, Clause 5.1.2.3

- For SDM simultaneous PDSCH transmissions through multi-TRP and by a single DCI indication, the UE is indicated a TCI codepoint associated with two TCI states, but the indicated DM-RS ports are within two CDM groups, where the first TCI state corresponds to the CDM group of the first antenna port, and the second TCI state corresponds to the other CDM group (TS 38.214, Clause 5.1.6.2). Here, FDRA bitfield indicates a PDSCH transmission occasion that is used by both TRPs, although different layers of that single TB are transmitted by different TRPs.

For multi-panel simultaneous PUSCH transmissions, a similar design as in Rel-16 PDSCH resource indication can be adopted, although given some differences between uplink and downlink resource indication, like lack of PRG bundling in uplink, some modifications should be considered. For example, for multi-panel simultaneous PUSCH transmission using a single DCI, resource allocation type 1 is prioritized. Next, for FDM PUSCH transmissions, the first NPRB2 PRBs are assigned to the first PUSCH and the remaining NPRB2 PRBs are assigned to the second PUSCH, where NPRB is the total number of allocated contiguous PRBs as given by FDRA.

Proposal 1: For FDM simultaneous PUSCH transmission, FDRA bit-field indicates NPRB allocated RBs, where ceil (NPRB/2), or alternatively floor (NPRB/2), is assigned to the first PUSCH and the rest of RBs belong to the second PUSCH.

Proposal 2: For simultaneous multi-panel PUSCH transmissions, partial overlap in frequency shall be deprioritized.

Proposal 3: For SDM simultaneous multi-panel PUSCH transmissions, the first (second) PUSCH is the PUSCH associated to the first (second) UL TCI state.

Figure 1: Examples for different resource allocations to simultaneous PUSCHs

## TB Determination

In Rel-16, multi-TRP based simultaneous PDSCH transmissions using a single DCI supports transmission of a single TB, with and without repetition. More precisely,

- For sDCI mTRP operation and SDM indication, a single TB size is determined similar to R15 sTRP PDSCH transmission, based on number of allocated REs, and MCS given by sDCI, although for SDM PDSCH transmissions, number of layers to determine the TB is the total number of layers across both TRPs. Simultaneous repetitions of the same TB are not supported for SDM.

- For sDCI mTRP operation and FDM indication, when a UE is configured by higher layer parameter repetitionScheme set to 'fdmSchemeA', same number of layers applied to PDSCH transmissions associated to each TCI, so TB is again determined similar to R15 single TRP PDSCH transmission, based on total REs associated to both TCIs, and MCS given by sDCI. Simultaneous repetitions of the same TB are not supported for 'fdmSchemeA'.

- For sDCI mTRP operation and FDM indication, when a UE is configured by higher layer parameter repetitionScheme set to 'fdmSchemeB', single TB is determined based on R15 procedure, but only REs associated to the first TCI state, i.e., the first PDSCH, are considered. The second PDSCH associated to the second TCI will carry a repetition of the same TB, with the same MCS and number of layers as of the first PDSCH, as indicated by DCI.

In Rel-17, multi-TRP based PUSCH was supported, where different repetitions in time are associated with different beams. A single TB is determined, based on the first PUSCH occasion.

To determine the TB size for multi-panel simultaneous PUSCH transmissions, a similar design as in Rel-16 simultaneous PDSCH transmission can be adopted, although some additional considerations should be taken into account. For example, whether or not to support two codewords each carried by one PUSCH. In addition, given that simultaneous transmission of different repetitions of the same TB will increase UE complexity, repetition-based scheme is deprioritized, or at least a combination of R17 repetition in addition to repetition in frequency/space is deprioritized (like R16 multi-TRP based PDSCH transmissions where joint configuration of different repetition types in time and frequency is not supported).

Proposal 4: For multi-panel simultaneous PUSCH transmissions indicated by a single DCI, for the case of single TB transmission, support a common MCS and the same number of layers for PUSCH transmissions associated to different TCIs.

Proposal 5: For multi-panel simultaneous PUSCH transmissions indicated by a single DCI, UE transmits a single TB which is calculated based on all REs associated to both TCIs, assuming a common MCS and the same number of layers for PUSCHs associated to each TCI.

- REs that are overlapped in frequency, they are counted once.

## PUSCH Mapping to REs

If UE is indicated to transmit a single TB through simultaneous PUSCH transmissions over multi-panel, it is preferred to have a single MCS bit-field and number of layers applicable to both PUSCHs. In this case, current specification can be applied, with possibly minor modifications for the SDM allocation, to determine the TB size, as discussed in the previous sub-section. For mapping of the coded symbols to the REs, different options can be discussed, as follows

- Option 1: start with the first allocated symbol, the coded symbols are mapped in frequency first to all resource elements (REs) associated to the 1st PUSCH and next to REs associated to the 2nd PUSCH. Next move to the next symbol.

- Option 2: start with 1st PUSCH, coded bits are mapped in all frequency and time REs associated to 1st PUSCH (frequency next time). Next, move to 2nd PUSCH.

- Option 3: For FDM, start with the first allocated symbol, the coded symbols are mapped to all REs (belong to 1st and 2nd PUSCH) in frequency, next to the next symbol and so on (similar to existing PUSCH mapping procedure).

Proposal 6: RAN1 should study different PUSCH mapping schemes in terms of UE complexity and/or PUSCH reliability.

## UL Beam and Precoding Indication

In Rel-17, when UE is indicated to perform PUSCH repetition in time with different UL beams associated to each repetition, SRI bit-field is extended to indicate beam association to each repetition. Rel-17 also specifies unified TCI framework, based on which when the UE is configured DLorJointTCIState or UL-TCIState, the UE shall perform PUSCH transmission according to the spatial relation, if applicable, with a reference to the RS for determining UL Tx spatial filter or the RS configured with qcl-Type set to 'typeD' of the indicated DLorJointTCIState or UL-TCIState (TS 38.214, Clause 6.1).

In Rel-18, unified TCI framework is assumed to be extended for UL beam indication for multi-panel simultaneous PUSCH transmissions. In our view, for multi-panel PUSCH beam indication, a combination of Rel-17 procedure and unified TCI framework can be envisioned as follows:

- UE is configured with two SRS resource sets both with the usage of value 'nonCodeBook', when ‘txConfig = nonCodeBook’, or two SRS resource sets with usage of value 'CodeBook', when ‘txConfig = CodeBook’, and

- SRS resource(s) in each SRS resource set is configured with DLorJointTCIState or UL-TCIState.

Based on the above discussion, we have the following proposal:

Proposal 7: For s-DCI, at least for FDM resource allocation of multi-panel simultaneous PUSCH transmissions, adopt R17 procedure to indicate UL beam where each SRS resource set is associated with each indicated joint or uplink TCI for PUSCH beam indication.

- If only 1 TCI is indicated, UE does not expect SRS resource set indicator bit-field to indicate codepoint 10, or 11.

Here we should note that simple extension of Rel-17 beam indication procedure has some drawbacks: In R17, PUSCHs on separate beams are not simultaneous but in different time resources, so Rel-17 based indication does not capture inter-panel interference for SDM (although OK for FDM). In addition, in R17, PUSCHs on different beams are simply repetitions in time which have same number of layers. While for simultaneous transmission of a single TB through multi-beams we do not support per-panel MCS/number of layers, R-17 indication needs modification if different number of layers per panel is needed, for example for two codeword case.

Proposal 8: For s-DCI, at least for two codewords multi-panel simultaneous PUSCH transmissions:

- For ‘txConfig = nonCodebook’: the second SRI bit field assumes all possible ranks that can be indicated by the corresponding SRS resource set, without dependency on the indicated rank by the first SRI.

- For ‘txConfig = Codebook’: the second TPMI bit-field indicates all possible number of layers for the corresponding beam/panel

Note that Proposal 8 may increase DCI size by up to 1 bit for ‘txConfig = nonCodeBook’. For ‘txConfig = CodeBook’, this may increase DCI size by up to 1 bit in most cases and 2 bits only for the special case of “4 antenna ports, disabled transform precoder, maxRank = 3 or 4, ul-FullPowerTransmission = fullpowerMode1, and codebookSubset =partialAndNonCoherent”. To reduce DCI size:

Proposal 9: UE indicates a maximum number of layers per panel for MP simultaneous PUSCHs

- The maximum rank for each panel, e.g. maximum rank for PUSCH associated with each SRS resource set, can be configured by RRC signalling. For example: (1,1), (2,1), (1,2), (2,2)

- Note: In (a,b) indication, neither a nor b can be larger than Lmax

## UL Antenna Port Indication

For s-DCI indication of FDM multi-panel simultaneous PUSCH transmissions, if different number of layers per panel is not allowed, then a single “Antenna ports” bit-field indicates DMRS ports for both PUSCHs. In this case, no new specification is needed.  For the case that different number of layers per panel is allowed (for example for two codewords scenario), we have the following proposal to indicate DMRS ports for PUSCHs:

Proposal 10: For s-DCI FDM simultaneous PUSCHs, “Antenna ports” bit-field indicates DMRS ports for the PUSCH with maximum number of layers

- Alt.1: DMRS ports for the panel with a smaller number of layers is determined from the smallest port indices within the set of ports associated to the panel with a greater number of layers

- Alt.2: reserved bits in DMRS tables for rank>1 is used to indicate the associated port index to the panel with smaller number of layers, based on modified tables in Fig. 2a and Fig. 2b.

As an example for Alt.1, consider the case that PUSCH associated to 1st panel has 1 layer, and PUSCH associated to 2nd panel has 2 layers. “Antenna ports” bit-field in DCI indicates DMRS ports 0,1 for the 2nd panel. Therefore, DMRS port for the 1st panel is 0. As an example for Alt.2, consider same example as for Alt.1, so we should look at modified Table 7.3.1.1.2-9, TS 38.212, as shown in Fig. 2a. Now if the value of “antenna port” bit-field is 1, the 2nd panel has DMRS ports 0,1 and 1st panel has DMRS port 0. On the other hand, if the value of “antenna port” bit-field is 4, the 2nd panel has DMRS ports 0,1 and 1st panel has DMRS port 0.

Also note that under Alt.2, when one of the PUSCHs is rank4, DMRS port indication to the other PUSCH under all possible ranks, i.e., 1-4, requires to extend the legacy Table 7.3.1.1.2-11 from 3 bits to 4 bits (as shown in Fig. 2b). Alternatively, some rows may be excluded, e.g., keep only the first 8 rows.

Figure 2a: DMRS port indication for both PUSCHs using “reserved” states for rank 2 and 3

Figure 2b: DMRS port indication for both PUSCHs using “reserved” states for rank 4

# Conclusion

In this contribution, we provided some views on multi-panel simultaneous PUSCH transmissions. Based on what we discussed, the following proposals are made:

Proposal 1: For FDM simultaneous PUSCH transmission, FDRA bit-field indicates NPRB allocated RBs, where ceil (NPRB/2), or alternatively floor (NPRB/2), is assigned to the first PUSCH and the rest of RBs belong to the second PUSCH.

Proposal 2: For simultaneous multi-panel PUSCH transmissions, partial overlap in frequency shall be deprioritized.

Proposal 3: For SDM simultaneous multi-panel PUSCH transmissions, the first (second) PUSCH is the PUSCH associated to the first (second) UL TCI state.

Proposal 4: For multi-panel simultaneous PUSCH transmissions indicated by a single DCI, for the case of single TB transmission, support a common MCS and the same number of layers for PUSCH transmissions associated to different TCIs.

Proposal 5: For multi-panel simultaneous PUSCH transmissions indicated by a single DCI, UE transmits a single TB which is calculated based on all REs associated to both TCIs, assuming a common MCS and the same number of layers for PUSCHs associated to each TCI.

- REs that are overlapped in frequency, they are counted once.

Proposal 6: RAN1 should study different PUSCH mapping schemes in terms of UE complexity and/or PUSCH reliability.

Proposal 7: For s-DCI, at least for FDM resource allocation of multi-panel simultaneous PUSCH transmissions, adopt R17 procedure to indicate UL beam where each SRS resource set is associated with each indicated joint or uplink TCI for PUSCH beam indication.

- If only 1 TCI is indicated, UE does not expect SRS resource set indicator bit-field to indicate codepoint 10, or 11.

Proposal 8: For s-DCI, at least for two codewords multi-panel simultaneous PUSCH transmissions:

- For ‘txConfig = nonCodeBook’: the second SRI bit field assumes all possible ranks that can be indicated by the corresponding SRS resource set, without dependency on the indicated rank by the first SRI.

- For ‘txConfig = CodeBook’: the second TPMI bit-field indicates all possible number of layers for the corresponding beam/panel

Proposal 9: UE indicates a maximum number of layers per panel for MP simultaneous PUSCHs

- The maximum rank for each panel, e.g. maximum rank for PUSCH associated with each SRS resource set, can be configured by RRC signalling. For example: (1,1), (2,1), (1,2), (2,2)

- Note: In (a,b) indication, neither a nor b can be larger than Lmax

Proposal 10: For s-DCI FDM simultaneous PUSCHs, “Antenna ports” bit-field indicates DMRS ports for the PUSCH with maximum number of layers

- Alt.1: DMRS ports for the panel with a smaller number of layers is determined from the smallest port indices within the set of ports associated to the panel with a greater number of layers

- Alt.2: reserved bits in DMRS tables for rank>1 is used to indicate the associated port index to the panel with smaller number of layers, based on modified tables in Fig. 2a and Fig. 2b.

# References

- R1-2204229, “Views on unified TCI framework for multi-TRP”, Apple Inc., May. 2022