3GPP TSG- RAN WG1 Meeting #109-e                                                 R1-2204238

e-Meeting, May. 9th – May. 20th, 2022

Agenda Item:	9.2.2.1

Source:	Apple Inc.

Title:	Evaluation for AI/ML based CSI feedback enhancement

Document for:	Discussion/Decision

# Introduction

AI/ML for physical layer has gained tremendous interest in academic research in recent years. The first 3GPP SI will study the use of AI/ML technology in air interface design, through three carefully selected use cases [1]. New evaluation methodology and corresponding evaluation are required to fully understand the benefit of AI/ML in comparison with traditional methods, and the associated potential specification.

Use cases to focus on:

- Initial set of use cases includes:

  - CSI feedback enhancement, e.g., overhead reduction, improved accuracy, prediction [RAN1]

  - Beam management, e.g., beam prediction in time, and/or spatial domain for overhead and latency reduction, beam selection accuracy improvement [RAN1]

  - Positioning accuracy enhancements for different scenarios including, e.g., those with heavy NLOS conditions [RAN1]

- Finalize representative sub use cases for each use case for characterization and baseline performance evaluations by RAN#98

  - The AI/ML approaches for the selected sub use cases need to be diverse enough to support various requirements on the gNB-UE collaboration levels

In this paper, we focus on the evaluation of CSI feedback enhancement use case.

# Evaluation methodology

## Auto-encoder/decoder based CSI feedback

Auto-encoder/decoder based CSI feedback enhancement have attracted a lot of research interest in academic and industry. The general structure is shown in Fig. 1. There are two main categories of the auto-encoder/auto-decoder design.

Fig. 1 General structure of auto-encoder/decoder based CSI feedback

The first main research direction explores spatial and frequency domain correlation.  The first paper applying neutral network to CSI feedback was presented in [3]. The main concept is to treat a channel matrix as an image, by considering the real and imagine part of the complex channel matrix as two channels of the image. Auto-encoder is used to compress the channel matrix in frequency and spatial domain into a vector, and decoder is used to reconstruct the channel matrix from the compressed feedback. CNN (Convolutional Neural Network) based neutral network (NN) are typically used. Domain knowledge is used to pre-process the input in frequency domain and angular domain to reduce the dimension before input to the neutral network. A set of research work further improve the accuracy, or reduce the complexity are developed [6,7].

The second main research direction explores time, spatial and frequency domain correlation, as shown in [4,5]. Higher compression ratio can be achieved, where combined CNN and LSTM are used.

The traditional codebook design in 3GPP explore the frequency and spatial domain correlation. In fact, the pre-processing techniques used in [3] has similar operation as R16 type II codebook design. In order to enable fair comparison between AI based approach and traditional design, R16 type II codebook can be used as the performance baseline.

Proposal 1:    R16 type II codebook’s performance can be used as the baseline for AI based CSI feedback.

For input to the neutral network, both eigen vector input and channel state estimation can be used. Using eigen vector as input enjoys much lower feedback overhead in low rank case, enable straightforward comparison with type II codebook design.  However, to calculate the per-subband eigen vector as the NN input itself is a complex task. Using channel state estimation from CSI-RS directly as the input to NN is simpler, but comparison with traditional method can be tricky.

Proposal 2: Prioritize subband based eigen-vectors as input to AI CSI evaluation.

Proposal 3: Channel measurement can be considered as input to AI CSI evaluation.

## Data set generation

Both field data and synthesized data from simulator can be used for CSI feedback enhancement evaluation. Synthesized data sets based on 3GPP agreed channel model [2] enable evaluation and cross validation of results of different NN designs. The synthesized data set need to be diverse enough in order to train a robust NN. In [2], both channel model for link level simulation, and channel model for system level simulation have been developed. The channel set generated from system level evaluation with large number of UE dropping, can be used to generate the data set for training and testing.

Proposal 4:  Synthesized data set from SLS is recommended.

## Performance metric

Typically, DL system throughput are used to evaluate DL MIMO codebook design. The same system level throughput can be reused to evaluate the benefit of AI/ML based approach. However, since system level throughput can be impacted by many factors, such as MU MIMO pairing and scheduling, MU MIMO precoder calculation, link adaptation etc, a simpler intermediate metric to be used to evaluate and calibrate the accuracy of the decoder output itself.

In [3], cosine similarity was defined to measure the quality of the reconstructed beamforming vector. Similarly, the cosine similarity for rank-1 eigen vector can be defined as (1), where wi is the rank-1 eigen-vector of subband i, and wi is the reconstructed eigenvector of subband i at the decoder output. Nc is the total number of subband for PMI feedback.

ρ=E1Nci=1Nc||wiHwi ||||wi||  ||wi||       (1)

For higher rank, the cosine similarity definition may need further study. A straightforward method is just to average over each rank, as shown in (2), where  wji is the rank-j eigen-vector of subband i, and wji is the reconstructed rank-j eigenvector of subband i at the decoder output.

ρ=E1NsNcj=1Nsi=1Nc||wjiHwji ||||wji||  ||wji||       (2)

In addition, a weighted average can be used, to scale each rank by its corresponding eigen value, as defined in (3)

ρ=E1NsNcj=1Nsλji=1Nc||wjiHwji ||||wji||  ||wji||       (3)

Proposal 5: Cosine similarity can be the metric for intermediate result calibration. Define rank-1 cosine similarity as ρ=E1Nci=1Nc||wiHwi ||||wi||  ||wi||  . Definition of cosine similarity for higher ranks can be further discussed.

Proposal 6: System level throughput can be used as the metric after the initial calibration stage.

# Evaluation assumptions and initial results

## Simulation assumption

It can be expected the SI will be simulation intensive. It is preferred to set up representative and manageable evaluation cases, so each case will be evaluated by multiple companies. As one of the main goals for CSI feedback along the line of Rel-16 enhanced Type II codebook design is to reflect efficiently frequency selectivity in the CSI feedback, the studied cases should be with high frequency selectivity. Otherwise the study won’t be as meaningful. Due to that consideration, we suggest the UMa channel model be selected. Other setups should be close to or identical to the setups in previous studies, so quick comparison with existing results would be possible. Pursuing the frequency selectivity consideration further, it is possible with a high channel bandwidth, e.g. 100 MHz, some phenomenon about the AI/ML may be revealed, such as whether the ability to compress channel information efficiently increase or decrease with channel bandwidth. However, simulation time and the overall evaluation effort should be not ignored. Therefore, it may be sufficient to study a channel bandwidth at 20 MHz, as there are 13 subbands in 20 MHz already.

As for antenna configurations, while it may be interesting to compare AI/ML schemes’ capability to handle 1D antenna array and 2D antenna array, a single choice such as 2D antenna array is preferred to save evaluation effort.

As comparison a conventional CSI feedback is key, the UE speed should be chosen so Rel-16 enhanced Type II codebook design performs reasonably well. For that, UE speed at 3 km/hr is preferred.

Overall, the recommended system level evaluation parameters are summarized in Table I.

Table I: Recommended evaluation assumption for AI enhanced CSI feedback

| Simulation configuration | Value |
| --- | --- |
|  | FR1 |
| Layout | 21 cells, ISD: 200m for dense Urban 21 cells, ISD: 500m for UMa |
| Carrier frequency | 2.1 GHz (FDD), 3.5 GHz(TDD) |
| Subcarrier spacing | FR1: 30 kHz |
| TDD split | DDDSU for TDD |
| BS noise figure | FR1: 5 dB |
| UE noise figure | FR1: 9 dB |
| UE receiver | MMSE-IRC |
| Channel estimation | Realistic |
| UE speed | 3 km/h |
| MCS | Up to 256QAM |
| BS Antenna parameters | 32 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (4,4,2,1,1;4,4) for dense urban and UMa |
| UE antenna parameters | 2 antennas for FDD (M, N, P, Mg, Ng; Mp, Np) = (1,1,2,1,1;1,1) 4 antennas for TDD (M, N, P, Mg, Ng; Mp, Np) = (1,2,2,1,1; 1,2) |
| BS Tx Power | FR1: 44 dBm per 20 MHz |
| UE Tx Power | FR1: 23 dBm |
| System bandwidth | 20 MHz |
| Scheduler | MU-MIMO PF Scheduler |
| CSI acquisition | Rel-16 enhanced Type II codebook, Auto-encoder/decoder |
| PHY Processing delay | UE PDSCH processing capability #1 |

Proposal 7: A set of SLS parameters should be agreed for evaluation. Urban Macro and Dense Urban are the focus for deployment scenarios, with UE speed at 3 Km/Hr.

## AI model description

Since the channel input matrix is treated as image for compression, CNN based network has been widely used in publication [3-8]. [3] designed CsiNet which uses convolutional layer and fully connected layer as a simple encoder, and multiple stage of refine block with skip connection at decoder. [6,7,8] fine turn the design with different filter sizes, dilation factor, number of layers etc. Similar efforts have been taken here.

A high-level neural network architecture is shown in Fig. 2. In the encoder side, number of CNN layers, and number of fully connected layers and other hyper-parameters are tested. The last fully connected layer compresses the input matrix to a vector of size N. A sigmod function is added after the last fully connected layer, similar to the approach used in [8]. A uniform quantization with range [0 1] is applied. The number of quantization bits B varies with different N value as shown in the evaluation for different configurations.

On the decoder side, after the de-quantization block, a fully connected layer to extract N value to the full matrix size, followed by a set of refine network as shown in Fig. 2(b). For each refine block, different the number is CNN layers before skip connection are tested as well. A few refine blocks are connected in the decoder side, and final output layer will have the reconstructed output. Normalized MSE is used as the loss function.

(a) CNN based encoder/decoder

(b) Refine block

Fig. 2. Encoder/decoder NN

## Initial evaluation results for calibration

Initial simulation evaluate the rank-1 eigen vector performance using cosine similarity as defined in (1).  The number of feedback bits are calculated corresponding to different type II codebook configuration. AI auto-encoder/decoder output can be set quite flexibly by adjusting the encoder output value N and the number of bits used in quantization B.  Table II summarize the feedback bit combinations that was tested.

Table II: Feedback bits

| Type II codebook configuration | Number of feedback bits based on Type II codebook configuration | AI encoder output |  |  |
| --- | --- | --- | --- | --- |
|  |  | Total output bits | N | B |
| 1 | 62 | 60 | 30 | 2 |
| 2 | 92 | 88 | 44 | 2 |
| 3 | 112 | 108 | 36 | 3 |
| 4 | 170 | 165 | 55 | 3 |
| 5 | 228 | 222 | 74 | 3 |
| 6 | 282 |  |  |  |
| 7 | 246 |  |  |  |
| 8 | 332 |  |  |  |

Fig. 3 shows the comparison of cosine similarity for different configurations. On average, we see around 45%overhead reduction with the same cosine similar value.

Fig. 3 Comparison of cosine similarity

Observation 1: Preliminary evaluation for rank-1 precoder shows around 45% overhead reduction based on cosine similarity

# Conclusion

In the paper, we discuss the evaluation methodology and initial evaluation result for AI based CSI enhancement.   The proposals and observations are:

Proposal 1: R16 type II codebook’s performance can be used as the baseline for AI based CSI feedback.

Proposal 2: Prioritize subband based eigen-vectors as input to AI CSI evaluation.

Proposal 3: Channel measurement can be considered as input to AI CSI evaluation.

Proposal 4:  Synthesized data set from SLS is recommended.

Proposal 5: Cosine similarity can be the metric for intermediate result calibration. Define rank-1 cosine similarity as ρ=E1Nci=1Nc||wiHwi ||||wi||  ||wi||  . Definition of cosine similarity for higher ranks can be further discussed.

Proposal 6: System level throughput can be used as the metric after the initial calibration stage.

Proposal 7: A set of SLS parameters should be agreed for evaluation. Urban Macro and Dense Urban are the focus for deployment scenarios, with UE speed at 3 Km/Hr.

Observation 1: Preliminary evaluation for rank-1 precoder shows around 45% overhead reduction based on cosine similarity

# Reference

- RP-213599, New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, Qualcomm, 3GPP TSG RAN meeting #94, Electronic meeting, Dec 6-17, 2022

- TR 38.901, “Study on channel model for frequencies from 0.5 to 100 GHz”

- “Deep learning for massive MIMO CSI feedback”, CK Wen, WT Shih, S. Jin, April 2018

- “Deep learning-Based CSI feedback approach for time-varying massive MIMO channels”, T Wang, CK Wen, S. Jin, and G. Ye Li, IEEE wireless comm letters, 2019 April

- “MIMO channel information feedback using deep recurrent network,” C. Lu et al., IEEE Commun. Lett., vol. 23, no. 1, pp. 188–191, Jan. 2019.

- “Multi-resolution CSI Feedback with deep learning in Massive MIMO System”, May 2021, arXiv:2105.00354v1

- “Dilated Convolution based CSI Feedback Compression for Massive MIMO Systems”, June 2021, arXiv

- “EVCsiNet: eigenvector-based CSI feedback under 3GPP link-level channels”, W. Liu, et al,  IEEE wireless communication letters, Vol 10, No 12, Dec 2021