3GPP TSG- RAN WG1 Meeting #109-e                                                 R1-2204247

e-Meeting, May. 9th – May. 20th, 2022

Agenda Item:	9.4.1.1

Source:	Apple Inc.

Title:	On channel access mechanism for sidelink on FR1 unlicensed spectrum

Document for:	Discussion/Decision

# Introduction

Sidelink in FR1 unlicenced bands (n46 and n96/n102) was approved in [1].

- Study and specify support of sidelink on unlicensed spectrum for both mode 1 and mode 2 where Uu operation for mode 1 is limited to licensed spectrum only [RAN1, RAN2, RAN4]

- Channel access mechanisms from NR-U shall be reused for sidelink unlicensed operation

  - Assess the applicability of sidelink resource reservation from Rel-16/Rel-17 to sidelink unlicensed operation within the boundaries of unlicensed channel access mechanism and operation

    - No specific enhancements for Rel-17 resource allocation mechanisms

    - If the existing NR-U channel access framework does not support the required SL-U functionality, WGs will make appropriate recommendations for RAN approval.

- Physical channel design framework: Required changes to NR sidelink physical channel structures and procedures to operate on unlicensed spectrum

  - The existing NR sidelink and NR-U channel structure shall be reused as the baseline.

- No specific enhancements for existing NR SL feature

- The study should focus on FR1 unlicensed bands (n46 and n96/n102) and is to be completed by RAN#98.

In this paper, we discuss design aspects related to channel access.

# Discussion

## Channel access mechanism

Channel access procedure for FR1 unlicensed band has been defined in LAA/eLAA/NR-U for Uu link. In general, the channel access procedure follows ETSI EN 301 893 [2]. Like all the unlicensed band operation, sidelink unlicensed devices should meet the regulation requirement defined in [2] as well.

While LAA/eLAA mainly follow requirement defined under LBE, NR-U enables both LBE and FBE, where FBE is mainly for the deployment where WiFi devices are not present. In FBE, a fixed frame duration of 1 to 10 ms can be defined, and an idle period equals to minimum of 5% of fixed frame duration or 100us, is required at the end of each frame. Although the fixed frame period may fit better with current the slot based sidelink design, there is a strong limitation that FBE work better when LBE devices including WiFi or even LAA/eLAA devices are not nearby. Due to the limitation, we propose that for R18 sidelink in unlicensed band, LBE channel access should be the design priority. If time allows, FBE can be considered.

Proposal 1:    Support LBE as defined in ETSI EN 301 893. Consider FBE as second priority.

In 37.213 type 1 channel access procedure, separate channel access priority class (CAPC) for DL or UL are defined.  DL CAPC table enables higher channel access chances for the gNB and longer MCOT for priority class 3 and 4.

The DL and UL CAPC definition follows the Supervising Devices and Supervised Devices rule in ETSI EN 301 893. For sidelink unlicensed transmission, although we can further define some sidelink UEs as the Supervising Devices and some sidelink UE as Supervised Devices, it further complicates the design. Due to the large amount of potential sidelink transmissions particularly for commercial D2D use cases, to have fair coexistence with existing unlicensed band devices including eLAA and NR-U, it is recommended that all sidelink UEs follow the Supervised Device rule, i.e., the CAPC defined for UL transmission in TS 37.213.

Proposal 2:    Type 1 UL channel access procedure and corresponding CAPC can be reused for sidelink type 1 channel access.

In Uu link, discovery burst transmission uses Type 2 channel access after one shot sensing with a gap of 25us. The transmission duration is at most 1ms, and discovery burst duty cycle is at most 1/20. In UL, for RACH transmission, type 1 UL channel access with priority type 1 can be used.

For sidelink SSB transmission, the channel access scheme needs to be similar or a bit more conservative comparing to gNB/eNB discovery burst transmission. Otherwise, it might cause too much interference to existing devices in the band.

Alternatively, the sidelink SSB can transmit similar to RACH, using type 1 uplink channel access with priority class 1.

Proposal 3:    S-SSB transmission can start after one shot 25us LBT, or type 1 priority 1 uplink channel access.

## Model 1 and mode 2 resource allocation

Mode 1 and mode 2 resource allocation are basic schemes enabled in NR sidelink. For mode 1 resource selection, the sidelink UE will request and gNB will allocate dynamic grant or configured grant to the sidelink transmission. The Uu link operation is limited to licensed band [1]. After the sidelink UE received the DG or CG, the sidelink UE will need to perform type 1 channel access procedure with corresponding priority class. Sidelink transmission starts after CCA success.

Proposal 4: For mode 1 resource allocation, type 1 channel access procedure is required before initiating DG and CG sidelink transmission.

Mode 2 resource allocation scheme is one of key optimizations for sidelink operation in licensed/dedicated band. For mode 2, the sidelink UE will perform sensing, select resources, and make reservation ahead of actual transmission. The sidelink UE performs type-1 channel access procedure before initial data transmission. The sidelink UE need to perform another type-1 channel access before transmission on the reserved resources.

Proposal 5: For mode 2 resource allocation, type-1 channel access procedure is required before transmission on reserved resource.

Once a resource is reserved, all the other UEs who received the reservation message will avoid selecting and transmitting on the reserved resource, to reduce collision probability. While the mechanism works well in licensed or dedicated band, in unlicensed band, it might have some limitation. There is a large mount of existing devices on markets already in the unlicensed band, including WiFi devices, LAA/eLAA/NR-U devices, who will not be able to listen to the reservation message sent by the sidelink UE, and will continue performing CCA regardless the resource is reserved or not. For other sidelink UEs that follow the reservation, the chances of getting CCA success is reduced. Therefore mode 2 resource selection is more useful when legacy devices are not nearby.

When model 2 UE operates within network coverage, the nearby Uu UEs will not listen to the reservation message as well. To reduce collision on the reserved resource, sidelink UE may forward the reservation signal to the gNB, so gNB can avoid scheduling UL transmission on the reserved resources.

Observation 1: For mode 2, reservation works better when other technologies is not present (No WiFi, eLAA/NR-U).

In eLAA/NR-U configured grant design, the starting point after type 1 channel access procedure is successful depends on whether this is full BW configuration, or partial BW configuration. In case of full BW configuration, random starting position is used, so if one UE starts the transmission, it will naturally block other UEs to start therefore avoiding collision. In case of partial BW, to avoid one UE transmission block another FDMed UE in the same 20MHz CCA bandwidth, the same starting position is used. For sidelink UE, similar mechanism can be used.

Proposal 6:  Separate starting position can be used for resource allocation with full BW or partial BW, similar to CG PUSCH in Uu link.

## COT sharing

When sidelink UE performs type 1 channel access procedure and acquires a COT, the COT can be shared to the responding sidelink UE without LBT if the gap between the transmissions is less than 16us, or one shot LBT if the gap is 25us. The responding device transmission includes unicast transmission to the initiating device only, or broadcast or groupcast transmission that include at least the initiating device.

Proposal 7: Allow COT sharing between initiating sidelink UE and responding sidelink UE.

To enable COT sharing, the channel access information can be included as part of SCI stage 1. For COT sharing, the responding device should transmit only the traffic with priority class higher than the priority class used to acquire the COT, the transmission power to be used is also lower than the Tx power used in EDT calculation for channel access. In addition, the remaining COT duration to be shared should be also included in SCI so that the responding UE knows how long the transmission can be. In addition, SCI might include the CCA type and CP extension, so the responding UE can transmit with CP extension to ensure either 16us or 25us gap is used between transmissions.

Proposal 8: SCI stage 1 includes sidelink COT sharing information such as access priority class/MCOT, remaining COT length, Sensing Power, Shared COT CCA type and CP extension etc.

# Conclusion

In the paper, we discuss the sidelink unlicensed band channel access design aspects. The proposals are:

Proposal 1:    Support LBE as defined in ETSI EN 301 893. Consider FBE as second priority.

Proposal 2:    Type 1 UL channel access procedure and corresponding CAPC can be reused for sidelink type 1 channel access.

Proposal 3:    S-SSB transmission can start after one shot 25us LBT, or type 1 priority 1 uplink channel access.

Proposal 4: For mode 1 resource allocation, type 1 channel access procedure is required before initiating DG and CG sidelink transmission.

Proposal 5: For mode 2 resource allocation, type-1 channel access procedure is required before transmission on reserved resource.

Observation 1: For mode 2, reservation works better when other technologies is not present (No WiFi, eLAA/NR-U).

Proposal 6:  Separate starting position can be used for resource allocation with full BW or partial BW, similar to CG PUSCH in Uu link.

Proposal 7: Allow COT sharing between initiating sidelink UE and responding sidelink UE.

Proposal 8: SCI stage 1 includes sidelink COT sharing information such as access priority class/MCOT, remaining COT length, Sensing Power, Shared COT CCA type and CP extension etc.

# Reference

- RP-213678, “New WID on NR sidelink evolution,” Dec. 2021.

- ETSI EN 301 893, 5 GHz RLAN; Harmonized standard covering the essential requirements of article 3.2 of directive 2014/53/EU, v2.1.1, May 2017.

- TS 37.213, Physical layer procedures for shared spectrum channel access.