3GPP TSG- RAN WG1 Meeting #109-e                                             R1- 2204251

e-Meeting, May 9th – 20th, 2022

Agenda Item:	9.5.1.1

Source:	Apple Inc.

Title:	Discussion on Sidelink positioning scenarios and requirements

Document for:	Discussion/Decision

# Introduction

A SID defining objectives for a Study on expanded and improved NR positioning was agreed to in RAN #94 [1] with the following justification for sidelink positioning:

- There is a need for 3GPP to study and develop sidelink positioning solutions that can support the use cases, scenarios and requirements identified for V2X, public safety, commercial and IIOT use cases.

and the following objective:

- Scenario/requirements

  - Coverage scenarios to cover: in-coverage, partial-coverage and out-of-coverage

  - Requirements: Based on requirements identified in TR38.845 and TS22.261 and TS22.104

  - Use cases: V2X (TR38.845), public safety (TR38.845), commercial (TS22.261), IIOT (TS22.104)

  - Spectrum: ITS, licensed

- Identify specific target performance requirements to be considered for the evaluation based on existing 3GPP work and inputs from industry forums [RAN1]

In this contribution, we address SL positioning scenarios and requirements including specific target performance requirements.

# Discussion

The coverage scenarios, use cases and spectrum have been addressed in the objectives in the SID [2]. However, there is a still a need to identify the specific requirements for the use cases to be considered for the evaluation.

Observation 1: The coverage scenarios, use cases and spectrum to be used in the study have been addressed in the SID. There is a still a need to identify the specific requirements for the use cases to be considered for the evaluation.

## Specific Target Performance Requirements

### SA1 positioning  service levels

In [2], Table 7.3.2.2-1,  SA1 defines performance requirements for horizontal and vertical positioning service levels with Service Levels 1 through 6 addressing absolute positioning and Service Level 7 addressing relative positioning. The Service Levels and their associated requirements are summarized below and serve as a reference for the requirement discussion in all the use cases to be discussed:

- Service Level 1: Absolute {10 m Horizontal, 3 m vertical, 1 second latency, 95% availability}

- Service Level 2: Absolute {3 m Horizontal, 3 m vertical, 1 second latency, 99% availability}

- Service Level 3: Absolute {1 m Horizontal,  2 m vertical, 1 second latency, 99% availability}

- Service Level 4: Absolute {1 m Horizontal, 2 m vertical, 15 msec latency, 99,9% availability}

- Service Level 5: Absolute {0,3 m Horizontal, 2 m vertical, 1 second latency, 99 % availability}

- Service Level 6: Absolute {0,3 m Horizontal, 2 m vertical, 10 msec latency, 99,9% availability}

- Service Level 7: Relative {0,2 m Horizontal, 0,2 m vertical, 1 second, latency, 99% availability}

### Use case 1: Commercial Use Case

This covers Indoor Office (IOO), Urban Macro (UMa), and Urban Micro (Umi) [3][4]. From  22.261[2], all Service Levels are applicable to the commercial use case. However, in the Rel-16 study on positioning support [3] and the Rel-17 positioning enhancement study [4] specific design requirements were agreed to.

In Rel-17 target positioning requirements for commercial use cases are defined as follows:

- Horizontal position accuracy (< 1 m) for 90% of UEs

- Vertical position accuracy (< 3 m) for 90% of UEs

- End-to-end latency for position estimation of UE (< 100 ms)

- Physical layer latency for position estimation of UE (< 10 ms)

These requirements can be re-used for the SL positioning evaluation.

Proposal 1: For SL position in the commercial use case, RAN1 should re-use the Rel-17 positioning requirements target:

- Horizontal position accuracy (< 1 m) for 90% of UEs

- Vertical position accuracy (< 3 m) for 90% of UEs

- End-to-end latency for position estimation of UE (< 100 ms)

- Physical layer latency for position estimation of UE (< 10 ms)

### Use case 2: Industrial IoT (IIOT)

This covers the Indoor Factory scenario [4]. In 22.104 [5], the positioning requirements map between Service Levels 2 through 7. However, in the Rel-17 positioning enhancement study [4] specific design requirements were agreed to.

In Rel-17 target positioning requirements for IIoT use cases are defined as follows:

- Horizontal position accuracy (< 0.2 m) for 90% of UEs

- Vertical position accuracy (< 1 m) for 90% of UEs

- End-to-end latency for position estimation of UE (< 100ms, in the order of 10 ms is desired)

- Physical layer latency for position estimation of UE (<10ms)

These requirements can be re-used for the SL positioning evaluation.

Proposal 2: For SL position in the IIoT use case, RAN1 should re-use the Rel-17 positioning requirements target:

- Horizontal position accuracy (< 0.2 m) for 90% of UEs

- Vertical position accuracy (< 1 m) for 90% of UEs

- End-to-end latency for position estimation of UE (< 100ms, in the order of 10 ms is desired)

- Physical layer latency for position estimation of UE (<10ms)

### Use case 3: Public Safety Use Case

This provides numerical positioning requirements for the "1st responders" use case. In {38.845}, the following requirements are reported from {22.261}:

- 1 m horizontal accuracy, 2 m (absolute) or 0.3 m (relative) vertical accuracy, 95 – 98 % positioning service availability.

These requirements should be used for the public safety use case. As this is a sub-set of the commercial use case, any design for the commercial use case can be used for public safety.

Proposal 3: For SL positioning in the Public Safety use case, the following requirements should be used:

- 1 m horizontal accuracy, 2 m (absolute) or 0.3 m (relative) vertical accuracy, 95 – 98 % positioning service availability (SA1)

  - NOTE: this is equivalent to approximately service level 4 absolute requirements and service level 7 relative requirements for the commercial use case. As such, any design for the commercial use case can be applicable to public safety.

### Use case 4: V2X Use Case

In [6], Absolute and Relative 2D positioning Support Levels up to 250 km/h are defined as follows:

- Set 1: 10 – 50 m with 68 – 95 % confidence level. This includes Group 1 (relatively lax accuracy for information provisioning) in [7]and Service level 1 in [2]

- Set 2: 1 – 3 m with 95 – 99 % confidence level. This includes Group 2 (lane level accuracy for day1 safety use cases as an example)  in [7], and Service level 2, 3, 4 in [2].

- Set 3: 0.1 – 0.5 m with 95 – 99 % confidence level. This includes Group 3 (below meter level accuracy for automated or teleoperated driving as an example) in [7],  and Service level 5, 6, 7 in [2].

Given this is the first effort at SL positioning for V2X scenarios, we propose to design to Set 2 with requirements as follows:

- Horizontal position accuracy (< 1  m) for 90% of UEs

- Vertical position accuracy: 2m (absolute), 0,2m (relative)

- PHY latency (< 15 msec)

Proposal 4: For SL positioning in the V2X use case, the following requirements should be used:

- Horizontal position accuracy (< 1  m) for 90% of UEs

- Vertical position accuracy: 2m (absolute), 0,2m (relative)

- PHY latency (< 15 msec)

The proposals are summarized in the Table below:

Table 1: SL Positioning Requirement Proposal Summary

| Commercial Use Case (TS 22.261, 38.857) | IIoT Use case (TS 22.104, 38.857) | Public Safety Use Case (38.845) | V2X Use Case (38.845) |
| --- | --- | --- | --- |
| Proposal: Re-use TR38.857 targets | Proposal: Re-use TR38.857 targets | Proposal: Use requirements from TR 38.845 | Proposal: Target Set 2 in TR 38.845 Horizontal < 1 m |

# Conclusion

In this contribution, we address SL positioning scenarios and requirements including specific target performance requirements with the following proposals:

Observation 1: The coverage scenarios, use cases and spectrum to be used in the study have been addressed in the SID. There is a still a need to identify the specific requirements for the use cases to be considered for the evaluation.

Proposal 1: For SL position in the commercial use case, RAN1 should re-use the Rel-17 positioning requirements target:

- Horizontal position accuracy (< 1 m) for 90% of UEs

- Vertical position accuracy (< 3 m) for 90% of UEs

- End-to-end latency for position estimation of UE (< 100 ms)

- Physical layer latency for position estimation of UE (< 10 ms)

Proposal 2: For SL position in the IIoT use case, RAN1 should re-use the Rel-17 positioning requirements target:

- Horizontal position accuracy (< 0.2 m) for 90% of UEs

- Vertical position accuracy (< 1 m) for 90% of UEs

- End-to-end latency for position estimation of UE (< 100ms, in the order of 10 ms is desired)

- Physical layer latency for position estimation of UE (<10ms)

Proposal 3: For SL positioning in the Public Safety use case, the following requirements should be used:

- 1 m horizontal accuracy, 2 m (absolute) or 0.3 m (relative) vertical accuracy, 95 – 98 % positioning service availability (SA1)

  - NOTE: this is equivalent to approximately service level 4 absolute requirements and service level 7 relative requirements for the commercial use case. As such, any design for the commercial use case can be applicable to public safety.

Proposal 4: For SL positioning in the V2X use case, the following requirements should be used:

- Horizontal position accuracy (< 1  m) for 90% of UEs

- Vertical position accuracy: 2m (absolute), 0,2m (relative)

- PHY latency (< 15 msec)

Proposal 5: Summary

| Commercial Use Case (TS 22.261, 38.857) | IIoT Use case (TS 22.104, 38.857) | Public Safety Use Case (38.845) | V2X Use Case (38.845) |
| --- | --- | --- | --- |
| Proposal: Re-use TR38.857 targets | Proposal: Re-use TR38.857 targets | Proposal: Use requirements from TR 38.845 | Proposal: Target Set 2 in TR 38.845 Horizontal < 1 m |

# Reference

- RP-213588, Revised SID on Study on expanded and improved NR positioning

- TS 22.261, Service requirements for the 5G system; Stage 1, (Release 18)

- TR 38.855, Study on NR positioning support, (Release 16)

- TR 38.857, Study on NR Positioning Enhancements; (Release 17)

- TS 22.104, Service requirements for cyber-physical control applications in vertical domains; Stage 1, (Release 18)

- 38.845, Study on scenarios and requirements of in-coverage, partial coverage, and out-of-coverage NR positioning use cases, (Release 17)

- RP-210040, LS reply to 3GPP RAN on requirements of in-coverage, partial coverage, and out-of-coverage positioning use cases, 5GAA, RAN #91-e