3GPP TSG- RAN WG1 Meeting #109-e                                             R1- 2204253

e-Meeting, May 9th – 20th, 2022

Agenda Item:	9.5.1.3

Source:	Apple Inc.

Title:	Discussions on Potential solutions for SL positioning

Document for:	Discussion/Decision

# Introduction

A SID defining objectives for a Study on expanded and improved NR positioning was agreed in RAN #94 [1],  with the following justification for sidelink positioning:

- There is a need for 3GPP to study and develop sidelink positioning solutions that can support the use cases, scenarios and requirements identified for V2X, public safety, commercial and IIOT use cases.

and the following objective:

- Study of sidelink reference signals for positioning purposes from physical layer perspective, including signal design, resource allocation, measurements, associated procedures, etc, reusing existing reference signals, procedures, etc from sidelink communication and from positioning as much as possible [RAN1]

- Study of sidelink reference signals for positioning purposes from physical layer perspective, including signal design, resource allocation, measurements, associated procedures, etc, reusing existing reference signals, procedures, etc from sidelink communication and from positioning as much as possible [RAN1]

In this contribution, we address the  positioning methods  (e.g. TDOA, RTT, AOA/D, etc) and the design of sidelink reference signals needed for SL positioning purposes.

# Discussion

## Techniques for Sidelink Positioning

Rel-16 and Rel-17 focus on Uu-based positioning with both RAT-based and RAT-independent positioning techniques. These need to be extended to sidelink positioning. The following table summarizes the RAT-based positioning techniques:

Table 1:Rel 16/Rel17 Uu Positioning Techniques

|  | DL based positioning | UL based positioning | DL and UL based positioning |
| --- | --- | --- | --- |
| Techniques | DL-TDOA, DL-AoD | UL-TDOA, UL-AoA | multi-cell RTT, E-CID |

As a first step in the study, RAN1 should decide which of the existing Uu RAN-based positioning techniques are suitable to be re-designed for SL positioning.

To re-design each of these positioning techniques for SL positioning requires

- a discussion of the SL positioning techniques based on the existing RAN-dependent techniques

- Updates to the signaling, measurements and procedures

- A discussion on the positioning entity (LMF) in the case of in-coverage, partial-coverage and out of-coverage cases

### SL positioning techniques

The following SL positioning in techniques may be considered:

Table 2: Candidate SL Positioning Techniques

|  | DL based positioning | UL based positioning | DL and UL based positioning |
| --- | --- | --- | --- |
| Techniques | SL-TDOA-1 (DL-like), SL-AoD | SL-TDOA-2 (UL-like), SL-AoA | SL-multi-cell RTT, SL-E-CID Type positioning |

#### SL-TDOA: Sidelink Time Delay of Arrival based Positioning

SL-TDoA positioning is based on the time delay of arrival to or from a SL UE target with

SL UEs in a positioning set (at least 2 for 2-D positioning and at least 3 for 3-D positioning). In hybrid SL-TDOA, one or more of the positioning set may be a gNB.

To enable SL-TDOA, the following need to be defined:

- Positioning set: set of UEs used to perform SL-TDOA (excluding target UE): May be (pre-) configured or dynamically set.

- Common synchronization source: For timing-based positioning, having a common reference is very important and there is a need to define a common synchronization reference for the UEs in the positioning set

- Positioning Reference: UE/gNB that serves as reference for SL-TDOA timing

- Directionality:

  - SL-TDOA-1: direction of signals from positioning set to target UE (equivalent to DL-TDOA for Uu case)

  - SL-TDOA-2: direction of signals from target UE to positioning set (equivalent to UL-TDOA for Uu case)

An example of the SL-TDOA-2 is shown in  Figure 1 with the target UE sending the SL-reference signal to the  SL positioning set.

Figure 1: SL-TDoA (Uplink Like) positioning

#### SL-AoA/AoD: Sidelink-Angle of Arrival/Angle of Departure based positioning

SL-AoA/AoD positioning  is based on the angle of arrival of transmission or angle of departure of transmission to or from a SL UE target. In hybrid SL-AoA/AoD, one or more of the positioning set may be a gNB.

To enable SL-AoA/AoD, the following need to be defined:

- Positioning set: set of UEs used to perform SL-AoA/AoD(excluding target UE): May be (pre-) configured or dynamically set.

- Directionality:

  - SL-AoD: The target UE receives the reference signals from the positioning set

  - SL-AoA: The target UE sends the reference signals to the positioning set

An example of the SL-AoAis shown in  Figure 2 with the target UE sending the SL-reference signal to the  SL positioning set.

Figure 2: SL AoA Positioning

#### Other Types:

Similar design updates can be made for multi-cell RTT (as a SL-RTT technique) and for e-CID (as a SL-e-CID type positioning-based SL measurements).

Proposal 1: As a first step, RAN1 should decide which of the existing Uu RAN-based positioning techniques are suitable to be re-designed for SL positioning. Candidate SL-positioning schemes are:

|  | DL based positioning | UL based positioning | DL and UL based positioning |
| --- | --- | --- | --- |
| Techniques | SL-TDOA-1 (DL-like), SL-AoD | SL-TDOA-2 (UL-like), SL-AoA | SL-multi-cell RTT, SL-E-CID Type positioning |

- Hybrid SL positioning with a mix of gNBs and SL-UEs in the positioning candidate set should also be considered.

### Signaling, Measurements and Procedures

For the different SL positioning techniques, updates to the existing measurements, procedures and signaling may be needed. These updates include:

- Identification of the positioning set,  the positioning reference UE, and the target UE

- Establishment of a common synchronization reference (for TDOA based schemes)

- Establishment of new SL measurements. Some examples of the signaling and measurements needed include:

  - For SL-TDOA-1 (“DL-like”), information that may be transferred from the LMF to the UE include the source IDs of the candidate UEs in the positioning set, SL-positioning reference signal configuration, and SL positioning reference signal only SL-UE indication.

  - For e-CID based techniques, new SL measurements for the S-SSB-RSRP and S-SSB-RSRQ may need to be defined.

- Configuration, transmission and measurement of SL positioning reference signals to or from the target UE

- Feedback of positioning measurement and assistance information to the positioning estimator

Proposal 2: RAN1 should discuss the specifics of  the SL positioning techniques based on the existing RAN-dependent techniques and update the associated signaling, measurements and procedures for the new SL-positioning schemes.. Issues to be addressed include:

- Identification of the positioning set and the target UE

- Establishment of a common synchronization reference and the positioning reference UE (for TDOA based schemes)

- Establishment of new SL measurements.

- Configuration, transmission and measurement of SL positioning reference signals to or from the target UE

- Feedback of positioning measurement and assistance information to the positioning estimator

### LMF design for out-of-coverage scenarios

For Uu-based positioning the Location Management Function (LMF) controls the location procedures either using the LPP (to the UE) or the NRPPa (to the gNB) and is located in the core network (see Figure 3). For SL-positioning in the in-coverage and partial coverage scenarios,  the LMF can retain its function (see Figure 4). However, for SL positioning in out-of-coverage scenarios, a new entity may be needed to perform this function e.g. a SL LMF. To enable the design of the different SL positioning techniques, RAN1 should initially assume the presence of a sidelink LMF to function in the same manner as  the LMF in the core in coverage/partial coverage scenarios (see Figure 5). The specifics of the SL-LMF design are assumed to be  to be discussed in RAN2/SA2 and RAN1 can confirm these at a later date.

Figure 3: LMF for Rel-16 Uu Positioning

Figure 4: LMF for in-coverage or partial coverage SL Positioning

Figure 5: LMF for out-of-coverage SL positioning

Proposal 3: RAN1 should discuss the assumption on  the positioning entity (LMF) in the case of in-coverage, partial-coverage and out of-coverage cases

## Design of sidelink reference signals for positioning

### Positioning Reference Signal Design

To leverage the Uu link design, Rel-16 PRS or P-SRS may be used as a starting point for the positioning RS design in SL positioning. Selection of each option has the following advantages and disadvantages:

|  | Advantage | Disadvantage |
| --- | --- | --- |
| SL-positioning based on DL-PRS | - Rx UE currently does not need to change Rx but uses same Rx as for Uu link | - No CDM in resources used for PRS which implies no sharing of resources by multiple Tx/Rx pairs - Higher PAPR than ZC-based SRS - UE will have to implement a DL-PRS transmitter. |
| SL-positioning based on P-SRS | - UE is able to multiplex in both FDM and CDM domains - Low PAPR transmission | - Rx SL-UE needs to decode SRS (not currently decoded based on Uu link) |

As shown from the table, there is no overwhelming advantage or disadvantage in either case. However, based on the if hybrid SL-Uu positioning is allowed, there may be a need for a Ue to transmit or receive a variant of both. As an example, a hybrid SL AoA technique will expect that the UE transmits an SRS type signal to the gNB while a hybrid SL-AoD technique will expect that the UE receive a PRS-type signal at the UE. This may motivate the need for a design that supports both. To limit UE complexity, a UE may indicate which type it supports based on a UE capability.

Proposal 4: DL-PRS and positioning SRS both considered as candidates for the SL-PRS.

  - UE may be configured with one or both of these variants.

  - UE may indicate its capability to support one or both of these variants.

### Resource Allocation

Time Domain: In  NR V2X, only certain slots are (pre-)configured to accommodate SL PSCCH/PSSCH/PSFCH transmissions. This is known as a “resource pool” consisting of contiguous PRBs and contiguous or non-contiguous slots that have been (pre-)configured for SL transmissions and defined within a SL BWP with a bitmap of size X. In NR V2X, S-SSB transmission slots are not contained in the time resource set configured by resource pool.

For the SL positioning reference signal transmissions, it is possible that the slots for transmitting the SL positioning reference signal are within the slots configured for the resource pool. Alternatively, the slots for transmitting SL positioning reference signal can be separate from the slots configured for the resource pool, e.g., in a similar manner as in for S-SSB transmission.

Proposal 5: Decide on whether or not the time resources for the SL positioning signal transmission are within the time resources of the slots for the resource pool.

Frequency Domain: In  NR V2X, a resource pool is divided into (pre-)configured number of L sub channels, where a sub-channel is a group of consecutive PRBs in a slot. The design should decide on the frequency resources used for SL positioning. This can either be the same size as the sub-channel for SL communication or set to a different size (either within the SL communications sub-channel, larger than the SL communications sub-channel or entirely independent).

Overall, the SL positioning reference signal should be in a wide band to achieve high positioning accuracy. This may be infeasible if the SL positioning reference signal is limited to  a single or a few sub-channels.

Proposal 6: Decide on the frequency resources for SL-positioning signals.

Slot structure: SL positioning reference signal can be configured in same slot as SL PSSCH/PSCCH/PSFCH. In this case, the SL positioning symbols can use the same resource allocation method as PSSCH/PSCCH/PSFCH. To enable this design, a discussion is needed on the modifications to the SL-slot structure e.g. should the signals be placed within the PSSCH region, the PSFCH region or in an independent region. In Figure 6, we show a scenario in which the positioning reference signal is placed in the PSSCH region of the slot (immediately after the DMRS symbol). Note that in this case, the positioning reference signal has the same frequency resources as the PSSCH and as such there is no need for a gap or AGC symbol. If they are set to different frequency resource sizes, additional symbols may be needed.

Figure 6: SL Positioning Signal Configured in same slot in PSSCH region

SL positioning reference signal can be configured in independent SL positioning slots. In this case, the SL RS uses independent resource allocation method that may need to be discussed and a discussion is needed on structure of new SL positioning slot as shown in Figure 7.

Figure 7: SL Positioning Signal configured in independent slot

Proposal 7: SL positioning reference signal can be configured

  - in same slot as SL PSSCH/PSCCH/PSFCH (uses same resource allocation method as PSSCH/PSCCH/PSFCH).

    - Discussion needed on modifications to SL-slot structure

  - in independent SL positioning slots (uses independent resource allocation method)

    - Discussion needed on structure of a new SL positioning slot

# Conclusion

Proposal 1: As a first step, RAN1 should decide which of the existing Uu RAN-based positioning techniques are suitable to be re-designed for SL positioning. Candidate SL-positioning schemes are:

|  | DL based positioning | UL based positioning | DL and UL based positioning |
| --- | --- | --- | --- |
| Techniques | SL-TDOA-1 (DL-like), SL-AoD | SL-TDOA-2 (UL-like), SL-AoA | SL-multi-cell RTT, SL-E-CID Type positioning |

- Hybrid SL positioning with a mix of gNBs and SL-UEs in the positioning candidate set should also be considered.

Proposal 2: RAN1 should discuss the specifics of  the SL positioning techniques based on the existing RAN-dependent techniques and update the associated signaling, measurements and procedures for the new SL-positioning schemes.. Issues to be addressed include:

- Identification of the positioning set and the target UE

- Establishment of a common synchronization reference and the positioning reference UE (for TDOA based schemes)

- Establishment of new SL measurements.

- Configuration, transmission and measurement of SL positioning reference signals to or from the target UE

- Feedback of positioning measurement and assistance information to the positioning estimator

Proposal 3: RAN1 should discuss the assumption on  the positioning entity (LMF) in the case of in-coverage, partial-coverage and out of-coverage cases

Proposal 4: DL-PRS and positioning SRS both considered as candidates for the SL-PRS.

  - UE may be configured with one or both of these variants.

  - UE may indicate its capability to support one or both of these variants.

Proposal 5: Decide on whether or not the time resources for the SL positioning signal transmission are within the time resources of the slots for the resource pool.

Proposal 6: Decide on the frequency resources for SL-positioning signals.

Proposal 7: SL positioning reference signal can be configured

  - in same slot as SL PSSCH/PSCCH/PSFCH (uses same resource allocation method as PSSCH/PSCCH/PSFCH).

    - Discussion needed on modifications to SL-slot structure

  - in independent SL positioning slots (uses independent resource allocation method)

    - Discussion needed on structure of a new SL positioning slot

# Reference

- RP-213588, Revised SID on Study on expanded and improved NR positioning