3GPP TSG-RAN1 Meeting #109-e                                             R1- 2204254

e-Meeting, May 9th – 20th, 2022

Agenda Item:	9.5.3

Source:	Apple Inc.

Title:	Discussions on Positioning for RedCap UEs

Document for:	Discussion/Decision

# Introduction

In RAN #94 [1], a SID defining objective on a Study on expanded and improved NR positioning was agreed on. In the SID, the following justification for sidelink positioning was identified:

- RedCap UEs could support NR positioning functionality but there is a gap in that the core and performance requirements have not been specified for the positioning related measurements performed by RedCap UEs, and no evaluation was performed to see how the reduced capabilities of RedCap UEs might impact eventual position accuracy. This gap is to be investigated by the present SI.

and the following objective was taken:

- Positioning support for RedCap UEs, considering the following:

  - Evaluate positioning performance of existing positioning procedures and measurements with RedCap UEs[RAN1]

  - Based on the evaluation, assess the necessity of enhancements and, if needed, identify enhancements to help address limitations associated with for RedCap UEs [RAN1, RAN2]

In this contribution, we evaluate the positioning performance of RedCap UEs using DL-TDOA positioning.

# Discussion

## RedCap UE overview

Release-17 has studied [2] and specified support for RedCap UEs with reduced bandwidth support and reduced complexity including reduced number of receive chains. Table 1 below summarizes the device capability  differences between Rel-16 baseline devices and Redcap devices and highlights the differences that will affect the RedCap UE positioning performance for FR1.

Table 1: Rel-16 baseline and Redcap device Capabilities

|  | Rel-16 Baseline Devices | Redcap Devices | Affects positioning performance |
| --- | --- | --- | --- |
| Maximum Bandwidth | 100 MHz | 20 MHz | Yes |
| Number of Rx branches | 2 | 1 or 2 | Yes |
| Maximum Number of DL MIMO layers | 4 | 1 or 2 | No |
| Maximum DL Modulation Order | 256 QAM | 64 QAM 256 QAM (optional) | No |
| Duplex Operations | FD-FDD, TDD | HD-FDD FD-FDD TDD | No |

From the table we see that only the maximum BW and the number of Rx antenna branches will affect the evaluation. As such, RAN1 should  set the evaluation parameters to 20 MHz bandwidth and 1 Rx antenna for RedCap UEs i.e. the parameters that map to the worst possible performance.

Observation 1: Of the device capability differences between Rel-16 baseline UEs and RedCap UEs, only the maximum bandwidth and the number of Rx antenna branches affect the positioning performance.

Proposal 1: For the RedCap UE evaluation, set the bandwidth of the Redcap devices to 20 MHz and the Number of Rx antennas to 1.

## Evaluation Methodology and Parameters

To enable a fair comparison of results, RAN1 should set a common evaluation methodology and common evaluation parameters including the specific use cases (commercial or IIoT) and scenarios (UMa, UMi, Indoor office or Indoor Factory) for evaluation.

In our evaluation, we use the commercial use case highlighted in TR 38.857 [4] and re-use the evaluation methodology in TR 38.855 [3] and evaluate the relative performance for the UMi street canyon scenario at FR1 with ISD 200 m. For the Rel-16 baseline, we set the maximum BW to 50 MHz at a carrier frequency of 2 GHz.

In addition, it is necessary to agree on the simulation assumptions for the configuration of the positioning reference signals including PRS parameters such as the Reference Signal Physical Structure and Resource Allocation length and if power boosting is applied or not.

The simulation metric is the horizontal positioning accuracy, defined as the difference between the calculated horizontal position and the actual horizontal position of a UE. The simulation parameters are detailed in the Appendix.

Proposal 2: RAN1 to set a common evaluation methodology and common evaluation parameters based on the parameters in 38.855 [4]  and 38.857 [3].

- it is necessary to agree on the simulation assumptions for the configuration of the positioning reference signals

## Evaluation Results

The following figure shows the difference in horizontal positioning accuracy between a RedCap UE and a baseline Rel-16 UE.

Figure 1: Horizontal Accuracy for RedCap UE vs Rel-16 baseline UE

Table 2: Horizontal Accuracy at 90th percentile

|  | RedCap UE | Rel-16 baseline UE |
| --- | --- | --- |
| Horizontal Accuracy (90%) | 16 m | 8.5m |

As seen, there is a positioning performance loss for this simple scenario. As the positioning reference signal can be repeated, RAN1 should study if modification of the reference signal parameters assumptions (e.g. repetition, boosting) can compensate for the loss.

Observation 2: There is a  positioning performance loss of RedCap UEs compared with Rel-16 baseline UEs in this simple scenario.

|  | RedCap UE | Rel-16 baseline UE |
| --- | --- | --- |
| Horizontal Accuracy (90%) | 16 m | 8.5m |

Proposal 3: RAN1 to investigate if modification of the reference signal parameters assumptions can compensate for performance loss based on a common set of evaluation parameters.

# Conclusion

In this contribution, we have evaluated the positioning performance of RedCap UEs using DL-TDOA positioning and have the following observations and proposals:

Observation 1: Of the device capability differences between Rel-16 baseline UEs and RedCap UEs, only the maximum bandwidth and the number of Rx antenna branches affect the positioning performance.

Proposal 1: For the RedCap UE evaluation, set the bandwidth of the Redcap devices to 20 MHz and the Number of Rx antennas to 1.

Proposal 2: RAN1 to set a common evaluation methodology and common evaluation parameters based on the parameters in 38.855 [4]  and 38.857 [3].

- it is necessary to agree on the simulation assumptions for the configuration of the positioning reference signals

Observation 2: There is a  positioning performance loss of RedCap UEs compared with Rel-16 baseline UEs in this simple scenario.

|  | RedCap UE | Rel-16 baseline UE |
| --- | --- | --- |
| Horizontal Accuracy (90%) | 16 m | 8.5m |

Proposal 3: RAN1 to investigate if modification of the reference signal parameters assumptions can compensate for performance loss based on a common set of evaluation parameters.

# Reference

- RP-213588, Revised SID on Study on expanded and improved NR positioning

- TR 38.875, Study on support of reduced capability NR devices, (Release 17)

- 38.855, Study on NR positioning support, (Release 16)

- 38.857, Study on NR Positioning Enhancements; (Release 17)

# Appendix

Table 2: Common scenario parameters applicable for all scenarios

|  | FR1 Specific Values |
| --- | --- |
| Carrier frequency, GHz | 2GHz, 4GHz – Note 1 |
| Bandwidth, MHz | 50MHz for 2GHz |
| Subcarrier spacing, kHz | 15kHz for 5MHz and 50MHz 30kHz for 100MHz |
| gNB model parameters |  |
| gNB noise figure, dB | 5dB |
| UE model parameters |  |
| UE noise figure, dB | 9dB – Note 1 |
| UE max. TX power, dBm | 23dBm – Note 1 |
| UE antenna configuration | Panel model 1 – Note 1 Mg = 1, Ng = 1, P = 2, dH = 0.5λ, (M, N, P, Mg, Ng) = (1, 2, 2, 1, 1) |
| UE antenna radiation pattern | Omni, 0dBi |
| PHY/link level abstraction | Explicit simulation of all links, individual parameters estimation is applied. Signal location parameters estimated based on DL-TDoA using Taylor Series approximation with a LS solution. |
| Network synchronization | Ideal |
| UE/gNB RX and TX timing error | Ideal |
| PRS signal | Comb 2, Length 4 |
| Note 1: According to TR 38.802 Note 2: According to TR 38.901 |  |

Table 3: Scenario 2: Urban micro (UMi) scenario parameters

|  | FR1 Specific Values |
| --- | --- |
| Layout | Hexagonal grid, 19 macro sites, 3 sectors per site, ISD = 200m, Note 1 19 cells with no wrap-around interference modeling Explicit link level simulation of 7 cells for positioning evaluation. |
| Total gNB TX power, dBm | 44dBm |
| gNB antenna configuration | (M, N, P, Mg, Ng) = (8, 8, 2, 1, 1), (dH, dV) = (0.5, 0.8)λ, - Note 1 |
| gNB antenna radiation pattern | Directional, 8dBi – Note 1 Table 6.1.1-5 |
| Channel model | UMi Street Canyon – Note 3 |
| Penetration loss | For outdoor UEs: 0dB |
| Number of floors | All UEs are on the ground. |
| UE drop procedure | 100% outdoor uniformly distributed over the horizontal area within center cell |
| UE mobility (for modeling Doppler effects) | Outdoor: 3km/h |
| UE height, m | 1.5m |
| Min. gNB-UE distance (2D), m | 10m |
| gNB antenna height | 10 m by default. Companies can bring results with uniform distribution [5-20]m​ |
| Note 1: According to 3GPP TR 38.802 Note 2: In case if interference considerations are not properly taken into account for 7 sites companies are encouraged to provide results for 19 sites. Note 3: According to 3GPP TR 38.901 |  |