3GPP TSG RAN WG1 #109-e		R1-2204255

e-Meeting, May 9th – 20th, 2022

Source: 	Apple Inc.

Title:                     On further RedCap UE complexity reduction features

Agenda item:	9.6.1

Document for:	Discussion and Decision

# 1 Introduction

In the RAN plenary 94 meeting, a new study item ‘Study on further NR RedCap UE complexity reduction’ was approved with the following objective [1]:

- Study further UE complexity reduction techniques based on Rel-17 evaluation methodology in TR 38.875 [RAN1] - Consider network impact, coexistence of Rel-17 and Rel-18 RedCap and non-RedCap UEs in a cell, UE impact, specification impact - Potential solutions, which may complement each other, for reducing device complexity are focusing on: - UE bandwidth reduction to 5MHz in FR1, - Possibly in combination with relaxed UE processing timeline for PDSCH and/or PUSCH and/or CSI - reduced UE peak data rate in FR1, - Possibly including restricted bandwidth for PDSCH and/or PUSCH - Possibly in combination with relaxed UE processing timeline for PDSCH and/or PUSCH and/or CSI

In this contribution, we discuss the aspects related to further Redcap UE complexity reduction features.

# 2. Discussions

Rel-17 Redcap design targets three use cases and the following requirements:

Table 1: Requirements of Rel-17 Redcap use cases

| Use cases | Data Rate | Battery Life |
| --- | --- | --- |
| Wearables | Peak rate: up to DL 150Mbps, up to UL 50Mbps, Reference bit rate: 5-50Mbps Mbps in DL and Minimum 2-5 Mbps in UL | At least several days and up to 1-2 weeks |
| Industrial Sensors | < 2 Mbps | Few years (≥5 yr in TR 22.832) |
| Video Surveillance | Economic video: 2-4Mbps High-end video: 7.5-25 Mbps | -- |

To maximize the benefit of economies of scale, a common NR redcap framework was established to address all these use cases by allowing a set of combinational cost reduction features:

- A reduced maximum UE bandwidth (20MHz for FR1, 100MHz for FR2)

- A reduced number of Rx branches (1 or 2 for FR1, 2 for FR2)

- A reduced number of DL MIMO layers (1 or 2)

- Relaxed maximum modulation order (DL 256QAM is optional in FR1)

- HD-FDD Type A operation

Given the reduced capabilities described above, a Rel-17 Redcap device with lowest achievable complexity ‘20MHz, 1 Rx, 1 DL MIMO layer, 64QAM’ supports a peak data rate of 82Mbps for FR1. It should be noted that, as listed in Table 1, these Rel-17 use cases have very different peak data requirements from 2Mbps (industrial sensor) to more than 50Mbps. A single Redcap UE type in Rel-17 results in an excessively overdesigned for low-tier devices such as industrial sensor, video surveillance and the smart grid, which intended to be further optimized in Rel-18 by further reducing the BW or peak data rate.

Observation 1: The common NR Redcap framework and a single Redcap device type results in an overdesigned device for the industrial sensors and video surveillance use cases.

According to the approved study item description [1], two potential complexity reduction features were identified for further study, one is UE bandwidth reduction to 5MHz in FR1 and the other is reduced UE peak data rate in FR1.

## 2.1 UE bandwidth reduction to 5MHz

As studied in Rel-17 and documented in Rel-17 Redcap TR 38.875, reducing UE maximum BW can reduce cost in both RF and Baseband perspectives with the following function blocks

- RF aspects

  - Power Amplifier, RF transceiver (including LNAs, mixer, and local oscillator)

- BB aspects

  - ADC/DAC, FFT/IFFT, Post-FFT data buffering, Receiver processing block, LDPC decoding and HARQ buffer.

Table 2 provides a rough estimation based on the Rel-17 breakdown cost reduction number captured in TR 38.875 and cost analysis methodologies. The values associated with ‘5MHz’ BW are computed using a linear extrapolation based on the cost reduction values in TR 38.875.

Table 2: Extrapolated cost analysis based on the Rel-17 cost breakdown

| FR1 FDD UE complexity reduction technique(s) | RF cost metric | BB cost metric | Total cost metric | Cost reduction relative to Rel-17 | RF reduction | BB reduction | Total reduction |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 20 MHz, 1 layer, 1 Rx [2] | 83.2% | 21.6% | 46.2% | - | 16.8% | 78.4% | 53.8% |
| 5 MHz, 1 layer, 1 Rx | ~81% | ~5.4% | ~35.6% | 8% | ~19% | ~94.6% | ~64.4% |

It should be noted that the reduced BW to 5MHz in both Baseband and RF, instead of BB only, can provide additional power saving benefit due to the reduced RF bandwidth, which is important for lower-tier industrial sensor devices to meet the >2 years battery life requirement.

On the other hand, reducing RF BW to 5MHz for eRedcap in FR1 requires new designs for several channels (e.g., PBCH, CORESET#0) and has significant impact on the existing specifications, especially for 30kHz SCS case. Although the spec changes can be largely avoided by supporting 15kHz SCS for 5MHz eRedcap devices, we believe it is not a feasible direction to go considering the wide deployment of 30kHz SCS on FR1.

Proposal 1: Support both 15kHz and 30kHz SCS configurations for Rel-18 eRedcap if maximum 5MHz reduced BW is supported.

## 2.2 On Reduced UE Peak Data Rate in FR1

Reducing UE peak data rate in FR1 for Redcap was extensively discussed during Rel-17 phase [3] and no conclusion was made eventually due to limited time left in Rel-17 timeframe. In Rel-15 NR, a scaling factor has been introduced to allow a mismatch between mandated RF BW and baseband processing capability for legacy eMBB UE to relax baseband processing requirement and provide implementation flexibility in cost. For a Rel-18 eRedcap device targeting to 10Mbps peak data rate, leveraging the scaling factor framework to further reduce the BB cost is the simplest way to minimize cost and fulfil the target peak data requirement for Rel-18 eRedcap use cases.

As already pointed out in Rel-17 discussions, there is currently a restriction in NR specification on the application of scaling factor at UE, as highlighted using yellow color below.

### 4.1.2 Supported max data rate for DL/UL For NR, the approximate data rate for a given number of aggregated carriers in a band or band combination is computed as follows. wherein J is the number of aggregated component carriers in a band or band combination Rmax = 948/1024 For the j-th CC, is the maximum number of supported layers given by higher layer parameter maxNumberMIMO-LayersPDSCH for downlink and maximum of higher layer parameters maxNumberMIMO-LayersCB-PUSCH and maxNumberMIMO-LayersNonCB-PUSCH for uplink. is the maximum supported modulation order given by higher layer parameter supportedModulationOrderDL for downlink and higher layer parameter supportedModulationOrderUL for uplink. is the scaling factor given by higher layer parameter scalingFactor and can take the values 1, 0.8, 0.75, and 0.4. […] The approximate maximum data rate can be computed as the maximum of the approximate data rates computed using the above formula for each of the supported band or band combinations. For single carrier NR SA operation, the UE shall support a data rate for the carrier that is no smaller than the data rate computed using the above formula, with J=1 CC and component vLayers(j)⋅Qmj⋅fj is no smaller than 4. NOTE: As an example, the value 4 in the component above can correspond to vLayers(j)=1, Qmj= 4 and fj=1.

For low-end Redcap device, e.g., industrial sensor, the following maybe implemented to minimize cost:

- Maximum number of DL layer: 1, correspondingly, vLayers(j)=1.

- Relaxed maximum modulation order: 64QAM, Qmj= 6

As one consequence, the restriction that vLayers(j)⋅Qmj⋅fj is no smaller than 4 for single carrier operation (see highlighted text from TS 38.306 copied above) prevent low-end Redcap devices from using the smaller value of fj=0.4.

Proposal 2: Remove the following restriction in clause 4.1.2 of TS 38.306 for Redcap device:

For single carrier NR SA operation, the UE shall support a data rate for the carrier that is no smaller than the data rate computed using the above formula, with J=1 CC and component vLayers(j)⋅Qmj⋅fj is no smaller than 4.

Assuming the restriction in TS 38.306 would be removed, the achievable maximum data rate with 15kHz SCS is approximately 86*0.4=34Mbp for 20MHz BW and 8Mbps for 5MHz BW, with following configurations:

- fj=0.4,

- Maximum number of DL layer: vLayers(j)=1.

- Relaxed maximum modulation order: 64QAM, Qmj= 6

Therefore, the current smallest scaling factor ‘0.4’ can be leveraged together with 5MHz reduced BW by industrial sensor and low-end video devices to scale down the max data rate supported by the device and thereby reduce its memory size (L2 and HARQ buffers) and interface bandwidth requirements.

On the other hand, given the trivial impacts on RAN1/RAN2 specification to introduce a smaller scaling factor and clear benefit for low-end Redcap devices, the possibility to introduce a smaller value for scaling factor parameter should be studied such that a wider range of useful scaling factors can be explored by Rel-18 eRedcap to cover all the intended use cases.

We therefore proposed the following:

Proposal 3: Study the need of introducing a smaller scaling factor fj=0.1 for Rel-18 eRedcap device.

## 2.3 On Relaxed UE processing time

In Rel-15, two set of UE processing capabilities N1/N2 were defined for NR, i.e., capability 1 and 2. Relaxing the processing time at the UE would reduce the hardware process parallelization at baseband part. Since lower rates need to be supported by the lower-tier target use cases, relaxations to the processing time should be considered.

As one example, capability 2 requires more aggressive processing timeline, which is mainly motivated to support URLLC application to meet the stringent latency requirement (e.g., 2ms E2E latency). This is not necessary for lower-tier eRedcap devices due to relaxed latency requirement as shown in Table 1 above. In addition, a new set of N1/N2 values larger than that of current NR system should be investigated for Rel-18 eRedcap devices, which can be comparable to LTE system with considering the request latency requirements.

Proposal 4: Support relaxed UE processing timelines by defining larger number of N1/N2 values and relaxed CSI computation time for Rel-18 eRedcap UEs.

# 3. Conclusion

In this contribution, we have presented our views on further NR RedCap UE complexity reduction features. Based on the discussions, the following was proposed:

Proposal 1: Support both 15kHz and 30kHz SCS configurations for Rel-18 eRedcap if maximum 5MHz reduced BW is supported.

Proposal 2: Remove the following restriction in clause 4.1.2 of TS 38.306 for Redcap device:

For single carrier NR SA operation, the UE shall support a data rate for the carrier that is no smaller than the data rate computed using the above formula, with J=1 CC and component vLayers(j)⋅Qmj⋅fj is no smaller than 4.

Proposal 3: Study the need of introducing a smaller scaling factor fj=0.1 for Rel-18 eRedcap device.

Proposal 4: Support relaxed UE processing timelines by defining larger number of N1/N2 values and relaxed CSI computation time for Rel-18 eRedcap UEs.

# References

- RP-213661		New SID on Study on further NR RedCap UE complexity reduction

- TR 38.875 V17.0.0		Study on support of reduced capability NR devices (Release 17)

- R1-2110591 	FL summary #5 on other aspects of UE complexity reduction for RedCap.   	   Moderator (Intel Corporation)