3GPP TSG RAN WG1 #109-e		R1-2204256

e-Meeting, May 9th – 20th, 2022

Agenda Item:	9.7.1

Source:	Apple

Title:	On network energy savings performance evaluation

Document for:	Discussion/Decision

# Introduction

A study item on network energy savings [1] has been approved for Rel-18, including the following objectives:

- Definition of a base station energy consumption model [RAN1]

- Adapt the framework of the power consumption modelling and evaluation methodology of TR38.840 to the base station side, including relative energy consumption for DL and UL (considering factors like PA efficiency, number of TxRU, base station load, etc), sleep states and the associated transition times, and one or more reference parameters/configurations.

- Definition of an evaluation methodology and KPIs [RAN1]

- The evaluation methodology should target for evaluating system-level network energy consumption and energy savings gains, as well as assessing/balancing impact to network and user performance (e.g. spectral efficiency, capacity, UPT, latency, handover performance, call drop rate, initial access performance, SLA assurance related KPIs), energy efficiency, and UE power consumption, complexity. The evaluation methodology should not focus on a single KPI, and should reuse existing KPIs whenever applicable; where existing KPIs are found to be insufficient new KPIs may be developed as needed.

Note: WGs will decide KPIs to evaluate and how.

In this contribution, we provide our views on the performance evaluation methodology and KPIs.

# Evaluation Methodology

The following example scenarios are provided in the SID as the potential targeted scenarios for this study:

- Urban micro in FR1, including TDD massive MIMO (note: this scenario can also model small cells)

- FR2 beam-based scenarios (note: this scenario can also model small cells)

- Urban/Rural macro in FR1 with/without DSS (no impact to LTE expected in case of DSS)

- EN-DC/NR-DC macro with FDD PCell and TDD/Massive MIMO on higher FR1/FR2 frequency

Even though these may be some widely deployed scenarios that are of interest for network energy savings, from evaluation point of view, we need to choose a minimum set of representative cases to make it informative but not unnecessarily over-complicated. Among the scenarios listed above, we think the first two scenarios, urban micro in FR1 and FR2 beam-based scenarios, can be considered as prioritized. Both of these scenarios can be evaluated using the dense urban scenario in TR 38.802, which was also used for UE power saving evaluation (TR 38.840) in R16/17.

Proposal 1: Use dense urban scenario (TR 38.840) for network energy savings evaluation.

For traffic model, at least FTP should be considered. It is not necessary to consider URLLC type of traffic for this SI. Whether additional traffic types should be included for evaluation can be further discussed.

Proposal 2: For traffic model, at least FTP should be considered.

Regarding the base station power model, we can maximally reuse the UE power model defined in TR 38.840, e.g. by defining a few different sleep states and the corresponding transition time/energy required. The model should focus on the energy consumption related to air interface operation only. A few aspects that are important to be discussed specifically for the base station includes:

- The relative power consumption values for different sleep states, transmission and reception, including the transition time/energy

  - This is similar to UE power model, but the actual values need to be determined for the base station.

- The power consumption vs the number of PRBs being actively used

  - This relationship needs to be understood to properly model the base station energy consumption with light-to-medium load.

- The power consumption vs the number of carriers (in case of CA) and the carrier bandwidth

- The power consumption vs the number of antenna elements/ports being actively used

  - This is important for understanding the potential benefit of spatial domain adaptation.

Proposal 3: Maximally leverage the UE power model defined in TR 38.840, focusing the discussion on determining the relative power consumption values for different sleep/active states, the dependency on the number of carriers, carrier bandwidth, the number of PRBs being used, and the number of antenna elements being used.

# KPIs

For the KPIs, it is expected that some of the techniques may have a wide impact on different performance aspects, as suggested in the objective description already.

Since the focus is on light-to-medium load, the spectral efficiency is not a critical performance metric in our view as there are sufficient resources to be used in the system and the immediate goal is not to maximize the efficiency. However, if the spectral efficiency is reduced due to some techniques, it will use more resources in time and/or frequency, which will be reflected in the network/UE power consumption. In this sense, we do not see spectral efficiency as a critical KPI to be included.

From user experience perspective, for FTP traffic, UPT should be included as one of the KPIs. If we additionally consider other traffic model, the corresponding performance metrics should be included as well (e.g. latency for VoIP), which can be further discussed.

Another important factor is the impact on UE power consumption, which should be included as one of the KPIs. UE power consumption directly affects user experience, and the network energy saving techniques should not be built on the cost of degraded user experience.

Some of the techniques may impact the initial access procedure, in which cases the initial access performance should be considered. Some of the other techniques may have direct coverage impact, in which cases the related metrics such as the coverage, handover performance, and call drop rate should be considered.

Proposal 4: Other than network energy consumption/efficiency, the KPIs should at least include:

- UPT for FTP traffic

  - FFS for other types of traffic if agreed

- UE power consumption

- Coverage, initial access performance, handover performance and call drop rate, if applicable to the proposed techniques

For the evaluation, it is important to note that the baseline for comparison should also consider what the gNB can do by implementation already without specification support.

Proposal 5: The baseline for performance evaluation should also consider the energy saving techniques that can be done purely by gNB implementation.

# Conclusion

In this contribution, we discussed the evaluation methodology for network energy savings and proposed the following:

Proposal 1: Use dense urban scenario (TR 38.840) for network energy savings evaluation.

Proposal 2: For traffic model, at least FTP should be considered.

Proposal 3: Maximally leverage the UE power model defined in TR 38.840, focusing the discussion on determining the relative power consumption values for different sleep/active states, the dependency on the number of carriers, carrier bandwidth, the number of PRBs being used, and the number of antenna elements being used.

Proposal 4: Other than network energy consumption/efficiency, the KPIs should at least include:

- UPT for FTP traffic

  - FFS for other types of traffic if agreed

- UE power consumption

- Coverage, initial access performance, handover performance and call drop rate, if applicable to the proposed techniques

Proposal 5: The baseline for performance evaluation should also consider the energy saving techniques that can be done purely by gNB implementation.

# Reference

- RP-220297, Revised SI: Study on network energy savings for NR, Huawei, RAN#95e, Mar. 2022.