3GPP TSG RAN WG1 #109-e		R1-2204257

e-Meeting, May 9th – 20th, 2022

Agenda Item:	9.7.2

Source:	Apple

Title:	Discussion on Network energy saving techniques

Document for:	Discussion/Decision

# Introduction

A study item on network energy savings [1] has been approved for Rel-18, with one of the objectives being:

Study and identify techniques on the gNB and UE side to improve network energy savings in terms of both BS transmission and reception, which may include:

- How to achieve more efficient operation dynamically and/or semi-statically and finer granularity adaptation of transmissions and/or receptions in one or more of network energy saving techniques in time, frequency, spatial, and power domains, with potential support/feedback from UE, and potential UE assistance information [RAN1, RAN2]

- Information exchange/coordination over network interfaces [RAN3]

Note: Other techniques are not precluded

In this contribution, we provide our initial views on what potential techniques can be investigated to improve network energy savings.

# Network energy saving techniques

There are different dimensions that can be investigated for network energy saving, including time, frequency, spatial, and power domains.

Time domain

Generally speaking, time domain adaptation, if possible, would provide the most energy saving by shutting off some or most of the components. Therefore, this is an important aspect to investigate.

For time domain, the gNB already has the flexibility to decide when to schedule UE data, and it can perform time domain adaptation by implementation. So the investigation should focus on the possible specification enhancements that can provide gNB some additional tools/information which allows gNB to perform time domain adaptation more effectively/efficiently than today.

For example, there are certain signaling that cannot be skipped, e.g. SSB. In addition, there may be some periodic DL/UL signaling being configured, such as CSI-RS/TRS/SRS. These signals should be transmitted or received, unless e.g. a different direction or ‘flexible’ is indicated by the gNB via dynamic SFI. All these signals may prevent gNB from transitioning into deep sleep state. It can be potentially useful to allow the gNB to indicate a complete Tx and/or Rx off state for a duration when the gNB does not intend to transmit and/or receive at all, which can potentially include the pause of the SSB transmissions. This would allow the gNB to enter deeper sleep state with longer idle duration, thus saving more power. Compared to the existing dynamic SFI, the new signaling can have more flexibility, because it does not have to follow or be restricted by any of the DL/UL slot format indication signaling that is already configured/transmitted.

Frequency domain

In a single-carrier case, the gNB has (almost) all the flexibility in determining the bandwidth to use, which PRBs to schedule within the bandwidth, etc. No further enhancements are needed.

In case of multi-carrier operation, the network can operate with one carrier in a lightly loaded case, and can use more carriers as needed. There are already some features available, e.g. fast SCell activation, that can facilitate network power saving while minimizing performance impact.

Overall we think the room for additional enhancements in frequency domain is limited. One potential investigation area that has been mentioned is SCells without SSB for intra-band CA. This allows the SCells to reduce the transmission when there is no data, which saves power. However, this is mostly RAN4 work, and RAN4 involvement is required to study the feasibility.

Spatial domain

As the network power consumption should heavily depends on the antenna configuration (especially for massive MIMO) used for transmission/reception, spatial domain adaptation is another area that can be used to achieve the network energy saving. This includes the adaptation on the number of Tx/Rx antenna elements/ports and the number of panels (if applicable).

One such example is the more dynamic change on the beam width, depending on how many antenna elements are activated. More antenna elements would allow narrower beam. But if there are a smaller number of UEs in the cell, the gNB can choose to reduce the number of antenna elements and use wider beam instead. Such adaptation is already possible by gNB implementation in a relatively slow fashion with the current specifications. The investigation should focus on the dynamic adaptation which allows the gNB to react to the traffic characteristics faster.

For more dynamic spatial adaptation, the potential impacted areas include SSB adaptation, CSI-RS adaptation, potential CSI measurement/reporting enhancements, and beam management, etc.

Figure 1 An example of base station antenna port adaptation

Power domain

It is typically assumed so far that PSD is kept constant for the base station. Power adaptation is being used in some sense already, e.g. when only some PRBs out of the whole bandwidth need to be used in a relatively low load case. Further power adaptation may involve reduced PSD. However, the benefit of power domain adaptation depends on how sensitive the base station power consumption is to the transmit power or PSD. Typically reducing the power would require longer time to deliver the same amount of information, and it may not necessarily provide a net gain to transmit with a lower power but with longer duration. This needs to be carefully investigated if power domain adaptation is to be considered.

Based on these discussions, we think we should mainly focus on time domain and spatial domain adaptation techniques. Of course, any techniques may have some performance impact and UE impact (such as complexity and power consumption), which needs to be carefully analyzed.

Proposal: At least the following enhancements can be considered further:

- Time domain adaptation

  - Techniques to enable the network to turn on/off more dynamically, flexibly, and/or for a longer period of time

- Spatial domain adaptation

  - Dynamic on/off of Tx/Rx ports and/or panels

    - Potential beam management enhancements, potential CSI enhancements, and signaling aspects

- Performance and UE impact needs to be carefully considered.

Observation: For frequency domain adaptation, if there is interest to pursue SCells without SSB for inter-band CA, RAN4 investigation on feasibility is needed.

# Conclusion

In this contribution, we discuss the possible enhancements for network energy savings, and propose the following:

Proposal: At least the following enhancements can be considered further:

- Time domain adaptation

  - Techniques to enable the network to turn on/off more dynamically, flexibly, and/or for a longer period of time

- Spatial domain adaptation

  - Dynamic on/off of Tx/Rx ports and/or panels

    - Potential beam management enhancements, potential CSI enhancements, and signaling aspects

- Performance and UE impact needs to be carefully considered.

Observation: For frequency domain adaptation, if there is interest to pursue SCells without SSB for inter-band CA, RAN4 investigation on feasibility is needed.

# Reference

- RP-220297, Revised SI: Study on network energy savings for NR, Huawei, RAN#95e, Mar. 2022.