3GPP TSG RAN WG1 #109-e		R1-2204258

e-Meeting, May 9th – 20th, 2022

Source: 	Apple Inc.

Title:                     Potential Side Control Information for NW-Controlled Repeater

Agenda item:	9.8.1

Document for:	Discussion and Decision

# 1 Introduction

In RAN#94, a new study item on NR Network-controlled Repeaters was approved [1] to efficiently extend the cell coverage by leveraging some side control information. According to [1], Rel-18 NW-Controlled Repeater (NC-Repeater) expects to have a lower cost/complexity compared to legacy IAB and a better coverage performance over a conventional RF repeater, focusing on the following scenarios and assumption:

- Network-controlled repeaters are inband RF repeaters used for extension of network coverage on FR1 and FR2 bands, while during the study FR2 deployments may be prioritized for both outdoor and O2I scenarios. - For only single hop stationary network-controlled repeaters - Network-controlled repeaters are transparent to UEs - Network-controlled repeater can maintain the gNB-repeater link and repeater-UE link simultaneously NOTE1: Cost efficiency is a key consideration point for network-controlled repeaters.

In addition, a variety of side control information were identified for further study as follows [1]:

Study and identify which side control information below is necessary for network-controlled repeaters including assumption of max transmission power [RAN1] - Beamforming information - Timing information to align transmission / reception boundaries of network-controlled repeater - Information on UL-DL TDD configuration - ON-OFF information for efficient interference management and improved energy efficiency - Power control information for efficient interference management (as the 2nd priority) Study and identify L1/L2 signaling (including its configuration) to carry the side control information [RAN1]

In this contribution, we discuss various side control information candidates such as beamforming information, UL-DL TDD configuration, ON-OFF operation, and power control as potential means for achieving the best coverage extension objective for FR2. The L1/L2 signaling to carry the potential side control information is not a focus of this contribution and elaborated in our companion paper [2].

# 2. Discussions

The main target use case of Rel-18 NC-Repeater is to offer the coverage extension for gNBs by amplifying and forwarding the received waveform. Same as Rel-17 RF repeater, the NC-Repeater would not be designed to distinguish desired signal from inference and noise to minimize the cost. On the other hand, by leveraging side control information, the NC-Repeater is able to perform the amplify-and-forward operation more efficiently with a better spatial directivity, which additionally limits any unnecessary interference and therefore benefits in terms of interference mitigation.

To facilitate the subsequent discussions, a signal or channel ‘X’ forwarded by the NC-Repeater over Uu Link to the associated UEs was termed as ‘R-X’. For example, SSB or CSI-RS signals that are generated by gNB and forwarded by NC-Repeater are termed as ‘R-SSB’ and ‘R-CSI-RS’. The UE connected to NC-Repeater is termed as ‘R-UE’ as depicted in FIG.1 below. It should be noted that ‘R-SSB’ or ‘R-CSI-RS’ is purely to facilitate the relevant discussions as NC-Repeater is transparent to the R-UE.

Figure 1: NC-Repeater to extend cell coverage based on Side Control Information

Toward the gNB, a NC-Repeater initially operates as a UE and perform initial access and RACH procedure. Once connection over backhaul link is established and the NC-Repeater is configured, the NC-Repeater uses subset of the ‘existing UE’ functionality for communication on the backhaul link and ‘NC-Repeater specific’ function for the Access link. The gNB needs to first identify the NC-Repeater and then provides side control information accordingly to manage the Tx/Rx directions on access link. A simple way is to indicate the device type in capability report. If a clear use case is identified, other means to earlier identify NC-Repeater can be considered (e.g., using dedicated PRACH resource or Msg3.

Proposal 1: A NC-Repeater reports the device type to gNB through capability report or RACH procedure.

## 2.1 Beamforming Information

For higher-frequency band, e.g., FR2, a large number of steerable antenna elements are implemented at the NC-Repeater and used for beamformed transmission and reception to extend the coverage. In addition, packet-data traffic for a given UE is often highly busty, with occasional period of transmission/reception followed by longer period of silence. With L1/L2-based beamforming side control information from the gNB, NC-Repeater is able to dynamically adapt its Tx/Rx beams in the direction of the target UEs and therefore maximize the cell coverage as well as improve the throughput performance.

Proposal 2: Support beamforming side control information for NC-Repeater to enable a dynamic beam adaptation for forwarding links.

Figure 2: Overall Beam Management Procedure in NR

In NR, multiple candidate SSBs are transmitted in different spatial directions (i.e., using different DL spatial beams) spanning the coverage area of a cell. A beam sweeping of multiple R-SSBs is required for NC-Repeater such that R-UEs can acquire the time and frequency synchronization during cell search, determine the cell ID and complete the initial beam establishment. Similar as the gNB, it is beneficial to allow a NC-Repeater implementing different set of beamforming coefficients and beam width for SSB and CSI-RS. The layout of the refined beams within a wider R-SSB beam has a dependence upon the antenna configuration of NC-Repeater. To facilitate a selection of the number of R-SSBs/R-CSI-RS resources and corresponding sweeping beams at gNB (e.g., to cover a certain intended area), a NC-Repeater may report its antenna array configuration (e.g., using <N1, N2, O1, O2> structure) to the gNB.

Proposal 3: For initial beam establishment of access link, study whether the beamforming information for R-SSB and R-CSI-RS sweeping is controlled by gNB using side control information to cover a certain intended area.

Proposal 4: Study the need of reporting antenna array configuration to gNB to facilitate the R-SSB and R-CSI-RS resource allocation and beam directions.

The R-SSBs and R-CSI-RS can be further used as source RS to indicate the Tx/Rx beam for DL and UL over access link, respectively. Like TCI-state configuration in Rel-15, a list of beam indications state (BI-State) can be first configured in a semi-static manner based on the structure of Table 1. The beam directions of access link can be dynamically indicated by BI-state in a DCI format over backhaul link depending on the location/direction of intended R-UEs.

Table 1: A beam state structure for beam direction indication

Proposal 5: Support a dynamic indication of Tx/Rx beams for access link to improve the efficiency and reduce latency.

## 2.2 On-Off Operation for Power Saving and Interference Management

In both LTE and NR, a DRX mode has been specified to reduce UE battery power consumption. The basic principle of C-DRX is that when there is no DL/UL transmission scheduled during in a certain time duration of inactivity timer, UE enters a DRX mode, and wakes up only periodically to monitor PDCCH. In Rel-16, the C-DRX operation was further enhanced to allow UE skipping the entire DRX cycle(s) based on the detected WUS.

In order not to interfere with other nodes more than necessarily and reduce power, a NC-repeater should preferably not transmit more than needed for the terminals that it serves. One way of achieving this would be to extend DRX operation for NC-repeater. In addition, similar as Rel-16 PDCCH-based power adaptation, a WUS signal can be used to dynamically on-off NC-Repeater DRX cycle based on traffic demanding. As depicted in FIG.3 below, a WUS can be configured to monitor within a window, which indicates whether the corresponding DRX cycle can be power off such that the power consumption in time domain can be reduced for NC-Repeater.

Figure 3: DRX extension for NC-Repeater to achieve ‘on-off’

We therefore propose the following:

Proposal 6: Study potential DRX functionality extension for NC-Repeater to reduce power consumption.

In addition, frequency selectivity function e.g., based on existing carrier aggregation or BWP framework, can be also considered, where some anchor carrier/BWP could be always available as long as the NC-Repeater is active and other carrier/BWPs could be dynamically on-off in case of need.

Proposal 7: Study the feasibility to support either CA or BWP-based frequency domain power adaptation.

# 3. Conclusion

In this contribution, we have presented our views on various side control information candidate. Based on the discussions, we proposed the following:

Proposal 1: A NC-Repeater reports the device type to gNB through capability report or RACH procedure.

Proposal 2: Support beamforming side control information for NC-Repeater to enable a dynamic beam adaptation for forwarding links.

Proposal 3: For initial beam establishment of access link, study whether the beamforming information for R-SSB and R-CSI-RS sweeping is controlled by gNB using side control information to cover a certain intended area.

Proposal 4: Study the need of reporting antenna array configuration to gNB to facilitate the R-SSB and R-CSI-RS resource allocation and beam directions.

Proposal 5: Support a dynamic indication of Tx/Rx beams for access link to improve the efficiency and reduce latency.

Proposal 6: Study potential DRX functionality extension for NC-Repeater to reduce power consumption.

Proposal 7: Study the feasibility to support either CA or BWP-based frequency domain power adaptation.

# References

- RP-213700	 	Study on NR Network-controlled Repeaters	ZTE

- R1-2204259	Discussion on L1/L2 signaling for side control information	Apple