3GPP TSG- RAN WG1 Meeting #109-e                                             R1- 2204259

e-Meeting, May 9th – 20th, 2022

Agenda Item:	9.8.2

Source:	Apple Inc.

Title:	Discussion on L1/L2 signaling for side control information

Document for:	Discussion/Decision

# Introduction

In RAN#94e, a new SI has been approved for network-controlled repeaters in NR Rel-18. One of the main objectives of the SI includes signaling of side control information between the network and repeater as follows [1]:

Study and identify which side control information below is necessary for network-controlled repeaters including assumption of max transmission power [RAN1]

- Beamforming information

- Timing information to align transmission / reception boundaries of network-controlled repeater

- Information on UL-DL TDD configuration

- ON-OFF information for efficient interference management and improved energy efficiency

- Power control information for efficient interference management (as the 2nd priority)

Study and identify L1/L2 signaling (including its configuration) to carry the side control information [RAN1]

Study the following aspects of network-controlled repeater management

- Identification and authorization of network-controlled repeaters [RAN2, RAN3]

NOTE2: Coordination with SA3 may be needed.

In this contribution, we provide our initial views on the signaling framework design required for side control information exchange between the network and repeaters.

# Discussion

## Views on signaling methods for side control information exchange

In our companion contribution [2], we discuss what potential side control information may be needed for network-controlled repeaters and in this contribution, we consider aspects related to how such information can be exchanged between network and repeater. Depending on specific side control information, the signaling could either be semi-static, dynamic or a combination of both. To enable dynamic signaling for side control information, one possibility is to introduce similar framework as for downlink control information exchange between network and UE, i.e., to encode the side control information into a format (DCI-like format) and use a physical control channel (PDCCH-like) for transmission. RAN1 should study the benefits and implications of introducing similar framework as downlink control for side control interface between network and repeater.

Proposal 1: Study the benefits and implications of introducing downlink control like framework for exchange of side control information between network and repeater.

Considering a downlink control like framework for side control information exchange, the design of control information format will need to be considered. Either existing DCI formats could be utilized to repurpose for exchange of side control information, or a new format could be considered. In our view, one of the key differences between the side control information and typical downlink control information is that repeater is not expected to receive any control information for encoding or decoding of physical channels that are just received and forwarded by repeater. Based on this understanding, the information fields in the existing DCI formats can be considered irrelevant for the side control information format.

Observation 1: For network-controlled repeaters, the side control information format, if introduced for the network-repeater link, is not expected to carry any information that is required for encoding/decoding physical channels for forwarding, unlike the scheduling DCI formats

Proposal 2: Study the need to introduce any new formats for side control information exchange between network and repeater.

If a new side control information format is possibly needed, then it needs to be discussed that which information fields will be included and how the exact side control information will be indicated using the control format. Based on the SID, it is quite likely that least following information will need to be indicated dynamically:

- Beamforming information for the repeater

- Timing alignment between reception and forwarding at the repeater

- TDD UL/DL information

In addition, other information relevant from power saving and interference management point of view might also need to be indicated. In our view, a good starting point on designing specific information fields in the side control information format should be to consider the similar information fields in the existing DCI formats as the baseline. For example, currently TCI framework with QCL assumption type-D is typically used to indicate the beams at the UE. Similar framework could possibly be extended for network-controlled repeaters.

Proposal 3: For network-controlled repeaters, if a new side control information format is considered, then the relevant information fields in the existing DCI formats should be used as baseline for indicating similar information via side control information format.

## Views on physical control channel design for repeaters

To facilitate the transmission of a side control information format, a physical channel is needed, like PDCCH. One direction could be simply reusing the existing PDCCH design framework in terms of monitoring, search space configuration, CORESETs, beamforming, etc. The PDCCH has been designed to be quite flexible and complex from UE implementation point of view. The motivation for the flexible and complex design of PDCCH has been to serve various use-cases and operations. However, a physical control channel for the purpose of side control information may not need such complex and flexible design. One of the main differences is that for the network-repeater link, both nodes are fixed and therefore the channel conditions will be typically static and time invariant. Moreover, the network planning would quite possibly account for a good quality link with LoS between gNB/TRP and repeaters. Moreover, multiple side control information formats might not be needed. Considering the reasonably fixed conditions, the control channel design for network-repeater link can be significantly simplified in comparison to PDCCH design for downlink control information.

Observation 2: For network-controlled repeaters, the design for the physical control channel for transmitting side control information format is not expected to serve large number of use-cases, unlink PDCCH.

Proposal 4: For network-controlled repeaters, if downlink control like framework is adopted, then RAN1 should strive to have a simplified control channel design in terms of supported parameters and configuration, for the transmission of side control information from gNB/TRP to repeaters.

# Conclusion

In this contribution, we have discussed our views on signaling framework design for exchange of side control information between network and repeater and have provided following observations/proposals:

Observation 1: For network-controlled repeaters, the side control information format, if introduced for the network-repeater link, is not expected to carry any information that is required for encoding/decoding physical channels for forwarding, unlike the scheduling DCI formats

Observation 2: For network-controlled repeaters, the design for the physical control channel for transmitting side control information format is not expected to serve large number of use-cases, unlink PDCCH.

Proposal 1: Study the benefits and implications of introducing downlink control like framework for exchange of side control information between network and repeater.

Proposal 2: Study the need to introduce any new formats for side control information exchange between network and repeater.

Proposal 3: For network-controlled repeaters, if a new side control information format is considered, then the relevant information fields in the existing DCI formats should be used as baseline for indicating similar information via side control information format.

Proposal 4: For network-controlled repeaters, if downlink control like framework is adopted, then RAN1 should strive to have a simplified control channel design in terms of supported parameters and configuration, for the transmission of side control information from gNB/TRP to repeaters.

# Reference

- RP-213700, “New SI: Study on NR Network-controlled Repeaters”, ZTE Corporation

- R1-2204258, “Potential side control information for NW-controlled repeater”, Apple Inc.