3GPP TSG RAN WG1 #109-e	                                         	           R1-2204262

e-Meeting, May 9th – 20th, 2022

Agenda Item:	9.10.1

Source:	Apple Inc.

Title:	On multi-cell PUSCH/PDSCH scheduling with a single DCI

Document for:	Discussion/Decision

# Introduction

In Rel-18, a work item was approved for multi-carrier enhancements (WID in RP-220834 [1]), and one of the objectives is to specify multi-cell scheduling DCI as follows:

1. Specify a solution for multi-cell PUSCH/PDSCH scheduling (one PDSCH/PUSCH per cell) with a single DCI [RAN1] - Identify the maximum number of cells that can be scheduled simultaneously - Consider both intra-band and inter-band CA operation - Consider both FR1 and FR2 - The single DCI shall be optimized for 3 or more cells for the multi-cell PUSCH/PDSCH scheduling

In this contribution, we discuss the issues that need to be addressed for multi-cell scheduling DCI.

# Discussions

As part of the objective, we need to identify the maximum number of cells that can be scheduled by a single DCI. This can be potentially determined at a later stage, but considering the following factors, we can make some preliminary target already:

- The WID says “the single DCI shall be optimized for 3 or more cells”.

- With the current polar code interleaver design for PDCCH (with interleaver length of 164), the DCI payload size (excluding CRC) should not exceed 140 bits.

- Larger number of cells scheduled by a DCI means larger DCI size and higher PDCCH load on the scheduling cell. The maximum number should not be too large to avoid potential PDCCH coverage and/or PDCCH capacity issue.

  - Regardless of the DCI design, there will be some DCI fields individually signalled per cell. Otherwise, there would be too much scheduling constraints, which affects the performance and could defeat the purpose of DL control overhead reduction.

  - It would be desirable not to increase the DCI size too much to avoid the potential PDCCH coverage issue.

- The motivation is mainly to address the fragmented spectrum. The number of segments of fragmented spectrum should not be too large.

With these considerations, we think from RAN1 specification perspective, the maximum number of scheduled cells should not be more than 4. Whether 4 cells can be supported depends on the discussion outcome of DCI content design (e.g., which fields are separate or common). In addition, the exact number of cells supported should depend on UE capability, the band or band combination, etc, which can be discussed in detail in a later stage.

Proposal 1: RAN1 specifications support a maximum of 3 or 4 cells (FFS 3 or 4) that can be scheduled simultaneously by a single DCI for both PDSCH and PUSCH.

- The actual maximum of cells scheduled by a single DCI should depend on UE capability and the band/band combinations.

In addition, as a high-level principle, multi-cell scheduling DCI should not introduce any out-of-order scheduling or out-of-order HARQ-ACK at least for single-TRP operation. For multi-TRP, it can be further discussed whether/how it is supported together with multi-cell scheduling DCI.

Proposal 2: Multi-cell scheduling DCI shall not introduce out-of-order PDSCH/PUSCH scheduling or out-of-order HARQ-ACK for any scheduled cell at least for single-TRP operation.

## DCI contents

One of the main design issues for the multi-cell scheduling DCI is the DCI contents. Given that multiple PDSCHs/PUSCHs on multiple cells are scheduled by a single DCI, we need to discuss whether each DCI field is separately indicated for each cell, or shared by multiple cells. If a DCI field is separately indicated for each cell, whether/how overhead optimization can be done to reduce the DCI overhead. This requires the per-DCI field analysis. For NR-U multi-PUSCH scheduling or multi-PDSCH/PUSCH scheduling for above 52.6GHz, the multiple PDSCHs/PUSCHs occur on the same cell, and some field values (e.g. MCS, FDRA, etc.) can be naturally shared. For multi-cell scheduling DCI, the conditions on each cell can be completely different, and it may no longer be appropriate to share the same field (e.g. can be overly restrictive and lead to significant performance degradation).

For PDSCH, the fields related to HARQ-ACK feedback are good candidates to be considered as shared by multiple PDSCHs. This includes PDSCH-to-HARQ_feedback timing indicator, PUCCH resource indicator, TPC command for scheduled PUCCH. By sharing these fields, it means that the HARQ-ACK feedback for the multiple PDSCHs scheduled by a single DCI is transmitted in the same PUCCH in the same HARQ-ACK codebook. This saves the DCI overhead without much impact on the performance. This could also simplify the Type-2 HARQ-ACK codebook construction as we discuss later.

Proposal 3: The multiple cells scheduled by a single DCI belong to the same PUCCH group, and the HARQ-ACK feedback for the PDSCHs scheduled by a single DCI is transmitted in the same PUCCH.

- There is a single field shared by all the PDSCHs for PDSCH-to-HARQ_feedback timing indicator, PUCCH resource indicator, and TPC command for scheduled PUCCH in the multi-cell scheduling DCI.

For many other scheduling related fields, such as FDRA, TDRA, MCS, NDI, RV, TCI, SRI, etc, sharing the field values may result in significant performance degradation, therefore these fields will require careful considerations.

Proposal 4: Further investigate whether to indicate the following fields separately for multiple PDSCHs/PUSCHs

- FDRA

- TDRA

- MCS

- NDI

- RV

- TCI

- SRI

## Type-2 HARQ-ACK codebook construction

Type-2 HARQ-ACK codebook needs to be enhanced to support multiple PDSCHs on multiple cells being scheduled by a single DCI. This has some similarities to the multi-PDSCH scheduling for above 52.6GHz. Therefore, we think the Type-2 HARQ-ACK codebook construction for above 52.6GHz can be used as the starting point for the design here. The basic idea is to have two sub-codebooks, one for single-PDSCH scheduling DCIs, and another one for multi-PDSCH scheduling DCIs.

Proposal 5: Use the Type-2 HARQ-ACK codebook construction mechanism for above 52.6GHz as the starting point for the Type-2 HARQ-ACK codebook construction with multi-cell scheduling DCI.

## DCI and number of BDs/CCEs

It needs to be considered whether a new DCI format is introduced or not. Depending on this decision, the considerations on the number of BDs/CCEs and the number of DCI sizes can be different.

There are two ways to specify the multi-cell scheduling DCI:

- Option 1: modify the existing DCI formats (e.g. 0_1/0_2/1_1/1_2) by modifying the existing fields and/or adding new fields

  - It should be noted that DCI formats 0_2/1_2 were designed for compact DCI size to ensure the PDCCH reliability for URLLC. Multi-cell scheduling DCIs will significantly increase the DCI size, and do not fit the purpose of DCI formats 0_2/1_2. Therefore, the multi-cell scheduling DCIs should be designed based on DCI formats 0_1/1_1.

- Option 2: introduce new DCI formats

Whether there is a need to introduce a new DCI format largely depends on whether we want to use multi-cell scheduling DCI and the legacy single-cell scheduling DCI simultaneously on the same cell (based on the same set of configurations). If yes, new DCI formats would be needed in order to differentiate the two. Otherwise, Option 1 would be sufficient.

As a simple example, consider a UE with 4 CCs, and multi-cell scheduling DCI is configured on CC1 that can schedule PDSCHs/PUSCHs on the 4 CCs. If we still want to support the legacy DCI format 0_1/1_1 on CC1 to perform self-scheduling at the same time, new DCI formats for multi-cell scheduling would be necessary.

If new DCI formats are introduced, the limit on the number of DCI sizes needs to be considered (e.g. whether additional DCI size alignment procedures are needed).

Proposal 6: The multi-cell scheduling DCI formats are designed based on DCI formats 0_1 and 1_1.

- FFS whether to introduce new DCI formats or modify DCI formats 0_1/1_1, and potentially the handling of the limit on the number of DCI sizes

How to count and define the limit for the number of BDs/CCEs also needs to be investigated. In legacy design, BDs/CCEs are counted for each scheduled cell and subject to the limit per scheduled cell. Now that one DCI can schedule multiple cells, towards which cell(s) we count these BDs/CCEs becomes a question. Further if one cell can be scheduled by DCIs on multiple cells (e.g. self-scheduling and scheduled by a multi-cell DCI on another cell), it also needs to be considered whether/how to split the BD/CCE budget between the multiple cells.

Proposal 7: Study the handling of BD/CCE limit, and whether one cell can be scheduled by multiple cells.

# Conclusion

In contribution, we provide our views on multi-cell PUSCH/PDSCH scheduling with a single DCI and propose the following:

Proposal 1: RAN1 specifications support a maximum of 3 or 4 cells (FFS 3 or 4) that can be scheduled simultaneously by a single DCI for both PDSCH and PUSCH.

- The actual maximum of cells scheduled by a single DCI should depend on UE capability and the band/band combinations.

Proposal 2: Multi-cell scheduling DCI shall not introduce out-of-order PDSCH/PUSCH scheduling or out-of-order HARQ-ACK for any scheduled cell at least for single-TRP operation.

Proposal 3: The multiple cells scheduled by a single DCI belong to the same PUCCH group, and the HARQ-ACK feedback for the PDSCHs scheduled by a single DCI is transmitted in the same PUCCH.

- There is a single field shared by all the PDSCHs for PDSCH-to-HARQ_feedback timing indicator, PUCCH resource indicator, and TPC command for scheduled PUCCH in the multi-cell scheduling DCI.

Proposal 4: Further investigate whether to indicate the following fields separately for multiple PDSCHs/PUSCHs

- FDRA

- TDRA

- MCS

- NDI

- RV

- TCI

- SRI

Proposal 5: Use the Type-2 HARQ-ACK codebook construction mechanism for above 52.6GHz as the starting point for the Type-2 HARQ-ACK codebook construction with multi-cell scheduling DCI.

Proposal 6: The multi-cell scheduling DCI formats are designed based on DCI formats 0_1 and 1_1.

- FFS whether to introduce new DCI formats or modify DCI formats 0_1/1_1, and potentially the handling of the limit on the number of DCI sizes

Proposal 7: Study the handling of BD/CCE limit, and whether one cell can be scheduled by multiple cells.

# Reference

[1] RP-220834, Multi-carrier enhancements for NR, RAN#95e, Mar. 2022.