3GPP TSG RAN WG1 #109-e	                                    	   R1-2204266

e-Meeting, May 9th -20th, 2022

Agenda Item:	9.11.3

Source:	Apple Inc.

Title:	Considerations on enhancements for XR

Document for:	Discussion/Decision

# 1. Introduction

In this contribution we provide our views on potential enhancements for XR in Rel-18.

# XR use cases and enhancements

eXtended Reality (XR) is a broad term covering Augmented Reality (AR), Mixed Reality (MR) and Virtual Reality (VR). Along with Cloud Computing, XR applications typically require high throughput and low latency. A part of end-to-end packet latency is contributed by those from radio network and transport network. With Edge Computing, the proportion of latency contributed by other parts in the end-to-end service can be reduced, and it makes more readily achievable for low latency.

The 5G system has been designed to deliver eMBB, URLLC and mMTC services. For NR, support of eMBB and URLLC was introduced in Rel-15, enhancements have been made in Rel-16 and are under way in Rel-17.

If the XR use cases are not distinct enough from existing eMBB/URLLC use cases, XR-specific enhancements are hardly justified, as there are many already for generic eMBB/URLLC use cases. From discussion in [5][6], SA4 study[8][9][10], and cited studies from [3][4], we do see some unique characteristics in the traffic profile of XR applications. Then XR-specific traffic characteristics [5], especially multiple periodic data streams with variable packet sizes as depicted in Figure 1 and Figure 2, should be the driving motif for NR enhancements for XR [12][14]. We have

Proposal 1: Rel-18 NR enhancements for XR should be motivated by XR services’ traffic characteristics, especially the multiple data flow aspects.

Figure 1 Multiple data flows in XR traffic

## Issues and potential enhancements

Achieving low latency, high throughput and high reliability is hard enough, even with abundant band-width supported by NR. When UE power consumption and system capacity, which can be quantized by the average number of satisfied UEs engaged in XR services in a cell, are considered, the problem is even harder. In our view, the Rel-18 XR enhancements for NR is about leveraging the findings from RAN1 and SA4 on XR traffic characteristics.

UE power consumption can be incurred for

- PDSCH/PUSCH processing

- PDCCH monitoring

- CSI feedback

- RRM and RLM

Among them, except item 4, which does not depend on the traffic characteristics, all other items have strong dependence on the traffic characteristics. With multiple flows at DL (the consideration for UL can be similarly developed), the gNB can choose to use separate PDSCHs to carry higher layer packets with different QoS requirements, or the gNB can use a single PDSCH to carry higher layer packets. Both approaches have their pros and cons. With the first approach, essentially TDM is used for different data flows, the latency for data flows can be long, and the UE’s on-time may be long and consequently UE power consumption can be large, yet the first approach does allow gNB much freedom in giving different treatments for different data flows, e.g. OLLA, retransmission, etc. The second approach may be preferred as the on-time for the UE can be shorter than that with the first approach. However, as different data flows have different QoS requirements, how to address different latency budgets for different data flows in a single PDSCH may require further investigation.

Ideally, when there is no uplink traffic or downlink traffic, there should be no activity on the UE side to minimize power consumption. Of course, some minimal functionalities need to be kept for the UE in the connected mode to allow nimble response from the UE once traffic arrives. Assume basic PDCCH monitoring for network connection is maintained by the UE, through CSS monitoring with CORESET #0 for example, then other PDCCH monitoring activities should be reduced as much as possible. Then when traffic arrives, the accompanying demand on PDCCH monitoring (e.g. for dynamic signaling for scheduling, retransmission for SPS/CG, etc) can be accommodated. In the current NR design, PDCCH monitoring is controlled through cDRX, PDCCH skipping and search space switching; whether they are suitable for multiple traffic flows with non-integer periodicity should be investigated.

### Traffic periodicity

From SA4 and RAN1 discussions, we can see support of video traffic is a salient component in XR service.  Video traffic has some unique characteristics:

- Non-integer periodicity, video at 60, 90 and 120 frames/second

- Time-varying packet size, e.g. due to I-frame or P-frame/B-frame

Figure 2 Multiple data streams with periodical traffic arrivals and time-varying packet size

As video frames can be generated at regular time epochs, DL SPS and UL configured grant should be the first candidates considered to accommodate the XR traffic in general, and video traffic in particular. However, the Rel-16 NR design has some limitations. Note the DL SPS periodicity and UL configured grant periodicity available in Rel-16 do not match well with video traffic’s:

- Supported periodicity for DL SPS in Rel-16 NR:

  - {1, …, 640} milliseconds for a NR system at 15 KHz subcarrier spacing,

  - {1/2, 1, 3/2, …, 640} milliseconds at 30 KHz, etc.

  - …

- Supported periodicities for UL configured grant in Rel-16 NR:

  - Multiple of 1 millisecond for 15 KHz up to 640 milliseconds, 2 symbols (1/7 milliseconds), 7 symbols (0.5 milliseconds)

  - Multiple of 1/2 millisecond for 30 KHz up to 640 milliseconds, 2 symbols (1/14 milliseconds), 7 symbols (0.25 milliseconds)

- With video frames generated at 30, 60, 90, 120 Hz, out of many periodicities supported currently in NR for SPS and CG, none is a good match for any of them.

### Time-varying packet size

When data streams can be generated at cadences which are not integer multiples of the NR’s basic timing (90 frames per seconds), DL SPS with over-provision has been considered a possible solution. As shown in Figure 3, with a number of DL SPS configurations configured for a UE, with some DL SPS occasions with no actual data transmission, the alignment latency in DL transmission can be controlled. However, for the transmission occasions not associated with any actual transmission, the UE still needs to generate HARQ feedback and transmit over the configured PUCCH resource. More importantly, as the UE does not have the a priori knowledge on whether there is actual transmission in a slot or not, some UE processing is still needed to make a determination:

- DMRS correlation

  - e.g. the UE performs correlation with the assumed DMRS to decide whether there is actual transmission or not.

- LDPC decoding

  - Depending on DL SPS’s MCS level (e.g. low MCS level, hence the required SINR can be rather low), the UE may not be able to decide there is actual transmission or not simply from DMRS correlation, further processing such as LDPC decoding may be needed.

In this case, UE power consumption is incurred for non-existent data transmission.

With over-provision, the UE is still required to send HARQ feedback over PUCCH for non-existent SPS PDSCH transmission, which wastes system resources; and leads to UL interference. There are proposals in 3GPP RAN1 to deal with those two issues, e.g. the UE is to skip the HARQ feedback for non-existent data transmission, shown in Figure 4. Note however the UE power consumption issue is not dealt with by the proposed solution with skipping HARQ feedback.

Figure 3 Over-provision for traffic with non-integer periodicity

Figure 4 Skipping of HARQ feedback

With time-varying packet size, while the gNB can always configure a UE with resources sufficient even for the largest packet size. However, that can lead to low system capacity, for example the number of UEs with XR service can be limited.

Also jitter in the traffic arrival for downlink can be expected. With time-varying packet size (or frame size for the video traffic for example), then some kind of signaling mechanism to indicate to the UE the reception occasions with actual data transmission can be beneficial from UE power saving point of view.

For UL, embedded signaling can be used to adapt the transport block size. For example, as shown in Figure 5, CG-UCI can be enhanced to indicate the MCS level or number of symbols in the PUSCH so the transport block size can be adopted according to the current need of XR traffic.

Figure 5 Using CG-UCI to adapt the transport block size

### PDCCH monitoring

From discussion in NR Rel-15/16, it can be seen PDCCH monitoring can account for much of UE power consumption. In Rel-16/Rel-17, PDCCH monitoring is adapted according to the current traffic need through newly introduced mechanisms such as wake-up signal, which can be a rather effective approach to save power if the XR traffic consists of a single flow, and the periodicity of the traffic arrival is not too small. With multiple data flows, adaptation to traffic becomes more challenging as those data flows may have non-commensurate periodicities. Then light-weighted control signaling embedded in the PDSCH reception occasions should be investigated to circumvent PDCCH monitoring and also make control signaling goes with traffic rather than making traffic follows control signaling.

As for DRX enhancements for XR, for a single data flow, matching the DRX periodicity with the periodicity of the packet arrival’s periodicity can be expected to be beneficial. When there are multiple data flows, especially when at least integer relationship does not exist among their periodicities, matching DRX’s periodicity to another data flow’s periodicity would still leave some data flows arrival potentially in DRX-Off duration. Then for potential enhancements in PDCCH monitoring, if dynamic grants are used as the main means to schedule data transmission, multiple DRX configurations or switching between DRX configurations can be considered. Another choice is to rely on SPS PDSCH and CG PUSCH as the main means for data transmission. For an initial SPS PDSCH reception, if an NACK is generated due to failed reception, then it would trigger the UE’s monitoring of PDCCH for retransmission. For some data flow, the latency budget can be tight, the initial transmission through SPS PDSCH may be the only chance of transmission for a transport block, then retransmission for a packet for such a data flow does not add anything to user’s better experience. Also for different data flows, the latency budget can be different, and how to exploit that for SPS/CG transmission should be investigated.

### HARQ/CSI feedback enhancements

CSI enhancements have been studied in the Rel-17 eIIoT/URLLC, without a clear direction where it should go. It can be also observed that proposed enhancements over there are motivated by quite diverse use cases. As XR traffic does provide some unique challenges and opportunities for system design, in our view, a component of CSI enhancements can be also included in the XR work item. Yet the objective should be clearly specified to target XR’s traffic characteristics, so to focus the discussion.

As some traffic stream of XR service can have a stringent latency requirement, the 2nd transmission is the only opportunity for the gNB to provide more coded bits to the UE, so they can be combined with previously received coded bits (LLRs) for successful decoding. As such, if a UE does not decode PDSCH successfully for the first transmission or for a retransmission when the latency bound is in danger of being exceeded, the more relevant information for the UE to provide is not merely the fact the UE fails to decode the transport block, rather how much more redundancy is needed from the gNB to allow the UE to decode the transport block in the next attempt, which can be the only chance for the UE to receive the transport block within the latency bound. From that, it is reasonable to allow the UE to indicate how much redundancy is needed further for the UE to decode the transport block. Also the UE can consider the current status of the soft buffer in its feedback to the gNB. In Rel-17 eIIoT/URLLC, a number of soft NACK/soft ACK schemes have been proposed; some of them target 10^-6 BLER which is not the typically required for XR. Nevertheless, we feel CSI enhancements can be important for achieving low latency & decent system capacity, and some of them may lead to suitable solution for XR.

For CSI measurement, as interference can fluctuate substantially between slots depending on traffic from neighboring cells, interference measurement which faithfully reflects the interference experienced by PDSCH carrying XR traffic should be supported. Periodic and semi-persistent CSI measurement resources matched with XR traffic arrival should be supported. Also for CSI reporting, while it is possible to use aperiodic CSI reporting to match the reporting occasions with XR traffic arrival, it can be also beneficial for periodic and semi-persistent CSI reporting’s occasions to be matched with XR traffic arrival, for faithful CSI measurements.

### Qos enhancements

Broadly we can see there can be roughly two approaches in providing traffic information to RAN:

in Approach 1, only high-level and semi-static traffic information (non packet specific traffic information) are provided to RAN. For each media data flow, with periodicity/offset/packet size distribution, etc., RAN enhancement for XR is already possible. It is noted the Rel-17 XR traffic models serve the study along the line of Approach 1 well, as essentially the traffic parameters in the Rel-17 XR traffic models can map nicely to the parameters which would be useful to RAN. The XR application client  at a UE (UE A) can acquire high-level and semi-static traffic information from XR server or its peer XR application client at another UE (UE B) for the downlink traffic of UE A’s, and the XR application client at UE A is aware its own uplink traffic information, thus the UE is in a position to provide the network with high level and semi-static traffic information for both downlink and uplink.  Hence the enhancement is mostly limited to RAN.

Beyond that, with Approach 2, real-time traffic information (packet-specific traffic information) is provided to RAN. For an IP packet, information such as ADU membership (PDU set), latency budget, remaining packet size, I/P frame/slices identification may provide further benefit, e.g. allowing selective packet discarding, etc. To support Approach 2, enhancements in RAN and core network are both necessary, e.g., what traffic information can be carried over the N6/N3 interfaces in the 5G core architecture.

While it may be intuitively true that the more traffic information is made available to RAN, the better the RAN can accommodate more UEs, the additional benefit of providing detailed traffic information needs to be studied and understood.

It is also obvious two approaches’ impact to network architecture are different, while Approach 1’s specification impact is limited to RAN, Approach 2 has much a wider impact. And by no means Approach 2 should be the only approach to study in Rel-18.

We have

Proposal 3: study the performance & benefit of two approaches:

Approach 1: providing high-level and semi-static traffic information (non packet specific traffic information) to RAN;

Approach 2: providing low-level and dynamic traffic information (packet specific traffic information) to RAN;

## Summary

In summary, we have

Proposal 2: Rel-18 XR should study the mechanism to adapt UE processing of data channel, control channel, CSI feedback to XR traffic flows:

- study enhancements in SPS/configured grant/DRX configurations to adopt to non-integer periodicity of XR traffic.

- study control signaling enhancements to adapt to time-varying packet size for XR traffic.

- study CSI enhancements for faithful CSI measurements.

- study CSI/HARQ feedback enhancements to achieve efficient transmission and satisfy stringent latency requirement.

Proposal 3: study the performance & benefit of two approaches:

Approach 1: providing high-level and semi-static traffic information (non packet specific traffic information) to RAN;

Approach 2: providing low-level and dynamic traffic information (packet specific traffic information) to RAN;

# Relationship between Rel-17 XR enhancement proposals and Rel-18 XR enhancement proposals

To clarify the relationship between Rel-17 XR enhancement proposals and Rel-18 XR enhancement proposals, we can first review the objectives in the Rel-17 XR SID:

The objective of this study item are as follows:

- Confirm XR and Cloud Gaming applications of interest

- Identify the traffic model for each application of interest taking outcome of SA WG4 work as input, including considering different upper layer assumptions, e.g. rendering latency, codec compression capability etc.

- Identify evaluation methodology to assess XR and CG performance along with identification of KPIs of interest for relevant deployment scenarios

- Once traffic model and evaluation methodologies are agreed, carry out performance evaluations towards characterization of identified KPIs

It is clear traffic model, KPIs and evaluation methodology were discussed in Rel-17. For evaluation, as inevitably some schemes such as DRX, MIMO transmission scheme, etc. were assumed so evaluation could be conducted, it is also understandable in the TR (38.838) some enhancement schemes are captured as well. Note studying enhancements were not the goal of Rel-17 per se, and the time to prepare the TR was quite short, certainly more time should be spent if RAN1 was allowed to reach a conclusion on XR enhancements in Rel-17. Admittedly, the enhancements proposed in Rel-17 did facilitate discussion among companies. Given this somewhat complicated situation, how to treat enhancement schemes proposed in Rel-17 becomes a question.

First, the conclusion of TR 38.838 reads:

- Various potential NR capacity enhancement schemes in support of XR services were proposed and evaluated by different companies.  Their results are collected in Clause 7.3.3

…

- The potential enhancement schemes for UE power saving were proposed and evaluated by different companies.  Their results and observations are given in Clause 8.3.3

It is important to note there is no conclusion drawn from RAN1’s perspective on any enhancement scheme in the TR. Given the history of enhancement discussion in Rel-17, then in our view enhancements discussed in Rel-17 probably provide early views from companies; but they don’t have any priority over enhancements to be brought by companies in Rel-18.

Then for the Rel-18 XR study, it is natural that traffic models, KPIs, and evaluation methodology don’t need to be discussed again, and they can be based on TR 38.838, which is reflected in the Rel-18 SID below.

The study is to be based on Release 17 TR 38.838, on corresponding Release 17 work from SA4 (as per SP-210043) and on Release 18 work from SA2 (as per SP-211166).

Objectives on XR-awareness in RAN (RAN2):

- Study and identify the XR traffic (both UL and DL) characteristics, QoS metrics, and application layer attributes beneficial for the gNB to be aware of.

- Study how the above information aids XR-specific traffic handling.

Objectives on XR-specific Power Saving (RAN1, RAN2):

- Study XR specific power saving techniques to accommodate XR service characteristics (periodicity, multiple flows, jitter, latency, reliability, etc...). Focus is on the following techniques:

  - C-DRX enhancement.

  - PDCCH monitoring enhancement.

Objectives on XR-specific capacity improvements (RAN1, RAN2):

- Study mechanisms that provide more efficient resource allocation and scheduling for XR service characteristics (periodicity, multiple flows, jitter, latency, reliability, etc…). Focus is on the following mechanisms:

  - SPS and CG enhancements;

  - Dynamic scheduling/grant enhancements.

It should be also clear that enhancements to be brought up in Rel-18 don’t need to be a continuation, refinement, etc. of any Rel-17 proposals, and they can be brought up independent of any proposal from Rel-17.

In summary, we have

Proposal 4: enhancement schemes captured in TR 38.838 are treated equally as enhancement schemes to be brought up in Rel-18.

# Conclusion

In this contribution, we provide our views on enhancement for XR. We have

Proposal 1: Rel-18 NR enhancements for XR should be motivated by XR services’ traffic characteristics, especially the multiple data flow aspects.

Proposal 2: Rel-18 XR should study the mechanism to adapt UE processing of data channel, control channel, CSI feedback to XR traffic flows:

- study enhancements in SPS/configured grant/DRX configurations to adopt to non-integer periodicity of XR traffic.

- study control signaling enhancements to adapt to time-varying packet size for XR traffic.

- study CSI enhancements for faithful CSI measurements.

- study CSI/HARQ feedback enhancements to achieve efficient transmission and satisfy stringent latency requirement.

Proposal 3: study the performance & benefit of two approaches:

Approach 1: providing high-level and semi-static traffic information (non packet specific traffic information) to RAN;

Approach 2: providing low-level and dynamic traffic information (packet specific traffic information) to RAN;

Proposal 4: enhancement schemes captured in TR 38.838 are treated equally as enhancement schemes to be brought up in Rel-18.

# References

- RP-201145, “Revised SID on XR Evaluations for NR”, Qualcomm, RAN #88e, June 29 -July 3, 2020

- TR 38.824, “Study on physical layer enhancements for NR ultra-reliable and low latency case (URLLC)”, March 2019

- H. Koumaras, C. Skianis, G. Gardikis, A. Kourtis,“Analysis of H.264 Video Encoded Traffic”, International Network Conference 2005, Samos, Greece.

- S. Tanwir and H. Perros, “A Survey of VBR Video Traffic Models,”, in IEEE Communications Surveys & Tutorials, vol. 15, no. 4, pp. 1778-1802, Fourth Quarter 2013

- R1-2008454, “Views on XR application, traffic models and evaluation methodology”, Apple, RAN1 #103-e, October 2020

- R1-2101365, “Views on XR traffic models”, Apple, RAN1 #104-e, January/February 2021

- R2-2100856, “Scheduling Assistance Information for support of new QoS”, Apple, RAN2 #113-e, January/February 2021

- R1-2101765, ”LS to on XR-Traffic Models”, SA4, SA4 #111-e, January, 2021

- S4aV200640, “[FS_XRTraffic] Summary of XR Traffic Models for RAN1 and Open Issues”, Qualcomm Incorporated (Rapporteur), Apple, Sony Mobile Communications, January 2021.

- S4aV200634, “[FS_XRTraffic] Traces and Configurations for VR2, CG and AR2”, Qualcomm, MediaTek, January 2021.

- RP-211655, “Moderator's summary for discussion [RAN93e-R18Prep-05] Enhancements for XR”, Nokia, RAN #93-e, September 2021.

- RP-212136, “Draft WID for Enhancements for XR support over NR”, Nokia, Nokia Shanghai Bell, RAN #93-e, September 2021.

- RP-201145, “Revised SID on XR Evaluations for NR”, Qualcomm, RAN #88-e, June 29 -July 3, 2020

- RP-220285, “Revised SID: Study on XR Enhancements for NR”, Nokia, RAN #95-e, March 17-23, 2022