3GPP TSG RAN WG1 #109-e			                          R1-2204273

e-Meeting, May 9th – 20th, 2022

Source: 	CMCC

Title:	Discussion on SA2 LS on UE capabilities for MBS

Agenda item:	5

Document for:	Discussion & Decision

- Introduction

In the LS from SA2, two questions about whether NG-RAN need to receive from 5GC information on NR UE capabilities of the target recipients of MBS data in MBS broadcast mode [1]. In this contribution, the reply of this LS will be discussed.

- Discussion

In SA2’s LS, two questions about UE capabilities for MBS are proposed to get RAN1’s feedback.

1. Overall Description: SA2 is working in rel.18 in the MBS enhancements study (FS_5MBS_Ph2) to define support for Group Communication (Key Issue #4) and Coexistence with existing power saving mechanisms for capability-limited devices (Key Issue #5) in TR 23.700-47. In rel.14 for eMTC and NB-IOT receiving SC-PTM SA2 received and LS from RAN1 (see R1-1704074/S2-174083) indicating that due to the different E-UTRAN UE categories/capabilities for SC-PTM transmission from RAN1 perspective, it is beneficial if the eNB has information regarding the maximum TBS/bandwidth and also potentially target coverage level to be used for a given MBMS service. SA2 defined the necessary functionalities described in TS 23.682 clause 4.5.20A and TS 23.246 clause 6.3.2 to indicate these capabilities (see also SA2 LS response in and SA2 response in S2-175071). Namely in order for E-UTRAN to know the UE categories and capabilities for MBMS bearer service, the UE Capability for MBMS (which includes UE Category for MBMS and optionally associated coverage level for MBMS) is provided by SCS/AS to the BMSC via the SCEF. Using PLMN specific QCI information, the characteristics are signalled by the BM-SC to E-UTRAN. Since in rel.18 SA2 plans to define support for MBS for group communication (KI#4 of TR 23.700-47) and power saving mechanisms for capability-limited UEs (KI#5 of TR 23.700-47), and we understand that only NR is in scope of RAN WI of rel.18 for MBS, SA2 would like to ask RAN1: Question 1: Whether, similarly to eMBMS case for eMTC/NB-IoT, would it be useful for NG-RAN to receive from 5GC information on NR UE capabilities (e.g. RedCap) of the target recipients of MBS data in MBS broadcast mode. Question 2: If the answer to Question 1 is yes, we would like to ask RAN1 which are the possible targeted NR UE capabilities that should be provided to NG-RAN by the 5GC. SA2 can further study mechanisms in the context of the MBS enhancements study (FS_5MBS_Ph2). 2. Actions: To RAN1: ACTION: SA2 kindly ask RAN1 to provide feedback on the 2 questions above.

Question 1:

In Rel-14, eMTC and NB-IoT UEs with different bandwidth/max TBS are all supported for SC-PTM transmission and it is beneficial if the eNB has information regarding the maximum TBS/bandwidth and also potentially target coverage level to be used for a given MBMS service.

In Rel-17, a new UE category, i.e., RedCap UE is introduced which can only support narrower bandwidth compared with non-RedCap UE. In addition, three CFR configurations are supported in Rel-17 NR MBS broadcast design including Case A, Case C and Case E. Since the broadcast service need to be transmitted to all broadcast service recipient UEs, the broadcast CFR configuration should consider all UEs’ supported bandwidth capabilities. The bandwidths of broadcast CFR of Case A and Case C are not larger than CORESET#0 which can both supported by RedCap UE and non-RedCap UE, but the bandwidth of broadcast CFR of Case E may be larger than RedCap UE’s maximum supported bandwidth. Therefore, if gNB knows one RedCap UE is a broadcast service recipient, it may not configure Case E CFR. From this perspective, it would be useful for NG-RAN to receive from 5GC information on NR UE capabilities (e.g. RedCap) of the target recipients of MBS data in MBS broadcast mode. However, it should be noted that, whether RedCap UE can support broadcast service reception, i.e., FG 33-1 has not been discussed in RAN1. If RedCap UE cannot support MBS broadcast service, it would be not necessary for NG-RAN to receive from 5GC information on NR UE capabilities (e.g. RedCap) of the target recipients of MBS data in MBS broadcast mode.

Proposal 1. Reply the LS to SA2 on Question 1: It would be useful for NG-RAN to receive from 5GC information on NR UE capabilities (e.g. RedCap) of the target recipients of MBS data in MBS broadcast mode. However, RAN1 has not discussed whether RedCap UE can support broadcast service, e.g., receiving DCI format 4_0.

Question 2：

As the discussion in Question 1, since RedCap UE cannot support Case E CFR configuration of broadcast service which is beyond its maximum supported bandwidth, whether a target broadcast service recipient UE is a RedCap UE or not should be provided to NG-RAN by the 5GC to assist gNB to configure a proper CFR bandwidth for all broadcast service recipients.

Proposal 2. Reply the LS to SA2 on Question 2: Whether a UE is a RedCap UE or not should be provided to NG-RAN by the 5GC.

- Conclusions

In this contribution, the answers of UE capabilities for MBS from SA2 LS are discussed, and the following proposals are made.

Proposal 1. Reply the LS to SA2 on Question 1: It would be useful for NG-RAN to receive from 5GC information on NR UE capabilities (e.g. RedCap) of the target recipients of MBS data in MBS broadcast mode. However, RAN1 has not discussed whether RedCap UE can support broadcast service, e.g., receiving DCI format 4_0.

Proposal 2. Reply the LS to SA2 on Question 2: Whether a UE is a RedCap UE or not should be provided to NG-RAN by the 5GC.

- References

- R1-2203218 (S2-2203020), UE capabilities for MBS, SA2, Qualcomm