3GPP TSG RAN WG1 #109-e			R1-2204275

e-Meeting, May 9th – 20th, 2022

Agenda item:	8.5.1

Source: 	CMCC

Title: 	Remaining issues on accuracy improvements

Document for:	Discussion and Decision

Introduction

In RAN1#108-e meeting, the remaining open issues on the UE/gNB Rx/Tx timing error mitigation were continually discussed, within which the issue on UE Tx TEG sweeping was intensively discussed [1]. Unfortunately, no consensus was reached. In this contribution, we further provide our views on this issue.

Remaining issues on timing error mitigation

In RAN1#107-e meeting, the following agreements were made [1]. It allows a UE with multiple Rx TEGs to optionally measure the same DL PRS resource with different Rx TEGs and report corresponding timing measurements, in order to let LMF obtain the timing error difference between two UE Rx TEGs:

Agreement Make the following modification on the previous agreement made in RAN#106bis-e: - Subject to UE capability, support the LMF to request a UE to optionally measure the same DL PRS resource of a TRP with N different UE Rx TEGs and report the corresponding multiple RSTD measurements. - N=[2, 3, 4, 6, 8] (FFS: other values), where the maximum value of N depends on UE capability, and applies to all DL PRS positioning frequency layers - Note: If N is not explicitly included in the request, it is up to UE to determine the number of different UE Rx TEGs to measure the same DL PRS resource within its capability - The TRP can be either a “RSTD” reference TRP or a neighbour TRP - FFS: details of the signalling, procedures, and UE capability - The timestamps of the multiple RSTD measurements in the same measurement report can be the same or different. - Note: All RSTD measurements are relative to a single reference timing - Support the LMF to request a TRP to optionally measure the same SRS resource of a UE with M different TRP Rx TEGs and report the corresponding multiple RTOA measurements. - M = [2, 3, 4, 6, 8] (FFS: other values) applies to all configured SRS resources for positioning - Note: If M is not explicitly included in the request, it is up to TRP to determine the number of different TRP Rx TEGs to measure the same SRS resources for positioning - FFS: details of the signalling, procedures The timestamps of the multiple RTOA measurements in the same measurement report can be the same or different. Agreement - Subject to UE capability, support the LMF to request a UE to optionally measure the same DL PRS resource of a TRP with N different UE Rx TEGs and report the corresponding multiple UE Rx-Tx time difference measurements. - N=[2, 3, 4, 6, 8], where the maximum value of N depends on UE capability, and applies to all DL PRS positioning frequency layers - Note: If N is not explicitly included in the request, it is up to UE to determine the number of different UE Rx TEGs to measure the same DL PRS resource within its capability - FFS: details of the signalling, procedures, and UE capability - The timestamps of the multiple UE Rx-Tx time difference measurements in the same measurement report can be the same or different. - Support the LMF to request a TRP to optionally measure the same SRS resource of a UE with M different TRP Rx TEGs and report the corresponding multiple gNB Rx-Tx time difference measurements. - M = [2, 3, 4, 6, 8] applies to all configured SRS resources. - Note: If M is not explicitly included in the request, it is up to TRP to determine the number of different TRP Rx TEGs to measure the same SRS resources - FFS: details of the signalling, procedures - The timestamps of the multiple gNB Rx-Tx time difference measurements in the same measurement report can be the same or different.

For a UE with multiple Tx TEGs, a symmetric mechanism should be supported. In the last RAN1 meeting, it was proposed to support SRS for positioning with “antenna switching” as an optional UE capability to solve this issue; however, due to limited inputs and comments, the issue was treated as low priority.

Support SRS for positioning with “antenna switching” enables a UE to transmit N SRS pos resources using N different Tx TEGs in turn towards a same TRP, and therefore allows the LMF to estimate and mitigate the UE Tx TEG timing error difference. In our views, as we have enabled the LMF to request the UE to optionally measure the same DL PRS resources of a TRP with N different UE Rx TEGs to mitigate the UE Rx TEG timing error difference, it would be reasonable to introduce a symmetric design for Tx TEG. In R17 positioning enhancement, one of the key targets is to improve the positioning accuracy for IIoT use cases, where it is highly possible that the new type of IIoT devices are implemented with multiple antenna panels, i.e., with multiple Tx TEGs. In such cases, the timing error differences between different UE Tx TEGs may have great impact on the performance of the timing-based positioning scheme, and to support the UE Tx TEG sweeping is a critical solution to mitigate the timing errors.

During the discussion in previous RAN1 meetings, some companies argued that the whole feature was not necessary, as the problem can be solved by NW implementation. In RAN1, we have agreed to support a UE to provide under capability the association information of UL SRS resources for positioning with Tx TEGs to the LMF if the UE has multiple Tx TEGs. With this mechanism, the TRPs report timing measurements associated with corresponding SRS pos resources, and then the LMF is able to link the timing measurements with the UE Tx TEGs. Ideally, as long as one TRP measures SRS pos resources sent by different UE Tx TEGs, the LMF can obtain the corresponding timing error differences. However, as no UE behaviour is defined, there may exist some case when the problem cannot be addressed by implementation. As shown in the following figures, suppose that the UE has two Tx TEGs, and the NW configures two SRS pos resources within a resource set. Up to UE implementation, the UE may send the SRS resources in the way shown in Figure 1(a). Then, the timing error difference between the two UE Tx TEGs can no longer be mitigated for RSTD between TRP 2/3 and TRP 1. On the other hand, by supporting UE Tx TEG sweeping as illustrated in Figure 1(b), the NW indicates the UE to transmit the two SRS resources using Tx TEG sweeping, then further mitigation on the timing error difference is feasible.

Figure 1(a). Up to UE implementation                   Figure 1(b). UE Tx TEG sweeping

Therefore, we propose to support UE Tx TEG sweeping as an optional UE capability.

Proposal 1: Support UE Tx TEG sweeping as an optional UE capability:

- A UE can be configured by serving gNB to use N different UE Tx TEGs for the transmission of the SRS resources for positioning

Conclusions

In this contribution, we provide our views on the remaining issues on UE/gNB Rx/Tx timing error mitigation, and the following proposals are made:

Proposal 1: Support UE Tx TEG sweeping as an optional UE capability:

- A UE can be configured by serving gNB to use N different UE Tx TEGs for the transmission of the SRS resources for positioning

References

- Chair’s Notes RAN1#108-e meeting.