# 3GPP TSG RAN WG1 #109-e		                         R1-2204284

# e-Meeting, May 9th – 20th, 2022

# Source: 	CMCC

# Title:	Discussion on UE features for RedCap

# Agenda item:	8.16.6

# Document for:	Discussion & Decision

- Introduction

This contribution discusses the remaining issues about RAN1 UE features associated with the Rel-17 RedCap WI based on RAN1 agreements made during previous RAN1 meetings. The UE feature discussion here are divided into FG28-1 related features and other features.

- Discussion on UE features for RedCap

In this section, we will discuss features related FG28-1 and other separate features.

Discussion on FG28-1 related feature

According to the following agreement of RAN1#108e, there is one FFS for separate initial DL BWP, which is highlighted below.

Agreement

Following capabilities are added as components in FG 28-1

- Separate initial UL BWP for RedCap UEs

  - It includes the configuration(s) needed for RedCap UE to perform random access

- Separate initial DL BWP for RedCap UEs

  - It includes CSS/CORESET for random access

  - FFS: For separate initial DL BWP used for paging, CD-SSB is included

  - For separate initial DL BWP only used for RACH, SSB may or may not be included

There are three cases for paging reception on separate initial DL BWP,

- Case 1: Separate initial DL BWP in idle/inactive mode. For this case, if it is configured for paging, CD-SSB is included.

- Case 2: Separate initial DL BWP with BWP#0 configuration option 1 in connected mode, paging can only be configured if it contains CD-SSB (and CORESET#0 for FR1).

- Case 3: Separate initial DL BWP with BWP#0 configuration option 2 in connected mode, if is configured for paging, CD-SSB/NCD-SSB is included for UE supporting mandatory FG 6-1 but not optional FG 6-1a.

- The reference agreements is copied as following. For components added to FG28-1 separate initial DL BWP, we understand it is for case 1 and case 2, since there will be dedicated UE feature group for UE-specific RRC configured BWP, either by reuse FG6-1/6-1a or define new features. According to TS38.331 B.2, BWP#0 configuration option 2 is considered as RRC-configured BWP, so it is reasonable to consider case 3 separately.

Agreement

- For BWP#0 configuration option 1,

  - For FR1,

    - For a separate initial DL BWP, for a RedCap UE in connected mode, paging can only be configured if it contains CD-SSB and the entire CORESET#0.

  - For FR2,

    - For a separate initial DL BWP, for a RedCap UE in connected mode, paging can only be configured if it contains CD-SSB.

- Note: For BWP#0 configuration option 2,

  - For FR1,

    - For a separate initial DL BWP in connected mode (if it does not include CD-SSB and the entire CORESET#0), if it is configured for paging,

      - A RedCap UE supporting mandatory FG 6-1 (but not optional FG 6-1a) expects it to contain NCD-SSB for serving cell but not CORESET#0/SIB

      - A RedCap UE supporting FG 6-1a does not expect it to contain SSB/CORESET#0/SIB

  - For FR2,

    - For a separate initial DL BWP in connected mode (if it does not include CD-SSB), if it is configured for paging,

      - A RedCap UE supporting mandatory FG 6-1 (but not optional FG 6-1a) expects it to contain NCD-SSB for serving cell but not CORESET#0/SIB

      - A RedCap UE supporting FG 6-1a does not expect it to contain SSB/CORESET#0/SIB

- So we prefer to modify the FFS to the following,

- Separate initial DL BWP for RedCap UEs

  - It includes CSS/CORESET for random access

  - For separate initial DL BWP used for paging in idle/inactive mode or in connected mode as BWP#0 configuration option 1, CD-SSB is included.

  - For separate initial DL BWP only used for RACH, SSB may or may not be included.

- Proposal 1: Include the following part to separate initial DL BWP component of FG 28-1，

- For separate initial DL BWP used for paging in idle/inactive mode or in connected mode as BWP#0 configuration option 1, CD-SSB is included.

Discussion on separate UE features other than FG 28-1

- SSB related UE capability

During the RAN1#108e, the following agreements are made,

Agreement

Replace the working assumption from RAN1#107e “Not need NCD-SSB: A RedCap UE can in addition optionally support relevant operation based on for CSI-RS (working assumption) and/or FG 6-1a by reporting optional capabilities” with the following agreement:

For FR1,

- For an RRC-configured active DL BWP in connected mode (if it does not include CD-SSB and the entire CORESET#0) from RAN1 perspective,

  - A RedCap UE supporting mandatory FG 6-1 (but not optional FG 6-1a) expects it to contain NCD-SSB for serving cell but not CORESET#0/SIB

  - A RedCap UE can indicate the following as optional capability:

    - Not need NCD-SSB: A RedCap UE can in addition optionally support relevant operation based on [FG 6-1a] with supporting CSI-RS, or [FG 6-1a]without supporting CSI-RS.

For FR2,

- For an RRC-configured active DL BWP in connected mode (if it does not include CD-SSB) from RAN1 perspective,

  - A RedCap UE supporting mandatory FG 6-1 (but not optional FG 6-1a) expects it to contain NCD-SSB for serving cell but not CORESET#0/SIB

  - A RedCap UE can indicate the following as optional capability:

    - Not need NCD-SSB: A RedCap UE can in addition optionally support relevant operation based on [FG 6-1a] with supporting CSI-RS, or [FG 6-1a] without supporting CSI-RS.

Note: The cases that CSI-RS in this agreement can support are left to RAN4

According to the agreements, there are two kinds of UE capabilities for RedCap UEs when an RRC-configured active DL BWP in connected mode does not include CD-SSB and the entire CORESET#0. One is that RedCap UEs doesn’t support optional FG6-1a, this kind of RedCap UE will expect the RRC configured active DL BWP to contain NCD-SSB. The other one is that RedCap support capability that NCD-SSB is not needed on the RRC configured active DL BWP.

For the first kind of capability, there are two options to define the UE feature,

- Alt.1: No separate new FG is introduced, while add a note in FG6-1(Basic BWP operation with restriction) that  “For RedCap UEs, BW of a UE-specific RRC configured BWP may or may not include BW of CD-SSB and the entire CORESET#0, if not, BW of the UE-specific RRC configured BWP includes NCD-SSB.

- Alt.2: A new FG 28-x1 is introduced, with the following component,

- BW of UE-specific RRC configured BWP may not include BW of CD-SSB and the entire CORESET#0, in this case, BW of the UE-specific RRC configured BWP includes NCD-SSB.

- Alt.1 introduces one case that BW of a UE-specific RRC configured BWP may not include BW of CD-SSB and the entire CORESET#0 into the traditional FG6-1, this is different from what legacy FG6-1 mentions. And there is a potential risk for Alt.1 that when R18 RedCap with 5MHz is introduced, will the mandatory feature be modified again? If so, the legacy UE feature 6-1 is better to keep as it is since R17.

- Alt.2 has no impact on legacy FG, and defining a separate FG keeps the possibility that the feature can be used by non-RedCap UEs in the future.

- Comparing these two options, we prefer Alt.2. The prerequisite feature group of this new feature can be FG 6-1, and since this is the default capability, RedCap UEs have to report this capability.

- Proposal 2: Define a new FG28-x1 for the capability of UE expect NCD-SSB when an RRC-configured active DL BWP in connected mode does not include CD-SSB and the entire CORESET#0.

- Proposal 3: The new FG28-x1 includes the following component, and the prerequisite feature group of this is FG 6-1, 6-2,6-3 or 6-4

- BW of UE-specific RRC configured BWP may not include BW of CD-SSB and the entire CORESET#0, in this case, BW of the UE-specific RRC configured BWP includes NCD-SSB.

- For the second kind of capability, RAN4 has made the following conclusion during RAN4#101b [5],

For an RRC-configured active DL BWP in connected mode (if it does not include CD-SSB and the entire CORESET#0):

- A RedCap UE that supports FG 6-1a but NOT support CSI-RS based L3 measurement operates in the BWP

  - the UE can support RLM, BFD, CBD and L1 RSRP measurement based on CSI-RS if UE reports the corresponding capabilities.

  - the UE can support SSB based L3 measurement, but cannot support CSI-RS based L3 measurement.

- A RedCap UE that supports FG 6-1a and CSI-RS based L3 measurement operates in the BWP

  - the UE can support RLM, BFD, CBD and L1 RSRP measurement based on CSI-RS if UE reports the corresponding capabilities.

  - the UE can support both SSB based L3 measurement and CSI-RS based L3 measurement with associated SSB.

    - RAN4 will not define CSI-RS L3 based measurement requirements for Redcap 1RX UE in Rel-17.

- For serving cell timing related requirements, RAN4 will not define requirements based on CSI-RS in Rel-17.

- Based on the above conclusion, RedCap UE can perform RLM, BFD, CBD and L1 RSRP measurement based on CSI-RS and L3 measurement based on CSI-RS with associated SSB if UE supports the corresponding capabilities. If the corresponding capabilities are not supported, UE may rely on re-tuning to CD-SSB for such measurement,which may be already supported as FG6-1a. So we propose to introduce a new FG 28-x2, to support this optional capability, which include the following components,

- BW of UE-specific RRC configured BWP may not include BW of SSB(including CD-SSB or NCD-SSB) and the entire CORESET#0.

- support measurement operation based on FG6-1a.

- support RLM, BFD, CBD and L1 RSRP measurement based on CSI-RS if UE reports the corresponding capabilities.

- support CSI-RS based L3 measurement with associated SSB if UE reports the corresponding capability.

- And the prerequisite feature group of this new feature can be FG 6-1a.

- Proposal 4: Define a new feature FG28-x2 for the capability of supporting operation without NCD-SSB when an RRC-configured active DL BWP in connected mode does not include CD-SSB and the entire CORESET#0.

- Proposal 5: The new FG28-x2 includes the following components, and the prerequisite feature group of this is FG 6-1a,

- BW of UE-specific RRC configured BWP may not include BW of SSB(including CD-SSB or NCD-SSB) and the entire CORESET#0.

- support measurement operation based on FG6-1a.

- support RLM, BFD, CBD and L1 RSRP measurement based on CSI-RS if UE reports the corresponding capabilities.

- support CSI-RS based L3 measurement with associated SSB if UE reports the corresponding capability.

- Conclusions

In this contribution, considerations on modification and definition RedCap UE feature group are discussed, the following proposals are made,

- Proposal 1: Include the following part to separate initial DL BWP component of FG 28-1，

- For separate initial DL BWP used for paging in idle/inactive mode or in connected mode as BWP#0 configuration option 1, CD-SSB is included.

- Proposal 2: Define a new FG28-x1 for the capability of UE expect NCD-SSB when an RRC-configured active DL BWP in connected mode does not include CD-SSB and the entire CORESET#0.

- Proposal 3: The new FG28-x1 includes the following component, and the prerequisite feature group of this is FG 6-1, 6-2,6-3 or 6-4

- BW of UE-specific RRC configured BWP may not include BW of CD-SSB and the entire CORESET#0, in this case, BW of the UE-specific RRC configured BWP includes NCD-SSB.

- Proposal 4: Define a new feature FG28-x2 for the capability of supporting operation without NCD-SSB when an RRC-configured active DL BWP in connected mode does not include CD-SSB and the entire CORESET#0.

- Proposal 5: The new FG28-x2 includes the following component, and the prerequisite feature group of this is FG 6-1a,

- BW of UE-specific RRC configured BWP may not include BW of SSB(including CD-SSB or NCD-SSB) and the entire CORESET#0.

- support measurement operation based on FG6-1a.

- support RLM, BFD, CBD and L1 RSRP measurement based on CSI-RS if UE reports the corresponding capabilities.

- support CSI-RS based L3 measurement with associated SSB if UE reports the corresponding capability.

- References

- R1-2110669, RAN1 agreements for Rel-17 NR RedCap.

- Final_Minutes_report_RAN1#107-e_v100.

- R1-2200780, Updated RAN1 UE features list for Rel-17 NR after RAN1 #107bis-e.

- R1-2112137,Summary on UE features for REDCAP.

- DRAFT Meeting Report for TSG RAN WG4 meeting: 101-bis-e.

- Appendix

- NR_redcap

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 28. NR_redcap | 28-1 | RedCap UE | 1. Maximum FR1 RedCap UE bandwidth is 20 MHz. 2. Maximum FR2 RedCap UE bandwidth is 100 MHz. 3. Early indication of RedCap UE in Msg.1 for 4-step RACH FFS whether to add any other basic features for RedCap UE |  | Yes |  | Network assumes the UE is not a RedCap UE | Per UE | No | No |  | RedCap UEs do not support carrier aggregation or dual connectivity. | Optional with capability signaling RedCap UE must indicate this FG is supported |
| 28. NR_redcap | 28-2 | Number of UE Rx branches and DL MIMO layers for RedCap UE | 1. For a RedCap UE with 1 Rx branch, 1 DL MIMO layer is supported. 2. For a RedCap UE with 2 Rx branches, 2 DL MIMO layers are supported. | 28-1 | Yes |  | Impact on UE complexity | Per band | No | No |  | For UE capability signalling, the number of Rx branches for RedCap is implicitly indicated by the corresponding capability parameter maxNumberMIMO-LayersPDSCH in the existing UE capability framework. Detailed signalling is up to RAN2. | Optional with capability signaling |
| 28. NR_redcap | 28-3 | Half-duplex FDD operation type A for RedCap UE | 1. Half-duplex FDD operation (instead of full-duplex FDD operation) type A for RedCap UE | 28-1 | Yes |  | UE is assumed to support FD-FDD in FDD bands | Per band | FDD only | FR1 only |  |  | Optional with capability signaling |