3GPP TSG RAN WG1 #109-e			                     R1-2204285

e-Meeting, May 9th – May 20th, 2022

Agenda item:	8.16.8

Source: 	CMCC

Title: 	Discussion on Rel.17 UE features for NR coverage enhancement

Document for:	Discussion and Decision

Introduction

This document is to discuss the question in FL summary for discussion in RAN1 108[2].

Overview on FGs for Coverage Enhancements

Proposal 1:

The bracket around K could be removed for the component part of FG 30-2.

Proposal 2:

Either remove or keep the 11-6 and 30-1 as one of the prerequisite FG for FG 30-2 would be fine. Since if UE want to support Rel-16 repetitions based on available slot or maximum 32 repetitions based on available slots, it should claim to support the FG 11-6 or FG 30-1.

Proposal 3:

The prerequisite of FG 30-3a contains FG 30-3.

Proposal 4：

It should not set up a dependency between FG 30-3 (TBOMS) and FG 30-2/2a (PUSCH Type A repetitions based on available slots)

Proposal 5:

No prerequisite for FG 30-3 is preferred.

Proposal 6:

The prerequisite FG of FG 30-4 should be vacant.

The prerequisite FG of FG 30-4a should be FG 30-4 and one of the {5-14,5-16,5-17}.

The prerequisite FG of FG 30-4b should be FG 30-4 and FG 11-6.

The prerequisite FG of FG 30-4c should be FG 30-4 and FG 30-3.

The prerequisite FG of FG 30-4d should be FG 30-4 and FG 4-23.

The prerequisite FG of FG 30-4e should be one of FG {30-4a, 30-4b or 30-4c}.

The prerequisite FG of FG 30-4f should be FG 30-4d.

Proposal 7:

FG 30-4a to 30-4h should be per band reported and differentiated between FR1 and FR2.

Proposal 8

No need to differentiate TDD and FDD for FG 30-4a to FG 30-4h.

Proposal 9:

Remove the bracket for Feature group	and Components in FG 30-4h.

Proposal 10:

Remove the bracket for Prerequisite feature groups in FG 30-5.

Proposal 11:

FG 30-6 should be reported as per UE level not per band. And it is not necessary to differentiate between TDD and FDD.

Proposal 12:

Remove the bracket for Components in FG 30-5.

Proposal 13:

No need to differentiate TDD and FDD for FG 30-6.

30-1 to 30-2a: Enhancements for PUSCH Type A repetitions

In [1], FGs 30-1 to 30-2a are captured as below.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 30. NR_cov_enh | 30-1 | Increased maximum number of PUSCH Type A repetitions | Maximum value of K (the number of repetitions) = 32 For DG PUSCH, the number of repetitions is indicated in a TDRA list. A row index of the TDRA list is indicated by a DCI. For Type 1 CG PUSCH, the number of repetitions is indicated by repK-r17 For Type 2 CG PUSCH, the number of repetitions is indicated in a TDRA list or by repK-r17. | 5-14, 5-16, or 5-17 | Yes | N/A | UE does not support more than 16 repetitions PUSCH. | Per band | N/A | N/A | N/A |  | Optional with capability signalling |
| 30. NR_cov_enh | 30-2 | PUSCH Type A repetitions based on available slots | Transmission occasions for [K] repetitions for dynamic and configured grant PUSCH are determined on the basis of available slots. | One of {5-14, 5-16, 5-17, [11-6, 30-1]} | Yes | N/A | UE does not support dynamic or configured grant PUSCH repetitions counted on the basis of available slots. | Per band | N/A | N/A | N/A |  | Optional with capability signalling |

As discussed in AI 8.8.1.1, the counting rule of the PUSCH repetition type A based on the available slot is concluded in the last meeting, when the AvailableSlotCounting is enabled and K =1. There is no ambiguity for UE behaviour. The bracket around K could be removed for the component part of FG 30-2.

Proposal 1:

The bracket around K could be removed for the component part of FG 30-2.

And there is a bracket for FG 30-2 in the prerequisite FG around 11-6 and 30-1. From our understanding, both Rel-16 repetition (FG 11-6) and maximum 32 repetitions (FG 30-1) could work with the PUSCH Type A repetitions based on available slots. At least, one of the FG 5-14/16/17 as the prerequisite of the FG 30-2 is enough. Once UEs want to support the Rel-16 repetitions and based on the available slot counting, the FG 11-6 should be also supported. And if UE want to support maximum 32 PUSCH repetitions based on available slot, the FG 30-1 should be supported. Then there is no need to add FG 11-6 and 30-1 into the prerequisite FG.

Proposal 2:

Either remove or keep the 11-6 and 30-1 as one of the prerequisite FG for FG 30-2 would be fine. Since if UE want to support Rel-16 repetitions based on available slot or maximum 32 repetitions based on available slots, it should claim to support the FG 11-6 or FG 30-1.

30-3: TB processing over multi-slot PUSCH

In [1], FG 30-3 is captured as below.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 30. NR_cov_enh | 30-3 | TB processing over multi-slot PUSCH | Support of TB processing over multi-slot PUSCH for DG and CG in RRC connected mode. | [11-6] | Yes | N/A | UE does not support TB processing over multi-slot PUSCH. | Per band | N/A | N/A | N/A |  | Optional with capability signalling |
| 30. NR_cov_enh | 30-3a | Repetition of TB processing over multi-slot PUSCH | Support repetition of TB processing over multi-slot PUSCH in RRC connected mode. | TBD | Yes | N/A |  | Per band | N/A | N/A | N/A |  | Optional with capability signalling |

Note that any contents highlighted in yellow mean FFS and to be discussed further.

Low priority question 3-7:

- Companies are encouraged to provide views on whether/how to revise the prerequisite feature groups for FG 30-3a

The prerequisite of the repetition of TBOMS should include FG 30-3. The FG 11-6 should not be a prerequisite for FG 30-3, since no strong correlation is considered between these two FGs. In addition, as discussed during the meeting, though TBOMS is counted based on the available slots. It is not necessary to set FG 30-2 and 2a as a prerequisite. When UE reports it could support TBOMS, the UE should support the determination of available slots. It is not necessary to set up a dependency between TBOMS and repetitions based on the available slots.

Proposal 3:

The prerequisite of FG 30-3a contains FG 30-3.

Proposal 4：

It should not set up a dependency between FG 30-3 (TBOMS) and FG 30-2/2a (PUSCH Type A repetitions based on available slots)

Proposal 5:

No prerequisite for FG 30-3 is preferred.

30-4 to 30-4g: DM-RS bundling

In [1], FGs 30-4 to 30-4g are captured as below.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 30. NR_cov_enh | 30-4 | The maximum duration for DM-RS bundling | The maximum duration during which UE is able to maintain power consisitency and phase continuity to support DM-RS bundling for PUSCH/PUCCH | FFS | Yes | N/A | UE does not support DM-RS bundling for PUSCH/PUCCH | Per band | N/A | N/A | N/A |  | Optional with capability signalling |
| 30. NR_cov_enh | 30-4a | DM-RS bundling for PUSCH repetition type A | Support DM-RS bundling for PUSCH repetition type A | [30-4], [30-1] or [30-2] | Yes | N/A | UE does not Support DM-RS bundling for PUSCH repetition type A | [Per UE] | FFS | [No] | [N/A] |  | Optional with capability signalling |
| 30. NR_cov_enh | 30-4b | DM-RS bundling for PUSCH repetition type B | Support DM-RS bundling for PUSCH repetition type B | [30-4], [11-5] [30-1] | Yes | N/A | UE does not Support DM-RS bundling for PUSCH repetition type B | [Per UE] | FFS | [No] | [N/A] |  | Optional with capability signalling |
| 30. NR_cov_enh | 30-4c | DM-RS bundling for TB processing over multi-slot PUSCH | Support DM-RS bundling for TB processing over multi-slot PUSCH | [30-4], [30-3] | Yes | N/A | UE does not Support DM-RS bundling for TB processing over multi-slot PUSCH | [Per UE] | FFS | [No] | [N/A] | Note: If a UE reports support of FG 30-3a and 30-4c, the UE supports DMRS bundling for the repetitions of TBoMS | Optional with capability signalling |
| 30. NR_cov_enh | 30-4g | [Restart DM-RS bundling after the events that violate power consistency and phase continuity] | [Support restarting DM-RS bundling after the events that violate power consistency and phase continuity] | [30-4] | Yes | N/A | [UE does not support restarting DM-RS bundling after the events that violate power consistency and phase continuity] | [Per UE] | FFS | [No] | [N/A] |  | Optional with capability signalling |
| 30. NR_cov_enh | 30-4h | DM-RS bundling for non-back-to-back transmission | Support DM-RS bundling for [non-back-to-back transmission for consecutive slots] for PUSCH and PUCCH only for [corresponding supported back-to-back transmission FGs (30-4a, 30-4b, 30-4c, or 30-4d)] | [30-4a, 30-4b, 30-4c, or 30-4d] | Yes | N/A | UE does not Support DM-RS bundling for non-back-to-back transmission | [Per UE] | FFS | [No] | [N/A] |  | Optional with capability signalling |

Note that any contents highlighted in yellow mean FFS and to be discussed further.

FG 30-4 is the basis of DMRS bundling, the prerequisite FG of this feature should be vacant. The prerequisite of FG 30-4a should include at least FG 30-4. And for the part of PUSCH repetition Type A in FG 30-4a, the prerequisite FG could be one of the {5-14,5-16,5-17}. No need to limited the DMRS bundling only work based on the Rel-16 or Rel-17 repetition enhancements. It is similar for FG 30-4b. The prerequisite of DM-RS bundling for PUSCH repetition type B should contain FG 30-4 and FG 11-6. The prerequisite of FG 30-4c the DMRS bundling of TBOMS should contain FG 30-4 for DMRS bundling and FG 30-3 for TBOMS. The prerequisite of FG 30-4d the DMRS bundling for PUCCH repetitions should contain FG 30-4 and FG 4-23 for PUCCH repetitions.

FG 30-4e the enhanced inter-slot frequency hopping for PUCCH repetitions with DMRS bundling should work under prerequisite of at least one of DMRS bundling scenarios, such as 30-4a, 30-4b or 30-4c. FG 30-4f is more straightforward with FG 30-4d as prerequisite.

Proposal 6:

The prerequisite FG of FG 30-4 should be vacant.

The prerequisite FG of FG 30-4a should be FG 30-4 and one of the {5-14,5-16,5-17}.

The prerequisite FG of FG 30-4b should be FG 30-4 and FG 11-6.

The prerequisite FG of FG 30-4c should be FG 30-4 and FG 30-3.

The prerequisite FG of FG 30-4d should be FG 30-4 and FG 4-23.

The prerequisite FG of FG 30-4e should be one of FG {30-4a, 30-4b or 30-4c}.

The prerequisite FG of FG 30-4f should be FG 30-4d.

And it was concluded that the FG 30-4 is per band reported. Then the FGs based on FG 30-4 should be also per band reported, such as FG 30-4a 30-4b. And consequently, for those per band reported FGs, FR1 and FR2 differentiation is needed.

Proposal 7:

FG 30-4a to 30-4h should be per band reported and differentiated between FR1 and FR2.

Considering the behavior of the DMRS bundling within the nominal and actual TDWs are the same for both TDD and FDD. There is no need to differentiate TDD and FDD for those features.

Proposal 8

No need to differentiate TDD and FDD for FG 30-4a to FG 30-4h.

FG 30-4h is the feature for UE support DM-RS bundling for non-back-to-back transmission. For a UE support DM-RS bundling, supporting non-back-to-back transmission is a advanced capability than the back-to-back transmission.

If a UE want support any non-back-to back PSUCH repetitions, it should support at least one of 30-4a, 30-4b, 30-4c, or 30-4d for the back to back PUSCH repetitions for the specific feature.

so the prerequisite of FG 30-4h should be  {30-4a, 30-4b, 30-4c, or 30-4d}.

Proposal 9:

Remove the bracket for Feature group	and Components in FG 30-4h.

30-5: Slot based dynamic PUCCH repetition indication

In [1], FG 30-5 is captured as below.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 30. NR_cov_enh | 30-5 | Slot based dynamic PUCCH repetition indication | Support slot based dynamic PUCCH repetition indication for PUCCH formats 0/1/2/3/4 support slot based dynamic PUCCH repetition for PUCCH formats 0/1/2/3/4 FFS whether to split FG 30-5 into 2 FGs; one for PUCCH formats 0/2 and the other for PUCCH formats 1/3/4 | [4-23 and/or 25-2] | Yes | N/A | UE does not support Dynamic PUCCH repetition indication | [Per UE] | FFSNo | No | N/A |  | Optional with capability signalling |

Note that any contents highlighted in yellow mean FFS and to be discussed further.

The Prerequisite FG for FG 30-5 should be 4-23 and/or 25-2. The former is for PUCCH format 1/3/4 and the latter is for PUCCH format 0/2. A UE should at least support one or both UE capabilities if it supports dynamic PUCCH repetition indication.

Proposal 10:

Remove the bracket for Prerequisite feature groups in FG 30-5.

30-6: Msg3 repetition

In [1], FG 30-6 is captured as below.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 30. NR_cov_enh | 30-6 | Repetition of PUSCH transmission scheduled by RAR UL grant and DCI format 0_0 with CRC scrambled by TC-RNTI | [Support of repetition of PUSCH transmission scheduled by RAR UL grant and DCI format 0_0 with CRC scrambled by TC-RNTI] |  | Yes | N/A | UE does not support repetition of PUSCH transmission scheduled by RAR UL grant and DCI format 0_0 with CRC scrambled by TC-RNTI. | [Per UE] | [No] | [No] | [N/A ] |  | Optional with capability signalling |

Note that any contents highlighted in yellow mean FFS and to be discussed further.

Medium priority question 6-2:

- Companies are encouraged to provide views on whether the type of FG 30-6 should be per UE or per band

This is similar as other repetition related features that it is band agnostic. Then there is not necessary to report per band. And it is also not necessary to differentiate between TDD and FDD.

Proposal 11:

FG 30-6 should be reported as per UE level not per band. And it is not necessary to differentiate between TDD and FDD.

Proposal 12:

Remove the bracket for Components in FG 30-5.

Proposal 13:

No need to differentiate TDD and FDD for FG 30-6.

References

[1] R1-2202928, Updated RAN1 UE features list for Rel-17 NR after RAN1 #108-e, Moderators (AT&T, NTT DOCOMO, INC.), 3GPP TSG RAN WG1 #108-e e-Meeting, February 21st – March 3rd, 2022.

[2] R1-2201517, Summary on UE features for NR coverage enhancement, Moderator (NTT DOCOMO, INC.), 3GPP TSG RAN WG1 #108-e e-Meeting, February 21st – March 3rd, 2022.