3GPP TSG RAN WG1 #109-e			                                  R1-2204288

e-Meeting, May 9th – May 20th, 2022

Agenda item:	9.1.1.2

Source: 	CMCC

Title: 	Discussion on two TAs for multi-DCI

Document for:	Discussion and Decision

Introduction

In the RAN #94-e meeting, the new WI of MIMO Evolution for Downlink and Uplink was agreed. The objective related to this agenda is listed below [1].

- Study, and if justified, specify the following - Two TAs for UL multi-DCI for multi-TRP operation - Power control for UL single DCI for multi-TRP operation where unified TCI framework extension in objective 2 is assumed. For the case of simultaneous UL transmission from multiple panels, the operation will only be limited to the objective 6 scenarios.

In this contribution, we discussed the related issues about two TAs for UL multi-DCI for multi-TRP operation.

Discussion

Before the detailed discussion about the technology, it should be clarified the scenario of the two TA for UL multiple DCI under the multiple TRP scenario. In the Rel-17 mTRP discussion, it was considered that the uplink transmission could work with a single TA under the two TRPs. But if the timing between the two TRPs are not aligned or the difference between the propagation delays to the two TRPs cannot be ignored, the two TAs are needed. But in the FR1, mTRP usually works under the two neighbor TRPs or gNBs, due to the large CP length, the propagation delay difference usually can be ignored. And the for the TDD system, the gNBs should also be synchronized to prevent the cross-link interferences. Then it is not clear whether we should discuss the two TA issue under the assumption of FR1. But in the FR2, due to the propagation loss of higher frequency, analog beamforming is widely used. The signal of different beamforming direction may experience different distance and propagation delay. And for even twp TRPs, the propagation delay could also be different. And the CP length of FR2 could be much less than that of FR1, it is also possible that the difference of propagation delay to the two TRPs is larger than the CP length. Then it is proposed focusing on FR2 scenario in which each TRP have a different TA.

Proposal 1:

- It is proposed focusing on the FR2 scenario in which each TRP have a different TA.

- The two TA scenario of FR1 needs to be clarified.

Currently the UE could only support one TA at least for one carrier. But the UE could also support multiple TAs across multiple carriers. But unlike in the CA scenario, the carriers that have different TAs may have different RF chains. Those RF chains could work separately. But in the current two TA under multiple TRP scenario, two TRP work under the same frequency. The UE should support two TAs in the same carrier. It is not clear whether this will put additional requirements for the realization. If the TA adjustment is related to the clock of UE, the retuning of the clock may require an additional process time between the switching between the TAs. If two TAs do not require to retune the clock, whether the UE can maintain two TAs and how to maintain the two TAs should be clarified.

Proposal 2:

It should be clarified that if the two TAs put additional requirements on the UE realization, such as clock retunings, process time of switching between the two TAs and whether there are additional requirements for UE to maintain two TAs.

In Rel-17, the multiple DCI under mTRP was assumed with non-ideal backhaul connections. And the collisions between two PUSCHs scheduled by two DCIs are not allowed. It is means that although the coordination between the TRPs is slow due to non-ideal backhaul, but there still needs coordination between the two TRPs because UEs are not expected to be scheduled with the overlapped time domain resources. This put additional requirements or the more limitations to the gNBs’ scheduling.

When only one TA is used for the two TRPs transmission, gNBs should try to schedule the UE with different time domain resources. But if two TAs are considered, the overlapping due to different TAs should be taken into account. Currently, in the specification, when two slots are overlapped due to different TA, the latter slot is reduced in the duration relative to the former slot.

Proposal 3:

It is proposed to discussed the overlapped transmission issues under the mTRP with two TAs. The legacy mechanisms could be a starting point.

Conclusions

In this contribution, we discussed the related issues about two TAs for UL multi-DCI for multi-TRP operation. The proposals are listed as below.

Proposal 1:

- It is proposed focusing on the FR2 scenario in which each TRP have a different TA.

- The two TA scenario of FR1 needs to be clarified.

Proposal 2:

It should be clarified that if the two TAs put additional requirements on the UE realization, such as clock retunings, process time of switching between the two TAs and whether there are additional requirements for UE to maintain two TAs.

Proposal 3:

It is proposed to discussed the overlapped transmission issues under the mTRP with two TAs. The legacy mechanisms could be a starting point.

References

- [1] RP-213598, New WID: MIMO Evolution for Downlink and Uplink, Samsung, 3GPP TSG RAN Meeting #94e, Electronic Meeting, Dec. 6 - 17, 2021