3GPP TSG RAN WG1 #109-e			                                  R1-2204292

e-Meeting, May 9th – May 20th, 2022

Agenda item:	9.1.4.1

Source: 	CMCC

Title: 	Discussion on UL precoding indication for multi-panel transmission

Document for:	Discussion and Decision

Introduction

In the RAN #94-e meeting, the new WI of MIMO Evolution for Downlink and Uplink was agreed. The objective related to this agenda is listed below [1].

- Study, and if needed, specify the following items to facilitate simultaneous multi-panel UL transmission for higher UL throughput/reliability, focusing on FR2 and multi-TRP, assuming up to 2 TRPs and up to 2 panels, targeting CPE/FWA/vehicle/industrial devices (if applicable) - UL precoding indication for PUSCH, where no new codebook is introduced for multi-panel simultaneous transmission - The total number of layers is up to four across all panels and total number of codewords is up to two across all panels, considering single DCI and multi-DCI based multi-TRP operation.

In this contribution, we discussed the related issues about UL precoding indication for the simultaneous multi-panel UL transmission.

Discussion

The UL simultaneous multiple panel UL transmission could bring additional throughput and enhance the reliability in the mTRP scenarios. Since two panels and two TRPs are considered, one panel to one TRP transmission and two panel to one TRP transmission could happen in this scenario. For the two panels simultaneous UL transmission with one panel to one TRP respectively, then throughput could be enhanced by allocating two different TBs or codewords to the two transmissions. And if a single TB or codeword is allocated to the two uplink transmissions, the reliability would be enhanced.

Proposal 1:

For the simultaneous multiple panel UL transmission, both two panel to two TRPs and single panel to two TRPs should be considered.

As one of the targets is to enhance the throughput, it should be allowed UE to transmit two codewords with different MCS to adapting its channel conditions. If the two codewords are allowed, for each CW, it could use different number of layers for the transmission. One reason is that, for each of the two panels, it may have different number of ports. And the gNB could determine the transmission layer numbers according to the port number of the panel and the rank of the channel. In the WID, only maximum 4 layers are assumed for the UL simultaneous transmission. Then, the combination of {2,2} and {1,3} should be supported, in which each value in the combination represents the layers number of one panel.

Proposal 2:

Two codewords with different layer numbers and MCS should be considered in the simultaneous multiple panel UL transmission.

In Rel-17 MIMO, only the port number of multiple panels with different values are reported to the gNB. For example, if the UE has two panel but with a same two ports for the uplink, the UE will only report single two ports to the gNB. The gNB cannot distinguish whether the UE have a single panel with a two uplink ports or two panels with two ports. But for the UE with different UL ports in the two panel, e.g. 2 and 4 uplink ports, the gNB can understand that the UE has two panels and one with 2 UL ports and the other with 4 UL ports. In this agenda, the UE capability reporting of two panel with the same number of the ports should be solved.

Proposal 3:

It should be discussed the UE capability reporting of multiple panels with a same UL ports number in each panel.

For the UL scheduling, both single and multiple DCIs should be allowed. In the single DCI, it should be support to indicated two UL transmissions with 2 CW and different layers. And for the multiple DCI, each DCI could schedule one transmission from one panel. For each DCI of the mDCI case, the indication of MCS and UL layer numbers is not different from the single TRP scenario. But the total number of the layer should not exceed four. Additional coordination and limitations should be considered for this case. And for the two UL grant DCIs, they should not schedule a same panel for the UL transmission. It also should be considered that whether the same resources or overlapped resources in frequency domain is allowed in the multiple DCIs scenarios. It is not clear whether the UE can support mapping two parts of the data to the same frequency domain resources in a short time. And in addition, if the two panel transmit their own uplink data to the two TRPs, the interference between two uplinks should be considered.

Proposal 4:

Several issues should be considered in the multiple DCI scenario, such as the total number of the layers should not exceed four and the two DCI should not schedule a same panel.

Proposal 5:

The resource collision or the overlapped frequency domain resources between the two panel simultaneous transmission should be discussed.

A last but not least issue is the UL power control. The maximum transmit power should not exceed the maximum transmit power. It may not be a issue under single DCI. But under the multiple DCI scenario, each DCI have its own power control command. It should prevent the sum of transmit power from two panels exceed the maximum power. The power limitation of each panel should be considered for the UL power control under multiple DCI scenarios.

Proposal 6:

The power limitation of each panel should be considered for the UL power control under multiple DCI scenarios.

Conclusions

In this contribution, we discussed the related issues about UL precoding indication for the simultaneous multi-panel UL transmission. The proposals are listed as below.

Proposal 1:

For the simultaneous multiple panel UL transmission, both two panel to two TRPs and single panel to two TRPs should be considered.

Proposal 2:

Two codewords with different layer numbers and MCS should be considered in the simultaneous multiple panel UL transmission.

Proposal 3:

It should be discussed the UE capability reporting of multiple panels with a same UL ports number in each panel.

Proposal 4:

Several issues should be considered in the multiple DCI scenario, such as the total number of the layers should not exceed four and the two DCI should not schedule a same panel.

Proposal 5:

The resource collision or the overlapped frequency domain resources between the two panel simultaneous transmission should be discussed.

Proposal 6:

The power limitation of each panel should be considered for the UL power control under multiple DCI scenarios.

References

- [1] RP-213598, New WID: MIMO Evolution for Downlink and Uplink, Samsung, 3GPP TSG RAN Meeting #94e, Electronic Meeting, Dec. 6 - 17, 2021