3GPP TSG RAN WG1 #109-e			                                                R1-2204295

e-Meeting, May 9th – 20th, 2022

Agenda item:	9.2.2.1

Title:	Discussion on evaluation on AI/ML for CSI feedback enhancement

Source:	CMCC

Document for:	Discussion and Decision

- Introduction

In RAN#94e, the Rel-18 SID for Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface was approved [1], the objective of this study item is to study the 3GPP framework for AI/ML for air-interface corresponding to each target use case regarding aspects such as performance, complexity, and potential specification impact. In this SID, one specific use case for AI/ML is CSI feedback enhancement.

Use cases to focus on: - Initial set of use cases includes: - CSI feedback enhancement, e.g., overhead reduction, improved accuracy, prediction [RAN1] - Beam management, e.g., beam prediction in time, and/or spatial domain for overhead and latency reduction, beam selection accuracy improvement [RAN1] - Positioning accuracy enhancements for different scenarios including, e.g., those with heavy NLOS conditions [RAN1] - Finalize representative sub use cases for each use case for characterization and baseline performance evaluations by RAN#98 - The AI/ML approaches for the selected sub use cases need to be diverse enough to support various requirements on the gNB-UE collaboration levels

In this contribution, we present our views on the evaluation methodology and KPI for AI/ML based CSI feedback enhancement and some initial evaluation results are also provided here.

- Evaluation methodology and preliminary results

AI based CSI feedback enhancement is aimed at CSI reporting overhead reduction and accuracy improvement. In our contribution [2], we have discussed the sub use case for CSI feedback enhancement. In this section, we show our specific evaluation methodology, KPI and preliminary results for the sub use case of AI based CSI compression and feedback.

As shown in Fig.1, for the use case of AI based CSI enhancement, UE obtain the joint eigenvectors form the channel matrix H, and then compress the eigenvectors to a bitstream via encoder and quantization. After receiving the compressed bitstream, gNB will recover the feedback bits to the eigenvectors through dequantization and decoder.

Fig.1 AI based CSI feedback framework

2.1 KPI

For the evaluation of AI based CSI enhancement, we use the square of generalized cosine similarity (SGCS) as the criterion to evaluate the difference between recovered channel and the original channel.

The SGCS is defined as:

wherein, the  is the original eigenvector of the n-th suband, the  is the recovered eigenvector of the n-th suband (n=1, …, N).

2.2 Data set

Statistical channels used in link-level simulation or system level simulation can be used to construct the data set for the evaluation of AI based CSI feedback. In this contribution, we use CDL-C-30 channel to generate channel data. The eigenvectors of each subband are used for model training, model validation and model testing.

The detail evaluation assumptions are shown below:

Table 1.  Key simulation parameters

| Parameter | Value |
| --- | --- |
| Carrier Frequency | 3.5GHz |
| Bandwidth | 10MHz |
| Subcarrier spacing | 15KHz |
| RB number | 48 |
| Subband number | 12 |
| BS antenna number | 32 |
| UE antenna number | 4 |
| Channel Model | CDL-C |
| UE speed | 3km/h |
| Delay spread | 30ns |
| Rank | 1 |
| UE number | 1000 |
| Slot number | 100 |

2.3 Preliminary results

The EVCsiNet model based on [3] and a transformer model are used in AI model training. Besides, uniform quantization is used in the evaluation. The AI models apply GCS as its loss function. The AI models for different feedback bits are different.

The square of generalized cosine similarity (SGCS) between recovered eigenvectors and original eigenvectors under different feedback bits are shown below. Besides, the SGCS for Type-I and enhanced Type-II codebook under approximate feedback bits are also given here for comparison.

Table 2. The SGCS of different CSI approaches over different feedback bits

| Approach | Feedback bits |  |
| --- | --- | --- |
| Type-I | 32 | 0.626 |
| EVCsiNet | 32 | 0.913 |
| Transformer | 32 | 0.900 |
| e Type-II, L=2 | 49 | 0.840 |
| EVCsiNet | 48 | 0.941 |
| Transformer | 48 | 0.942 |
| e Type-II, L=4 | 128 | 0.941 |
| EVCsiNet | 120 | 0.977 |
| Transformer | 120 | 0.980 |

According the evaluation results above, it can be observed that:

- With the same feedback bits, AI based approach could obtain 4%~40% performance gain over traditional codebook in the square of generalized cosine similarity;

- With the same performance in the square of generalized cosine similarity, AI based approach could reduce 30%~60% feedback bits.

Observation 1: Compared with traditional codebook, AI/ML based CSI feedback schemes could improve the CSI accuracy with the same feedback bits.

Observation 1: Compared with traditional codebook, AI/ML based CSI feedback schemes could reduce CSI feedback bits with the same CSI accuracy.

- Conclusion

In this contribution, we share our views on the evaluation methodology and KPI for AI based CSI compression and the preliminary evaluation results are also provided. The observations are summarized below:

Observation 1: Compared with traditional codebook, AI/ML based CSI feedback schemes could improve the CSI accuracy with the same feedback bits.

Observation 1: Compared with traditional codebook, AI/ML based CSI feedback schemes could reduce CSI feedback bits with the same CSI accuracy.

- References

- RP-213599, New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface

- R1-2204296, Discussion on other aspects on AI/ML for CSI feedback enhancement

- W. Liu, W. Tian, H. Xiao, S. Jin, X. Liu and J. Shen, "EVCsiNet: Eigenvector-based CSI Feedback under 3GPP Link-Level Channels," IEEE Wireless Communications Letters, doi: 10.1109/LWC.2021.3112747.