3GPP TSG RAN WG1 #109-e			R1-2204306

e-Meeting, May 9th – 20th, 2022

Agenda item:	9.4.1.1

Source: 	CMCC

Title: 	Discussion on channel access mechanism for sidelink on unlicensed spectrum

Document for:	Discussion and Decision

Introduction

In RANP#94 and #95 meetings, the following objective in NR sidelink evolution SID/WID [1] has been achieved, which leads RAN1 to study and specify support of sidelink on unlicensed spectrum (SL-U). The most important motivation is to increase the sidelink data rate and fulfil the requirements of commercial use cases, it will be a more interesting scenario in Rel-18 although NR sidelink was originally developed for V2X applications.

Study and specify support of sidelink on unlicensed spectrum for both mode 1 and mode 2 where Uu operation for mode 1 is limited to licensed spectrum only [RAN1, RAN2, RAN4] - Channel access mechanisms from NR-U shall be reused for sidelink unlicensed operation - Assess the applicability of sidelink resource reservation from Rel-16/Rel-17 to sidelink unlicensed operation within the boundaries of unlicensed channel access mechanism and operation - No specific enhancements for Rel-17 resource allocation mechanisms - If the existing NR-U channel access framework does not support the required SL-U functionality, WGs will make appropriate recommendations for RAN approval. - Physical channel design framework: Required changes to NR sidelink physical channel structures and procedures to operate on unlicensed spectrum - The existing NR sidelink and NR-U channel structure shall be reused as the baseline. - No specific enhancements for existing NR SL feature - The study should focus on FR1 unlicensed bands (n46 and n96/n102) and is to be completed by RAN#98.

In this contribution, we would like to share our preliminary views on channel access mechanism for SL-U, since this is the first meeting of Rel-18, we suggest that RAN1 should first study the basic design principle for channel access mechanism in SL-U during this meeting, with the existing NR sidelink and NR-U design as baseline.

Channel access mechanism in SL-U

In NR-U, both Load Based Equipment (LBE) and Frame Based Equipment (FBE) are supported. The former is a dynamic channel access mechanism which is more suitable for the scenarios where the existence of other RATs cannot be guaranteed; the latter is a semi-static mechanism intended for environments where the existence of other RATs can be guaranteed. From our point of view, the aforementioned two cases may still exist in SL-U, so both of the LBE and FBE schemes are still needed to be supported.

Proposal 1: RAN1 should study and support both LBE and FBE in SL-U.

For LBE scheme, there are four Listen Before Talk (LBT) types have been supported in NR-U to achieve channel access, type 1 mechanism is mainly used for initiating a Channel Occupancy Time (COT) by a gNB/UE; type 2A/2B/2C mechanism is mainly used for sharing a COT by a UE/gNB. The rough descriptions about these channel access mechanisms can be found in the below table:

Table 1 LBT types in NR-U

| LBT type | Gap requirement | Sensing duration |
| --- | --- | --- |
| Type 1 | No | Random, based on random backoff and CWS adjustment principle. |
| Type 2A | 25us | Fixed, successful access requires the channel to be idle for 25us. |
| Type 2B | 16us | Fixed, successful access requires the channel to be idle for 16us. |
| Type 2C | <16us | No need to sense the channel but the COT shall be less than 584us. |

According to the SID/WID, channel access mechanisms from NR-U shall be reused for SL-U, since we will also propose to support UE-to-UE COT sharing (which will be introduced in a later section) in Rel-18, we think all of the four types channel access mechanisms can be reused in SL-U.

Proposal 2: RAN1 should study and support to reuse NR-U channel access mechanisms, including all of the LBT type 1, type 2A, type 2B, type 2C, in SL-U.

Resource allocation in SL-U

Consecutive resource allocation in mode 1 and mode 2

In NR-U, consecutive multiple PUSCHs scheduling is supported in NR-U, the purpose is to continuously occupy the channel and reduce the impact of LBT failure, for similar considerations, this feature should also be supported in SL-U for both mode 1 and mode 2.

Figure 1 Illustration of consecutive multiple PUSCHs scheduling in SL-U

For mode 1, the design of DCI format 0_1 and CG configuration in NR-U could be a reference, through some similar enhancements on DCI format 3_0 or a new DCI format for SL-U scheduling, or enhancements on SL CG configuration, consecutive PSSCH resource allocation in SL-U can be realized.

For mode 2, by using current Rel-16 resource selection mechanism, only discrete resources can be selected and this may have bad impacts on the performance of SL-U due to more LBT checks and higher probability of losing channel occupancy. Therefore, some enhancements should be done on mode 2 resource allocation mechanism to make sure consecutive resources can be selected during resource selection procedure.

Observation 1: Only discrete resources can be selected during Rel-16 resource selection procedure, which may have bad impacts on the performance of SL-U due to more LBT checks and higher probability of losing channel occupancy.

Proposal 3:	RAN1 should study the enhancements on resource allocation mechanism to ensure consecutive channel occupancy in SL-U:

- For mode 1, enhancements on both DG and CG can be considered, the design of DCI format 0_1 and CG configuration in NR-U can be a reference;

- For mode 2, enhancements on Rel-16 resource selection procedure can be considered, to make sure consecutive resources are selected after performing the procedure.

Relationship b/w Sensing/reservation mechanism and LBT

In NR sidelink, for mode 2, UE performs SCI-based sensing in sensing window and selects available resources in the corresponding selection window, this procedure can provide long-term sensing results and avoid collision with other SL users; whereas, LBT is a short-term listening mechanism which is mandated by regulations when deployed in unlicensed bands, in order to eliminate collisions with the users from other RATs. From our point of view, these two operations cannot replace with each other, and if they can work well in coordination, it will help the UE to eliminate both internal and external interference, then the performance of SL-U can be increased.

Figure 2 Illustration of sensing/reservation mechanism in NR sidelink

Figure 3 Illustration of LBT procedure in NR-U

Observation 2: Sensing/reservation mechanism can provide long-term sensing results and eliminate the internal interference while LBT can provide short-term sensing results and eliminate the external interference, these two operations cannot replace with each other.

Proposal 4:	RAN1 should study the feasibility of support both sensing/reservation mechanism and LBT in SL-U, and the potential enhancements to make them work together better.

UE-to-UE COT sharing

In NR-U, COT sharing between gNB and UE is supported. The purpose is to transmit as much data as possible after accessing the channel. If the COT initiator’s own transmission is completed, the remaining COT can be shared to the peer device for the transmission in opposite direction.

- DL-to-UL COT sharing: gNB attempts to access the channel to perform downlink transmission through LBT. After LBT successful, it can indicate the UE to perform uplink transmission in the remaining COT time through dynamic scheduling (DG), and the parameters for the UE to perform uplink channel access are also indicated by the gNB;

- UL-to-DL COT sharing: UE uses semi-statically configured resources (CG) and attempt to access the channel to perform uplink transmission through LBT. After LBT successful, it can indicate the relevant parameters by carrying them in CG-UCI. GNB can perform downlink transmission in the follow-up time of the COT.

Figure 4 Illustration of COT sharing b/w gNB and UE in NR-U

In SL-U, it is not possible to support COT sharing b/w gNB and UE because gNB can only work on licensed band in Rel-18. However, UE-to-UE COT sharing can be considered because sidelink is a UE-to-UE type transmission. For instance, a COT initiator can first perform a certain time of SL transmission after successful access the channel by using type 1 LBT procedure, then it may share the remaining time of the COT to one or multiple other UEs, the corresponding parameters, such as LBT type, CPE length and CAPC, can be included in the SCI from the COT initiator UE. The target UE can perform the channel access procedure according to these parameters and then transmit their own sidelink data after successful occupy the channel.

Figure 5 Illustration of UE-to-UE COT sharing

To support UE-to-UE COT sharing, RAN1 may need to further study the design details and make clarifications on some potential issues. In our views, there are at least three potential issues should be clarified in RAN1 study:

- Which UE can share a COT initiated by another UE?

  - Option 1-1: only the UEs inside a same sidelink communication session can share the COT with each other.

  - Option 1-2: every UE surrounding the COT initiator can share the COT.

For option 1-1, in our companion views, since gNB can only perform DL unicast transmission to the COT initiator in NR-U, similarly, it seems option 1-1 is more feasible for a COT sharing target UE to perform at least PSCCH/PSSCH transmission, but since there are also some other transmissions may need to be performed, e.g., PSFCH, RAN1 should further study this option with the consideration of variable characteristics of different channels in SL. One further issue should be studied or discussed is which or which combinations of cast types are supported in this case, in our views, RAN1 should at least support COT sharing b/w the peer UEs in a unicast session, but whether and how to support COT sharing feature for groupcast may need more discussions since it may bring more complexities. Broadcast may not be applicable in this case because UE has no information about the intended receiver of the transmission.

For option 1-2, there seems no need to limit the cast type during a COT for both the transmissions from the COT initiator and the COT sharing target UE. In this case, each slot in the COT can be separately used for COT initiator or COT sharing target UE to perform SL transmission for any possible destination ID. However, one further issue for this option is how to decide which UE is the COT sharing target, and how to resolve the collision issue when multiple UEs would like to be the COT sharing target UE simultaneously.

Figure 6(a) Illustration of intra-session COT sharing       6(b) Illustration of inter-session COT sharing

- Whether some prerequisites are required for COT sharing operation, with the considerations of hidden node issue?

As aforementioned, a COT sharing target (UE B) can try to access the channel by using type 2A or 2B or 2C LBT procedure, which has higher possibility for successful occupying the channel. However, if another UE C surrounding UE B also would like to transmit data by initiating a new COT with type 1 LBT, and it cannot be sensed by the COT initiator UE (UE A), the transmissions from UE B and UE C are more like to be blocked by each other in this case. For instance, hidden node issue occurs more frequently in SL than Uu deployment, so it is worthwhile to consider some prerequisites for COT sharing operation in SL-U.

Figure 7 Hidden node issue in COT sharing scenario

- Whether separate resource selection mechanism should be deployed for in-COT and out-of-COT case?

- Option 3-1: in-COT and out-of-COT case use a unified resource selection mechanism, such as the legacy mode 2 resource selection procedure defined in Rel-16.

- Option 3-2: separate mechanism should be designed for in-COT case, e.g., a mode 2(d) like resource allocation mechanism can be considered.

In the first option, every UE shall perform sensing/reservation procedure regardless of whether the transmission is within a COT; in the second option, the COT initiator can reserve a set of resources in a COT by using sensing/resource selection and LBT procedure, then it can allocate the resources for the COT sharing target UE which may not need to perform legacy sensing procedure which is designed in NR sidelink. From our perspective, both of the two options are open to be studied and discussed in RAN1 because either of them has its own pros and cons.

Proposal 5: RAN1 should study how to support UE-to-UE COT sharing in SL-U, with the considerations of at least the following issues:

- Which UE can share a COT initiated by another UE, both intra-session and inter-session operation can be considered;

- Whether some prerequisites are required for UE-to-UE COT sharing, to address the hidden node issue in SL deployment;

- Whether the resource selection mechanism needs to be carried out separately for in-COT and out-of-COT cases, e.g., a mode 2(d) like mechanism is specially designed for in-COT scenario.

Conclusions

In this contribution, we have shared our views on the channel access mechanism in SL-U, the following observations and proposals are provided:

Observation 1: Only discrete resources can be selected during Rel-16 resource selection procedure, which may have bad impacts on the performance of SL-U due to more LBT checks and higher probability of losing channel occupancy.

Observation 2: Sensing/reservation mechanism can provide long-term sensing results and eliminate the internal interference while LBT can provide short-term sensing results and eliminate the external interference, these two operations cannot replace with each other.

Proposal 1: RAN1 should study and support both LBE and FBE in SL-U.

Proposal 2: RAN1 should study and support to reuse NR-U channel access mechanisms, including all of the LBT type 1, type 2A, type 2B, type 2C, in SL-U.

Proposal 3:	RAN1 should study the enhancements on resource allocation mechanism to ensure consecutive channel occupancy in SL-U:

- For mode 1, enhancements on both DG and CG can be considered, the design of DCI format 0_1 and CG configuration in NR-U can be a reference;

- For mode 2, enhancements on Rel-16 resource selection procedure can be considered, to make sure consecutive resources are selected after performing the procedure.

Proposal 4:	RAN1 should study the feasibility of support both sensing/reservation mechanism and LBT in SL-U, and the potential enhancements to make them work together better.

Proposal 5: RAN1 should study how to support UE-to-UE COT sharing in SL-U, with the considerations of at least the following issues:

- Which UE can share a COT initiated by another UE, both intra-session and inter-session operation can be considered;

- Whether some prerequisites are required for UE-to-UE COT sharing, to address the hidden node issue in SL deployment;

- Whether the resource selection mechanism needs to be carried out separately for in-COT and out-of-COT cases, e.g., a mode 2(d) like mechanism is specially designed for in-COT scenario.

References

- RP-220300, WID revision: NR sidelink evolution, 3GPP TSG RAN Meeting #95e.