3GPP TSG RAN WG1 #109-e			R1-2204307

e-Meeting, May 9th – 20th, 2022

Agenda item:	9.4.1.2

Source: 	CMCC

Title: 	Discussion on physical channel design framework for sidelink on unlicensed spectrum

Document for:	Discussion and Decision

Introduction

In RAN#94-e meeting, a new WID on NR sidelink evolution was approved [1], in which the application of NR sidelink is further expanded to the commercial use cases with increased sidelink data rate.

To achieve the increased data rate, the support of sidelink over unlicensed spectrum was included as one of the objectives in Rel-18 sidelink evolution, and will be investigated from the beginning of the work item phase:

- Study and specify support of sidelink on unlicensed spectrum for both mode 1 and mode 2 where Uu operation for mode 1 is limited to licensed spectrum only [RAN1, RAN2, RAN4] - Channel access mechanisms from NR-U shall be reused for sidelink unlicensed operation - Assess the applicability of sidelink resource reservation from Rel-16/Rel-17 to sidelink unlicensed operation within the boundaries of unlicensed channel access mechanism and operation - No specific enhancements for Rel-17 resource allocation mechanisms - If the existing NR-U channel access framework does not support the required SL-U functionality, WGs will make appropriate recommendations for RAN approval. - Physical channel design framework: Required changes to NR sidelink physical channel structures and procedures to operate on unlicensed spectrum - The existing NR sidelink and NR-U channel structure shall be reused as the baseline. - No specific enhancements for existing NR SL feature - The study should focus on FR1 unlicensed bands (n46 and n96/n102) and is to be completed by RAN#98.

In this contribution, we provide our preliminary views on the physical channel design framework of SL-U.

Discussion

- 2.1 Special limits for unlicensed spectrum

For unlicensed spectrum, some essential regulation requirements are specified [2], which should be taken into account when designing the physical channel structure for sidelink over unlicensed spectrum:

- Nominal channel bandwidth: In unlicensed band, the nominal channel bandwidth for a single operation channel shall be 20MHz;

- Occupied channel bandwidth (OCB): OCB is the bandwidth containing 99% of the power of the signal, and device operating in unlicensed spectrum should meet the OCB limits, which requires that the OCB shall be between 80%~100% of the nominal channel bandwidth;

- Listen before talk (LBT): A device shall implement an LBT based channel access mechanism to detect the presence of other RLAN transmissions on an operating channel before using the channel. The basic unit for sensing is a sensing slot with a duration of 9us. The device can then occupy the channel if it is sensed to be idle.

- Channel occupancy time (COT): A device can have multiple transmissions within a COT, where the gaps between the transmissions is smaller than 16us or 25us. Otherwise, other devices may access to the channel if the LBT succeeds and therefore the device may lose the COT.

- 2.2 Resource pool configuration

Considering the special limits for the unlicensed spectrum mentioned in Section 2.1, we provide our views on the SL-U resource pool configuration.

Frequency domain

In NR sidelink, the bandwidth and starting position of a sidelink carrier are configured by higher payer, and a single bandwidth part (SL BWP) for sidelink transmission is configured in the sidelink carrier.

Within the SL BWP, a UE can be configured by higher layers with one or more sidelink resource pools for transmission/reception.

In frequency domain, a sidelink resource pool consists of N continuous sub-channels, and a sub-channel consists of M continuous PRBs.

Figure 1: NR sidelink resource pool in frequency domain

To facilitate the discussion of the SL-U resource pool configuration enhancements in frequency domain, the enhancements in NR-U is firstly reviewed in the following.

1) BWP and RB sets

According to the definition of the nominal channel bandwidth for unlicensed spectrum, a single operation channel is 20MHz (The 20MHz operation channel is defined as resource block set, RB set, in NR-U specification). In contrast to unlicensed band, NR system design allows a UE to operate on a wider bandwidth part to support higher data rate traffic. To maintain the NR configuration flexibility in frequency domain for NR-U, a UE can be configured with a BWP equal to or larger than 20MHz, with the limitation of the BWP shall include N continuous RB sets, where N is a positive integer.

Figure 2: NR-U BWP and RB sets

2) Interlace design for UL signals/channels

To meet the OCB requirements and maximize the transmit power under maximum power spectral density requirements restriction in unlicensed band, NR-U introduces interlace based waveform for UL signals/channels. To be specific, the interlaced waveform is defined for SCS 15kHz and 30kHz. An interlace consists of the ith resource blocks in every M PRBs (1≤i≤M), where M is 10 for 15kHz SCS and 5 for 30kHz SCS. The number of resource blocks in an interlace within a BWP shall be no less than 10.

Figure 3: Interlace design for NR-U UL signals/channels

Based on the above enhancements in NR-U, when designing the SL-U resource pool configuration in frequency domain, its relationship with the RB sets should be considered again. In addition, as sidelink can only operate on the semi-persistently configured uplink resources, the interlace design for UL signals/channels in NR-U should also be taken into consideration for SL-U resource pool configuration.

In our views, SL-U resource pool can consist one or more continuous RB sets in frequency domain. In addition, both interlaced and non-interlaced resource allocation should be supported in SL-U, and it can depend on resource pool (pre-)configuration. For a resource pool in which the interlaced waveform is enabled, the resources occupied by the sidelink physical channels are the intersections of allocated RB sets, indicated interlaces, and the corresponding guard bands in-between the RB sets.

Figure 4: Illustration of SL-U resource pool configuration in frequency domain

Proposal 1: SL-U resource pool can consist one or more continuous RB sets in frequency domain.

Proposal 2: Both interlaced and non-interlaced resource allocation mechanisms should be supported in SL-U.

Proposal 3: For a resource pool in which the interlaced waveform is enabled, the resources occupied by the sidelink physical channels are the intersections of allocated RB sets, interlaces, and the guard band.

Time domain

In NR sidelink, the set of slots that belong to a sidelink resource pool is determined by a bitmap after excluding slots with S-SSB and slots of which at least one of the symbols determined by startSLsymbols and lengthSLsymbols are not semi-persistently configured as UL. In other words, the logical slots may not be continuous from the perspective of physical.

Figure 5: NR sidelink resource pool in time domain

Note that the gaps between two logical slots are in slot level, and much longer than 16us or 25us, therefore, the discontinuity of the logical slots will prevent a UE to keep the occupancy of the channel, which is not feasible to maintain a COT.

Observation 1: NR sidelink resource pool configuration in time domain may not be continuous in physical slots, and is difficult to maintain a COT in unlicensed spectrum.

To solve this problem, the most straightforward solution is to enhance the resource pool configuration in time domain, and ensure that a resource pool contains contiguous physical slots.

Proposal 4: The resource pool configuration in time domain should be enhanced to ensure that a SL-U resource pool contains contiguous physical slots.

- 2.3 Slot structure

Transmission occasions within a slot

In NR sidelink, a minimum resource allocation unit in time domain is a slot. A UE can only transmit PSSCH in the symbols available for sidelink within a slot, which are (pre-)configured per SL BWP by higher layer parameters startSLsymbols and lengthSLsymbols, where startSLsymbols is the symbol index of the first symbol of lengthSLsymbols consecutive symbols.

According to the LBT procedures defined for NR-U, the basic unit for sensing is a sensing slot with a duration of 9us. A device can occupy the channel if it is sensed to be idle. Note that the sensing granularity is much smaller than the duration of a slot, it is possible that a channel is sensed to be idle in the middle of a sidelink slot.

Observation 2: The sensing unit of the LBT procedure is much smaller than the duration of a slot, it is possible that a channel is sensed to be idle in the middle of a sidelink slot.

A potential solution is to (pre-)configure more than one startSLsymbols and lengthSLsymbols per SL BWP, and hence introduces multiple channel access opportunities in a single sidelink slot. Note that with the increasing transmission occasions within one slot, the UE needs to blind detect more potential PSCCH candidates in time domain, and may lead to the increasement of UE complexity of PSCCH blind detection for both sensing and PSSCH reception. At the starting point of SL-U, we can first consider 2 sets of startSLsymbols and lengthSLsymbols higher layer parameters.

Figure 6: Illustration of multiple PSCCH/PSSCH transmission candidate occasions in a single slot

Proposal 5: (Pre-)configuration of N sets of starting symbol and length per SL BWP can be studied. N = 2 can be considered as a starting point.

Gap symbols

In NR sidelink, a gap symbol is introduced for all sidelink physical signals/channels, including PSCCH/PSSCH, PSFCH, and S-SS/PSBCH block, to allow a UE to perform potential Tx/Rx switching. As the duration of a symbol is longer than 16us or 25us, a similar issue of UE losing the channel occupancy is also identified.

Observation 3: The gap symbol for Tx/Rx switching may lead to a UE loses the channel.

To solve this issue, one straightforward way is to use the CP extension in a gap symbol to reduce the duration of the gap.

Proposal 6: Support CP extension in a gap symbol to reduce the duration for maintaining the channel occupancy.

- 2.4 PSCCH/PSSCH multiplexing

In NR sidelink, sub-channel size is per resource pool (pre-)configured, and the candidate values are {10, 15, 20, 25, 50, 75, 100} PRBs. The frequency domain size of PSCCH is also per resource pool (pre-)configured, and the candidate values are {10, 12, 15, 20, 25} PRBs. PSCCH is limited in only one sub-channel, and occupies the lowest PRBs of the lowest sub-channel of the associated PSSCH. In addition, the number of PSCCH symbols is per resource pool (pre-)configured, and the candidate values are {2, 3} symbols. PSCCH is transmitted in the same slot as the associated PSSCH.

PSCCH and PSSCH are multiplexed in a TDM+FDM manner, i.e., a part of PSCCH and the associated PSSCH are transmitted using overlapping time resources in non-overlapping frequency resources, but another part of the associated PSSCH and/or another part of the PSCCH are transmitted using non-overlapping time resources, as shown in the following figure.

Figure 7: Illustration of PSCCH/PSSCH multiplexing

In SL-U, with the interlace design to meet the OCB requirements, the PSCCH/PSSCH should also map to the physical resources in interlaced waveform if it is enabled. In this sense, the PSCCH/PSSCH multiplexing should be further investigated.

One potential issue is that whether the PSCCH/PSSCH multiplexing design principle in NR sidelink where the PSCCH occupies the lowest PRB of the lowest subchannel of the associated PSSCH, should be reused in SL-U. If so, the most straightforward method is to use the lowest interlace of the lowest RB set of the associated PSSCH for PSCCH transmission. However, it may lead to limited PRB size of the PSCCH, as an interlace only contains 10 or 11 PRBs, which may lose the configuration flexibility of NR sidelink PSCCH size and may also impact the coverage of the SL-U control channel. On the other hand, if the PSCCH is expanded to occupy more than one interlaces or RB sets, the benefit is more PRBs can be allocated to PSCCH; however, it may have risks of increasing the UE complexity to blind detect the PSCCH. For example, if the PSCCH can occupy more than one interlaces within an RB set, as in NR-U, in the case where PSSCH is allocated with non-contiguous interlaces, UE blind detection complexity is significantly increased compared to NR sidelink. Considering the case of 30kHz SCS, and a UE is allocated with 2 interlaces for PSCCH, there are C52=10 detection candidates for PSCCH with interlaced waveform.

Proposal 7: PSCCH/PSSCH multiplexing rule should be studied for interlaced waveform in SL-U.

- 2.5 HARQ-ACK feedback on PSFCH

PSFCH format and resource determination

In NR sidelink, only one PSFCH format (PSFCH format 0) is specified which is a sequence-based physical channel. The sequence maps to the physical resource of 1 PRB in frequency domain and 1 symbol in time domain (The PSFCH contains two consecutive symbols in total as the first symbol is the duplication of the second symbol and used as AGC). Only 1 ACK/NACK bit can be carried by PSFCH format 0.

In time domain, PSFCH resources occurs periodically in a resource pool, the candidate periodicity values are {0, 1, 2, 4} slots. In a slot that contains PSFCH resources, two symbols are used for PSFCH transmission wherein the first one is used for AGC. In frequency domain, PSFCH resources is firstly indicated by higher layer with a set of PRBs. Then the configured PRBs can be divided into some PRB sets first by ascending order of the PSSCH slot index and then by ascending order of the PSSCH subchannel index. In a PRB set, FDM and CDM are used to multiplex different UEs’ PSFCH resources which can be further distinguished by UE source ID and member ID.

Figure 8: NR sidelink PSFCH resources determination.

Similar as PSCCH/PSSCH, to meet the OCB requirements, interlaced waveform design should also be considered for PSFCH.

One issue is whether only PSFCH format 0 is reused in SL-U. If so, with the interlaced PSFCH waveform, PSFCH format 0 in frequency domain should be extended from one PRB to one interlace. Note that the interlaced PSFCH will lead to inefficiency of resource utilization, new PSFCH resource determination rules should be considered, so that a resource pool has enough PSFCH transmission resources and HARQ-ACK multiplexing capability.

On the other hand, due to the limited payload size of PSFCH format 0 in SL-U, and also considering a UE has multiple continuous transmissions for multiple TBs within a COT, new PSFCH format may need to be further studied. In this sense, more enhancements including the PSFCH resource indication and HARQ codebook should be considered as well.

Proposal 8: Interlaced design for PSFCH format 0 should be supported in SL-U with the consideration of enhancements on PSFCH resource determination.

Proposal 9: New PSFCH format in addition to PSFCH format 0 may need to be studied.

Proposal 10: If one or more new PSFCH formats are supported in SL-U, enhancements including PSFCH resource indication and HARQ codebook design should be further studied.

- 2.6 PHY procedures

According to Rel-16 NR sidelink, PHY procedures also need to be studied in SL-U, such as CSI-feedback, prioritization of transmissions/receptions, open-loop power control, etc. The key issue to be discussed is also the impacts of LBT failure on these procedures, and how to do the enhancements accordingly.

Proposal 11: RAN1 should study whether enhancements need to be done for PHY procedures in SL-U, including HARQ-ACK feedback, CSI-feedback, prioritization of transmissions/receptions and open-loop power control, and focusing on the impacts of LBT failure on these procedures.

For example, in NR sidelink, PSFCH can only be dropped due to prioritization, in addition, PSFCH dropping would be more frequent in SL-U due to LBT failure, which will be more harmful for reliability. To be specific, in NR-U, UE can store the un-transmitted HARQ-ACK bits in a PUCCH occasion and try to transmit them again in next PUCCH occasion, the same logic can be followed, then the issue is how to find more PSFCH occasions in SL-U because only one-to-one mapping is supported in Rel-16 NR sidelink.

Proposal 12: RAN1 should strive to define more PSFCH occasions corresponding to one PSSCH transmission in SL-U, to mitigate the impacts of PSFCH dropping due to LBT.

Conclusions

In this contribution, we provide our views on the physical channel design framework of SL-U, and the following observations and proposals are made:

Observation 1: NR sidelink resource pool configuration in time domain may not be continuous in physical slots, and is difficult to maintain a COT in unlicensed spectrum.

Observation 2: The sensing unit of the LBT procedure is much smaller than the duration of a slot, it is possible that a channel is sensed to be idle in the middle of a sidelink slot.

Observation 3: The gap symbol after the sidelink channels for Tx/Rx switching may lead to a UE loses the channel.

Proposal 1: SL-U resource pool can consist one or more continuous RB sets in frequency domain.

Proposal 2: Both interlaced and non-interlaced resource allocation mechanisms should be supported in SL-U.

Proposal 3: For a resource pool in which the interlaced waveform is enabled, the resources occupied by the sidelink physical channels are the intersections of allocated RB sets, interlaces, and the guard band.

Proposal 4: The resource allocation pool configuration in time domain should be enhanced to ensure that a SL-U resource pool contains contiguous physical slots. Proposal 5: (Pre-)configuration of N sets of starting symbol and length per SL BWP can be studied. N = 2 can be considered as a starting point.

Proposal 6: Support CP extension in a gap symbol to reduce the duration of the gapfor maintaining the channel occupancy.

Proposal 7: PSCCH/PSSCH multiplexing rule should be studied for interlaced waveform in SL-U.

Proposal 8: Interlaced design for PSFCH format 0 should be supported in SL-U with the consideration of enhancements on PSFCH resource determination.

Proposal 9: New PSFCH format in addition to PSFCH format 0 may need to be studied.

Proposal 10: If one or more new PSFCH formats are supported in SL-U, enhancements including PSFCH resource indication and HARQ codebook design should be further studied.

Proposal 11: RAN1 should study whether enhancements need to be done for PHY procedures in SL-U, including HARQ-ACK feedback, CSI-feedback, prioritization of transmissions/receptions and open-loop power control, and focusing on the impacts of LBT failure on these procedures.

Proposal 12: RAN1 should strive to define more PSFCH occasions corresponding to one PSSCH transmission in SL-U, to mitigate the impacts of PSFCH dropping due to LBT.

References

- RP-213678, New WID on NR sidelink evolution, OPPO, LG Electronics, RAN#94-e.

- ETSI EN 301 893, 5 GHz RLAN; Harmonised Standard covering the essential requirements of article 3.2 of Directive 2014/53/EU.