3GPP TSG RAN WG1 #109-e			R1-2204310

e-Meeting, May 9th – 20th, 2022

Agenda item:	9.5.1.3

Source: 	CMCC

Title: 	Discussion on potential solutions for SL positioning

Document for:	Discussion and Decision

Introduction

In RANP#94 meeting, the following objective [1] has been achieved which leads RAN1 to study support of NR sidelink positioning. Wherein, there is a need for RAN1 to study and develop sidelink positioning solutions to support the use cases, scenarios and requirements identified during parallel RAN1 discussions, which will also be mentioned in another contribution from our company [2].

- Study solutions for sidelink positioning considering the following: [RAN1, RAN2] - Scenario/requirements - Coverage scenarios to cover: in-coverage, partial-coverage and out-of-coverage - Requirements: Based on requirements identified in TR38.845 and TS22.261 and TS22.104 - Use cases: V2X (TR38.845), public safety (TR38.845), commercial (TS22.261), IIOT (TS22.104) - Spectrum: ITS, licensed - Identify specific target performance requirements to be considered for the evaluation based on existing 3GPP work and inputs from industry forums [RAN1] - Define evaluation methodology with which to evaluate SL positioning for the uses cases and coverage scenarios, reusing existing methodologies from sidelink communication and from positioning as much as possible [RAN1]. - Study and evaluate performance and feasibility of potential solutions for SL positioning, considering relative positioning, ranging and absolute positioning: [RAN1, RAN2] - Evaluate bandwidth requirement needed to meet the identified accuracy requirements [RAN1] - Study of positioning methods (e.g. TDOA, RTT, AOA/D, etc) including combination of SL positioning measurements with other RAT dependent positioning measurements (e.g. Uu based measurements) [RAN1] - Study of sidelink reference signals for positioning purposes from physical layer perspective, including signal design, resource allocation, measurements, associated procedures, etc, reusing existing reference signals, procedures, etc from sidelink communication and from positioning as much as possible [RAN1] - Study of positioning architecture and signalling procedures (e.g. configuration, measurement reporting, etc) to enable sidelink positioning covering both UE based and network based positioning [RAN2, including coordination and alignment with RAN3 and SA2 as required] Note: When the bandwidth requirements have been determined and the study of sidelink communication in unlicensed spectrum has progressed, it can be reviewed whether unlicensed spectrum can be considered in further work. Checkpoint at RAN#97 to see if sufficient information is available for this review.

In this contribution, we would like to provide our preliminary views on selecting the positioning methods from the candidate methods (e.g. TDOA, RTT, AOA/D, etc), and the design of reference signals, for sidelink positioning.

Sidelink positioning methods

In Rel-16, positioning methods including TDOA, RTT, AOA, and AOD have been specified for NR positioning. To enable sidelink positioning, RAN1 should first study and evaluate which or which combination of positioning methods can fulfil the sidelink positioning requirements and are able to be reused, by identifying the bottleneck of using these methods in sidelink positioning.

First, for timing-based methods, i.e., TDOA and RTT, better accuracy performance requires larger BW for positioning RS. As mentioned in our company’s companion [2], to avoid the further involvements of unlicensed spectrum for positioning RS design, which is also in a starting phase in the study item of NR sidelink evolution, and introducing a very stressful workload, the evaluation of TDOA and RTT should be based on only ITS band (20/40MHz) and licensed band in FR1 in Rel-18.

Observation 1: TDOA and RTT may requires larger BW for positioning RS.

Proposal 1: The evaluation of TDOA and RTT should be based on ITS band (20/40MHz) and licensed band in FR1in Rel-18 SL positioning.

Moreover, for TDOA, strict synchronization between anchor devices are required. This is very difficult to be achieved if the anchor devices are other vehicles. However, one possible applicable scenario is when the anchor devices are RSUs, in this case, the synchronization between anchor devices can be maintained well.

Figure 1 Illustration of TDOA for RSU-reference case

Observation 2: TDOA may only be applicable for the case where the anchor devices are RSUs.

For angle-based method (AoA/AoD), it is not sensitive to the bandwidth; however, it requires UE to support multiple antennas, which will increase the requirement for UE capabilities.

Observation 3: AOA/AOD may increase the requirement for UE capabilities since multiple antennas should be supported.

For AoD, in addition, positioning RS should support multi-beam operation, of which no baseline has been defined in sidelink because the study of FR2 in the study item of NR sidelink evolution is put on hold until further checking in RAN#97. It may also have impacts on sidelink PRS resource selection, i.e., multiple-slot candidate resources may need to be supported.

Observation 4: AoD requires positioning RS to support multi-beam while the study of FR2 in NR sidelink evolution is put on hold until further checking in RAN#97.

The bottlenecks of the positioning methods are summarized in the following table:

Table 1 Bottlenecks of the positioning methods in sidelink

| Method | Bottleneck 1 | Bottleneck 2 |
| --- | --- | --- |
| TDOA | Larger BW for positioning RS may be required. | Strict synchronization between positioning reference devices is required |
| RTT |  |  |
| AOA | The requirement for UE capabilities may be increased due to support of multiple antennas. |  |
| AOD |  | Multi-beam operation may be required. |

According to above analysis, we propose that RAN1 should further study which or which combination of positioning methods (e.g. TDOA, RTT, AOA/D, etc) should be selected for NR SL positioning, based on the evaluation results of positioning accuracy performance and feasibility of resolving the corresponding bottlenecks in Rel-18.

Proposal 2: RAN1 should further study which or which combination of positioning methods (e.g. TDOA, RTT, AOA/D, etc) should be selected for NR SL positioning, based on the evaluation results of:

- Positioning accuracy performance by using each method;

- Feasibility of resolving the respective method’s bottleneck in Rel-18 of each method.

In our initial views, based on above analysis, at least RTT can be supported in Rel-18 SL positioning due to strict synchronization is not necessary in this case, for both absolute positioning and relative positioning/ranging; Besides, TDOA can be used when the RSU type anchor device is identified, due to RSU is a kind of gNB-like UE, absolute positioning can be guaranteed to come true; however, whether to support AoA/AoD for both absolute positioning and relative positioning/ranging, should be subject to UE capability.

Proposal 3: Initial views from our company on which methods to be supported in Rel-18 SL positioning are:

- RTT can be supported in Rel-18 SL positioning for both absolute positioning and relative positioning /ranging;

- TDOA can be supported only in the case where the anchor devices are RSUs in Rel-18 SL positioning, both absolute positioning;

- Whether AoA/AoD can be supported should be subject to UE capability, for both absolute positioning and relative positioning/ranging.

Furthermore, it is straightforward to consider some joint scheme b/w NR sidelink Uu and SL positioning for some scenarios, as shown in Figure 2. As sidelink is more likely to be a LOS dominant and more ideal in propagation environment, the positioning accuracy performance can be improved in terms of the measurements results from both links.

Figure 2 Example scenarios for joint scheme b/w NR sidelink Uu and SL positioning

Proposal 4: Joint scheme b/w NR Uu and SL positioning can be considered to facilitate the positioning accuracy performance.

Sidelink positioning RS

Design baseline (PRS or SRS-Pos)

In Uu positioning, PRS is transmitted in downlink and used by UE to do measurement for the purpose of positioning; SRS-Pos is transmitted in uplink and used by TRP to do measurement for positioning. The advantage of using PRS is that it follows the design principle of using downlink signals as reference in NR sidelink; however, the advantage of using SRS-Pos is that it can provide lower PAPR. Thus, from our point of view, neither of these two should be precluded at this stage, RAN1 should further study which one should be used as design baseline for sidelink positioning RS. Another aspect should be considered is that which type of RS in time domain should be supported in Rel-18, including periodic, semi-persistent, and aperiodic.

Proposal 5: RAN1 should further study which signal should be used as design baseline for sidelink positioning RS, b/w PRS and SRS-Pos.

Proposal 6: RAN1 should further study which type of RS in time domain should be supported in Rel-18, including periodic, semi-persistent, and aperiodic.

PHY structure of sidelink positioning RS

In a NR sidelink slot, for each sidelink signal/channel, the first symbol is a copy of the second one for the purpose of AGC. Besides, the last symbol is used as a guard for Tx/Rx switching. Moreover, PSCCH may be needed in a SL positioning RS transmission slot, which will be introduced in latter part of this tdoc, 2 or 3 symbols are needed due to resource pool configuration, then, the remaining symbols can be regarded as candidate for sidelink positioning RS.

Figure 3 Potential slot structure of SL positioning RS slot

Proposal 7: Slot structure in NR sidelink should be reused as much as possible for sidelink positioning RS slot, which including AGC symbol, GP symbol and the potential PSCCH symbols, the remaining symbols can be regarded as candidates for positioning RS.

For positioning RS, the design in sidelink cannot be supported as so much flexible as NR Uu, because without gNB scheduling, some big issues may be caused such as resource collision, AGC performance degradation, and so on. This issue may have bad impacts on the positioning performance. Therefore, we propose to live the parameters of SL positioning RS, e.g., number of symbols, RS comb size, and RS BW, with resource pool level configuration. Furthermore, for the RS BW, RAN1 should further evaluate whether it should be directly equal to the bandwidth of the BWP or resource pool, in order to provide higher positioning accuracy. On the other hand, since there is only one BWP can be supported in sidelink, the configuration of SL positioning RS should be tied to BWP.

Proposal 8: Parameters of SL positioning RS, e.g., number of symbols, RS comb size, and RS BW, should be (pre)configured on resource pool level.

Proposal 9: Configuration of SL positioning RS should be tied to BWP.

In NR sidelink, available resources are restricted by resource pool. If SL positioning RS can be multiplexed with data in a same resource pool, more collision will be additionally caused due to resource overlap b/w RS and data. Therefore, it is more appropriate to configure a separate resource pool for SL positioning RS from our point of views. Moreover, to avoid introducing the issue for processing data and positioning RS simultaneously again in Rel-18, we propose to only support TDM configuration b/w SL data resource pool and positioning RS resource pool.

Figure 4 TDM resource pool configuration b/w data and positioning RS

Proposal 10: In Rel-18, dedicated resource pool for SL positioning RS should be (pre)configured other than multiplexing with data in a same resource pool, and only TDM configuration is supported for SL data and positioning RS resource pools.

Resource allocation for sidelink positioning RS

As aforementioned, a dedicated resource pool is (pre-)configured for SL positioning RS. In such a case, one open issue is whether the SL positioning RS is a standalone RS as defined in Uu positioning, or transmitted along with PSCCH as defined in NR sidelink. In our views, the transmission of sidelink positioning RS should inherit the framework of NR sidelink. The reasons are twofold. First, SL positioning RS associated with PSCCH can make the design for both Mode1 and Mode 2 resource allocation mechanisms unified. In addition, the associated PSCCH of PSSCH can be used to reserve further resources and mitigate the resource collision possibility by performing resource selection procedure; otherwise, resource collision b/w different UEs’ positioning RS may also happen frequently if only standalone RS is transmitted.

Proposal 11: SL positioning RS should also be transmitted along with PSCCH to reserve further resources and mitigate the resource collision possibility.

In NR sidelink, the starting position of the frequency domain of the PSCCH is the lowest PRB of the lowest sub-channel of the scheduled PSSCH, thus a relationship between PSCCH and PSSCH resource is established. However, RE level resource multiplexing is supported in frequency domain for the positioning RS defined in NR positioning, which is quite different from the framework in NR sidelink where only sub-channel level multiplexing is supported. Then, multiplexing rule b/w PSCCH and positioning RS resources may need to be re-designed for NR SL positioning.

Figure 5 Illustration of PSCCH and PSSCH resource in NR sidelink

Figure 6 Illustration for RE level multiplexing b/w positioning RS from different UEs

Proposal 12: Multiplexing rule b/w PSCCH and positioning RS resources may need to be re-designed for NR SL positioning.

Regarding the resource allocation for SL positioning PRS, for mode 1, it is very straightforward and natural for the gNB to take the responsibility for resource allocation; for mode 2, legacy mode 2 resource selection procedure can be reused as much as possible, but one potential enhancement may be needed if the RE-level multiplexing for SL positioning RS is reused.

Figure 7 Illustration of mode 2 resource selection procedure in Rel-16

Proposal 13: Resource allocation for SL positioning PRS should reuse the Rel-16 mechanism in NR sidelink as much as possible.

- For mode 1, gNB should take the responsibility for resource allocation;

- For mode 2, take the legacy mode 2 resource selection procedure as baseline with the consideration of potential enhancements towards RE-level multiplexing of SL positioning RS.

Another method for mode 2 resource selection is to use a Mode 2(d) like method which has been discussed in Rel-16, so as to perform centralized scheduling of RS resources. The benefits are higher reliability can be obtained compared to UE autonomous selection procedure. Meanwhile, RE-level sensing/resource selection procedure may be possible to be avoided, because a scheduler UE can obtain resources in a sub-channel level granularity, and then allocate them to member UEs for performing RE-level multiplexing.

Figure 8 Mode 2(d) like mechanism for SL positioning RS

Proposal 14: Centralized scheduling mechanism, e.g., mode 2(d) like method discussed in Rel-16 NR sidelink, can be considered for resource allocation for SL positioning PRS.

For the resource configuration/indication procedure, we think the procedure for CSI-RS measurement in Rel-16 NR sidelink could be a reference. Taking the case of a positioning initiating UE receiving and anchor UEs transmitting SL positioning RS as an example, after receiving the location request from positioning initiating UE, the anchor UE would determine its own resources for SL positioning RS, by using aforementioned methods. Then it can notice the selected resources to initiating UE by exchanging assistance data, after that the initiating UE can acquire the selected resource information of SL positioning RS, and perform RS reception and measurement accordingly. This framework can avoid the usage of 2nd SCI for reception with the limitation of separate resource pool configuration b/w SL positioning RS and data.

Proposal 15: Procedure for CSI-RS measurement in Rel-16 NR sidelink could be a reference for SL positioning RS resource configuration/indication.

- The usage of 2nd SCI for reception can be precluded by defining a similar procedure to CSI-RS measurement for SL positioning RS.

Conclusions

In this contribution, we have shared our initial views on SL positioning, including the potential methods and RS design, the following observations and proposals are provided:

Observation 1: TDOA and RTT may requires larger BW for positioning RS.

Observation 2: TDOA may only be applicable for the case where the anchor devices are RSUs.

Observation 3: AOA/AOD may increase the requirement for UE capabilities since multiple antennas should be supported.

Observation 4: AoD requires positioning RS to support multi-beam while the study of FR2 in NR sidelink evolution is put on hold until further checking in RAN#97.

Proposal 1: The evaluation of TDOA and RTT should be based on ITS band (20/40MHz) and licensed band in FR1in Rel-18 SL positioning.

Proposal 2: RAN1 should further study which or which combination of positioning methods (e.g. TDOA, RTT, AOA/D, etc) should be selected for NR SL positioning, based on the evaluation results of:

- Positioning accuracy performance by using each method;

- Feasibility of resolving the respective method’s bottleneck in Rel-18 of each method.

Proposal 3: Initial views from our company on which methods to be supported in Rel-18 SL positioning are:

- RTT can be supported in Rel-18 SL positioning for both absolute positioning and relative positioning /ranging;

- TDOA can be supported only in the case where the anchor devices are RSUs in Rel-18 SL positioning, both absolute positioning;

- Whether AoA/AoD can be supported should be subject to UE capability, for both absolute positioning and relative positioning/ranging.

Proposal 4: Joint scheme b/w NR Uu and SL positioning can be considered to facilitate the positioning accuracy performance.

Proposal 5: RAN1 should further study which signal should be used as design baseline for sidelink positioning RS, b/w PRS and SRS-Pos.

Proposal 6: RAN1 should further study which type of RS in time domain should be supported in Rel-18, including periodic, semi-persistent, and aperiodic.

Proposal 7: Slot structure in NR sidelink should be reused as much as possible for sidelink positioning RS slot, which including AGC symbol, GP symbol and the potential PSCCH symbols, the remaining symbols can be regarded as candidates for positioning RS.

Proposal 8: Parameters of SL positioning RS, e.g., number of symbols, RS comb size, and RS BW, should be (pre)configured on resource pool level.

Proposal 9: Configuration of SL positioning RS should be tied to BWP.

Proposal 10: In Rel-18, dedicated resource pool for SL positioning RS should be (pre)configured other than multiplexing with data in a same resource pool, and only TDM configuration is supported for SL data and positioning RS resource pools.

Proposal 11: SL positioning RS should also be transmitted along with PSCCH to reserve further resources and mitigate the resource collision possibility.

Proposal 12: Multiplexing rule b/w PSCCH and positioning RS resources may need to be re-designed for NR SL positioning.

Proposal 13: Resource allocation for SL positioning PRS should reuse the Rel-16 mechanism in NR sidelink as much as possible.

- For mode 1, gNB should take the responsibility for resource allocation;

- For mode 2, take the legacy mode 2 resource selection procedure as baseline with the consideration of potential enhancements towards RE-level multiplexing of SL positioning RS.

Proposal 14: Centralized scheduling mechanism, e.g., mode 2(d) like method discussed in Rel-16 NR sidelink, can be considered for resource allocation for SL positioning PRS.

Proposal 15: Procedure for CSI-RS measurement in Rel-16 NR sidelink could be a reference for SL positioning RS resource configuration/indication.

- The usage of 2nd SCI for reception can be precluded by defining a similar procedure to CSI-RS measurement for SL positioning RS.

References

- RP-213588, Revised SID on Study on expanded and improved NR positioning, 3GPP TSG RAN Meeting #94e.

- R1-2204309, Discussion on SL positioning scenarios and requirements, RAN1#109-e meeting, CMCC.