3GPP TSG RAN WG1 #109-e			                                  R1-2204322

e-Meeting, May 9th – May 20th, 2022

Agenda item:	9.8.2

Source: 	CMCC

Title: 	Discussion on L1/L2 signaling for side control information

Document for:	Discussion and Decision

Introduction

In the RAN #94-e meeting, the network-controlled repeaters study item was agreed to be set up. The objectives related to RAN1 are listed below [1].

The study on NR network-controlled repeaters is to focus on the following scenarios and assumptions: - Network-controlled repeaters are inband RF repeaters used for extension of network coverage on FR1 and FR2 bands, while during the study FR2 deployments may be prioritized for both outdoor and O2I scenarios. - For only single hop stationary network-controlled repeaters - Network-controlled repeaters are transparent to UEs - Network-controlled repeater can maintain the gNB-repeater link and repeater-UE link simultaneously NOTE1: Cost efficiency is a key consideration point for network-controlled repeaters. Study and identify which side control information below is necessary for network-controlled repeaters including assumption of max transmission power [RAN1] - Beamforming information - Timing information to align transmission / reception boundaries of network-controlled repeater - Information on UL-DL TDD configuration - ON-OFF information for efficient interference management and improved energy efficiency - Power control information for efficient interference management (as the 2nd priority) Study and identify L1/L2 signaling (including its configuration) to carry the side control information [RAN1]

The main functions that need to be supported by the side control information are discussed in the companion contribution [2]. In this contribution, we discussed the L1/L2 signalling and its configuration to carry the side control information.

Discussion

As discussed in the companion contribution [2], the MT part of Network-controlled repeater could reuse UE’s chipset. And a full protocol stack containing RRC, MAC and PHY layers should be supported.

2.1 NCR specific RNTI /C-RNTI

Since NCR should receive the control information from gNB at the MT part, there should be an identification for the MT part or the NCR. Two options could be considered. One is to define an NCR specific RNTI for the side control information. With a new defined RNTI, new DCI format for NCR could be defined, which will not reuse current DCI formats and the issues of DCI field repurposing. Since it is an NCR specific RNTI, this will not occupy additional blind decoding numbers. The other option is to reuse the C-RNTI. This option could also work. But there will be more discussion how to deal with the legacy DCI formats from the perspective of NCR. And whether repurposing some filed of the legacy format or introducing new DCI formats for NCR needs more discussions. Both options could work. But the option 1 seems simpler and have less impact to the specification and legacy UEs.

Proposa1 1:

It is proposed to discussed the RNTI issue of NCR considering the two options below.

- Option 1: Define a Network-controlled repeater’s RNTI.

- Option 2: reuse the C-RNTI for MT part of NCR.

2.2 RRC configurations

The NCR need to acquire the TDD configuration from the gNB. But either read from the system information as normal UE or configured by the gNB through the side control information could work for the NCR. That depends on the discussion on the working procedure of the NCR.

Observation 1:

The NCR could acquire the TDD configuration from gNB either through reading the system information or through the configuration with side control information.

As we discussed in the other contribution, we consider the gNB could configure the time domain resources for NCR. In those resources, the NCR will amplify and forward the data to the access UEs. Between the NCR and the access UEs, some information of beam managements are needed to maintain the connections. Then, at least the time domain resources allocated for the NCR and the beam management information should be contained in the RRC configurations to the NCR.

Proposal 2:

At least the time domain resources allocated for the NCR and the beam management information should be contained in the RRC configurations to the NCR.

2.2 MAC CE and DCI indication

As it is possible that some urgent transmissions or the latency sensitive traffic need to be forwarded through NCR. Then the dynamic indication of forwarded resources and the related beam information are needed. If the close loop of power control or amplifier gain control are needed, those information should be also transferred in a dynamic indications.

Proposal 3:

- The information of allocated resources to the NCR and related beam management information could be indicated in a dynamic way, such as in MAC CE and or DCI indications

- The close loop of power control and amplifier control could also be indicated in a dynamic way.

Conclusions

In this contribution, we discussed the L1/L2 signalling and its configuration to carry the side control information. The observation and proposals are listed as below.

Observation 1:

The NCR could acquire the TDD configuration from gNB either through reading the system information or through the configuration with side control information.

Proposa1 1:

It is proposed to discussed the RNTI issue of NCR considering the two options below.

- Option 1: Define a Network-controlled repeater’s RNTI.

- Option 2: reuse the C-RNTI for MT part of NCR.

Proposal 2:

At least the time domain resources allocated for the NCR and the beam management information should be contained in the RRC configurations to the NCR.

Proposal 3:

- The information of allocated resources to the NCR and related beam management information could be indicated in a dynamic way, such as in MAC CE and or DCI indications

- The close loop of power control and amplifier control could also be indicated in a dynamic way.

References

- [1] RP-213700, New SI: Study on NR Network-controlled Repeaters, ZTE Corporation , 3GPP TSG RAN Meeting #94e, Electronic Meeting, Dec. 6 - 17, 2021

- [2] R1-2204321, Discussion on Side control information to enable NR network-controlled repeaters, CMCC, 3GPP TSG RAN WG1 #109-e, e-Meeting, May 9th – May 20th, 2022