3GPP TSG RAN WG1 #109-e			                          R1-2204323

e-Meeting, May 9th – 20th, 2022

Source: 	CMCC

Title:	Discussion on NR PDCCH reception in symbols with LTE CRS REs

Agenda item:	9.9.1

Document for:	Discussion & Decision

- Introduction

In the WID [1], the following objective is included in Rel-18 DSS enhancement WID:

- Study and if needed specify NR PDCCH reception in symbols with LTE CRS REs. [RAN1]

  - Investigate enabling LTE CRS to puncture NR PDCCH, including the impact to NR PDCCH DMRS if there is the performance gain from the additional PDCCH resources.

In this contribution, the detail physical layer design of NR PDCCH reception on overlapping symbols with LTE CRS REs will be discussed.

- Discussion

NR PDCCH can not be monitored on the OFDM symbols overlapping with LTE CRS in current NR design. In DSS scenario, the NR PDCCH capacity may be the bottleneck which enable LTE CRS puncturing NR PDCCH is a potential method to increase NR PDCCH capacity. But this method not only punctures DCI but also NR PDCCH DMRS as the illustration in Figure 1 which the decrease of DMRS density may cause the demodulation performance degradation of NR PDCCH. One method to avoid such issue is shifting one NR DMRS to another sub-carrier to maintain the same NR DMRS density as legacy NR PDCCH which is illustrated in Figure 2.

Figure 1. LTE CRS puncture NR PDCCH in one RB    Figure 2. NR PDCCH DMRS shift in one RB

To verify the new NR PDCCH DMRS pattern, link level simulation is took considering the following three cases as in Figure 3, which the baseline is legacy NR PDCCH design, pattern 1 is LTE CRS puncture NR PDCCH DCI as well as DMRS, pattern 2 is the new NR PDCCH DMRS shift pattern. The detailed simulation parameters are listed in Table 1.

Baseline          Pattern 1          Pattern 2

Figure 3. Simulation cases

Table 1. Simulation parameters

| Parameter | Value |
| --- | --- |
| Carrier frequency | 2.6GHz |
| Bandwidth | 10MHz |
| SCS | 15kHz |
| MIMO | 2T2R precoder cycling |
| DM-RS channel estimation | Realistic |
| Channel model | TDLC DS = 300ns, Speed = 3km/h |
| DCI length (excluding 24bits CRC) | 39 bits |
| Aggregation level | 4，8 |
| CORESET symbol | 2 |
| CORESET bandwidth | 48RB |
| Mapping type | Interleaved |
| REG bundle size | 6 |
| Interleaver size | 2 |

The simulation results are illustrated as in Figure 4 and Figure 5, which the Figure 4 is the PDCCH detection performance and Figure 5 is the MSE of channel estimation.

Figure 4. PDCCH detection performance

Figure 5. MSE of channel estimation

From the simulation results in Figure 4 we can see that for AL=8, the detection performance of pattern 2 is almost the same as baseline and better than pattern 1 by 0.5dB. For AL=4, the detection performance of pattern 2 is comparable with pattern 1 which are both worse than baseline, it is because that although the DMRS density of pattern 2 is larger than pattern 1, the coding rate is also higher at AL=4. For AL=8, the difference of coding rate between pattern 1 and pattern 2 is small, the larger DMRS density of pattern 2 can provide PDCCH detection gain than pattern 1. From the simulation results in Figure 5, we can see that the accuracy of channel estimation of pattern 2 is the same as baseline and better than pattern 1. Therefore, considering the PDCCH detection performance gain at AL=8, we think if NR PDCCH reception in symbols with LTE CRS REs is supported in DSS, the NR PDCCH DMRS pattern can be enhanced to maintain three DMRS REs in one RB.

Proposal 1. If NR PDCCH reception in symbols with LTE CRS REs is supported, the NR PDCCH DMRS pattern can be enhanced to maintain three DMRS REs in one RB after the puncture of LTE CRS.

- Conclusion

In this contribution, NR PDCCH reception on overlapping symbols with LTE CRS REs is discussed, and the following proposal is made.

Proposal 1. If NR PDCCH reception in symbols with LTE CRS REs is supported, the NR PDCCH DMRS pattern can be enhanced to maintain three DMRS REs in one RB after the puncture of LTE CRS.

- Reference

- RP-213575, New WI: Enhancement of NR Dynamic spectrum sharing (DSS)