3GPP TSG RAN WG1 #109-e			                          R1-2204326

e-Meeting, May 9th – 20th, 2022

Source: 	CMCC

Title:	Discussion on XR-specific power saving techniques

Agenda item:	9.11.1

Document for:	Discussion & Decision

- Introduction

In the updated SID [1], one of the objectives of XR enhancement study is as the following:

Objectives on XR-specific Power Saving (RAN1, RAN2):

- Study XR specific power saving techniques to accommodate XR service characteristics (periodicity, multiple flows, jitter, latency, reliability, etc...). Focus is on the following techniques:

  - C-DRX enhancement.

  - PDCCH monitoring enhancement.

In this contribution, some potential XR-specific power saving techniques will be discussed.

- Discussion

2.1 C-DRX enhancement

As the justification part in SID [1], “It is understood that the current DRX configurations do not fit well for (i) the non-integer XR traffic periodicity, (ii) variable XR data rate and (iii) quasi-periodic XR periodicity, hence enhancements would be beneficial in this area.”, the most challenge of current fixed one DRX cycle can not be adapted to the XR non-integer traffic periodicity and jitter.

2.1.1 Enhanced DRX configuration

In current NR design, one UE can only be configured with one or two unicast DRX configurations with a single set of parameters, including the DRX cycle, on duration timer and so on. For XR service, considering the non-integer and quasi-periodic service periodicity, the fixed DRX ON duration starting point cannot be aligned with each XR DL traffic arrival time, which causes unnecessary PDCCH monitoring if the DL traffic arrives after the DRX ON or service delay if the DL traffic arrives before the DRX ON.

Some enhanced DRX configuration can be considered for XR service to solve this problem. For example, multiple DRX parameters can be configured for one DRX configuration, including multiple DRX cycles or DRX ON starting offset, e.g., drx-SlotOffset and each XR data arrival time can match the best suitable DXR configuration parameters. Regarding which DRX configurations to be used in each DRX cycle, both explicit way using indication signalling and implicit way to define some DRX configuration parameters selection rules can be considered.

Proposal 1. Multiple DRX parameters for one DRX configuration can be considered for XR service, e.g., multiple DRX cycles or multiple DRX ON duration start positions to align the XR DL traffic arrival and DRX ON.

Proposal 2. Both explicit and implicit ways can be considered to indicate UE which DRX configuration parameter to be used.

- Explicit way: using dedicated signalling to indicate the used DRX parameter.

- Implicit way: defining DRX parameter selection rule for each DRX cycle.

2.1.2 Jitter handling

Due to variable video encoding time, network transfer time, etc, XR traffic arrival at gNB may have unpredictable jitter. If traffic arrives during DRX non-active time, the UE shall wait for the packet transmission until “on Duration” of the next DRX cycle (i.e. until drx-onDurationTimer is started), which increases the latency for the packet transmission (see upper part of Figure 1).

Figure 1: Legacy DRX vs. New DRX

To minimize the impact of jitter, the DRX Active Time should be aligned with actual DL traffic arrival time as much as possible. One method to realize this is extending the on duration timer to cover the time distribution of jitter, but it will bring extra PDCCH monitoring power consumption. Another method is that gNB uses signalling to indicate UE to wake up before the next DRX cycle instead of waiting for data transmission until next on duration time. For example, a new "Early Wake-up Indication" can be carried in DCI format 2_6 and UE can monitor PDCCH immediately after receiving the "Early Wake-up Indication" or after a time gap for DCI format 2_6 processing to reduce the service latency (see lower part of Figure 1). Without extending on duration timer, DCI format 2_6 with early wake-up indication can be monitored at the end of the jitter time distribution and hence reduce unnecessary PDCCH monitoring. In addition, multiple PDCCH monitoring occasions for detection of DCI format 2_6 can be configured to further reduce the latency.

Figure 2: DCI format 2_6 with Early Wake-up Indication

Figure 2 is an illustration that for an XR UE a new 1-bit "Early Wake-up Indication" field can be added and followed the corresponding "Wake-up indication + SCell dormancy indication" block in DCI format 2_6 to indicate whether UE should wake up to monitor PDCCH immediately after receiving it or after a time gap.

Figure 3: Data arrives earlier than expected arrival time

Furthermore, if DL data arrives earlier than the expected arrival time and no more data is expected to be sent on the next DRX cycle, gNB can indicate UE to wake up early by the "Early Wake-up Indication" field and not to start the next on duration timer by the Wake-up Indication field (see Figure 3). As the DCI for the data is sent during the “Early active time” and the duration of which is expected to be shorter than that of “on Duration”, the overall UE power consumption will be reduced.

Proposal 3. A new "Early Wake-up Indication" field in DCI format 2_6 to indicate UE whether to wake up early from DRX ON can be considered for XR service.

2.2 PDCCH monitoring enhancement

In addition to C-DRX enhancement, the PDCCH monitoring enhancement can also be considered to further align the PDCCH monitoring occasion with XR traffic arrival.

In Rel-17, both PDCCH skipping and SSSG switching are supported for licensed band to save PDCCH monitoring power consumption. But one issue is still under discussion that whether the Rel-17 PDCCH skipping and SSSG switching can only be applied in DRX active time or not. Considering for some XR service with multiple flows or dense traffic arrival rate, it may not suitable to always configure DRX. Therefore, the PDCCH skipping and SSSG switching can be extended to non-DRX scenario which can also facilitate the adaptive PDCCH monitoring to align with XR DL traffic more accurately.

Proposal 4. PDCCH skipping and SSSG switching can be considered to be applied when DRX is not configured to save PDCCH monitoring power consumption.

In addition, only scheduled DCI is supported as the PDCCH skipping and SSSG switching indication DCI in Rel-17. Considering the jitter of XR service, the current PDCCH skipping duration candidates may not be sufficient. One possible solution is to extend the PDCCH skipping duration candidates and the bitlength of PDCCH skipping indication, but this will cause the increase of DCI size. Thus, non-scheduling DCI can also be considered as the PDCCH monitoring adaptation indication, for example, repurpose some fields in DL/UL grant as the PDCCH skipping or SSSG switching indication.

Proposal 5. Non-scheduling DCI can also be considered as PDCCH skipping and SSSG switching indication, e.g., repurpose some fields in DL/UL grant.

- Conclusions

In this contribution, some potential XR-specific power saving techniques are discussed, and the following proposals are made.

Proposal 1. Multiple DRX parameters for one DRX configuration can be considered for XR service, e.g., multiple DRX cycles or multiple DRX ON duration start positions to align the XR DL traffic arrival and DRX ON.

Proposal 2. Both explicit and implicit ways can be considered to indicate UE which DRX configuration parameter to be used.

- Explicit way: using dedicated signalling to indicate the used DRX parameter.

- Implicit way: defining DRX parameter selection rule for each DRX cycle.

Proposal 3. A new "early wake-up indication" field in DCI format 2_6 to indicate UE whether to wake up early from DRX ON can be considered for XR service.

Proposal 4. PDCCH skipping and SSSG switching can be considered to be applied when DRX is not configured to save PDCCH monitoring power consumption.

Proposal 5. Non-scheduling DCI can also be considered as PDCCH skipping and SSSG switching indication, e.g., repurpose some fields in DL/UL grant.

- References

- RP-220285, Revised SID: Study on XR Enhancements for NR, Nokia (Rapporteur)