3GPP TSG RAN WG1 #109-e			                          R1-2204327

e-Meeting, May 9th – 20th, 2022

Source: 	CMCC

Title:	Discussion on XR-specific capacity enhancements techniques

Agenda item:	9.11.2

Document for:	Discussion & Decision

- Introduction

In the updated SID [1], one of the objectives of XR enhancement study is as the following:

Objectives on XR-specific capacity improvements (RAN1, RAN2):

- Study mechanisms that provide more efficient resource allocation and scheduling for XR service characteristics (periodicity, multiple flows, jitter, latency, reliability, etc…). Focus is on the following mechanisms:

  - SPS and CG enhancements;

  - Dynamic scheduling/grant enhancements.

In this contribution, some potential XR-specific capacity enhancements techniques will be discussed.

- Discussion

2.1 SPS and CG enhancements

2.1.1 SPS/CG scheduling adaptation

As SPS/CG can reduce the overhead of signaling and PDCCH monitoring, it is often used for periodic services with constant packet size. Once an SPS/CG configuration is activated, the TBS and MCS of data transmitted on the SPS/CG occasions will not change unless the SPS/CG configuration is reactivated. Because of this, legacy SPS/CG may not be suitable for (quasi-) periodic XR traffic with variable packet sizes. One possible way to achieve this is to reactivate or update SPS/CG configuration (timing/frequency resources, MCS, etc.) using DCI with CS-RNTI when the packet size is changed, but this will add extra PDCCH monitoring overhead.

To enhance SPS/CG to support variable packet sizes, adaptive SPS/CG scheduling be introduced for XR service, that is the network can change the timing/frequency resources, MCS, etc. dynamically for one SPS/CG configuration. To reduce the overhead of DCI and PDCCH monitoring, a new simplified DCI with limited payload size and number of candidates can be considered as the dynamical indication signaling which can be monitored in specific search space.

Proposal 1. Adaptive SPS/CG scheduling with simplified DCI can be considered for XR service.

2.1.2 SPS/CG configuration

For XR service, the packet arrival rate is determined by the frame generation rate, e.g., 60fps equals to 16.6667ms, which is not aligned with the integer SPS/CG periodicity configuration. In addition, due to the impact of random jitter, the data arrival may not be not be aligned with the SPS/CG occasions. There are several implementation methods, e.g., multiple SPS/CG configurations, but this is not an efficient way. Therefore, some enhancement on SPS/CG configuration is necessary.

One potential enhancement is to extend the current SPS/CG periodicity configuration, e.g., multiple periodicities for one SPS configuration and the sum of multiple periodicities equals to an integer number. For example, one SPS configuration can contain two periodicities, 16ms and 17ms and are used in the order of 16ms, 17ms, 17ms in turn. Thus, the total time of one round is 16+17+17=50ms which can satisfy transmitting 60 frames in 1s.

Proposal 2. Multiple periodicities configuration for one SPS configuration can be considered for XR service, which the multiple periodicities are used by turns.

2.2 Dynamic scheduling/grant enhancements

2.2.1 Multi-TB scheduling with single DCI on the same cell

Some advanced XR services have multiple traffic streams (e.g. video, audio, haptic data, etc.), which are always mapped to different QoS flows. Packets within the same traffic stream with different packet characteristics (e.g. packet importance, frame type, etc.) may also be mapped to different QoS flows. These QoS flows have different QoS characteristics and may be transmitted on the same TTI. If data from different QoS flows is multiplexed in the same TB, a lower code rate may be used to ensure the reliability of the data with high importance, hence reducing overall throughput.

Some multiple TB transmission enhancements have been supported in NR, e.g., multi-TTI PUSCH/PDSCH scheduling is supported in Rel-16/17 and single DCI scheduling multiple PDSCH/PUSCH on multiple cells will also be supported in Rel-18. Different from these scenarios, the latency requirement of multiple traffic streams transmission of XR service is stricter, how to schedule multi-flow transmission more finely on the same TTI needs to be studied. One potential solution is extending the UE capability to support processing more than one DL grants in the one span/slot but the DCI signalling overhead is also very huge.

Therefore, multi-TB scheduling with single DCI on the same cell are be further considered for XR service, which different QoS flows with different requirements (e.g. importance, reliability, etc.) can be mapped to different TBs on the same TTI, and different transmission parameters (e.g. MCS, frequency resources, etc.) can be indicated for different TBs. This method can not only ensure the reliable transmission of data with high importance but also transmit data with low importance with the best effort and also reduce the DCI signalling overhead as well as not impact on UE’s DCI processing capability.

Proposal 3. Multi-TB scheduling with single DCI on the same cell can be considered to schedule multi-flow transmission of XR service.

2.2.2 Cross-carrier HARQ enhancement

One of the biggest challenges of XR service for gNB is how to provide high date rate as well as low latency. In traditional CA scenario, the TDD frame structures are almost aligned in DL or UL, although this CA deployment can provide larger bandwidth, the latency of service may not be always satisfied. Therefore, CA with opposite TDD configurations can be considered for XR service, e.g., DL slot in one carrier and UL slot in another carrier at the same time with cross-carrier HARQ feedback and cross-carrier retransmission enhancement which can use all the resources in two carriers more efficiently to reduce the HARQ feedback latency and retransmission latency.

For enabling cross-carrier HARQ feedback, the Rel-17 PUCCH carrier switching can be used as the baseline to dynamically or semi-statically changed the PUCCH carrier for HARQ feedback. To enable cross-carrier retransmission, some enhancement may be needed on retransmission scheduling indication, e.g., DCI needs to indicate the retransmission TB belongs to which cell if a same HARQ process is used by two cells.

Proposal 4. Cross-carrier HARQ feedback and cross-carrier retransmission can be considered for XR service.

- Conclusions

In this contribution, some potential XR-specific capacity enhancements techniques are discussed, and the following proposals are made.

Proposal 1. Adaptive SPS/CG scheduling with simplified DCI can be considered for XR service.

Proposal 2. Multiple periodicities configuration for one SPS configuration can be considered for XR service, which the multiple periodicities are used by turns.

Proposal 3. Multi-TB scheduling with single DCI on the same cell can be considered to schedule multi-flow transmission of XR service.

Proposal 4. Cross-carrier HARQ feedback and cross-carrier retransmission can be considered for XR service.

- References

- RP-220285, Revised SID: Study on XR Enhancements for NR, Nokia (Rapporteur)