# 3GPP TSG RAN WG1 #109-e			                               R1-2204329

# e-Meeting, May 9th – 20th, 2022

Source: 	CMCC

Title:	Discussion on disabling of HARQ feedback for IoT NTN

Agenda item:	9.12.2

Document for:	Discussion & Decision

- Introduction

During RAN#94-e and RAN#95-e meeting, a new work item is proposed to define further enhancements for NB-IoT NTN and eMTC NTN [1]. The justification and objectives that the work item focuses on are as follows

Justification In Release 17, a work item is carried out to enable NB-IoT and eMTC to support Non-Terrestrial Networks (NTN) under the following assumptions: - Transparent payload based GEO and NGSO network scenarios addressing at least 3GPP power class 3 UE with GNSS capability in both Earth fixed &/or moving cell configurations. As part of release 18, a new work item is proposed to define further enhancements for NB-IoT NTN and eMTC NTN in order to: - Improve mobility aspects - Improve performance in terms of throughput - Optimize the GNSS operation with sparse use of GNSS and power efficiency for long-term connection (compared to Rel-17) Further enhance support for discontinuous coverage (subject to Rel-17 IoT-NTN status on discontinuous coverage at RAN#96e/June 2022) Objective The work item aims to specify further enhancements for E-UTRA (LTE-RAN) based NTN (non-terrestrial networks) according to the following assumptions: - GEO and NGSO (LEO and MEO). - Earth fixed Tracking area. Earth fixed & Earth moving cells for NGSO - FDD mode - UEs with GNSS capabilities The detailed objectives are to specify enhanced NB-IoT NTN and eMTC NTN radio interfaces and E-UTRAN/NG-RAN as follows: The first objective is that IoT-NTN Performance Enhancements in Rel-18 to address remaining issues from Rel-17 This work considers Rel-17 IoT-NTN as baseline as well as Rel-17 NR-NTN outcome and the further IoT-NTN performance enhancements objectives are listed below: - Disabling of HARQ feedback to mitigate impact of HARQ stalling on UE data rates [RAN1,RAN2] - Study and specify, if needed, improved GNSS operations for a new position fix for UE pre-compensation during long connection times and for reduced power consumption. Simultaneous GNSS and NTN NB-IoT/eMTC operation is not assumed. [RAN1] - NOTE: The need for RAN4 Core requirements for this objective will be identified after the conclusion on the need for improvements.

In this contribution, we will share our views on disabling of HARQ feedback, enhancement on HARQ, and some other issues for IoT NTN.

- Discussion

  - 2.1 Issue-1: Disabling HARQ feedback

Disabling of HARQ feedback is beneficial to throughput improvement and latency reduction with the cost of reduced reliability. When disabling of HARQ feedback is configured for transmission, there is no need to wait for the HARQ feedback before the new data transmission, which obviously avoids HARQ stalling and consequently increases the data rate. In addition, disabling of HARQ feedback may especially bring more benefits to the GEO network scenarios (e.g. GEO with RTT about 500 ms) due to the transmission waiting time is reduced.

Note that UL data transmission consumes the most amount of energy for a UE. If HARQ feedback is disabled, higher repetition number may be configured, which may significantly increase the power consumption for DL data reception. Note that the main requirements for IoT devices are low complexity and low power consumption, whether disabling of HARQ has benefits to save UE power consumption needs to be further studied. From our understanding, this may depend on whether reliability reduction is acceptable.

Considering that disabling of HARQ feedback may result in the issue on reduced reliability, re-transmission at RLC layer (i.e. RLC ARQ) can be considered to meet reliability requirements as IoT services are usually delay-tolerant. Nevertheless, re-transmission at RLC layer may significantly increase power consumption at least for UL transmission. UE may need to awake for a longer time to wait for the potential re-transmission scheduling signaling triggered by RLC layer, which may increase the power consumption for PDCCH monitoring.

Based on above discussion, disabling HARQ feedback is beneficial to increase the data rate with the cost of reduced reliability or increased power consumption.

Observation 1: Disabling HARQ feedback is beneficial to throughput improvement and latency reduction with the cost of reduced reliability or increased power consumption.

Proposal 1. The disabling of HARQ feedback should be supported for IoT NTN.

Proposal 2. The impact of disabling HARQ feedback on power consumption, as well as whether reliability reduction is acceptable in IoT NTN, needs further study.

  - 2.2 Issue-2: Enhancement on disabling HARQ feedback

It is obvious that disabling HARQ feedback will increase the residual block error rate because HARQ feedback based re-transmissions are not allowed anymore. In order to guarantee the data transmission reliability when HARQ disabled, some enhancement mechanisms should be considered.

Different high layer parameters can be configured for HARQ process with/without feedback. In RAN1 #102-e meeting, the following agreement on HARQ feedback transmission for NR NTN was achieved [2].

Agreement Enabling/disabling on HARQ feedback for downlink transmission should be at least configurable per HARQ process via UE specific RRC signaling.

The mechanism on HARQ feedback disabling agreed for NR NTN can be reused, and disabling of HARQ feedback can be configured per HARQ process via UE specific RRC signaling signaling for IoT NTN.

Proposal 3. Enabling/disabling on HARQ feedback can be configured per HARQ process via UE specific RRC signaling for IoT NTN.

Furthermore, we prefer to support different configuration on aggregation factor for HARQ process with/without feedback. Nevertheless, other parameters need more clarification. For example,

- For block error rate target, it only depends on network implementation with no specification impact.

- For MAC table, one MCS table may be sufficient, since network can schedule proper MCS value via DCI for HARQ process with/without feedback.

Proposal 4. Support different configuration on aggregation factor for HARQ process with/without feedback.

  - 2.3 Issue-3: HARQ codebook enhancements

If disabling HARQ feedback mechanism is supported for IoT NTN, some enhancement mechanisms on HARQ codebook should be considered.

Discussion on the potential enhancements for HARQ codebook in NR NTN are conducted up to RAN1#108-e meeting, and several agreements on HARQ codebook enhancements for NTN were achieved as follows [3].

Type-1 HARQ codebook: Agreement For Type-1 HARQ codebook, the UE will consistently report NACK-only for the feedback-disabled HARQ process regardless of decoding results of corresponding PDSCH. Type-2 HARQ codebook: Agreement For Type-2 HARQ codebook in NTN: Reduce codebook size with HARQ-ACK codebook only including HARQ-ACK of PDSCH with feedback-enabled HARQ processes FFS: The details of C-DAI and T-DAI counting for DCI of PDSCH with feedback-enable/disabled HARQ processes Agreement For the DCI of PDSCH with feedback-disabled HARQ processes, only one of following is supported for Type-2 codebook: - Option-1: The C-DAI and T-DAI are the count of feedback-enabled processes, despite they are not incremented, and are taken into account by the UE for type 2 codebook generation. - Option-2: The C-DAI and T-DAI are ignored by the UE regardless of the value for Type 2 codebook generation. Type-3 HARQ codebook: Agreement For Type-3 HARQ codebook in NTN, the UE should skip the codebook feedback for a feedback-disabled HARQ processes Note: The Type-3 codebook size is reduced by excluding the bit positions of disabled HARQ processes

From our understanding, the codebook enhancement mechanisms of feedback-disabled HARQ process for IoT NTN can refer to the codebook enhancement method of NTN.

For HARQ codebook in NB-IoT over NTN and semi-persistent HARQ codebook in eMTC over NTN, the UE can consistently report NACK-only for the feedback-disabled HARQ process regardless of decoding results of corresponding PDSCH.

Proposal 5. For HARQ codebook in NB-IoT over NTN, the UE can consistently report NACK-only for the feedback-disabled HARQ process regardless of decoding results of corresponding PDSCH.

Proposal 6. For semi-persistent HARQ codebook in eMTC over NTN, the UE can consistently report NACK-only for the feedback-disabled HARQ process regardless of decoding results of corresponding PDSCH.

Besides, for UEs configured by higher layers with codebooksizeDetermination-r13 = dai in eMTC over NTN, similar to Type-2 HARQ codebook enhancement in NR NTN, the details of C-DAI and T-DAI counting for DCI with feedback-enable/disabled HARQ processes should be further discussed.

As discussed in our company’s previous contribution in NTN, we support Option 1 of the above conclusion of Type-2 codebook enhancement for NR NTN to relieve the last DCI missing issue.

Regarding in the bellowing cases, network may expect different HARQ-ACK feedback behavior, while the UE cannot distinguish.

- Case 1: the first DCI is with feedback-enabled HARQ process and other DCIs are with feedback-disabled HARQ processes, and the first DCI is missing detected;

- Case 2: all DCIs are with feedback-disabled HARQ processes.

To achieve the same understanding between network and UE on codebook size based on the value of DAI, Option 1 is fine to us.

Proposal 7. For HARQ codebook depending on the DAI for eMTC over NTN, the C-DAI and T-DAI are the count of feedback-enabled processes.

- Conclusions

In this contribution, we share our views on disabling of HARQ feedback, enhancement on the HARQ process, and some other issues for IoT NTN. The observations and proposals are summarized as follows:

Observation 1: Disabling HARQ feedback is beneficial to throughput improvement and latency reduction with the cost of reduced reliability or increased power consumption.

Proposal 1. The disabling of HARQ feedback should be supported for IoT NTN.

Proposal 2. The impact of disabling HARQ feedback on power consumption, as well as whether reliability reduction is acceptable in IoT NTN, needs further study.

Proposal 3. Enabling/disabling on HARQ feedback can be configured per HARQ process via UE specific RRC signaling for IoT NTN.

Proposal 4. Support different configuration on aggregation factor for HARQ process with/without feedback.

Proposal 5. For HARQ codebook in NB-IoT over NTN, the UE can consistently report NACK-only for the feedback-disabled HARQ process regardless of decoding results of corresponding PDSCH.

Proposal 6. For semi-persistent HARQ codebook in eMTC over NTN, the UE can consistently report NACK-only for the feedback-disabled HARQ process regardless of decoding results of corresponding PDSCH.

Proposal 7. For HARQ codebook depending on the DAI for eMTC over NTN, the C-DAI and T-DAI are the count of feedback-enabled processes.

- References

- RP-220979, Revised WID on IoT NTN enhancements, RAN#95.

- R1-2007311, “Summary#3 of AI 8.4.3 for HARQ in NTN”, August 17th - 28th, 2020.

- R1-2202910, “3GPP TSG-RAN WG1 Agreements under 8.4 up to eMeeting RAN1#108-e”, February 21th - March 3rd, 2022.