3GPP TSG RAN WG1 #109	R1-2204337

e-Meeting, May 9th – 20th, 2022

Source:	NTT DOCOMO, INC

Title:	Remaining issues on SRS enhancement

Agenda Item:	8.1.3

Document for: 	Discussion and Decision

- Introduction

In this contribution, we discuss the remaining issues for SRS enhancements.

- Flexible aperiodic SRS triggering

In an agreement made at RAN1#104-e, there was an FFS point on rules to handle the case of multiple SRS resource sets with overlapping symbols and/or triggered by a same DCI.

Agreement (#104-e) Confirm the following working assumption with modifications An “available slot” is a slot satisfying there are UL or flexible symbol(s) for the time-domain location(s) for all the SRS resources in the resource set and it satisfies UE capability on the minimum timing requirement between triggering PDCCH and all the SRS resources in the resource set. - From the first symbol carrying the SRS request DCI and the last symbol of the triggered SRS resource set, UE does not expect to receive SFI indication, UL cancellation indication or dynamic scheduling of DL channel/signal(s) on flexible symbol(s) that may change the determination of “available slot”. - Note: Collision handling between the triggered SRS and any other UL channel/signal is performed after the determination of available slot. - FFS: Rules to handle the case of multiple SRS resource sets with overlapping symbols and/or triggered by a same DCI

This has been discussed but no conclusion has been reached. Therefore, in the latest specifications, UE behavior when such collision happens is not described, which results in that all the collided SRS transmissions may not be performed by considering such a case as error. In our view, at least one of the SRS transmissions collided should be performed as configured; otherwise, the configuration itself can be inefficient.

Then the question is how to determine which SRS transmission to be transmitted among the collided transmissions. The following is the latest status in RAN1; which we think seems reasonable.

FL Proposal 2-1: Introduce dropping rule when collision happens among multiple aperiodic SRS resource sets in a same CC or different CCs. - Adopt the following priority rules (with priority level from high to low) - Usage > CC ID > Set ID - For usages, priority order is AS > CB > NCB > BM - For CC ID/set ID, lower ID has higher priority than higher ID - The new dropping rule is a UE optional feature - UE will take collision as error case if UE does not support this feature

Meanwhile, we understand some companies do not prefer the priority rule above since the order of usage would be difference case-by-case. For this issue, we are open to discuss on whether to consider usage here.

Proposal 1:

- Support to introduce dropping rule when collision happens among multiple aperiodic SRS resource sets in a same CC or different CC

  - The priority rule should consider at least CC ID and Set ID

  - Usage configured for each SRS resource set can also be considered

- SRS switching for DL CSI acquisition with up to 8 Rx antenna ports

For guard symbols needed for SRS antenna switching, below were agreed in the previous e-meetings:

Agreement For two SRS resource sets of an xTyR antenna switching located in two consecutive slots, if UE is capable of transmitting SRS in all symbols in one slot, a minimum gap period of Y symbols exists between the last OFDM symbol occupied by the SRS resource set in the first slot and the first OFDM symbol occupied by the SRS resource set in the second slot - The value of Y is same as the inter-resource GP defined in Rel-15 - FFS: Whether or not the minimum GP exists can be RRC configurable subject to UE capability - Whether this inter-set GP is needed for 4T6R can be discussed later per the decision on 4T6R configuration. - FFS: How/Whether to handle the case where the interval between SRS resource sets is larger than Y Agreement For inter-set guard period, UE does not transmit any other signal on any symbols of the interval if the interval between SRS resource sets is Y symbols. - When both the SRS resource on all of the corresponding symbols prior to the gap and the SRS resource on all of the corresponding symbols after the gap are dropped due to collision handling, the gap period is also dropped with same priority and can be used for UL transmission. - The above is the only collision handling rule to be introduced in Rel-17 for antenna switching guard period

The two agreements have been reflected in 38.214 [1] as follows:

#### 6.2.1.2 UE sounding procedure for DL CSI acquisition [omitted] The UE is configured with a guard period of Y symbols, in which the UE does not transmit any other signal, in the case the SRS resources of a set are transmitted in the same slot. The guard period is in-between the SRS resources of the set. For two SRS resource sets of an antenna switching located in two consecutive slots, if UE is capable of transmitting SRS in all symbols in one slot, a guard period of Y symbols exists between the last OFDM symbol occupied by the SRS resource set in the first slot and the first OFDM symbol occupied by the SRS resource set in the second slot. For the inter-set guard period, the UE does not transmit any other signal on any symbols of the interval if the interval between SRS resource sets is Y symbols. - When both the SRS resource on all of the corresponding symbols prior to the gap and the SRS resource on all of the corresponding symbols after the gap are dropped due to collision handling, the gap period is also dropped with same priority and can be used for UL transmission. [omitted]

With above, in case that Y symbols exist between SRS resources, regardless of the SRS resources are configured in the same SRS resource set or different SRS resource sets, UE behavior would be clear, i.e., any other UL signal is not transmitted during the Y symbols. It is also clarified that such Y-symbol guard period always exists between SRS resources.

Another point which is still unclear would be whether/how to handle the case where interval between SRS resource sets is larger than Y. For this issue, RAN4 has sent an LS [2] to this RAN1 e-meeting, in which the following information is shared:

- RAN4 thinks not sending PUSCH and PUCCH between SRS resources belonging to different sets where usage is set to antenna switching is inefficient. - RAN4 clarifies that the transient period between SRS resources is 15us.

As RAN4 indicates, we believe not sending PUSCH/PUCCH between SRS resources belonging to different sets where usage is set to antenna switching is inefficient when such resources are more than the ones needed for compensating transient period for SRS antenna switching (15us) in time domain. It should be clarified in the specification that the resources between SRS transmissions for antenna switching, at least other than Y-symbol guard period, would be available for other UL transmissions. Thus, we propose the following TP to 38.214.

Proposal 2:

- Adopt the following TP#1:

  - Reason for change: Whether/how much resources between SRS resources associated with different SRS-ResourceSets, for which “usage” is configured as “antennaSwitching”, is available for any other signal is not clear

  - Summary of change: Clarify Y symbols before the latter SRS resource is not available for any other signal

  - Consequences if not approved: All the resources, including the symbols not needed for antenna switching, cannot be used for transmission of other signal.

TP#1

#### 6.2.1.2 UE sounding procedure for DL CSI acquisition [Unchanged Parts Omitted] The UE is configured with a guard period of Y symbols, in which the UE does not transmit any other signal, in the case the SRS resources of a set are transmitted in the same slot. The guard period is in-between the SRS resources of the set. For two SRS resource sets of an antenna switching located in two consecutive slots, if UE is capable of transmitting SRS in all symbols in one slot, a guard period of Y symbols exists between the last OFDM symbol occupied by the SRS resource set in the first slot and the first OFDM symbol occupied by the SRS resource set in the second slot. For the inter-set guard period, the UE does not transmit any other signal on any symbols of the interval if the interval between SRS resource sets is Y symbols. If the interval between SRS resource sets is larger than Y symbols, the UE can be scheduled to transmit any other signals or channels in the symbols within the interval except for Y symbols before the latter SRS resource. - When both the SRS resource on all of the corresponding symbols prior to the gap and the SRS resource on all of the corresponding symbols after the gap are dropped due to collision handling, the gap period is also dropped with same priority and can be used for UL transmission. [Unchanged Parts Omitted]

Moreover, for Y-symbol guard period between SRS resources for antenna switching, we see a potential conflict between the text in 6.2.1 and 6.2.1.2 in [1], as shown below:

### 6.2.1 UE sounding procedure [omitted] For PUCCH and SRS on the same carrier, a UE shall not transmit SRS when semi-persistent or periodic SRS is configured in the same symbol(s) with PUCCH carrying only CSI report(s), or only L1-RSRP report(s), or only L1-SINR report(s). A UE shall not transmit SRS when semi-persistent or periodic SRS is configured or aperiodic SRS is triggered to be transmitted in the same symbol(s) with PUCCH carrying HARQ-ACK, link recovery request (as defined in clause 9.2.4 of [6, 38.213]) and/or SR. In the case that SRS is not transmitted due to overlap with PUCCH, only the SRS symbol(s) that overlap with PUCCH symbol(s) are dropped. PUCCH shall not be transmitted when aperiodic SRS is triggered to be transmitted to overlap in the same symbol with PUCCH carrying semi-persistent/periodic CSI report(s) or semi-persistent/periodic L1-RSRP report(s) only, or only L1-SINR report(s). [omitted] When the UE is configured with the higher layer parameter usage in SRS-ResourceSet set to 'antennaSwitching', and a guard period of Y symbols is configured according to Clause 6.2.1.2, the UE shall use the same priority rules as defined above during the guard period as if SRS was configured. [omitted]

#### 6.2.1.2 UE sounding procedure for DL CSI acquisition [omitted] The UE is configured with a guard period of Y symbols, in which the UE does not transmit any other signal, in the case the SRS resources of a set are transmitted in the same slot. The guard period is in-between the SRS resources of the set. For two SRS resource sets of an antenna switching located in two consecutive slots, if UE is capable of transmitting SRS in all symbols in one slot, a guard period of Y symbols exists between the last OFDM symbol occupied by the SRS resource set in the first slot and the first OFDM symbol occupied by the SRS resource set in the second slot. For the inter-set guard period, the UE does not transmit any other signal on any symbols of the interval if the interval between SRS resource sets is Y symbols. - When both the SRS resource on all of the corresponding symbols prior to the gap and the SRS resource on all of the corresponding symbols after the gap are dropped due to collision handling, the gap period is also dropped with same priority and can be used for UL transmission. [omitted]

Yellow part in the upper extraction of Clause 6.2.1 describes that the UE has to treat the symbols during Y-symbol guard period “as if the corresponding SRS was transmitted”. It means the usage of the Y symbols is determined based on the priority rule applied for the corresponding SRS itself when the SRS are actually transmitted.

On the other hand, the two cyan parts in the lower extraction of Clause 6.2.1.2 describes that a guard period of Y symbols is not available for any other signal transmitted by the UE, as long as the corresponding SRS resources are transmitted.

We view the above texts inconsistent when we consider the situation shown in Figure 1 below, i.e. PUCCH resource overlaps only with Y-symbol guard period. In this case, if we follow Clause 6.2.1, depending on time-domain behavior of the corresponding SRS transmissions and contents carried on the PUCCH, the PUCCH transmission can be prioritized over the usage as guard period. However, Clause 6.2.1.2 does not allow the UE to transmit any other signal during the guard period.

Figure 1. Priority rule between PUCCH and guard period usage in Y-symbol

In our understanding, a Y-symbol guard period is always necessary between SRS resources configured for antenna switching to switch the used antenna(s), at least when the SRS resources are actually transmitted. In this sense, the Y symbols between SRS resources for antenna switching should be kept for antenna switching, i.e., transmission of any other signal, i.e., PUCCH transmission, should be avoided. Meanwhile, fixing this issue for guard period between SRS resources associated with the same SRS resource set may result in making the different behavior between Rel-15/16 UE and Rel-17 UE, which is not very preferable in our view. We think it would be good to focus on Rel-17 UE behavior only by, e.g., focusing on guard period between inter-set SRS resources.

Note that, although this issue may happen since Rel-15, we think it is more significant in Rel-17. Considering that we have to consider Y-symbol guard period between SRS resources even associated with different SRS resource sets, this case would happen more frequently. Therefore, we believe this issue has to be fixed even only for Rel-17.

Based on the discussion above, we propose the following TP on Clause 6.2.1 in [1]:

Proposal 3:

- Adopt the following TP#2:

  - Reason for change: Contradicting specification texts in Clause 6.2.1 and 6.2.1.2 in 38.214, regarding whether PUCCH can be transmitted during Y-symbol guard period between SRS resources for antenna switcing

  - Summary of change: Clarify Y-symbol guard period between SRS resources in different SRS-ResourceSet is not available for any other signal, including PUCCH, as long as the corresponding SRS resources are actually transmitted

  - Consequences if not approved: Transmission during switching the used antenna can be enforced

TP#2

### 6.2.1 UE sounding procedure [omitted] When the UE is configured with the higher layer parameter usage in SRS-ResourceSet set to 'antennaSwitching', and a guard period of Y symbols is configured between SRS transmissions associated with the same SRS-ResourceSet according to Clause 6.2.1.2, the UE shall use the same priority rules as defined above during the guard period as if SRS was configured. When the UE is configured with the higher layer parameter usage in SRS-ResourceSet set to 'antennaSwitching', and a guard period of Y symbols is configured between SRS resources associated with different SRS-ResourceSet according to Clause 6.2.1.2, and if the SRS transmission before and after the guard period of Y symbols are not dropped, the UE does not transmit any other signal above during the guard period as if SRS was configured. [omitted]

- Summary

In this contribution, we have discussed possible approaches for enhancing SRS transmission in NR Rel. 17. Based on the discussion, we have made following proposals.

Proposal 1:

- Support to introduce dropping rule when collision happens among multiple aperiodic SRS resource sets in a same CC or different CC

  - The priority rule should consider at least CC ID and Set ID

  - Usage configured for each SRS resource set can also be considered

Proposal 2:

- Adopt the following TP#1:

  - Reason for change: Whether/how much resources between SRS resources associated with different SRS-ResourceSets, for which “usage” is configured as “antennaSwitching”, is available for any other signal is not clear

  - Summary of change: Clarify Y symbols before the latter SRS resource is not available for any other signal

  - Consequences if not approved: All the resources, including the symbols not needed for antenna switching, cannot be used for transmission of other signal.

TP#1

#### 6.2.1.2 UE sounding procedure for DL CSI acquisition [Unchanged Parts Omitted] The UE is configured with a guard period of Y symbols, in which the UE does not transmit any other signal, in the case the SRS resources of a set are transmitted in the same slot. The guard period is in-between the SRS resources of the set. For two SRS resource sets of an antenna switching located in two consecutive slots, if UE is capable of transmitting SRS in all symbols in one slot, a guard period of Y symbols exists between the last OFDM symbol occupied by the SRS resource set in the first slot and the first OFDM symbol occupied by the SRS resource set in the second slot. For the inter-set guard period, the UE does not transmit any other signal on any symbols of the interval if the interval between SRS resource sets is Y symbols. If the interval between SRS resource sets is larger than Y symbols, the UE can be scheduled to transmit any other signals or channels in the symbols within the interval except for Y symbols before the latter SRS resource. - When both the SRS resource on all of the corresponding symbols prior to the gap and the SRS resource on all of the corresponding symbols after the gap are dropped due to collision handling, the gap period is also dropped with same priority and can be used for UL transmission. [Unchanged Parts Omitted]

Proposal 3:

- Adopt the following TP#2:

  - Reason for change: Contradicting specification texts in Clause 6.2.1 and 6.2.1.2 in 38.214, regarding whether PUCCH can be transmitted during Y-symbol guard period between SRS resources for antenna switcing

  - Summary of change: Clarify Y-symbol guard period between SRS resources in different SRS-ResourceSet is not available for any other signal, including PUCCH, as long as the corresponding SRS resources are actually transmitted

  - Consequences if not approved: Transmission during switching the used antenna can be enforced

TP#2

### 6.2.1 UE sounding procedure [omitted] When the UE is configured with the higher layer parameter usage in SRS-ResourceSet set to 'antennaSwitching', and a guard period of Y symbols is configured between SRS transmissions associated with the same SRS-ResourceSet according to Clause 6.2.1.2, the UE shall use the same priority rules as defined above during the guard period as if SRS was configured. When the UE is configured with the higher layer parameter usage in SRS-ResourceSet set to 'antennaSwitching', and a guard period of Y symbols is configured between SRS resources associated with different SRS-ResourceSet according to Clause 6.2.1.2, and if the SRS transmission before and after the guard period of Y symbols are not dropped, the UE does not transmit any other signal above during the guard period as if SRS was configured. [omitted]

References

- 38.214 V17.1.0

- R1-2200895, “LS on Rel-17 FeMIMO SRS related impact”, 3GPP RAN WG1, February 2022