3GPP TSG RAN WG1 Meeting #109-e			    R1-2204340

Electronic Meeting, May 9th – 20th, 2022

Source:	NTT DOCOMO, INC.

Title:	PDSCH/PUSCH enhancements for NR from 52.6 to 71 GHz

Agenda Item:	8.2.3

Document for: 	Discussion and Decision

- Introduction

In this contribution, we discuss remaining issues on PDSCH/PUSCH enhancements for NR from 52.6 – 71 GHz, including:

- Scheduling/HARQ

- Scheduling/HARQ

Here we discuss some remaining aspects for multi-PDSCH/PUSCH scheduling by single DCI.

  - “Configured” vs “valid” for Out-of-Order scheduling judgement

For out-of-order scheduling, following agreement and conclusion were made at RAN1#106bis-e and RAN1#107bis-e, respectively.

Agreement (@RAN1#106bis-e): For two multi-PDSCH (or two multi-PUSCH) scheduling DCIs, UE does not expect any of the scheduled PDSCHs (or PUSCHs) and the scheduling DCI to lead to out-of-order scheduling. - FFS: whether to allow OOO scheduling for the following two cases: - for the case of one multi-PDSCH (or multi-PUSCH) scheduling DCI and one single-PDSCH (or single-PUSCH) scheduling DCI, where multi-PDSCH (or multi-PUSCH) scheduling DCI schedules more than one PDSCH (or PUSCH) - for the case where two multi-PDSCH (or multi-PUSCH) scheduling DCIs end in the same symbol but two multi-PDSCH (or multi-PUSCH) scheduling DCIs have overlapping spans, where the span is defined from the beginning of the first scheduled SLIV till the end of the last scheduled SLIV - Note: The above FFS aspect applies only to multi-PDSCH and multi-PUSCH scheduling with single DCI Conclusion (@RAN1#107bis-e) - UE does not expect any of the scheduled PDSCHs (or PUSCHs) and the scheduling DCIs to lead to out-of-order scheduling, also for the case of one multi-PDSCH (or multi-PUSCH) scheduling DCI and one single-PDSCH (or single-PUSCH) scheduling DCI, where multi-PDSCH (or multi-PUSCH) scheduling DCI schedules more than one PDSCH (or PUSCH). - This may not have specification impact. - Note: It is separately discussed whether the scheduled PDSCHs (or PUSCHs or SLIV) is based on configured SLIV or valid SLIV. Agreement (@RAN1#108-e): - The case where two multi-PDSCH (or multi-PUSCH) scheduling DCIs end in the same symbol but two multi-PDSCH (or multi-PUSCH) schedulings have overlapping spans, where the span is defined from the beginning of the first scheduled SLIV till the end of the last scheduled SLIV, is considered as out-of-order scheduling and is not expected by UE. - This applies also when one of two DCIs is single-PDSCH (or single-PUSCH) scheduling DCI, including the case that one DCI schedules multi-slot PDSCH (or PUSCH repetition type A or B). - Note: This doesn’t apply when each of two DCIs schedules multi-slot PDSCH (or PUSCH repetition type A or B) as in Rel-15/Rel-16 - Note: This doesn’t apply when each of the two DCIs schedules single PDSCH (or single PUSCH) as in Rel-15/Rel-16 - Note: It is separately discussed whether the scheduled SLIV is based on configured SLIV or valid SLIV.

Accordingly, the following texts have been implemented in 38.214:

#### 5.1.2.1 Resource allocation in time domain [omitted] If a UE is configured with pdsch-TimeDomainAllocationListForMultiPDSCH-r17 in which one or more rows contain multiple SLIVs for PDSCH on a DL BWP of a serving cell, when any two DL DCIs end in the same symbol and at least one of the DCIs schedules multiple PDSCHs, the UE does not expect that the scheduled PDSCH(s) by the two DCIs have overlapping spans, where the span associated with a DCI is defined from the beginning of the first scheduled PDSCH or up to the end of the last scheduled PDSCH. [omitted] #### 6.1.2.1 Resource allocation in time domain [omitted] If a UE is configured with pusch-TimeDomainAllocationListForMultiPUSCH-r17 in which one or more rows contain multiple SLIVs for PUSCH on a UL BWP of a serving cell, when any two UL DCIs end in the same symbol and at least one of the DCIs scheduling multiple PUSCHs, the UE does not expect that the any scheduled multiple PUSCHs have overlapping spans, where the span associated with a DCI is defined from the beginning of the first scheduled PUSCH till the end of the last scheduled PUSCH. [omitted]

As noted in the conclusion/agreement shown above, which to consider, “configured SLIV” or “valid SLIV”, for OoO scheduling judgement may need to be clarified. Our interpretation of the current specification is that “any scheduled multiple PDSCH/PUSCH” above corresponds to all the SLIVs included in a TDRA row indicated by the scheduling DCI, i.e., “configured SLIV” is considered for OoO scheduling judgement. When we keep the current text as it is, allowed scheduling is judged based on configured SLIV, although a SLIV among the configured ones may be invalid due to collision with e.g., TDD configuration. In this case, a TDRA row can be considered unavailable due to OoO scheduling judgement based on not actual transmission. It can be deemed as inflexible scheduling/resource utilization because of unnecessary restriction. One might argue that it would be problematic given that the number of configurable rows is limited, and the exact K0s/SLIVs in each row has only RRC-level configurability.

When OoO scheduling judgement is based on “valid SLIV”, the issue above would not happen. Any row would be available even if a part of the configured rows collides with e.g., TDD configuration. However, this direction would cause some follow-up issues which may impact the specification. For instance, when Type-1 HARQ codebook and time-domain HARQ-ACK bundling are configured, there are some cases where HARQ-ACK bundling can be considered across PDSCHs scheduled by more than a DCI per the agreements. Since it is not considered in the latest specification (i.e., bundling can be performed across PDSCHs scheduled by a DCI), some further discussions can be required to clarify UE behavior. It would be a bit unclear to us whether/how scheduling inflexibility argued for “configured SLIV”-based judgement is significant. Moreover, we would like to point out that we are in the very late stage of Rel-17 maintenance. Any critical change should be avoided in our view for now.

Proposal 1: For OoO scheduling judgement, support to consider “configured SLIV”

- No TP is needed

  - Condition to indicate SCell dormancy

Some DCIs may indicate no actual data scheduling. For example, the agreement below is related to the DCI indicating TCI state update.

Agreement When a DCI format indicates TCI state update without scheduling PDSCH reception, the PDCCH indicates a TDRA row index including only one SLIV.

The agreement above came from the discussion in the last e-meeting, which was actually associated with another aspect, i.e., DCI to indicate SCell dormancy with no date scheduling.

In our view, difference between DCI for TCI state update and for SCell dormancy would stem only from the indicated functionality. Therefore, it would be a bit strange for us if RAN1 would take another direction for SCell dormancy indicating DCI from TCI state update DCI. One could indeed argue that such restriction on SLIV allowed for DCI for particular use cases may cause inflexible configurations. However, again, whether/how significant such inflexibility would be not very clear. Moreover, considering the agreement above, we think just to take an unified direction for similar functionalities would be implementation friendly.

Given the above consideration, our view is as follows:

Proposal 2: Adopt TP#1 for TS 38.213 for DCI format indicating SCell dormancy without scheduling PDSCH reception

- Reason for change: UE behavior for receiving SCell dormancy indication via DCI 1_1 when multi-PDSCH scheduling is configured is not clear

- Summary of change: Clarify that DCI format 1_1 including SCell dormancy indication indicates a TDRA row with only one SLIV entry

- Consequence if not approved: SCell dormancy may not work in FR2-2

## 10.3 PDCCH monitoring indication and dormancy/non-dormancy behaviour for SCells <Unchanged part omitted> If a UE is provided search space sets to monitor PDCCH for detection of DCI format 1_1, and if - the CRC of DCI format 1_1 is scrambled by a C-RNTI or a MCS-C-RNTI, and if - a one-shot HARQ-ACK request field is not present or has a '0' value, and if - the UE detects a DCI format 1_1 on the primary cell that does not include a carrier indicator field, or detects a DCI format 1_1 on the primary cell that includes a carrier indicator field with value equal to 0, and if - the time domain assignment field in DCI format 1_1 indicates a TDRA row including only a SLIV entry, and if - resourceAllocation = resourceAllocationType0 and all bits of the frequency domain resource assignment field in DCI format 1_1 are equal to 0, or - resourceAllocation = resourceAllocationType1 and all bits of the frequency domain resource assignment field in DCI format 1_1 are equal to 1, or - resourceAllocation = dynamicSwitch and all bits of the frequency domain resource assignment field in DCI format 1_1 are equal to 0 or 1 the UE considers the DCI format 1_1 as indicating SCell dormancy, not scheduling a PDSCH reception, and for transport block 1 interprets the sequence of fields of - modulation and coding scheme - new data indicator - redundancy version and of - HARQ process number - antenna port(s) - DMRS sequence initialization as providing a bitmap to each configured SCell, in an ascending order of the SCell index, where - a '0' value for a bit of the bitmap indicates an active DL BWP, provided by dormantBWP-Id, for the UE for a corresponding activated SCell - a '1' value for a bit of the bitmap indicates - an active DL BWP, provided by firstWithinActiveTimeBWP-Id, for the UE for a corresponding activated SCell, if a current active DL BWP is the dormant DL BWP - a current active DL BWP, for the UE for a corresponding activated SCell, if the current active DL BWP is not the dormant DL BWP - the UE sets the active DL BWP to the indicated active DL BWP <Unchanged part omitted>

- Conclusion

Proposal 1: For OoO scheduling judgement, support to consider “configured SLIV”

- No TP is needed

Proposal 2: Adopt TP#1 for TS 38.213 for DCI format indicating SCell dormancy without scheduling PDSCH reception

- Reason for change: UE behavior for receiving SCell dormancy indication via DCI 1_1 when multi-PDSCH scheduling is configured is not clear

- Summary of change: Clarify that DCI format 1_1 including SCell dormancy indication indicates a TDRA row with only one SLIV entry

- Consequence if not approved: SCell dormancy may not work in FR2-2

## 10.3 PDCCH monitoring indication and dormancy/non-dormancy behaviour for SCells <Unchanged part omitted> If a UE is provided search space sets to monitor PDCCH for detection of DCI format 1_1, and if - the CRC of DCI format 1_1 is scrambled by a C-RNTI or a MCS-C-RNTI, and if - a one-shot HARQ-ACK request field is not present or has a '0' value, and if - the UE detects a DCI format 1_1 on the primary cell that does not include a carrier indicator field, or detects a DCI format 1_1 on the primary cell that includes a carrier indicator field with value equal to 0, and if - the time domain assignment field in DCI format 1_1 indicates a TDRA row including only a SLIV entry, and if - resourceAllocation = resourceAllocationType0 and all bits of the frequency domain resource assignment field in DCI format 1_1 are equal to 0, or - resourceAllocation = resourceAllocationType1 and all bits of the frequency domain resource assignment field in DCI format 1_1 are equal to 1, or - resourceAllocation = dynamicSwitch and all bits of the frequency domain resource assignment field in DCI format 1_1 are equal to 0 or 1 the UE considers the DCI format 1_1 as indicating SCell dormancy, not scheduling a PDSCH reception, and for transport block 1 interprets the sequence of fields of - modulation and coding scheme - new data indicator - redundancy version and of - HARQ process number - antenna port(s) - DMRS sequence initialization as providing a bitmap to each configured SCell, in an ascending order of the SCell index, where - a '0' value for a bit of the bitmap indicates an active DL BWP, provided by dormantBWP-Id, for the UE for a corresponding activated SCell - a '1' value for a bit of the bitmap indicates - an active DL BWP, provided by firstWithinActiveTimeBWP-Id, for the UE for a corresponding activated SCell, if a current active DL BWP is the dormant DL BWP - a current active DL BWP, for the UE for a corresponding activated SCell, if the current active DL BWP is not the dormant DL BWP - the UE sets the active DL BWP to the indicated active DL BWP <Unchanged part omitted>