3GPP TSG RAN WG1 Meeting #109			     R1-2204341

E-Meeting, May 9th – 20th, 2022

Source:	NTT DOCOMO, INC.

Title:	Remaining issues on Channel access mechanism for NR in FR2-2

Agenda Item:	8.2.4

Document for: 	Discussion and Decision

- Introduction

The status report [1] has declared that RAN1 completed its work for NR 52.6-71GHz WI. This contribution describes our view on the maintenance for channel access mechanism for NR in FR2-2.

- Discussion

  - Channel access type indication in fallback DCI

For 52.6 – 71 GHz unlicensed band operation, three types of channel access procedures are supported: Type 1 (LBT with random back-off without CWS adjustment), Type 2 (LBT without random back-off) and Type 3 (no LBT). To determine a type to be used for UL transmission at UE, gNB needs to indicate which type to be used by DCI. For non-fallback DCI formats, RAN1 has agreed the following already:

Agreement For Non-Fallback DCI formats, for FR2-2 operation, for the configuration of the ChannelAccess-CPext field in DCI to indicate the channel access type only, new tables are introduced indicating channel access types for FR2-2, with entries “Type 1 channel access in 4.4.1 of 37.213”, “Type 2 channel access in 4.4.2 of 37.213” and “Type 3 channel access in 4.4.3 of 37.213”.

A remaining issue is how to support the indication by fallback DCI formats. Our best preference is to support the same manner as non-fallback DCI formats, i.e. having two bits for the indication of channel access type even in fallback DCI formats. In the previous e-meetings, some companies argued that only one bit with Type 1 and Type 3 indication is sufficient for fallback DCI formats since Type 2 channel access procedure is an optional capability. However, we would like to point out that which type of LBT is mandatory/optional and which type of LBT is deemed beneficial for operation would depend on multiple aspects, e.g., local regulation, interference condition, etc. As fallback DCIs can be used for RRC_CONNECTED UEs, which can indicate its support of both Type 1 and Type 2 channel access procedures, we believe in some situations two-bit indication would be beneficial even for fallback DCI formats.

Meanwhile, if only one bit is deemed necessary for channel access type indication in fallback DCI formats, we prefer to define the configurable (or variable) association between each entry and indicated channel access type. For example, depending on which type(s) of channel access procedure the UE supports and/or region, an association between an entry and the exact channel access type to be indicated (or, the table defining the associations itself) can be adapted to another association which can be more proper.

Proposal 1: For channel access type indication by fallback DCI formats, adapt either of the following TPs:

- TP#1 Alt-1: Support 2-bit indication to cover all the three channel access types

- TP#1 Alt-2:  Support 1-bit indication, and the association between entries and the indicated types to be configurable, where default table covers Type 1 and Type 3

- Reason for change: Details of channel access type indication via fallback DCI is not clear

- Summary of change: Clarify the number of bits, and association between each entry and channel access type

- Consequence if not approved: Any uplink transmission granted by fallback DCI cannot be transmitted

TP#1 Alt-1:

-------------------------- Start of Text Proposal for TS 38.212 -------------------------- <Unchanged parts omitted> ##### 7.3.1.1.1 Format 0_0 DCI format 0_0 is used for the scheduling of PUSCH in one cell. The following information is transmitted by means of the DCI format 0_0 with CRC scrambled by C-RNTI or CS-RNTI or MCS-C-RNTI: … - ChannelAccess-CPext – 2 bits indicating combinations of channel access type and CP extension as defined in Table 7.3.1.1.1-4 for FR1 or Table 7.3.1.1.1-4B for FR2-2, or Table 7.3.1.1.1-4A if ChannelAccessMode-r16 = "semistatic" is provided, for operation in a cell with shared spectrum channel access; 0 bit otherwise. … The following information is transmitted by means of the DCI format 0_0 with CRC scrambled by TC-RNTI: … - ChannelAccess-CPext – 2 bits indicating combinations of channel access type and CP extension as defined in Table 7.3.1.1.1-4 for FR1 or Table 7.3.1.1.1-4B for FR2-2, or Table 7.3.1.1.1-4A if ChannelAccessMode-r16 = "semistatic" is provided, for operation in a cell with shared spectrum channel access; 0 bit otherwise … Table 7.3.1.1.1-4: Channel access type & CP extension for DCI format 0_0 and DCI format 1_0 for frequency range 1 Bit field mapped to index; Channel Access Type; The CP extension T_"ext" index defined in Clause 5.3.1 of [4, TS 38.211]. 0; Type2C-ULChannelAccess defined in [clause 4.2.1.2.3 in 37.213]; 2. 1; Type2A-ULChannelAccess defined in [clause 4.2.1.2.1 in 37.213]; 3. 2; Type2A-ULChannelAccess defined in [clause 4.2.1.2.1 in 37.213]; 1. 3; Type1-ULChannelAccess defined in [clause 4.2.1.1 in 37.213]; 0. Table 7.3.1.1.1-4A: Channel access type & CP extension if ChannelAccessMode-r16 = "semistatic" is provided Bit field mapped to index; Channel Access Type; The CP extension T_"ext" index defined in Clause 5.3.1 of [4, TS 38.211]; Initiator of the channel occupancy associated with the UL transmission as described in Clause x.x in TS 37.213. 0; No sensing as defined in Clause 4.3 in TS 37.213; 0; gNB. 1; No sensing as defined in Clause 4.3 in TS 37.213; 2; gNB. 2; 9us sensing within a 25us interval as defined in Clause 4.3 in TS 37.213; 0; gNB. 3; Sensing as defined in Clause x.x in TS 37.213; 0; UE. Note: Row index 3 is only applicable if ue-SemiStaticChannelAccessConfig is provided. Otherwise, the row is reserved.. Table 7.3.1.1.1-4B: Channel access type for DCI format 0_0 and DCI format 1_0 for frequency range 2-2 Bit field mapped to index; Channel Access Type. 0; Type 1 channel access defined in clause 4.4.1 of 37.213. 1; Type 2 channel access defined in clause 4.4.2 of 37.213. 2; Type 3 channel access defined in clause 4.4.3 of 37.213. <Unchanged parts omitted> -------------------------- End of Text Proposal for TS 38.212 --------------------------

TP#1 Alt 2:

-------------------------- Start of Text Proposal for TS 38.212 -------------------------- <Unchanged parts omitted> ##### 7.3.1.1.1 Format 0_0 DCI format 0_0 is used for the scheduling of PUSCH in one cell. The following information is transmitted by means of the DCI format 0_0 with CRC scrambled by C-RNTI or CS-RNTI or MCS-C-RNTI: … - ChannelAccess-CPext – number of bits determined by the following: - 2 bits indicating combinations of channel access type and CP extension as defined in Table 7.3.1.1.1-4, or Table 7.3.1.1.1-4A if ChannelAccessMode-r16 = "semistatic" is provided, for operation in a cell with shared spectrum channel access in FR1. - 1 bit indicating channel access type as defined in Table 7.3.1.1.1-4B or Table 7.3.1.1.1-4C if [RRC parameter] is configured, or as defined in Table 7.3.1.1.1-4B if [RRC parameter] is not configured, for operation in a cell with shared spectrum channel access in FR2-2 - 0 bit otherwise. … The following information is transmitted by means of the DCI format 0_0 with CRC scrambled by TC-RNTI: … - ChannelAccess-CPext – number of bits determined by the following: - 2 bits indicating combinations of channel access type and CP extension as defined in Table 7.3.1.1.1-4 for FR1 or Table 7.3.1.1.1-4B for FR2-2, or Table 7.3.1.1.1-4A if ChannelAccessMode-r16 = "semistatic" is provided, for operation in a cell with shared spectrum channel access in FR2. - 1 bit indicating channel access type as defined in Table 7.3.1.1.1-4B or Table 7.3.1.1.1-4C if [RRC parameter] is configured, or as defined in Table 7.3.1.1.1-4B if [RRC parameter] is not configured, for operation in a cell with shared spectrum channel access in FR2-2. - 0 bit otherwise. … Table 7.3.1.1.1-4: Channel access type & CP extension for DCI format 0_0 and DCI format 1_0 for frequency range 1 Bit field mapped to index; Channel Access Type; The CP extension T_"ext" index defined in Clause 5.3.1 of [4, TS 38.211]. 0; Type2C-ULChannelAccess defined in [clause 4.2.1.2.3 in 37.213]; 2. 1; Type2A-ULChannelAccess defined in [clause 4.2.1.2.1 in 37.213]; 3. 2; Type2A-ULChannelAccess defined in [clause 4.2.1.2.1 in 37.213]; 1. 3; Type1-ULChannelAccess defined in [clause 4.2.1.1 in 37.213]; 0. Table 7.3.1.1.1-4A: Channel access type & CP extension if ChannelAccessMode-r16 = "semistatic" is provided in FR1 Bit field mapped to index; Channel Access Type; The CP extension T_"ext" index defined in Clause 5.3.1 of [4, TS 38.211]; Initiator of the channel occupancy associated with the UL transmission as described in Clause x.x in TS 37.213. 0; No sensing as defined in Clause 4.3 in TS 37.213; 0; gNB. 1; No sensing as defined in Clause 4.3 in TS 37.213; 2; gNB. 2; 9us sensing within a 25us interval as defined in Clause 4.3 in TS 37.213; 0; gNB. 3; Sensing as defined in Clause x.x in TS 37.213; 0; UE. Note: Row index 3 is only applicable if ue-SemiStaticChannelAccessConfig is provided. Otherwise, the row is reserved.. Table 7.3.1.1.1-4B: Channel access type for DCI format 0_0 and DCI format 1_0 for frequency range 2-2 Bit field mapped to index; Channel Access Type. 0; Type 1 channel access defined in clause 4.4.1 of 37.213. 1; Type 3 channel access defined in clause 4.4.2 of 37.213. Table 7.3.1.1.1-4C: Channel access type for DCI format 0_0 and DCI format 1_0 for frequency range 2-2 Bit field mapped to index; Channel Access Type. 0; Type 1 channel access defined in clause 4.4.1 of 37.213. 1; Type 2 channel access defined in clause 4.4.2 of 37.213. <Unchanged parts omitted> -------------------------- End of Text Proposal for TS 38.212 --------------------------

  - Short control signaling exemption for UL transmission

Based on the following agreement and conclusion, short control signaling rule defined in BRAN can be applied only for msg1 or MsgA in UL:

Agreement: - Contention Exempt Short Control Signaling rules apply to the transmission of msg1 for the 4 step RACH and MsgA for the 2-step RACH for all supported SCS. - Note restriction for short control signalling transmissions apply (10% over any 100ms intervals) - Alt 1: The 10% over any 100ms interval restriction is applicable to all available msg1/msgA resources configured (not limited to the resources actually used) in a cell - Alt 2: The 10% over any 100ms interval restriction is applicable to the msg1/msgA transmission from one UE perspective - FFS: Other UL signals/channels can be transmitted with Contention Exempt Short Control Signaling rule, such as msg3, SRS, PUCCH, PUSCH without user plain data, etc Conclusion In Rel.17, there is no consensus to apply contention exemption short control signalling to UL transmissions other than msg1 and msgA.

One FFS point still remains in the above agreement; how to interpret the rule of 10% over 100 ms. We believe it should be “per-device” (i.e., Alt 2) because of the following reasons:

- Short control signaling is defined in BRAN regulation, which describes what needs to be ensured by “a device”. Meanwhile, there is no restriction on the behavior of the whole cell. If we follow the exact definition in BRAN, it is undoubtedly straightforward to interpret this rule as in Alt 2.

- Because of the point above, we think there is another RAT which implements some signals and/or channels without LBT by applying short control signaling rule per device. If NR shares a band with such RAT, and if only NR interprets this rule as “per cell”, the actual coexistence would be unfair.

- If we do not consider the coexistence issue described above, “per-cell” interpretation actually does not have any benefit, while it has much more specification impacts. For example, one approach to implement “per-cell” interpretation would be to limit available Ros as proposed in the last e-meeting. We do not think just to limit the available indexes of RO would be reasonable since whether a RO is used or not is totally up to the actual number of UEs that want to perform RACH procedure in the cell. Even if there is no limitation, any RO configuration can eventually comply with the short control signaling exemption rule depending on the actual situation. If we try to optimize the RO limitation to avoid the issue above, there will be much more specification impacts needed, which is not preferable at this stage at all in our view.

With above, we draw the following proposal:

Proposal 2: Define short control signaling by interpreting the exemption rule as “per device”

- No TP is needed

Moreover, we believe both in RRC and SIB1, there should be a signaling that configures whether short control signaling is applicable for msg1 and MsgA; otherwise, a UE cannot be aware that whether msg1/MsgA can be transmitted without performing LBT.

Proposal 3: Support a signaling to configure whether short control signaling is applicable or not

  - DCI format 2_0 enhancement

There was a proposal that DCI format 2_0 with COT duration indication supports an indication of beam(s) to indicate for which beam(s) the COT duration information is considered. We generally see two use cases for DCI 2_0 with COT duration, one is P-/SP-CSI-RS validation and the other is the upgrade of channel access type. For both of which we do not see the significant need to explicitly introduce this feature. Firstly, depending on the exact beam(s) for which COT duration indication is applicable, there would be no need of such explicit indication. For example, if COT duration indication is defined to be applicable only for the same beam used for DCI 2_0, it would be sufficient to define this behavior without any impact on the contents of DCI 2_0 as the beam for DCI 2_0 would be known to the receiver UEs. In other words, an explicit indication in DCI 2_0 can be motivated only if we want to support more than 1 beam indication. Another point that we believe should be well considered is the relationship between the number of beams and DCI 2_0 payload limitation. When we are to define the indication of multiple beams in DCI 2_0, we should determine how many beams have to be covered. Given the high frequency, we believe so many beams are not necessary in a single DCI 2_0 in practice. However, the sufficient number would be unclear (and we assume would be much divergent among companies), so we do not think RAN1 can reach a consensus on issue within the remaining time in WI. Instead, just to cover maximum beams (e.g. 64 as assumed for SSB) might be another possibility. However, we do not prefer this approach since it leads to significant impact on DCI 2_0 payload.

With above, we propose the following:

Proposal 4: Not support to introduce beam indication in DCI 2_0.

- To define COT duration indication in DCI 2_0 to be applicable only for the beam used by the DCI 2_0 is sufficient to achieve the desired behavior

Together with Proposal 6, we also draw the following proposal for CSI-RS validation and upgrading the type of channel access:

Proposal 5: Support both P-/SP-CSI-RS validation and upgrading the type of channel access based on COT duration indication DCI 2_0

- Support to define the rule so that COT duration indication is applicable only for the beam used for the corresponding DCI 2_0

- Conclusion

Proposal 1: For channel access type indication by fallback DCI formats, adapt either of the following TPs:

- TP#1 Alt-1: Support 2-bit indication to cover all the three channel access types

- TP#1 Alt-2:  Support 1-bit indication, and the association between entries and the indicated types to be configurable, where default table covers Type 1 and Type 3

- Reason for change: Details of channel access type indication via fallback DCI is not clear

- Summary of change: Clarify the number of bits, and association between each entry and channel access type

- Consequence if not approved: Any uplink transmission granted by fallback DCI cannot be transmitted

TP#1 Alt-1:

-------------------------- Start of Text Proposal for TS 38.212 -------------------------- <Unchanged parts omitted> ##### 7.3.1.1.1 Format 0_0 DCI format 0_0 is used for the scheduling of PUSCH in one cell. The following information is transmitted by means of the DCI format 0_0 with CRC scrambled by C-RNTI or CS-RNTI or MCS-C-RNTI: … - ChannelAccess-CPext – 2 bits indicating combinations of channel access type and CP extension as defined in Table 7.3.1.1.1-4 for FR1 or Table 7.3.1.1.1-4B for FR2-2, or Table 7.3.1.1.1-4A if ChannelAccessMode-r16 = "semistatic" is provided, for operation in a cell with shared spectrum channel access; 0 bit otherwise. … The following information is transmitted by means of the DCI format 0_0 with CRC scrambled by TC-RNTI: … - ChannelAccess-CPext – 2 bits indicating combinations of channel access type and CP extension as defined in Table 7.3.1.1.1-4 for FR1 or Table 7.3.1.1.1-4B for FR2-2, or Table 7.3.1.1.1-4A if ChannelAccessMode-r16 = "semistatic" is provided, for operation in a cell with shared spectrum channel access; 0 bit otherwise … Table 7.3.1.1.1-4: Channel access type & CP extension for DCI format 0_0 and DCI format 1_0 for frequency range 1 Bit field mapped to index; Channel Access Type; The CP extension T_"ext" index defined in Clause 5.3.1 of [4, TS 38.211]. 0; Type2C-ULChannelAccess defined in [clause 4.2.1.2.3 in 37.213]; 2. 1; Type2A-ULChannelAccess defined in [clause 4.2.1.2.1 in 37.213]; 3. 2; Type2A-ULChannelAccess defined in [clause 4.2.1.2.1 in 37.213]; 1. 3; Type1-ULChannelAccess defined in [clause 4.2.1.1 in 37.213]; 0. Table 7.3.1.1.1-4A: Channel access type & CP extension if ChannelAccessMode-r16 = "semistatic" is provided Bit field mapped to index; Channel Access Type; The CP extension T_"ext" index defined in Clause 5.3.1 of [4, TS 38.211]; Initiator of the channel occupancy associated with the UL transmission as described in Clause x.x in TS 37.213. 0; No sensing as defined in Clause 4.3 in TS 37.213; 0; gNB. 1; No sensing as defined in Clause 4.3 in TS 37.213; 2; gNB. 2; 9us sensing within a 25us interval as defined in Clause 4.3 in TS 37.213; 0; gNB. 3; Sensing as defined in Clause x.x in TS 37.213; 0; UE. Note: Row index 3 is only applicable if ue-SemiStaticChannelAccessConfig is provided. Otherwise, the row is reserved.. Table 7.3.1.1.1-4B: Channel access type for DCI format 0_0 and DCI format 1_0 for frequency range 2-2 Bit field mapped to index; Channel Access Type. 0; Type 1 channel access defined in clause 4.4.1 of 37.213. 1; Type 2 channel access defined in clause 4.4.2 of 37.213. 2; Type 3 channel access defined in clause 4.4.3 of 37.213. <Unchanged parts omitted> -------------------------- End of Text Proposal for TS 38.212 --------------------------

TP#1 Alt 2:

-------------------------- Start of Text Proposal for TS 38.212 -------------------------- <Unchanged parts omitted> ##### 7.3.1.1.1 Format 0_0 DCI format 0_0 is used for the scheduling of PUSCH in one cell. The following information is transmitted by means of the DCI format 0_0 with CRC scrambled by C-RNTI or CS-RNTI or MCS-C-RNTI: … - ChannelAccess-CPext – number of bits determined by the following: - 2 bits indicating combinations of channel access type and CP extension as defined in Table 7.3.1.1.1-4, or Table 7.3.1.1.1-4A if ChannelAccessMode-r16 = "semistatic" is provided, for operation in a cell with shared spectrum channel access in FR1. - 1 bit indicating channel access type as defined in Table 7.3.1.1.1-4B or Table 7.3.1.1.1-4C if [RRC parameter] is configured, or as defined in Table 7.3.1.1.1-4B if [RRC parameter] is not configured, for operation in a cell with shared spectrum channel access in FR2-2 - 0 bit otherwise. … The following information is transmitted by means of the DCI format 0_0 with CRC scrambled by TC-RNTI: … - ChannelAccess-CPext – number of bits determined by the following: - 2 bits indicating combinations of channel access type and CP extension as defined in Table 7.3.1.1.1-4 for FR1 or Table 7.3.1.1.1-4B for FR2-2, or Table 7.3.1.1.1-4A if ChannelAccessMode-r16 = "semistatic" is provided, for operation in a cell with shared spectrum channel access in FR2. - 1 bit indicating channel access type as defined in Table 7.3.1.1.1-4B or Table 7.3.1.1.1-4C if [RRC parameter] is configured, or as defined in Table 7.3.1.1.1-4B if [RRC parameter] is not configured, for operation in a cell with shared spectrum channel access in FR2-2. - 0 bit otherwise. … Table 7.3.1.1.1-4: Channel access type & CP extension for DCI format 0_0 and DCI format 1_0 for frequency range 1 Bit field mapped to index; Channel Access Type; The CP extension T_"ext" index defined in Clause 5.3.1 of [4, TS 38.211]. 0; Type2C-ULChannelAccess defined in [clause 4.2.1.2.3 in 37.213]; 2. 1; Type2A-ULChannelAccess defined in [clause 4.2.1.2.1 in 37.213]; 3. 2; Type2A-ULChannelAccess defined in [clause 4.2.1.2.1 in 37.213]; 1. 3; Type1-ULChannelAccess defined in [clause 4.2.1.1 in 37.213]; 0. Table 7.3.1.1.1-4A: Channel access type & CP extension if ChannelAccessMode-r16 = "semistatic" is provided in FR1 Bit field mapped to index; Channel Access Type; The CP extension T_"ext" index defined in Clause 5.3.1 of [4, TS 38.211]; Initiator of the channel occupancy associated with the UL transmission as described in Clause x.x in TS 37.213. 0; No sensing as defined in Clause 4.3 in TS 37.213; 0; gNB. 1; No sensing as defined in Clause 4.3 in TS 37.213; 2; gNB. 2; 9us sensing within a 25us interval as defined in Clause 4.3 in TS 37.213; 0; gNB. 3; Sensing as defined in Clause x.x in TS 37.213; 0; UE. Note: Row index 3 is only applicable if ue-SemiStaticChannelAccessConfig is provided. Otherwise, the row is reserved.. Table 7.3.1.1.1-4B: Channel access type for DCI format 0_0 and DCI format 1_0 for frequency range 2-2 Bit field mapped to index; Channel Access Type. 0; Type 1 channel access defined in clause 4.4.1 of 37.213. 1; Type 3 channel access defined in clause 4.4.2 of 37.213. Table 7.3.1.1.1-4C: Channel access type for DCI format 0_0 and DCI format 1_0 for frequency range 2-2 Bit field mapped to index; Channel Access Type. 0; Type 1 channel access defined in clause 4.4.1 of 37.213. 1; Type 2 channel access defined in clause 4.4.2 of 37.213. <Unchanged parts omitted> -------------------------- End of Text Proposal for TS 38.212 --------------------------

Proposal 2: Define short control signaling by interpreting the exemption rule as “per device”

- No TP is needed

Proposal 3: Support a signaling to configure whether short control signaling is applicable or not

Proposal 4: Not support to introduce beam indication in DCI 2_0.

- To define COT duration indication in DCI 2_0 to be applicable only for the beam used by the DCI 2_0 is sufficient to achieve the desired behavior

Proposal 5: Support both P-/SP-CSI-RS validation and upgrading the type of channel access based on COT duration indication DCI 2_0

- Support to define the rule so that COT duration indication is applicable only for the beam used for the corresponding DCI 2_0

- Reference

- RP-212990, “SR for Extending current NR operation to 71GHz”, Qualcomm CDMA Technologies, RAN#94-e, 2020

- TS 37.213 V17.0.0, 3GPP TSG RAN, December 2021