3GPP TSG RAN WG1 #109-e			     R1-2204344

e-Meeting, May 9th  – 20th, 2022

Source:	NTT DOCOMO, INC.

Title:	Remaining issues on intra-UE multiplexing/prioritization for Rel.17 URLLC

Agenda Item:	8.3.2

Document for: 	Discussion and Decision

- Introduction

At RAN#88e meeting, revised WID on enhanced industrial internet of things (IoT) and ultra-reliable and low latency communication (URLLC) support for NR was approved with the objective as follows [1]:

- Study, identify and specify if needed, required Physical Layer feedback enhancements for meeting URLLC requirements covering - UE feedback enhancements for HARQ-ACK [RAN1] - CSI feedback enhancements to allow for more accurate MCS selection [RAN1] Note: DMRS-based CSI feedback is not in scope of this WI - Uplink enhancements for URLLC in unlicensed controlled environments [RAN1, RAN2]: - Specify support for UE-initiated COT for FBE with minimum specification effort - Harmonizing UL configured-grant enhancements in NR-U and URLLC introduced in Rel-16 to be applicable for unlicensed spectrum - Intra-UE multiplexing and prioritization of traffic with different priority based on work done in Rel.16 [RAN1]: - Specify multiplexing behavior among HARQ-ACK/SR/CSI and PUSCH for traffic with different priorities, including the cases with UCI on PUCCH and UCI on PUSCH. - Specify PHY prioritization of overlapping dynamic grant PUSCH and configured grant PUSCH of different PHY priorities on a BWP of a serving cell including the related cancelation behavior for the PUSCH of lower PHY priority, taking the solution developed during Rel-16 as the baseline - Enhancements for support of time synchronization: - RAN impacts of SA2 work on uplink time synchronization for TSN, if any. [RAN2] - Propagation delay compensation enhancements (including mobility issues, if any). [RAN2, RAN1, RAN3, RAN4] - RAN enhancements based on new QoS related parameters if any, e.g. survival time, burst spread, decided in SA2. [RAN2, RAN3]

In this contribution, we share our views on the remaining issues for Rel-17 intra-UE multiplexing/prioritization.

- Discussions

  - Overlapping handling for HP PUCCH carrying SR and HP HARQ-ACK of PF 0/1 and LP PUCCH carrying HARQ-ACK and CSI/SR

- Reason for change

In RAN1#108-e meeting [2], the following agreement was achieved for overlapping handling for HP PUCCH carrying SR and HP HARQ-ACK of PF2/3/4 and LP PUCCH carrying HARQ-ACK and CSI/SR if any:

Agreement To resolve overlapping between a HP PUCCH carrying SR and HP HARQ-ACK with PUCCH format 2/3/4 and a LP PUCCH carrying HARQ-ACK and CSI/SR, only LP HARQ-ACK is multiplexed with HP UCI (i.e., SR and HARQ-ACK) and LP CSI/SR are dropped. - FFS HP PUCCH with PUCCH format 0/1

Regarding the FFS point, we think it is related with the following proposal, which was made and discussed at the last meeting for the case without LP CSI/SR.

Proposal 1: When a PUCCH carrying explicit/implicit HP SR and HP HARQ-ACK with PUCCH format 0/1 overlaps with a PUCCH carrying LP HARQ-ACK, where the original PUCCH carrying the HP HARQ-ACK before Step 1 overlaps with K HP SRs which are all negative or include at least one positive, - In Step 1, 1-bit HP SR is multiplexed to HP HARQ-ACK bits (based on Rel-15 rules) for determining the resultant PUCCH resource. - Note: The description of Step 1 here is only for information purpose, which has no spec impact. - In Step 2, down-select from the two options: - Option 0: LP HARQ-ACK is dropped. When a PUCCH carrying positive HP SR and HP HARQ-ACK with PUCCH format 0/1 overlaps with a PUCCH carrying LP HARQ-ACK, the PUCCH carrying LP HARQ-ACK is dropped. - Option 2: If HP HARQ-ACK is dynamic HARQ-ACK or SPS HARQ-ACK with sps-PUCCH-AN-List, 1-bit HP SR is appended to HP HARQ-ACK bits. The number of HP UCI bits is OUCI=OACK,1+1 - The resultant PUCCH resource for multiplexing HP SR + HP HARQ-ACK + LP HARQ-ACK is either of PF2, PF3, or PF4. If HP HARQ-ACK is SPS HARQ-ACK with n1PUCCH-AN, LP HARQ-ACK is dropped.

Between Option 0 and Option 2, Option 2 is preferred to achieve better performance by transmitting LP HARQ-ACK in addition to HP SR and HP HARQ-ACK. If Option 2 is supported for the case without LP CSI/SR, the same procedure should be applied for PF 0/1 case as PF 2/3/4 case for the case with LP CSI/SR, i.e., only LP HARQ-ACK is multiplexed with HP SR+HARQ-ACK, and LP CSI/SR are dropped.

- Summary of change

Add the corresponding changes to make the specification complete.

- Consequences if not approved

The spec is incomplete.

- Text proposal 1:

-------------------------- Start of Text Proposal for TS 38.213 -------------------------- <Unchanged parts omitted> # 9 UE procedure for reporting control information <Unchanged parts omitted> if // this is for cases the UE supports multiplexing information of different priorities in a PUCCH/PUSCH transmission - a PUCCH transmission with HARQ-ACK information, without repetitions, with smaller priority index overlaps with a PUCCH transmission only with HARQ-ACK information, without repetitions, with larger priority index, or - a PUCCH transmission without repetitions that includes HARQ-ACK information of smaller priority index overlaps with a PUCCH transmission without repetitions using a PUCCH resource with PUCCH format 2/3/4 with that includes HARQ-ACK information and SR of larger priority index, or - a PUCCH transmission with HARQ-ACK information, without repetitions, with smaller or larger priority index overlaps, respectively, with a PUSCH transmission with larger or smaller priority index the UE - multiplexes HARQ-ACK information of different priority indexes and SR information of larger priority index, if any, in a same PUCCH transmission of larger priority index, or multiplexes HARQ-ACK information the UE would provide in a PUCCH transmission of smaller or larger priority index in a PUSCH transmission of larger or smaller priority index, respectively, and applies the procedures in clause 9.2.5.3 or 9.3, respectively, and - drops CSI and/or SR carried in the PUCCH of smaller priority index, if any - drops HARQ-ACK information of smaller priority index if the UE would multiplex the HARQ-ACK information of smaller priority index in a PUSCH where the UE multiplexes Part 1 CSI reports and Part 2 CSI reports of larger priority index - drops Part 2 CSI reports of smaller priority index if the UE would multiplex the HARQ-ACK information of smaller and larger priority indexes in a PUSCH where the UE multiplexes Part 1 CSI reports and Part 2 CSI reports of smaller priority index <Unchanged parts omitted> -------------------------- End of Text Proposal for TS 38.213 --------------------------

  - Handling of LP HARQ-ACK on HP PUSCH with CG-UCI

- Reason for change

In RAN1#108-e meeting [2], the following agreement was achieved:

Agreement If LP HARQ-ACK without HP HARQ-ACK would be transmitted on HP PUSCH, down-select from the options: - Option 2: UE follows the same behaviour as that in case of PUSCH with HP HARQ-ACK assuming two bits - FFS for CG-UCI PUSCH.

How to handle CG-UCI PUSCH is FFS when LP HARQ-ACK without HP HARQ-ACK would be transmitted on HP PUSCH. Two options can be considered; one is CG-UCI is jointly encoded with LP HARQ-ACK, and the other is CG-UCI is separately encoded from LP HARQ-ACK. The former would lead to worse performance of CG-UCI as LP HARQ-ACK beta-offset will be used for rate-matching of LP HARQ-ACK and CG-UCI. Therefore, we prefer separate encoding of CG-UCI and LP HARQ-ACK.

- Summary of change

Add the corresponding changes to make the specification complete.

- Consequences if not approved

The spec is incomplete.

- Text proposal 2:

-------------------------- Start of Text Proposal for TS 38.212 -------------------------- <Unchanged parts omitted> 6.3.2.1.5 UCI with different priority indexes If UCI-MuxWithDifferentPriority is configured, and HARQ-ACK bits associated with priority index 0, and CSI part 1 if any are transmitted on a PUSCH associated with priority index 1, the following UCI bit sequences are generated, a0(1), a1(1), a2(1), a3(1), …,aA(1)-11, and a0(2), a1(2), a2(2), a3(2), …,aA(2)-12 if any, according to the following: - If CSI part 1 is also transmitted on the PUSCH, - Set ai(1) for i=0,1,…,A(1)-1 as the bit sequence of CSI part 1, where the CSI fields of all CSI reports, in the order from upper part to lower part in Table 6.3.2.1.2-6, are mapped to the UCI bit sequence a0(1), a1(1), a2(1), a3(1), …,aA(1)-11 starting with a0(1). - Set ai(2)=oiACK-LP for i=0,1,…,OACK-LP-1 and A(2)=OACK-LP, where the HARQ-ACK bit sequence o0ACK-LP, o1ACK-LP, …, oOACK-LP-1ACK-LP associated with priority index 0 is given by Clause 9.1 of [5, TS 38.213]. - Otherwise, set ai(1)=oiACK-LP for i=0,1,…,OACK-LP-1 and A(1)=OACK-LP, where the HARQ-ACK bit sequence o0ACK-LP, o1ACK-LP, …, oOACK-LP-1ACK-LP associated with priority index 0 is given by Clause 9.1 of [5, TS 38.213]. If UCI-MuxWithDifferentPriority and cg-UCI-Multiplexing are configured, and HARQ-ACK bits associated with priority index 0, and CG-UCI are transmitted on a PUSCH associated with priority index 1, the following UCI bit sequences are generated, a0(1), a1(1), a2(1), a3(1), …,aA(1)-11, and a0(2), a1(2), a2(2), a3(2), …,aA(2)-12 if any, according to the following: - Set ai(1)=oiCG-UCI for i=0,1,…,OCG-UCI-1 and A(1)=OCG-UCI, where the CG-UCI bit sequence o0CG-UCI, o1CG-UCI, …, oOCG-UCI-1CG-UCI is given by Table 6.3.2.1.3-1, mapped in the order from upper part to lower part.. - Set ai(2)=oiACK-LP for i=0,1,…,OACK-LP-1 and A(2)=OACK-LP, where the HARQ-ACK bit sequence o0ACK-LP, o1ACK-LP, …, oOACK-LP-1ACK-LP associated with priority index 0 is given by Clause 9.1 of [5, TS 38.213]. <Unchanged parts omitted> ###### 6.3.2.4.1.6 UCI with different priority indexes If UCI-MuxWithDifferentPriority is configured, and HARQ-ACK bits associated with priority index 0, and CSI part 1 if any are transmitted on a PUSCH associated with priority index 1: - If CSI part 1 is also transmitted on the PUSCH, - Perform rate matching for CSI part 1 according to clause 6.3.2.4.1.2, by assuming the number of HARQ-ACK information bits to be transmitted on PUSCH in clause 6.3.2.4.1.2 is 0 bit. - Perform rate matching for HARQ-ACK with priority index 0 according to clause 6.3.2.4.1.3, by taking HARQ-ACK with priority index 0 as CSI part 2 and replacing βoffsetPUSCH by βoffsetHARQ-ACK-LP, and assuming the number of HARQ-ACK information bits to be transmitted on PUSCH in clause 6.3.2.4.1.3 is 0 bit. - Otherwise, perform rate matching for HARQ-ACK with priority index 0 according to clause 6.3.2.4.1.2, by taking HARQ-ACK with priority index 0 as CSI-part 1 and replacing βoffsetPUSCH by βoffsetHARQ-ACK-LP, and assuming the number of HARQ-ACK information bits to be transmitted on PUSCH in clause 6.3.2.4.1.2 is 0 bit. If UCI-MuxWithDifferentPriority and cg-UCI-Multiplexing are configured, and HARQ-ACK bits associated with priority index 0, and CG-UCI are transmitted on a PUSCH associated with priority index 1: - Perform rate matching for CG-UCI according to clause 6.3.2.4.1.1, by taking CG-UCI as HARQ-ACK and replacing βoffsetPUSCH by βoffsetCG-UCI. - Perform rate matching for HARQ-ACK with priority index 0 according to clause 6.3.2.4.1.2, by taking HARQ-ACK with priority index 0 as CSI part 1 and replacing βoffsetPUSCH by βoffsetHARQ-ACK-LP. <Unchanged parts omitted> -------------------------- End of Text Proposal for TS 38.212 --------------------------

- Conclusion

In this contribution, we proposed followings for intra-UE UL multiplexing/prioritization enhancements.

Text proposal 1:

-------------------------- Start of Text Proposal for TS 38.213 -------------------------- <Unchanged parts omitted> # 9 UE procedure for reporting control information <Unchanged parts omitted> if // this is for cases the UE supports multiplexing information of different priorities in a PUCCH/PUSCH transmission - a PUCCH transmission with HARQ-ACK information, without repetitions, with smaller priority index overlaps with a PUCCH transmission only with HARQ-ACK information, without repetitions, with larger priority index, or - a PUCCH transmission without repetitions that includes HARQ-ACK information of smaller priority index overlaps with a PUCCH transmission without repetitions using a PUCCH resource with PUCCH format 2/3/4 with that includes HARQ-ACK information and SR of larger priority index, or - a PUCCH transmission with HARQ-ACK information, without repetitions, with smaller or larger priority index overlaps, respectively, with a PUSCH transmission with larger or smaller priority index the UE - multiplexes HARQ-ACK information of different priority indexes and SR information of larger priority index, if any, in a same PUCCH transmission of larger priority index, or multiplexes HARQ-ACK information the UE would provide in a PUCCH transmission of smaller or larger priority index in a PUSCH transmission of larger or smaller priority index, respectively, and applies the procedures in clause 9.2.5.3 or 9.3, respectively, and - drops CSI and/or SR carried in the PUCCH of smaller priority index, if any - drops HARQ-ACK information of smaller priority index if the UE would multiplex the HARQ-ACK information of smaller priority index in a PUSCH where the UE multiplexes Part 1 CSI reports and Part 2 CSI reports of larger priority index - drops Part 2 CSI reports of smaller priority index if the UE would multiplex the HARQ-ACK information of smaller and larger priority indexes in a PUSCH where the UE multiplexes Part 1 CSI reports and Part 2 CSI reports of smaller priority index <Unchanged parts omitted> -------------------------- End of Text Proposal for TS 38.213 --------------------------

Text proposal 2:

-------------------------- Start of Text Proposal for TS 38.212 -------------------------- <Unchanged parts omitted> 6.3.2.1.5 UCI with different priority indexes If UCI-MuxWithDifferentPriority is configured, and HARQ-ACK bits associated with priority index 0, and CSI part 1 if any are transmitted on a PUSCH associated with priority index 1, the following UCI bit sequences are generated, a0(1), a1(1), a2(1), a3(1), …,aA(1)-11, and a0(2), a1(2), a2(2), a3(2), …,aA(2)-12 if any, according to the following: - If CSI part 1 is also transmitted on the PUSCH, - Set ai(1) for i=0,1,…,A(1)-1 as the bit sequence of CSI part 1, where the CSI fields of all CSI reports, in the order from upper part to lower part in Table 6.3.2.1.2-6, are mapped to the UCI bit sequence a0(1), a1(1), a2(1), a3(1), …,aA(1)-11 starting with a0(1). - Set ai(2)=oiACK-LP for i=0,1,…,OACK-LP-1 and A(2)=OACK-LP, where the HARQ-ACK bit sequence o0ACK-LP, o1ACK-LP, …, oOACK-LP-1ACK-LP associated with priority index 0 is given by Clause 9.1 of [5, TS 38.213]. - Otherwise, set ai(1)=oiACK-LP for i=0,1,…,OACK-LP-1 and A(1)=OACK-LP, where the HARQ-ACK bit sequence o0ACK-LP, o1ACK-LP, …, oOACK-LP-1ACK-LP associated with priority index 0 is given by Clause 9.1 of [5, TS 38.213]. If UCI-MuxWithDifferentPriority and cg-UCI-Multiplexing are configured, and HARQ-ACK bits associated with priority index 0, and CG-UCI are transmitted on a PUSCH associated with priority index 1, the following UCI bit sequences are generated, a0(1), a1(1), a2(1), a3(1), …,aA(1)-11, and a0(2), a1(2), a2(2), a3(2), …,aA(2)-12 if any, according to the following: - Set ai(1)=oiCG-UCI for i=0,1,…,OCG-UCI-1 and A(1)=OCG-UCI, where the CG-UCI bit sequence o0CG-UCI, o1CG-UCI, …, oOCG-UCI-1CG-UCI is given by Table 6.3.2.1.3-1, mapped in the order from upper part to lower part.. - Set ai(2)=oiACK-LP for i=0,1,…,OACK-LP-1 and A(2)=OACK-LP, where the HARQ-ACK bit sequence o0ACK-LP, o1ACK-LP, …, oOACK-LP-1ACK-LP associated with priority index 0 is given by Clause 9.1 of [5, TS 38.213]. <Unchanged parts omitted> ###### 6.3.2.4.1.6 UCI with different priority indexes If UCI-MuxWithDifferentPriority is configured, and HARQ-ACK bits associated with priority index 0, and CSI part 1 if any are transmitted on a PUSCH associated with priority index 1: - If CSI part 1 is also transmitted on the PUSCH, - Perform rate matching for CSI part 1 according to clause 6.3.2.4.1.2, by assuming the number of HARQ-ACK information bits to be transmitted on PUSCH in clause 6.3.2.4.1.2 is 0 bit. - Perform rate matching for HARQ-ACK with priority index 0 according to clause 6.3.2.4.1.3, by taking HARQ-ACK with priority index 0 as CSI part 2 and replacing βoffsetPUSCH by βoffsetHARQ-ACK-LP, and assuming the number of HARQ-ACK information bits to be transmitted on PUSCH in clause 6.3.2.4.1.3 is 0 bit. - Otherwise, perform rate matching for HARQ-ACK with priority index 0 according to clause 6.3.2.4.1.2, by taking HARQ-ACK with priority index 0 as CSI-part 1 and replacing βoffsetPUSCH by βoffsetHARQ-ACK-LP, and assuming the number of HARQ-ACK information bits to be transmitted on PUSCH in clause 6.3.2.4.1.2 is 0 bit. If UCI-MuxWithDifferentPriority and cg-UCI-Multiplexing are configured, and HARQ-ACK bits associated with priority index 0, and CG-UCI are transmitted on a PUSCH associated with priority index 1: - Perform rate matching for CG-UCI according to clause 6.3.2.4.1.1, by taking CG-UCI as HARQ-ACK and replacing βoffsetPUSCH by βoffsetCG-UCI. - Perform rate matching for HARQ-ACK with priority index 0 according to clause 6.3.2.4.1.2, by taking HARQ-ACK with priority index 0 as CSI part 1 and replacing βoffsetPUSCH by βoffsetHARQ-ACK-LP. <Unchanged parts omitted> -------------------------- End of Text Proposal for TS 38.212 --------------------------

References

- RP-201310, Revised WID: Enhanced Industrial Internet of Things (IoT) and ultra-reliable and low latency communication (URLLC) support for NR, June 2020, Nokia, Nokia Shanghai Bell.

- 3GPP RAN1 #108-e meeting, chairman’s notes