3GPP TSG RAN WG1 #109-e			R1-2204345

e-Meeting, May 9th – 20th , 2022

Source:	NTT DOCOMO, INC.

Title:	Remaining issues on NR NTN

Agenda Item:	8.4

Document for: 	Discussion and Decision

- Introduction

In this contribution, we will provide our views on the remaining issues of NTN. The remaining issues mainly focus on UL time and frequency synchronization, including NTA at Initial access, UE behaviour w.r.t validity timer expiry, TACommonDriftVariation value range, and ambiguity in interpretation of SFN indicating.

- Issue#1: NTA at initial access

The issue of initial TA over-compensation was discussed during the RAN1#107-e meeting and the following working assumption has been made.

Working asssumption: When TAC (TA) in msg2/msgB is received, UE receives the first adjustment and NTA is updated as: - Option 1: NTA=TA⋅16⋅642μ. Where, TA is the TAC field in msg2/msgB

Based on the discussions in previous meetings, a timing offset could be included in Common TA and no explicit indication is needed. In this case, NTA_old should be zero during RACH procedure. Hence, the working assumption should be confirmed as it is.

Proposal 1: Confirm the working assumption made in 107-e meeting: When TAC (TA) in msg2/msgB is received, UE receives the first adjustment and NTA is updated as:

NTA=TA. 16.642μ  , where TA is the TAC field in msg2/msgB

With this proposal, no TP will be needed according to the following text in TS 38.213 [1].

<Unchanged Text Omitted> For a SCS of 2μ∙15kHz, the timing advance command for a TAG indicates the change of the uplink timing relative to the current uplink timing for the TAG in multiples of 16∙64∙Tc2μ. The start timing of the random access preamble is described in [4, TS 38.211]. A timing advance command [11, TS 38.321] in case of random access response or in an absolute timing advance command MAC CE, TA, for a TAG indicates NTA values by index values of TA = 0, 1, 2, ..., 3846, where an amount of the time alignment for the TAG with SCS of 2μ∙15 kHz is NTA=TA∙16∙642μ. NTA is defined in [4, TS 38.211] and is relative to the SCS of the first uplink transmission from the UE after the reception of the random access response or absolute timing advance command MAC CE. In other cases, a timing advance command [11, TS 38.321], TA, for a TAG indicates adjustment of a current NTA value, NTA_old, to the new NTA value, NTA_new, by index values of TA = 0, 1, 2,..., 63, where for a SCS of 2μ∙15 kHz, NTA_new=NTA_old+TA-31∙16∙642μ.

- Issue#2 Combination of open and closed loop TA control

As shown in Figure 1, when updating the open-loop TA related parameters (common TA, and/or satellite ephemeris and/or GNSS fix), there will be a sudden change for open-loop TA values. However, the closed-loop TA value NTA the UE have was indicated from NW side based on the old open-loop TA values, which will cause a large TA error.

Figure 1 Impacts of common TA updates on overall TA in RRC_CONNECTED state

During RAN1#106-bis and 107-e meetings, RAN1 discussed the possible solutions to avoid the TA jump caused by double correction of open and closed loop TA component. Meanwhile, an LS was sent to RAN4 on combination of open and closed loop TA control in NTN. RAN4 has discussed this issue and concluded that RAN4 defines a requirement to ensure the impact on NTN UE UL timing accuracy due to “double-correction” issue is properly addressed.

According to RAN4 LS reply (R1-2203020) [2], in RAN4#102-e, following agreement has been made:

Gradual timing adjustment requirement Agreement: - When the transmission timing error between the UE and the reference timing exceeds ±Te_NTN then the UE is required to adjust its timing to within ±Te_NTN. - The reference timing shall be (NTA+NTA,UE-specific+NTA,common+NTA,offset)×Tc before the downlink timing of the reference cell. - All adjustments made to the UE uplink timing shall follow these rules: - The maximum amount of the magnitude of the timing change, apart from a change of NTA,UE-specific due to satellite position update and NTA,common between the previous transmission and the current transmission, in one adjustment shall be Tq. - The minimum aggregate adjustment rate, apart from a change of NTA,UE-specific due to satellite position update and NTA,common during the last one second, shall be Tp per second. - The maximum aggregate adjustment rate, apart from a change of NTA,UE-specific due to satellite position update and NTA,common during the last 200ms, shall be Tq per 200 ms. - where the maximum autonomous time adjustment step Tq and the aggregate adjustment rate Tp are specified in Table. - The maximum autonomous time adjustment step Tq and the aggregate adjustment rate Tp are specified in Table below. Frequency Range; SCS of uplink signals (kHz); Tq; Tp. 1; 15; [5.5]*64*Tc; [5.5]*64*Tc. 30; [5.5]*64*Tc; [5.5]*64*Tc. 60; NA; NA. NOTE: Tc is the basic timing unit defined in TS 38.211. - (Note) Te_NTN is a timing error limit of NTN UE initial transmission timing error.

With these agreements, UE is required to adjust its timing error between UE and reference timing to within ±Te_NTN, by which the “double-correction” issue can be solved and there is no need for RAN1 to find other solutions on this issue.

Proposal 2: The issue on combination of open and closed loop TA control is up to the UE implementation to meet the RAN4 gradual timing adjustment requirement. Further discussion is not needed in RAN1.

- Issue#3. UE behaviour w.r.t validity timer expiry

In RAN1#106bis-e meeting, following agreement has been made [3].

Agreement: The UE assumes that it has lost uplink synchronization if new or additional assistance information (i.e. serving satellite ephemeris data or Common TA parameters) is not available within the associated validity duration. - FFS: details on how to acquire new or additional assistance information

The UE can always re-acquire new assistance information (re-read the NTN-specific SI) before expiry of UL validity timer. If not, there will be two cases:

- Case1: New assistance information is not available before expiry of the UL validity timer.

- Case2: New assistance information is available but not within the associated validity duration.

Regarding these corner cases, the issue of UE behavior w.r.t validity timer expiry was further discussed on RAN1#108-e meeting [4].

From our point of view, the aforementioned cases can be resolved by implementation, i.e., UE should be capable of realizing when to read SIB before validity timer expiry to avoid these cases from happening. Meanwhile, RAN2 has already made agreement on validity timer expiry issue [5], and specified it in latest version of TS38.331 [6], as follows:

Agreement (RAN2) Upon validity timer expiry, UE shall suspend uplink transmission and re-acquire SI (FFS whether or not UE needs to flush HARQ buffer) TS38.331 5.2.2.4.xx Actions upon reception of SystemInformationBlockTypeXX Upon receiving SystemInformationBlockTypeXX (SystemInformationBlockTypeXX), the UE shall: 1> instruct the lower layers to start or restart ntnUlSyncValidityDuration from the subframe indicated by epochTime; NOTE: UE should attempt to re-acquire SystemInformationBlockTypeXX before the end of the duration indicated by ntnUlSyncValidityDuration and epochTime by UE implementation.

According to RAN2 agreement and text from TS38.331, the expiry of validity duration issue is stated clear enough and no need to be further discussed in RAN1.

Proposal 3: Regarding the issue of validity timer expiry, it is clear enough in current RAN2 spec., and there is no need to further discuss it in RAN1.

- Issue#4: Maintenance on TACommonDriftVariation value range

The following agreement on on granularity and bits allocation for Common TA parameters was made in RAN1#107-e meeting [7].

Agreement: Confirm the Working assumption on granularity and bits allocation for Common TA parameters: Value range, granularity and bits allocation of Higher-layer parameters TACommon, TACommonDrift, TACommonDriftVariation are as follows:

In RAN1#108-e meeting, according to company’s simulation results, it has been found that NTACommonDriftVariation can be negative value for GEO. If the NTACommonDriftVariation < 0 and the range does not include negative numbers, then it cannot be signaled [3]. Following updated proposal has been brought up:

Updated Proposal 14: Value range and bits allocation of TACommonDriftVariation are modified as follows: - Value range : - 0.60 µs/s2 … + 0.60 µs/s2 - Bit allocation: 16 bits

We think it’s okay to modify the incorrect value range of TACommonDriftVariation with 1 bit added, and the Updated Proposal 14 can be supported. However, it is unclear for us whether the common TA drift variation is really necessary for GEO. If not deemed necessary, keeping the current value range would be fine as well. Meanwhile, if it is modified, the modification of TACommonDriftVariation range should be captured in TS38.331, and RAN1 could send LS to RAN2 to inform the modification.

Proposal 4: Either to modify the value range and bits allocation of TACommonDriftVariation as value range of - 0.60 µs/s2  … + 0.60 µs/s2, and bit allocation of 16 bits, or keep the current value range could be supported. If the value range is modified, send LS to RAN2 to inform this modification.

- Issue#5: Ambiguity in interpretation SFN indicating Epoch time

The issue of ambiguity in SFN interpretation has been discussed in RAN1#108-e meeting with a motivation of avoiding system failure of UE pre-diction. According to the discussions, following two versions of updated proposals have been brought up:

Updated Proposal 15 (rev 3): - If indicated explicitly by a SFN and subframe number, the UE considers this frame to be the frame which is nearest to the frame where the message is received Updated Proposal 15 (rev 4): - Indicated SFN for Epoch time is current SFN or the next upcoming SFN after the frame where the message indicating the Epoch time is received.

It can be found that for rev-3, the validity duration got reduced when indicated epoch time is in the past, while for rev-4, it has less flexibility of epoch time configuration. For comparison, updated proposal 15 (rev-4) is slightly preferred. Meanwhile, if this proposal can be agreed, the information element ‘epoch time’ of NTN-Config needs to be revised. RAN1 could send LS to RAN2 to inform the modification on Epoch time.

Proposal 5: Indicated SFN for Epoch time is current SFN or the next upcoming SFN after the frame where the message indicating the Epoch time is received. Send LS to RAN2 to inform this modification.

- Conclusion

In this contribution, we discussed the remaining issues on UL time and frequency synchronization in NR NTN, including NTA at initial access, UE behaviour w.r.t validity timer expiry, TACommonDriftVariation value range, and ambiguity in interpretation of SFN indicating. Based on the discussion, we made following observation and proposals.

Proposal 1: Confirm the working assumption made in 107-e meeting: When TAC (TA) in msg2/msgB is received, UE receives the first adjustment and NTA is updated as:

NTA=TA. 16.642μ  , where TA is the TAC field in msg2/msgB

Proposal 2: The issue on combination of open and closed loop TA control is up to the UE implementation to meet the RAN4 gradual timing adjustment requirement. Further discussion is not needed in RAN1.

Proposal 3: Regarding the issue of validity timer expiry, it is clear enough in current spec., and there is no need to further discuss it in RAN1.

Proposal 4: Either to modify the value range and bits allocation of TACommonDriftVariation as value range of - 0.60 µs/s2  … + 0.60 µs/s2, and bit allocation of 16 bits, or keep the current value range could be supported. If the value range is modified, send LS to RAN2 to inform this modification.

Proposal 5: Indicated SFN for Epoch time is current SFN or the next upcoming SFN after the frame where the message indicating the Epoch time is received. Send LS to RAN2 to inform this modification.

References

- 3GPP, TS38.213

- R1-2203020, “Reply LS on combination of open and closed loop TA control in NTN”, RAN WG4, Mar. 2022

- 3GPP, RAN1#106bis-e, RAN1 Chairman’s Notes, Oct. 2021.

- R1-2202908, “FL Summary #4: Maintenance on UL time and frequency synchronization for NR”, Thales, March 2022.

- 3GPP, RAN2#116bis-e, RAN2 Chairman’s Notes, Oct. 2021.

- 3GPP, TS38.331

- R1-2111127, “FL Summary #4 on enhancements on UL time and frequency synchronization for NR NTN”, Thales, Nov. 2021.