3GPP TSG RAN WG1#109-e	 	 	R1-2204346

e-Meeting, May 9th – 20th, 2022

Source:	NTT DOCOMO, INC.

Title:	Remaining issue on accuracy improvements

Agenda Item:	8.5.1

Document for: 	Discussion and Decision

- Introduction

In this contribution, we present our view on remaining issue regarding accuracy improvements for Rel-17 NR positioning.

- Periodic UE Tx TEG reporting for Multi-RTT

At the last meeting, periodic UE Tx TEG reporting for Multi-RTT was discussed in agenda item of mitigating UE Rx/Tx and/or gNB Rx/Tx timing delays, and the following proposal was made [1].

(Final) Proposal 2.4 (H)

Modify the previous agreement as follows:

- If a Tx TEG ID is reported with any UE Rx-Tx time difference measurement, the UE should also report the association of the Tx TEG ID to the UL SRS resource(s) that have already been transmitted and are associated with the Tx TEG ID.

  - The maximum number of associations per report can be 64.

  - All the UL SRS resource(s) that have already been transmitted and are associated with the Tx TEG ID may be reported for the association as long as the number of associations (ie. The number of NR-SRS-TxTEG-Element-r17) per report does not exceed 64.

  - It is up to RAN2 to decide how to indicate the change of the Tx TEG association

  - It is up to RAN2 to optimize the signalling when there is no change of the Tx TEG association

- Send LS to RAN2 the signaling details to continue the signalling design

  - FFS: how the the association of the Tx TEG ID to the UL SRS resource(s) is determined by UE.

  - FFS: details of the signalling

Regarding reporting of UE Tx TEG association, periodic association reporting for UL-TDOA was agreed in the previous meeting. However, periodic association reporting for Multi-RTT was not clarified. In our understanding, periodic association reporting for Multi-RTT should be also supported in Rel-17, and the corresponding clarification is necessary to help RAN2 work. Therefore, the above proposal should be discussed in maintenance phase. Given that the maximum number of association reporting for Multi-RTT was agreed with 64, we think the first and second sub bullets are needed.

At the previous meeting, whether a UE needs to report the association information of all the UL SRS resources even if the association is not changed was discussed. However, RAN1 did not reach a conclusion. We think the association reporting of all UL SRS resources at every time should be avoid, however, moderately periodic association reporting can inform the association change to NW immediately, and we believe the function would be beneficial. Since indication and signaling design regarding the association reporting have been discussed in RAN2, RAN1 should make an LS and share the considerations from RAN1 perspective. Therefore, the third and fourth sub bullets and the second main bullet would be reasonable.

Proposal 1:

- Regarding periodic UE Tx TEG reporting for Multi-RTT, the previous agreement should be modified as follows:

- If a Tx TEG ID is reported with any UE Rx-Tx time difference measurement, the UE should also report the association of the Tx TEG ID to the UL SRS resource(s) that have already been transmitted and are associated with the Tx TEG ID. - The maximum number of associations per report can be 64. - All the UL SRS resource(s) that have already been transmitted and are associated with the Tx TEG ID may be reported for the association as long as the number of associations (ie. The number of NR-SRS-TxTEG-Element-r17) per report does not exceed 64. - It is up to RAN2 to decide how to indicate the change of the Tx TEG association - It is up to RAN2 to optimize the signalling when there is no change of the Tx TEG association - Send LS to RAN2 the signaling details to continue the signalling design - FFS: how the the association of the Tx TEG ID to the UL SRS resource(s) is determined by UE. - FFS: details of the signalling

- Conclusion

In this contribution, we discussed on remaining issue regarding accuracy improvements for Rel-17 NR positioning. Based on the discussion, we made following proposal.

Proposal 1:

- Regarding periodic UE Tx TEG reporting for Multi-RTT, the previous agreement should be modified as follows:

- If a Tx TEG ID is reported with any UE Rx-Tx time difference measurement, the UE should also report the association of the Tx TEG ID to the UL SRS resource(s) that have already been transmitted and are associated with the Tx TEG ID. - The maximum number of associations per report can be 64. - All the UL SRS resource(s) that have already been transmitted and are associated with the Tx TEG ID may be reported for the association as long as the number of associations (ie. The number of NR-SRS-TxTEG-Element-r17) per report does not exceed 64. - It is up to RAN2 to decide how to indicate the change of the Tx TEG association - It is up to RAN2 to optimize the signalling when there is no change of the Tx TEG association - Send LS to RAN2 the signaling details to continue the signalling design - FFS: how the the association of the Tx TEG ID to the UL SRS resource(s) is determined by UE. - FFS: details of the signalling

Reference

- R1-2202500, 3GPP RAN1#108-e, “FL Summary #4 for accuracy improvements by mitigating UE Rx/Tx and/or gNB Rx/Tx timing delays”, Moderator (CATT)